3GPP TSG-RAN WG1 Meeting #118	R1-2407486

# Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0059 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on reference section numbers for PSFCH prioritization in multi-channel access procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a UE would perform multiple PSFCHs on more than one RB set, both multi-channel access and PSFCH prioritization are performed. The processing order has been described in 4.5.6 of 37.213 based on the following agreement that was reached at RAN1#115 meeting. Agreement When UE intends to transmit PSFCH, after performing PSFCH prioritization: - if LBT fails on all RB set(s) where the UE attempts to transmit PSFCH: - UE drops PSFCH transmission - No RAN1 specification impact - if TypeA and TypeB LBT fails on part of RB set(s) where the UE attempts to transmit PSFCH: - The UE may transmit PSFCH on RB set(s) where LBT was successful. Regarding the spec descriptions in 4.5.6 of 37.213, PSFCH prioritization behavior is referred as 16.2.4.2 of 38.213, where multiple PSFCH transmissions are handled. However, ‘PSFCH prioritization’ in the agreement should mean any prioritization relevant to PSFCH transmission; otherwise, UE behavior is unclear in some cases. In the current specifications, UL/SL prioritization handling is defined and the prioritization includes PSFCH transmission case. Co-existense between SL-U and NR are not precluded; thus, this handling must be referred for the processing order with multi-channel access. In addition, there is also PSFCH prioritization due to the limited total transmission power and/or the limited capability of simultaneous PSFCH transmissions in section 16.2.3 of TS 38.213 (i.e., PSFCH power control). Hence, Section 16.2.3 should be captured as well. |
| Summary of change: |  | Multi-channel access for PSFCH transmissions is performed after prioritization relevant to PSFCH transmission, i.e., all of 16.2.3, 16.2.4.2 and 16.4.2.3 of TS 38.213. Section numbers 16.2.3 and 16.2.4.3 of reference [7] are added for the reference of PSFCH prioritization behavior. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is not defined when the case UE would perform multiple PSFCH transmissions on multiple RB sets and they are overlapped with UL transmission, and the case when there is a limited total transmission power and/or the limited capability of simultaneous PSFCH transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

## 4.5.6	Channel access procedures for transmission(s) on multiple channels

If a UE

-	is scheduled to transmit on a set of channels C, and if the SL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or

-	intends to perform sidelink transmissions on configured resources on the set of channels C, and if the SL transmissions are configured to start transmissions at the same time on all channels in the set of channels C, or

-	intends to perform sidelink transmissions on selected resources on the set of channel C, and if SL transmissions are to start at the same time on all channels in the set of channels C

the followings are applicable:

-	Type A or Type B procedures described in clause 4.5.6.1 and 4.5.6.2 can be used for accessing multiple channels only for PSFCH or S-SSB transmissions.

-	A UE can access multiple channels on which SL transmissions are performed, according to the procedures described in clause 4.5.6.3.

When a UE performs Type A or Type B channel access procedures to transmit PSFCH transmissions on multiple channels after performing associated prioritization for the PSFCH as described in clause 16.2.3, 16.2.4.2 and 16.2.4.3 of [7], if the channel access procedures fail on part of the channel(s) but succeed on other part of the channel(s), the UE may transmit the PSFCH transmission(s) on the part of the channel(s) where the corresponding channel access was successful.

< End of change request >