3GPP TSG-RAN WG1 Meeting #118	R1-2407541

# Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0060 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on applicable CAPC for PSFCH and S-SSB transmissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung, NTT DOCOMO, vivo, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the current specification description, the CAPC value is defined for only PSFCH or only S-SSB transmission to initiate a channel occupancy using Type 1 channel access procedures. For the following two cases, it is still unclear the CAPC value that should be used for PSFCH and S-SSB transmissions within an initiated channel occupancy. - When UE intends to transmit PSFCH and followed by S-SSB - When UE intends to transmit PSFCH or S-SSB with PSCCH/PSSCH in an initiated channel occupancy |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the applicable channel access priority class for the PSFCH and S-SSB is p=1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec remains ambiguous on the CAPC value that should be applied in the case when a UE transmits both PSFCH and S-SSB, or PSFCH/S-SSB with PSCCH/PSSCH, in a same channel occupancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

4.5	Sidelink channel access procedures

A UE operating in sidelink resource allocation mode 1 or mode 2 and performing SL transmission(s) on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed.

In this clause, transmissions from a UE are considered as separate SL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in clause 4.5.5 when applicable.

A UE can access a channel on which SL transmission(s) are performed according to one of Type 1 or Type 2 SL channel access procedures as described in clauses 4.5.1 and 4.5.2, respectively.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s), the applicable channel access priority class (CAPC) is defined in Table 4.5-1.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSSCH with user plane data and associated PSCCH, the UE determines the corresponding SL channel access priority class p  in Table 4.5-1 following the procedures described in Clause 16.9.9.2 in [9].

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSFCH or S-SSB transmission(s), the applicable channel access priority class for the PSFCH and S-SSB is p=1 in Table 4.5-1.

A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tslmcot,p where the channel access procedure is performed based on the channel access priority class p  associated with the UE transmissions, as given in Table 4.5-1.

< End of change request >