3GPP TSG-RAN WG1 Meeting #118	R1-2407542

# Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0061 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL-U reference duration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), CATT, CICTCI, Samsung, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The contention window adjustment procedures for SL-U are divided into two cases. One is used for SL transmission with explicit HARQ-ACK feedback including 'ACK/NACK' and the other is used for SL transmission which is not associated with explicit HARQ-ACK feedback(s). For SL transmission with explicit HARQ-ACK feedback, the CW is determined based on the HARQ feedback situation corresponding to the PSSCH transmission in the reference duration. However, when there is no reference duration for the latest channel occupancy initiated by the UE, which can be used to adjust the contention window of the current SL transmission, the CW adjustment procedure is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 4.5.4, it is clarified that when reference duration is not available for the latest channel occupancy initiated by the UE, in order to perform contention window adjustment procedures for a SL transmission(s) including at least one PSSCH enabled with explicit HARQ-ACK feedback including 'ACK/NACK', step 6 is applied, i.e., for every priority class p∈1,2,3,4, maintain CWp as it is. - Editorial correction: “a HARQ-ACK feedback” -> “HARQ-ACK feedback”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is unclear how to adjust the contention window when no reference duration can be determined for the latest channel occupancy initiated by a UE, in the case that UE intends to transmit a SL transmission with explicit HARQ-ACK feedback including 'ACK/NACK'. - Editorial errors remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

4.5.4	Contention window adjustment procedures for SL transmissions

If a UE transmits a SL transmission(s) including at least one PSSCH enabled with explicit HARQ-ACK feedback including 'ACK/NACK' using Type 1 channel access procedures associated with the channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.5.1 for the SL transmission(s) applying the following procedures:

1)	For every priority class p∈1,2,3,4, set CWp=CWmin,p.

2)	If HARQ-ACK feedback corresponding to the PSSCH(s) for unicast SL transmission(s) in the reference duration for the latest channel occupancy initiated by the UE, is available:

-	If the HARQ-ACK feedback includes only 'ACK', go to step 1; otherwise go to step 5.

3)	If HARQ-ACK feedback corresponding to the PSSCH(s) for groupcast SL transmission(s) in the reference duration for the latest channel occupancy initiated by the UE, is available:

-	If harq-ACK-FeedbackRatioforCW-AdjustmentGC-Option2-r18 is provided by higher layers:

-	The UE calculates the ratio between the number of received 'ACK' in the HARQ-ACK feedback and the number of UE(s) from which the corresponding 'ACK'/'NACK' in the HARQ-ACK feedback is expected. If the calculated ratio is equal to or larger than harq-ACK-FeedbackRatioforCW-AdjustmentGC-Option2-r18, go to step 1; otherwise go to step 5.

-	Otherwise:

-	If the HARQ-ACK feedback includes at least an 'ACK', go to step 1; otherwise go to step 5.

4)	If HARQ-ACK feedback corresponding to the PSSCH(s) in the reference duration for the latest channel occupancy initiated by the UE is not available or reference duration for the latest channel occupancy initiated by the UE is not available, go to step 6.

5)	Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value.

< End of change request >