3GPP TSG-RAN WG1 Meeting #118	R1-2407540

# Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0205 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL-U COT sharing flag indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), vivo, CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The fields of CAPC, etc., are presented only if the 'COT sharing flag' field in SCI format 1-A is present and set to '1'. The current spec seems to imply that these fields are still present when 'COT sharing flag' field in SCI format 1-A is set to '0', which is not aligned with the RAN1 agreement. - Since the 'COT sharing flag' is not explicitly defined in TS 37.213, it will be confusing to say “1 bit as defined in [14, TS 37.213]” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the fields of CAPC, COT sharing cast type, COT sharing additional ID, Remaining COT duration are present only when the 'COT sharing flag' field in SCI format 1-A is present and set to '1'. - Remove “as defined in [14, TS 37.213]”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The presence of the fields of CAPC, COT sharing cast type, COT sharing additional ID, and Remaining COT duration are still present when 'COT sharing flag' field in SCI format 1-A is set to '0', which is not aligned with the RAN1 agreement. The reference to TS 37.213 for COT sharing flag leads to confusion as there is nowhere to find an explicit definition of 'COT sharing flag' in TS 37.213 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1, 8.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

#### 8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

<Unchanged parts are omitted>

-	COT sharing flag – 0 or 1 bit

-	1 bit if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured;

-	0 bit otherwise.

<Unchanged parts are omitted>

#### 8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

<Unchanged parts are omitted>

If the 'COT sharing flag' field in SCI format 1-A is present and set to '1', all the remaining fields are present and set as follows:

< End of change request >