3GPP TSG-RAN WG1 Meeting #118	R1-2407487

# Maastricht, Netherlands, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0614 | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on interlace RB-based transmission in partial sensing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#117, it was already agreed to support the combination of interlace RB-based transmission and partial sensing in SL-U. However, only the correction for periodic transmission in partial sensing was implemented. The corresponding correction for aperiodic transmission in partial sensing is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of candidate single-slot resource and candidate multi-slot resource for interlace RB-based transmission when UE performs aperiodic transmission in partial sensing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to determine a candidate single-slot resource or a candidate multi-slot resource for interlace RB-based transmission when UE performs aperiodic transmission in partial sensing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change request >

## 8.1.4	Channel access procedures for transmission(s) on multiple channels

< Unchanged parts are omitted >

The UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool within the time interval n+T1,n+T2 correspond to one candidate multi-slot resource for UE performing full sensing. The UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate multi-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets included in the corresponding resource pool in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource or the UE shall assume that any set of LsubCH contiguous sub-channels or LsubCH contiguous sub-channels in LRBset contiguous RB sets in Nslot,MCSt consecutive slots included in the corresponding resource pool in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate multi-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where

< End of change request >