3GPP TSG-RAN5 Meeting #104	R5-244900

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5520 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of reference sensitivity CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reference sensitivity test requirements for the following CA configurations are incorrect: - CA_2A-2A-29A-66A - CA_2A-2A-29A-30A-66A - CA_2A-2A-29A-66A-66A |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test requirements have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.9.5, 7.3A.10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.3A.9.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

< Unchanged tables omitted >

For combinations of intra-band non-contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.9.5-6. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.9.5-6: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration |  | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |  |
| CA_2A-2A-4A-5A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
| CA_2A-2A-5A-12A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  |  | 12 |  |  | -96.3 | -93.3 |  |  |  |
| CA_2A-2A-4A-71A6,7 |  | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  |  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  |  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-2A-4A-71A9 |  | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-4A-71A10 |  | 2 |  |  | -95.8 | -93 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-5A-30A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |  |
| CA_2A-2A-5A-66A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_2A-2A-12A-30A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 12 |  |  | -96.3 | -93.3 |  |  |  |  |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |  |
| CA_2A-2A-12A-66A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 12 |  |  | -96.3 | -93.3 |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_2A-2A-29A-66A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.64 | -89.44 |  |  |  |
|  |  | 29 |  |  | -96.3 | -93.3 |  |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |  |
| CA_2A-2A-30A-66A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_2A-2A-66A-71A6.7 |  | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |  |
|  |  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-2A-66A-71A9 |  | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-2A-66A-71A10 |  | 2 |  |  | -95.8 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-4A-4A-5A |  | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  |  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -97.24 | -96.04 |  |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
| CA_2A-4A-7A-7A |  | 2 |  |  | -97.3 | -94.3 | -92.5 | -91.3 | FDD |  |
|  |  | 4 |  |  | -99.3 | -96.3 | -94.2 | -93.3 |  |  |
|  |  |  |  |  | -102.04 | -99.04 | -96.94 | -96.04 |  |  |
|  |  | 7 |  |  | -97.3 | -94.3 | -92.5 | -91.3 |  |  |
| CA_2A-4A-7A-7A (Note 8) |  | 2 | - | - | - | - | - | -86.3 | FDD |  |
|  |  | 2 | - | - | - | - | - | -83.620 |  |  |
|  |  | 4 | - | - | -99.3 | - | - | - |  |  |
|  |  |  |  |  | -102.04 |  |  |  |  |  |
|  |  | 7 | - | - | - | - | - | -91.3 |  |  |
|  |  | 2 | - | - | -97.3 | - | - | - | FDD |  |
|  |  | 4 | - | - | -94.3 | - | - | - |  |  |
|  |  |  |  |  | -97.04 |  |  |  |  |  |
|  |  | 7 | - | - | - | - | - | -91.3 |  |  |
| CA_2A-5A-66A-66A |  | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |  |
|  |  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |  |
|  |  | 5 |  |  | -97.3 | -94.3 |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_2A-12A-66A-66A6,7 |  | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |  |
|  |  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  |  | 66 |  |  | -89.5 | -89.0 | -88.5 | -88.0 |  |  |
|  |  |  |  |  | -92.24 | -91.74 | -91.24 | -90.74 |  |  |
| CA_2A-29A-66A-66A |  | 2 |  |  | -97,3 | -94,3 | -92.5 | -91.3 | FDD |  |
|  |  | 29 |  |  | -96.3 | -93.3 |  |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_2A-66A-66A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91.3 | FDD |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |  |
| CA_2A-66A-66A-71A6.7 |  | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |  |
|  |  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -69.7 | -69.7 | -69.7 | -69.7 |  |  |
| CA_2A-66A-66A-71A9 |  | 2 |  |  | -92.4 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -95.14 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_2A-66A-66A-71A10 |  | 2 |  |  | -95.8 | -93.0 | -91.5 | -90.4 | FDD |  |
|  |  |  |  |  | -98.54 | -95.74 | -94.24 | -93.14 |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 |  |  |
| CA_4A-4A-12A-30A6,7 |  | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  |  |  |  |  | -92.74 | -92.24 | -91.74 | -91.24 |  |
|  |  | 12 |  |  | -96.5 | -93.5 |  |  |  |
|  |  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  |  |  |  |  | -101.24 | -98.24 |  |  |  |
| CA_4A-4A-29A-30A |  | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  |  |  |  |  | -102.34 | -99.34 | -97.54 | -96.34 |  |
|  |  | 29 |  |  | -97.0 | -94.0 |  |  |  |
|  |  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  |  |  |  |  | -101.24 | -98.24 |  |  |  |
| CA_5A-30A-66A-66A |  | 5 |  |  | -97.3 | -94.3 |  |  | FDD |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_12A-30A-66A-66A |  | 12 |  |  | -96.3 | -93.3 |  |  | FDD |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_14A-30A-66A-66A |  | 14 |  |  | -96.3 | -93.3 |  |  | FDD |  |
|  |  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |
|  |  |  |  |  | -101.04 | -98.04 |  |  |  |  |
|  |  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  | - | - | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
| CA_29A-30A-66A-66A | 29 |  |  | -96.3 | -93.3 |  |  | FDD |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |  |
| CA_29A-66A-66A-70A |  | 29 |  |  | -96.3 | -93.3 |  |  | FDD |  |
|  |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |
|  |  | 70 |  |  | -99.3 | -96.3 | -94.5 |  |  |  |
| CA_66A-66A-70A-71A |  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 | FDD |  |  |
|  |  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |  |  |
|  |  | 70 |  |  | -99.3 | -96.3 | -94.5 | - |  |  |  |
|  |  |  |  |  | -89.36,7 | -88.86,7 | -88.56,7 | - |  |  |  |
|  |  | 71 |  |  | -96.5 | -93.5 | -91.2 | -86.8 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 7: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 8: These requirements apply when UL CA is configured and the uplink is active in Band 2 and Band 4. NOTE 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.3A.10.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.10.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

< Unchanged tables omitted >

Table 7.3A.10.5-5: Reference sensitivity QPSK PREFSENS for CA_XA-XA-XD or XA-XC-XC or XA-YC-YC or XA-YA-YA-YC or XA-XA-YD or XA-XA-YA-YC or XA-XA-YA-YA-YA or XA-XA-XA-YC or XA-YA-ZA-ZA-ZA or XA-YA-YA-ZC or XA-YA-YA-ZA-ZA or XA-YA-ZA-RA-RA or CA_XA-XA-YA-ZA-RA or XA-XA-YA-YA-ZA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-2A-12A-66A-66A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  | -94.74 | -92.44 | -90.94 | -89.44 |  |
|  | 12 |  |  | -96.3 | -96.5 |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 | -88 |  |
|  |  |  |  | -92.24 | -91.74 | -91.24 | -90.74 |  |
| CA_2A-2A-5A-66A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-5A-30A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-12A-30A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-5A-66A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-5A-30A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-12A-30A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 12 |  |  | -96.3 | -93.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-29A-30A-66A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  | -94.74 | -92.44 | -90.64 | -89.44 |  |
|  | 29 |  |  | -96.3 | -93.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-2A-29A-66A-66A | 2 |  |  | -92.0 | -89.7 | -87.9 | -86.7 | FDD |
|  |  |  |  | -94.74 | -92.44 | -90.64 | -89.44 |  |
|  | 29 |  |  | -96.3 | -93.3 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_2A-5A-30A-66A-66A | 2 |  |  | -97,3 | -94,3 | -92.5 | -91,3 | FDD |
|  |  |  |  | -100.04 | -97.04 | -95.24 | -94.04 |  |
|  | 5 |  |  | -97.3 | -94.3 |  |  |  |
|  | 30 |  |  | -98.3 | -95.3 |  |  |  |
|  |  |  |  | -101.04 | -98.04 |  |  |  |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
| CA_29A-66A-66A-70C | 29 |  |  | -96.3 | -93.3 |  |  | FDD |
|  | 66 |  |  | -98.8 | -95.8 | -94 | -92.8 |  |
|  |  |  |  | -101.54 | -98.54 | -96.74 | -95.54 |  |
|  | 70 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

< End of changes >