3GPP TSG RAN Meeting #105								       RP-242405

Melbourne, Australia, Sept 9-12, 2024

Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name | New SID: Study on solutions for Ambient IoT (Internet of Things) in NR |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: | Performance part: | Testing part: |
| Acronym | FS_Ambient_IoT_solutions |  |  |  |
| Unique ID | 1020085 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240826 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2024 (No change) | Core part: | Performance part: | Testing part: |
| Overall Completion level | Study Item: 60 % | Core part: | Performance Part: | Testing part: |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Shen |
|  | Company | CMCC |
|  | Email | shenxiaodong@chinamobile.com |
|  | Name | Matthew Webb |
|  | Company | Huawei |
|  | Email | matthew.webb@huawei.com |
|  | Name | John Humbert |
|  | Company | T-Mobile USA |
|  | Email | John.Humbert2@T-Mobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

TR updates

Agreement

R1-2407489 is endorsed for producing v0.1.0 of TR 38.769. TR in R1-2407513.

LS response to SA2

Agreement

RAN1 answer to the first question from SA2 is the following:

Whether or not an Ambient IoT device can incorporate a non-volatile memory in the device design i.e., include a non-volatile memory in the Bill-of-Materials (BoM)

RAN1 is studying several different A-IoT device architectures, which all are assumed to include a memory block described as follows:

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Therefore, it can be assumed that an A-IoT device can incorporate an NVM in the device design, i.e., include an NVM in the BoM.

Agreement

RAN1 answer to the second question from SA2 is the following:

Whether an Ambient IoT device will be able to update its non-volatile memory based on its energy status and energy storage capabilities at some point after receiving a trigger from the Reader

RAN1 would like to provide the following answer:

- An Ambient IoT device will be able to update its non-volatile memory at some point after receiving a trigger from the Reader. Writing may not always be possible at all times.

- The power consumption during writing to the NVM is higher than during reading.

- RAN1 thinks that frequent or recurring writing to NVM should be avoided.

- RAN1 has not discussed energy status and energy storage capabilities in relation to an Ambient IoT device ability to update its non-volatile memory.

Agreement

Final LS in R1-2407364.

2.1.1.1	Evaluation assumptions and results

Agreement

Definition of the latency is refined as follows,

- For the use case of “inventory-only”:

  - The time interval between the time that the A-IoT paging is sent from reader to a A-IoT device and the time that the inventory report is successfully received at reader from the A-IoT device, i.e., for completing Step A and Step B (as per RAN2 agreements).

- For the use case of “inventory and command”:

  - The time interval between the time that the A-IoT paging is sent from reader and the time that

      - the command is successfully received at A-IoT device, i.e., for completing Step A, Step B, Step C (as per RAN2 agreements), if Step D is optional and not used (note: pending RAN2 decision on optionality of Step D).

      - the response is successfully received at Reader, i.e., for completing Step A, Step B, Step C, and Step D (as per RAN2 agreements).

- Note: the “successfully received” (i.e., decoding successfully)

- Expected value of latency is calculated according a X% re-attempt probability to each attempt.

- Note: the latency is evaluated for a single A-IoT device.

- Note: Time for energy harvesting is not included in the definition of latency.

Agreement

The following performance metric is considered for evaluation purpose only,

- Inventory completion time for multiple A-IoT devices

  - For inventory-only use case, the  ‘Inventory completion time for multiple A-IoT devices’ is defined as the time a reader successfully completed the inventory process for at least 99% of all A-IoT devices within the coverage of the reader, assuming device density of 1.5 devices per m2.

Note: RAN1 will not define a target for the inventory completion time

Agreement

Confirm following working assumption with following update.

Working assumption:

- For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth.

- FFS: where Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power.

- On/off keying backscatter loss is not taken into account in the LLS and is included in link budget table [1H].

Agreement

Update the agreement on [0m] Reference data rate as follows:

1 kbps (M), 5 ~ 7 kbps (M), 48 ~ 60 kbps (O), 0.1 kbps for message size of 20 or 96 bits (O)

- Other data rate can be reported by companies

- Note1: companies to report the exact data rate.

- Note 2: the exact data rate is close to the values listed above.

- Note 3: The exact data rate is calculated by dividing the message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present).

- Note 4: the exact data rate may be related to coding scheme, repetition and etc.

- Note 5: all data rates considered are for evaluation purpose only.

Agreement

Confirm the working assumption on [0q] Sampling frequency with the following modifications:

Working assumption on [0q] (sampling frequency) in link level simulation table

Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any)

Initial SFO (Sampling Frequency Offset) (Fe):

- (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2,

- (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1,

- (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2,

- FFS: Optionally evaluate a fixed value SFO for device 1 and 2

- Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution

- Note: Above values are only for sampling purpose.

- FFS other values

- Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R].

The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T.

- Note: Accuracy can be improved after clock calibration for at least device 2.  FFS applicable for device 1

- Note: SFO after clock calibration can be applied to Fe.

- FFS other models

CFO for device 2b.

- [(100ppm(M), 200ppm(O), 1000ppm(O, only as initial CFO), with drift rate of [TBD]ppm/s)]”

- Note: companies to report whether they assumed the value as initial CFO or after calibration

Note: Above assumptions are for LLS evaluation purpose only

Agreement

Companies should report their assumptions for evaluation (if any) of the inventory completion time for multiple devices. Companies can refer to the table in section 2.1 of R1-2407327 for information.

Agreement

Companies to report the calculation of single-device latency according to the follow tables

Table XX: calculation of single-device latency for “inventory-only”

| Steps | Procedures | Time (us) |
| --- | --- | --- |
| Step A: A-IoT paging | A-IoT paging message |  |
| Gap A (if any) |  |  |
| Step B: D2R data transmission. | A-IoT Random access procedure |  |
|  | Gap B1 |  |
|  | D2R data transmission |  |
| Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B |  |  |
| Average time considering all the re-attempts, e.g., T2 = T1 /(1-X) |  |  |
| Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS |  |  |

Table YY: calculation of single-device latency for “inventory and command”

| Steps | Procedures | Time (us) |
| --- | --- | --- |
| Step A: A-IoT paging | A-IoT paging message |  |
| Gap A (if any) |  |  |
| Step B: D2R data transmission. | A-IoT Random access procedure |  |
|  | Gap B1 |  |
|  | D2R data transmission |  |
| Gap B (if any) |  |  |
| Step C: Data transmission | R2D data transmission |  |
| Gap C (if any) |  |  |
| Step D (optional, pending RAN2 decision) | D2R data transmission |  |
| Total time for one attempt, T1= Tstep_A+TGap_A+ Tstep_B+TGap_B+ Tstep_C+ +TGap_C +Tstep_D |  |  |
| Average time considering all the re-attempts, e.g., T2 = T1 / (1-X) |  |  |
| Note: the following is assumed - Data rate: 1kbps or 7kbps - The rate for re-attempt probability X = [10] % - Message size for each procedure: FFS |  |  |

2.1.1.2	Ambient IoT device architectures

Agreement

For device 2a with RF-ED, make following update.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present.

    - At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA.

For device 2b with RF-ED, make following update.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present.

For device 2b with ZIF, make following update.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present.

For device 2b with IF, make following update.

  - FFS: LNA for improving signal strength and sensitivity of receiver, if present.

Agreement

For Device 2b with RF-ED/ZIF/IF, make following update.

- FFS: Power amplifier (PA) amplifies tx signal, if present

Agreement

For the architecture of Device 2b with RF-ED receiver, make following updates.

- Local oscillator (LO) for carrier frequency generation

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

    - FLL(/PLL) may not exist depending on architecture

For the architecture of Device 2b with ZIF receiver, make following update:

- Local oscillator (LO) for generating carrier frequency for Tx and Rx

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

  - FFS: o One LO or separate LOs for Tx and Rx.

For the architecture of Device 2b with ZIF receiver, make following update:

- Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx generating carrier frequency for Tx and Rx

  - FFS: PLL/FLL

  - FLL(/PLL) can be used for the frequency synthesis and calibration

  - FFS: o One LO or separate LOs for Tx and Rx

Agreement

Adopt the following table for clock

|  | Description | Applicable device types | Clock speed | Power consumption | Initial clock accuracy | Accuracy after clock sync / calibration | Clock drift |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock | Sampling | Device 1, 2a, 2b | a few MHz | < [1]uW for device 1 FFS for device 2a/2b | [10^4 ~ 10^5] ppm for device 1 [10^3~ 10^4] ppm for device 2a/2b | FFS (if applicable for device 1) | FFS |
| Purpose #2 of the clock | Small frequency shift | At least for device 1 and device 2a |  |  |  |  |  |
| [Purpose #3 of the clock] | [Time counting (if supported)] | [Device 1, 2a, 2b] | FFS | FFS the same or different for different devices | FFS | FFS (if applicable) | FFS |
| Purpose #4 of the clock | Large frequency shift (if supported for device 2a) | Device 2a | 10s MHz | [10s] uW | FFS | FFS | FFS |
| Purpose #5 of the clock | LO for carrier frequency (for up/down conversion) | Device 2b | e.g., [900] MHz | [10s ~ 100s] uW | FFS | FFS | FFS |
| Note: It does not necessarily imply that different purposes of LOs/clocks correspond to separate discrete LOs/clocks, which is up to implementation. |  |  |  |  |  |  |  |

Agreement

Make the following update for device 2a diagram.

- FFS: Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency).

- Note: this does not mean that RAN1 has concluded on the feasibility of Large Frequency shifter for device 2a

Agreement

Add following additional observations for large frequency shifter for device 2a.

- Large frequency shift may allow the reader to avoid implementing in-band full duplex capability for scenario e.g., D1T1-A2 and D2T2-A2

- Large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz

Conclusion

RAN1 will not further discuss the following:

Adopt following diagram as example tx modulators. Provided diagrams are not intended to constrain actual implementation.

Following blocks could be used as an alternative tx architecture in device architecture diagrams.

- Device 1/2a with OOK/BPSK

  - Tx modulator is based on impedance switching between two states.

  - OOK/PSK can be realized by the choice of two impedance values.

- Device 1/2a with BFSK

  - Tx modulator is based on impedance switching between two states.

  - BFSK can be realized by the choice of IF frequency f1 and f2 based on baseband information bits.

- Device 2b with OOK

  - Baseband information bits control the switch between LO and output.

  - [Matching network may or may not exist.]

- Device 2b with BPSK

  - Baseband information bits select a phase of differential carrier frequency signal.

- Device 2b with BFSK

  - Baseband input signal directly controls the choice of carrier frequency f1 and f2 generated from LO.

Agreement

Companies are encouraged to provide receiver sensitivity numbers used in their evaluations with justification of feasibility to agenda 9.4.1.1, along with their evaluation assumptions.

2.1.1.3	General aspects of physical layer design

Agreement

The following table is a starting point for the study of M values and the associated minimum Btx,R2D value

- Reader can use any R2D transmission bandwidth >= minimum Btx,R2D

- FFS: Impacts, if any, of CP handling solutions, and other impacts

- Note: depending on further study, the maximum value of M may be less than 32

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 2 |
| 32 | 3 |

Agreement

Small frequency shifts for D2R are studied for OOK and BPSK:

  - For applying with Manchester line codes

    - Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition.

    - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift.

        - Companies to report how they perform multiplying for option 2

  - For applying with Miller line codes, according to Figure 6-13 of UHF RFID standard.

  - For FM0, small frequency shift is not defined

  - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift.

  - Potential purposes include:

    - FDMA of D2R, if supported

    - CW interference avoidance, if supported

- Note: small frequency shifts for D2R are studied for the same potential purposes for relevant identified BFSK variant(s)

Agreement

For D2R FEC study, the LTE convolutional code polynomials are a reference. Other designs can be studied subject to:

- Constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study

- Mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study

- FFS other details, e.g. final code rate by puncturing, shift-register initialization/termination.

Agreement

For D2R line codes, the study assumes the following codewords corresponding to an information bit 0 or bit 1, before considering potential small frequency-shifting:

- For FM0:

  - According to Figures 6-8 and 6-9 of UHF RFID standard

- For Miller:

  - According to Figure 6-12 of UHF RFID standard.

Agreement

- OOK and Binary PSK for baseband modulation are feasible for D2R for all devices.

- The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK)

  - FFS: whether MSK is feasible for all devices

2.1.1.4	Frame structure and timing aspects

Agreement

When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max].

- FFS: whether there is a necessity to define different maximum times for different A-IoT devices

Agreement

Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following

- How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1

- How a device determines the frequency domain resource for the D2R transmissions for Msg.1

Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1

Agreement

For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769:

- Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

- Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

- Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed.

- Note: Direction 1 and Direction 2 are not for down-selection.

For Direction 1, following can be the template to capture the solution details proposed by companies, if any.

| Source | Details |
| --- | --- |
| Source [x] | Solution description Observations or Analysis or Evaluations Specification impacts, if any |
| Source [y] | Solution description Observations or Analysis or Evaluations Specification impacts, if any |

For Direction 2, following can be the template to capture the solution details proposed by companies, if any.

| Source | Details |
| --- | --- |
| Source [x] | Solution description Observations or Analysis or Evaluations Specification impacts, if any |
| Source [y] | Solution description Observations or Analysis or Evaluations Specification impacts, if any |

2.1.1.5	Downlink and uplink channel/signal aspects

Agreement

For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH.

Agreement

For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities:

- SFO estimation

- CFO estimation

- Channel estimation

- Interference estimation

Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported

FFS: Other functionalities, if any

Conclusion

Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

Agreement

For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: length/pattern of ON/OFF.

- FFS: when TD2R_min is applicable, whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2

Agreement

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH:

- Time domain resources

- Frequency domain resources

- MCS-like information

- Chip duration

- ID associated with device(s)

- Repetitions

FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Agreement

For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH:

- ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported)

- FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

2.1.1.6	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

Observation

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [0, 8] dB or [-0.2, 3] dB frequency diversity gain at 1% or 10% BLER target in a fading channel for a 1Rx reader and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - In a TDL-A fading channel with 30ns delay spread

    - For the gap between [75KHz, 900KHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [0, 1.5] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB.

    - For the gap between [1.08MHz, 4.2MHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [3, 5.8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [0.4, 2.5] dB.

    - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 5 sources are within [5, 8] dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within [1.3, 4] dB.

  - In a TDL-D fading channel with 30ns delay spread

    - For 10MHz gap, 1 sources (R1-2405855) observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain. (Note: loss due to the power split in TDL-D)

  - In a TDL-A fading channel with 150ns delay spread

    - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [1, 3] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [0, 2.5] dB.

    - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 5.5] dB.

    - For the gap is 5MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 3] dB.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

Observation

For the D2R transmission BW corresponding to the CW waveforms, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform requires 2 times the frequency domain resources for D2R transmission, if the frequency gap between the two tones is no smaller than the transmission bandwidth of the corresponding D2R transmission.

Observation

For interference suppression complexity for different CW waveforms at D2R receiver, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones requires individual cancellation for each of the two tones, e.g. two RF or IF narrow-band bandpass filters.

Observation

For the gap between two tones to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

Agreement

For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, capture the following table in the TR.

- Further observations (if any) on these CW transmission cases are not precluded

| CW Transmission case | observations |
| --- | --- |
| Case 1-1 | - NO need for BS to support full-duplex capability in DL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in DL spectrum for scenario ‘A2’. - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum. |
| Case 1-2 | - NO need for BS to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side for scenario ‘A1’. - BS needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum. |
| Case 1-4 | - NO need for BS to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at BS side. - Cross-link interference handing for CW at BS side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at BS). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum. |

Agreement

For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, capture the following table in the TR.

- Further observations (if any) on these CW transmission cases are not precluded

| CW Transmission case | Observations |
| --- | --- |
| Case 2-2 | - NO need for intermediate UE to support full-duplex capability in UL spectrum for scenario ‘A1’ - Spatial isolation is possible for scenario ‘A1’, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side for scenario ‘A1’. - Intermediate UE needs to support full-duplex capability (including self-interference suppression for CW) in UL spectrum for scenario ‘A2’. - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum. |
| Case 2-3 | - NO need for intermediate UE to support full-duplex capability in DL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum. |
| Case 2-4 | - NO need for intermediate UE to support full-duplex capability in UL spectrum - Spatial isolation is possible, reducing the received CW interference power at intermediate UE side. - Cross-link interference handling for CW at intermediate UE side. - Estimation of CW interference may be needed for successful D2R reception (i.e., at intermediate UE). - Lower CW transmission power can be assumed in the UL spectrum than that of in the DL spectrum. |

Observation

With respect to the following RAN guidance:

- Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

    - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

- From RAN1 perspective, the following CW characteristics are identified:

    - When CW is transmitted or not transmitted

    - Transmission Power

    - Frequency resources

    - FFS other characteristics

2.1.2	Remaining Open issues

The following opens need to be addressed in RAN1:

- Evaluation assumptions

- Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1].

  - Clause 5.3: Applicable maximum distance target values(s)

  - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs

  - Clause 5.8: 2D distribution of devices

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

- Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1]

- Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology.

NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.

NOTE: strive to minimize evaluation cases in RAN1.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).

- RAN1-led:

For the Ambient IoT DL and UL:

  - Frame structure, synchronization and timing, random access

  - Numerologies, bandwidths, and multiple access

  - Waveforms and modulations

  - Channel coding

  - Downlink channel/signal aspects

  - Uplink channel/signal aspects

  - Scheduling and timing relationships

  - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

For Topology 2, no difference in physical layer design from Topology 1.

2.2	RAN2

2.2.1	Agreements

2.2.1.1	Stage 2 General aspects

RAN2#125bis

Agreements

- Unless explicitly stated all agreements apply to all device types and for both topologies.

- From RAN2 perspective, the aim is that the design on the interface between reader and A-IoT device is common for topology 1 and topology 2.

- RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

- Baseline procedure:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

Step C: The device may perform the data communication with the reader as needed,: Details FFS

- We will study the support of both “inventory” and “command” in the same procedure.

- FFS if Initial Trigger Message can also include “command”.

- RAN2 will continue the study of ambient IoT assuming no support of AS security until SA3 provides further input.

RAN2#126

Agreements

1	As baseline, the “inventory only” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

2	As baseline, the “inventory and command” case is supported by the procedure:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message.  This doesn’t mean that inventory and command are received by the reader at the same time from upper layer.

3	From RAN2 point of view we will study “Command only” use case.

FFS the options on how to support it :

Initial trigger message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

2.2.1.2	Protocol and Functionality aspects

RAN2#125bis

Agreements for control plane

1	RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

2	RRM L3 measurement reporting is not supported by Ambient IoT devices.

3	RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding.

4	Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

5	RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

Agreements for user plane

1	SDAP is not supported for UP protocol stack.

2	PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

3	RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

4	No HARQ and RLC AM

5	FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

6	RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

RAN2#126

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

RAN2#127

Agreements on functionality

1	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size. The LS is approved in R2-2407831.

2	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

3	RAN2 will study the need and use of a simple message “size” reporting to the reader

4	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

5	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

2.2.1.3	Paging

RAN2#125bis

Agreements

1.	Legacy paging message for device will not be supported.

2.	Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

3.	RAN2 assumes that the device will not support tracking/RAN area update procedure.

4.	For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

RAN2#126

Agreements

1	RAN2 will study the following cases for AIoT paging message:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

2	AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

3	From RAN2 perspective, we assume the device can receive as long as there is enough energy.  We will wait for RAN1 further progress on device monitoring details.

RAN2#127

Agreements

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices.

2	Wait for further RAN1 progress on indication of the start of access occasion.

2.2.1.4	Random Access

RAN2#125bis

Agreement

1	RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

2	We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

3	Random Access is triggered by the reader

4	Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

5	Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

6	Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

7	For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

RAN2#126

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

1	A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

RAN2#127

Agreements

1	RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design).  Wait for RAN1 for further details.

2	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

3	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

4	For 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

5	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

6	FFS if reader assigns the AS ID for scheduling purposes.

2.2.1.5	Topology 2

RAN2#127

Agreements

1	RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

2	A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

3	RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

4	RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

5	NW is in control of resources to be used by AIoT air interface.

6	Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

2.2.2	Remaining Open issues

- RAN2-led:

  - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions.

For example:

    - Paging

    - Random access

    - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope

    - Interactions with upper layers

For functionalities not listed above, they are studied only if found essential.

2.3	RAN3

2.3.1	Agreements

2.3.1.1	RAN Architecture

RAN3#123bis

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

[Topo1] AIoT RAN node:

Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848;

A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2.

[Topo2] UE Reader:

A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848);

Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

For Topology 1, RAN3 starts with AIoT RAN node being aggregated.

RAN3#124

Agreements of RAN Architecture in RAN3#124 meeting are captured in the following document:

- R3-243962	[TP for TR 38.769] CB:#AIoT1_Architecture, Ericsson, Huawei

RAN3#125

Agreements of RAN Architecture in RAN3#125 meeting are captured in the following document:

- R3-244837	(TP for TR 38.769) RAN architecture aspects, Huawei

2.3.1.2	RAN-CN Interface Impact

RAN3#123bis

AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command).

RAN3#124

Agreements of signaling and procedures for CN-RAN interface in RAN3#124 meeting are captured in the following document:

- R3-243963	Inventory and Command between AIoT CN and AIoT RAS, Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC

RAN3#125

Agreements of signalling and procedures for CN-RAN interface in RAN3#125 meeting are captured in the following document:

-	R3-244838	(TP for TR38.769) NG interface impact for Ambient-IoT, ZTE, Xiaomi

2.3.1.3	Others

RAN3#123bis

Use cases for locating an AIoT device:

-	Find an appropriate “reader” close to the A-IoT device;

-	Find where the A-IoT device is.

Support locating the A-IoT device at “reader” granularity.

RAN3#124

Agreements of locating an Ambient IoT device in RAN3#124 meeting are captured in the following document:

- R3-243964	(TP for TR38.769) Locating Ambient-IoT device, CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC

RAN3#125

Agreements of locating an Ambient IoT device in RAN3#125 meeting are captured in the following document:

-	R3-244840	(TP for TR 38.769) Locating Ambient-IoT device, CMCC

#### 2.3.2	Remaining Open issues

- RAN3-led:

  - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable:

    - Paging

    - Device context management

    - Data transport

  - Identify RAN architecture aspects.

  - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

2.4	RAN4

2.4.1	Agreements

Agreements in RAN4#110bis meeting are captured in the following document:

- R4-2406714	WF on Ambient IoT in NR, CMCC

Agreements in RAN4#111 meeting are captured in the following documents:

- R4-2410567	WF on co-existence study for ambient IoT and NR/LTE, CMCC

- R4-2410597	WF on impacts of A-IoT on RF requirements, Huawei

- R4-2410596	TP to TR38.769 skeleton for RF part, Huawei

Agreements in RAN4#112 meeting are captured in the following documents:

- R4-2414304	WF on Co-existence study for ambient IoT, CMCC

- R4-2414305	WF on RF requirements for A-IoT, Huawei

- R4-2414306	draft TP to TR 38.769 for Co-existence evaluation assumptions, Huawei

2.4.2	Remaining Open issues

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2400328	Ambient IoT Study Item work plan	CMCC, Huawei, T-Mobile USA

RAN1#118

- R1-2405800	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

- R1-2405801	Discussion on Rel-19 Ambient IoT device architecture	FUTUREWEI

- R1-2405802	Discussion on physical layer design for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2405803	Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

- R1-2405804	D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

- R1-2405805	Discussion on External Carrier Waveform Characteristics for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2405818	Evaluation assumptions and results for Ambient IoT	Nokia

- R1-2405819	Ambient IoT device architectures	Nokia

- R1-2405820	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2405821	Frame structure and timing aspects for Ambient IoT	Nokia

- R1-2405822	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

- R1-2405823	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

- R1-2405824	Evaluation assumptions and results for Ambient IoT	Ericsson

- R1-2405825	Ambient IoT device architectures	Ericsson

- R1-2405826	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2405827	Frame structure and timing aspects for Ambient IoT	Ericsson

- R1-2405828	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2405829	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

- R1-2405850	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

- R1-2405851	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

- R1-2405852	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2405853	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon

- R1-2405854	Physical channels and signals for Ambient IoT	Huawei, HiSilicon

- R1-2405855	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

- R1-2405910	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum Communications

- R1-2405911	Discussion on Ambient IoT device architectures	Spreadtrum Communications

- R1-2405912	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum Communications

- R1-2405913	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum Communications

- R1-2405914	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum Communications

- R1-2405915	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum Communications

- R1-2405938	Evaluation assumption and link level simulation of AIoT	Tejas Networks Limited

- R1-2405939	Study the Ambient IoT Device Architecture	Tejas Networks Limited

- R1-2405965	Discussion on frame structure and timing aspects of A-IoT	Tejas Networks Limited

- R1-2405967	Discussion on CW waveform characteristics of A-IoT	Tejas Networks Limited

- R1-2405968	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2405969	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	TCL

- R1-2405970	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

- R1-2405987	Discussion on evaluation methodology and assumptions	CMCC

- R1-2405988	Discussion on Ambient IoT device architectures	CMCC

- R1-2405989	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2405990	Discussion on frame structure and timing aspects for A-IoT	CMCC

- R1-2405991	Discussion on downlink and uplink channel/signal aspects	CMCC

- R1-2405992	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2406011	Discussions on frame structure and timing aspects for A-IoT	Intel Corporation

- R1-2406012	Discussions on physical channel and signals for A-IoT	Intel Corporation

- R1-2406013	Discussions on waveform characteristics of carrier-wave for A-IoT	Intel Corporation

- R1-2406071	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

- R1-2406072	Discussion on channel/signal aspects for Ambient IoT	Lenovo

- R1-2406073	Discussion on external carrier wave for Ambient IoT	Lenovo

- R1-2406082	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2406089	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2406090	Discussion on Ambient IoT device architectures	China Telecom

- R1-2406091	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2406092	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2406093	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

- R1-2406094	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Telecom

- R1-2406143	Considerations for downlink and uplink channel/signal aspects	Semtech Neuchatel SA

- R1-2406144	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Semtech Neuchatel SA

- R1-2406184	Evaluation methodologies assumptions and results for Ambient IoT	vivo

- R1-2406185	Discussion on Ambient IoT Device architectures	vivo

- R1-2406186	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2406187	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT	vivo

- R1-2406188	Discussion on  Downlink and uplink channel/signal aspects	vivo

- R1-2406189	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

- R1-2406240	Discussion on evaluation assumptions and results for A-IoT	OPPO

- R1-2406241	Discussion on device architecture for A-IoT device	OPPO

- R1-2406242	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2406243	Discussion on frame structure and timing aspects of A-IoT communication	OPPO

- R1-2406244	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

- R1-2406245	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

- R1-2406266	Discussion on A-IoT Frame Structure and Timing Aspects	Panasonic

- R1-2406286	Evaluation methodology& assumptions and results for Ambient IoT	Xiaomi

- R1-2406287	Discussion on ambient IoT device architectures	Xiaomi

- R1-2406288	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2406289	Discussion on frame structure and timing aspects for Ambient IoT	Xiaomi

- R1-2406290	Discussion on downlink and uplink channel and signal aspects for Ambient IoT	Xiaomi

- R1-2406291	Discussion on waveform characteristics of carrier-wave	Xiaomi

- R1-2406315	Consideration on general aspects of physical layer	Fujitsu

- R1-2406316	Discussion on frame structure and timing aspects	Fujitsu

- R1-2406317	Discussion on downlink and uplink channel/signal aspects	Fujitsu

- R1-2406318	Analyses on interference between AIoT and NR	Fujitsu

- R1-2406370	The evaluation methodology and preliminary results of Ambient IoT	CATT

- R1-2406371	Study of the Ambient IoT devices architecture	CATT

- R1-2406372	Discussion on general aspects of physical layer design	CATT

- R1-2406373	Study of Frame structure and timing aspects for Ambient IoT	CATT

- R1-2406374	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

- R1-2406375	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

- R1-2406403	Discussion on Ambient IoT evaluations	ZTE Corporation, Sanechips

- R1-2406404	Discussion on Ambient IoT device architectures	ZTE Corporation, Sanechips

- R1-2406405	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2406406	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE Corporation, Sanechips

- R1-2406407	Discussion on channel and signal  for Ambient IoT	ZTE Corporation, Sanechips

- R1-2406408	Discussion on carrier wave for Ambient IoT	ZTE Corporation, Sanechips

- R1-2406421	Discussion on frame structure and timing aspects for Ambient IoT	BUPT

- R1-2406429	Discussion on downlink and uplink channels and signals for A-IoT	Panasonic

- R1-2406430	Discussion on waveform characteristics of carrier-wave for Ambient IoT device	Panasonic

- R1-2406433	Analysis of cfo drift for Ambient IoT	Wiliot Ltd.

- R1-2406445	On General Physical Layer Design Considerations for Ambient IoT (internet of things) Applications	Lekha Wireless Solutions

- R1-2406453	Discussion on Frame Structure and Timing Aspects for Ambient IoT	IIT, Kharagpur

- R1-2406456	Frame structure and timing aspects of Ambient IoT	InterDigital, Inc.

- R1-2406457	Downlink and uplink channels aspects of Ambient IoT	InterDigital, Inc.

- R1-2406473	Ambient IoT evaluations	Sony

- R1-2406474	General aspects of Ambient IoT physical layer design	Sony

- R1-2406475	Frame structure and timing aspects for Ambient IoT	Sony

- R1-2406476	Downlink and uplink channel / signal aspects for Ambient IoT	Sony

- R1-2406505	Considerations on Control Information, Proximity Determination, and Intermediate UE for A-IoT	Continental Automotive

- R1-2406557	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2406558	Discussion on frame structure and timing for ambient IoT	NEC

- R1-2406559	Discussion on downlink and uplink channel for ambient IoT	NEC

- R1-2406560	Discussion on the waveform of carrier-wave for the Ambient IoT	NEC

- R1-2406566	Discussion on ambient IoT device architectures	TCL

- R1-2406579	Discussion on evaluation assumptions and results for Ambient IoT	HONOR

- R1-2406580	Discussion on frame structure and timing aspects for Ambient IoT	HONOR

- R1-2406581	Discussion on downlink and uplink channel/signal aspects for A-IoT	HONOR

- R1-2406599	Initial evaluation by link level simulation for Ambient IoT R2D	Panasonic

- R1-2406600	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2406602	Discussion on Ambient IoT evaluation	LG Electronics

- R1-2406603	Discussion on Ambient IoT device architectures	LG Electronics

- R1-2406604	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2406605	Frame structure and timing aspects for Ambient IoT	LG Electronics

- R1-2406606	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

- R1-2406607	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

- R1-2406652	Considerations for evaluation assumptions and results	Samsung

- R1-2406653	Considerations for Ambient-IoT device architectures	Samsung

- R1-2406654	Considerations on general aspects of Ambient IoT	Samsung

- R1-2406655	Considerations for frame structure and timing aspects	Samsung

- R1-2406656	Considerations for downlink and uplink channel/signal aspect	Samsung

- R1-2406657	Considerations for Waveform characteristics of carrier-wave	Samsung

- R1-2406692	Discussion on ambient IoT evaluation framework	NEC

- R1-2406693	Device architecture requirements for ambient IoT	NEC

- R1-2406728	Discussion on general aspects of physical layer design	ETRI

- R1-2406729	Discussion on frame structure and timing aspects	ETRI

- R1-2406730	Discussion on downlink and uplink channel/signal aspects for A-IoT	ETRI

- R1-2406731	Discussion on carrier-wave for Ambient IoT	ETRI

- R1-2406752	Update for TR 38.769 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei

- R1-2406771	Evaluation assumptions and results for A-IoT	MediaTek Inc.

- R1-2406772	Ambient IoT device architectures	MediaTek Inc.

- R1-2406773	General aspects of physical layer design	MediaTek Inc.

- R1-2406774	Frame structure and timing aspects	MediaTek Inc.

- R1-2406775	Downlink and uplink channel/signal aspects	MediaTek Inc.

- R1-2406776	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	MediaTek Inc.

- R1-2406811	Discussion on the evaluation assumptions for Ambient IoT devices	Lenovo

- R1-2406812	Discussion on the Ambient IoT device architectures	Lenovo

- R1-2406813	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2406838	On evaluation assumptions and link budget analysis for AIoT	Apple

- R1-2406839	On device architecture for AIoT	Apple

- R1-2406840	On general physical layer design aspects for AIoT	Apple

- R1-2406841	Frame structure and timing aspects for Ambient IoT	Apple

- R1-2406842	On physical channels/signals and proximity determination for AIoT	Apple

- R1-2406843	On carrier waveform and interference handling for AIoT	Apple

- R1-2406844	FL summary#1 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2406845	FL summary#2 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2406846	FL summary#3 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2406878	Discussion on general aspects of physical layer design	Sharp

- R1-2406879	Discussion on frame structure and timing aspects	Sharp

- R1-2406880	Discussion on downlink and uplink channel/signal aspects	Sharp

- R1-2406890	Evaluations for Ambient IoT	InterDigital, Inc.

- R1-2406891	Device architectures for Ambient IoT	InterDigital, Inc.

- R1-2406892	On the general aspects of physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2406893	On the carrier-wave for Ambient IoT	InterDigital, Inc.

- R1-2406932	Study on evaluation assumptions and results for Ambient IoT	NTT DOCOMO, INC.

- R1-2406933	Study on device architectures for Ambient IoT	NTT DOCOMO, INC.

- R1-2406934	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2406935	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2406936	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2406937	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

- R1-2407031	Evaluation Assumptions and Results	Qualcomm Incorporated

- R1-2407032	Ambient IoT Device Architecture	Qualcomm Incorporated

- R1-2407033	General aspects of physical layer design	Qualcomm Incorporated

- R1-2407034	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2407035	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

- R1-2407036	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

- R1-2407070	Discussion on Frame structure and timing aspects for A-IoT	China Unicom

- R1-2407071	Discussion on downlink and uplink channel aspects for A-IoT	China Unicom

- R1-2407072	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Unicom

- R1-2407087	Discussion on Evaluation assumptions and results	CEWiT

- R1-2407088	Discussion on General aspects of physical layer design	CEWiT

- R1-2407089	Discussion on Frame structure and timing aspects	CEWiT

- R1-2407090	Discussion on Downlink and Uplink channel/signal aspects	CEWiT

- R1-2407091	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

- R1-2407095	Evaluation assumptions and results for Ambient IoT	Indian Institute of Tech (M), IIT Kanpur

- R1-2407096	Discussion on Ambient IoT device architectures	Indian Institute of Tech (M), IIT Kanpur

- R1-2407107	Discussion on frame structure and timing aspects for Ambient IoT	TCL

- R1-2407113	Discussion on frame structure and timing aspects	Google

- R1-2407114	Discussion on downlink and uplink transmission aspects	Google

- R1-2407119	General aspects of physical layer design for Ambient IoT	ITL

- R1-2407124	Controllable Carrier Wave Characteristics	Sony

- R1-2407128	Discussion on Carrier wave related aspects for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407129	Discussion on Downlink and Uplink channel signal aspects for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407130	Frame structure and timing aspects of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407131	Discussion on General aspects of physical layer design of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407133	Views on Architecture of Ambient IoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407134	Evaluation assumption and results for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2407137	Discussion on frame structure and timing aspect	ASUSTeK

- R1-2407141	Discussion on downlink and uplink channel/signal aspect	ASUSTeK

- R1-2407175	Ambient IoT device architecture	Sony

- R1-2407185	Discussion on evaluation methodology and assumptions	CMCC

- R1-2407202	FL summary #1 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2407203	FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2407204	FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2407248	Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2407249	Feature Lead Summary #2 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2407250	Feature Lead Summary #3 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2407273	RAN1#118 FL Summary #1 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2407274	RAN1#118 FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2407275	RAN1#118 FL Summary #3 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2407276	RAN1#118 FL Summary #4 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2407312	FL summary#1 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2407313	FL summary#2 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2407325	FL summary #1 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2407326	FL summary #2 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2407327	FL summary #3 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2407363	Draft Reply LS on Clarification of requirements for Ambient IoT	Ericsson

- R1-2407364	Reply LS on Clarification of requirements for Ambient IoT	RAN1, Ericsson

- R1-2407365	Moderator summary for RAN1 reply to SA2 LS on Clarification of requirements for Ambient IoT	Moderator (Ericsson)

- R1-2407399	Draft Reply LS on Clarification of requirements for Ambient IoT	Ericsson

- R1-2407426	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2407468	FL summary#3 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2407469	FL summary#4 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2407479	Session notes for 9.4 (Study on solutions for Ambient IoT in NR)	Ad-Hoc Chair (Huawei)

- R1-2407489	Update for TR 38.769 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei

- R1-2407490	FL summary #4 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2407513	TR 38.769 v0.1.0 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei

- R1-2407532	Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2407544	Final FL summary on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2407555	Final summary on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2407560	Final FL summary for Ambient IoT evaluation	Moderator (CMCC)

RAN2#127

- R2-2406237	LS on Clarification of requirements for Ambient IoT (S2-2407231; contact: Ericsson)	SA2	LS in	  	 	To:SA1, RAN1, SA3	Cc:RAN2, RAN3

- R2-2406271	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA	discussion

- R2-2406362	Discussion on protocol stack for ambient IOT	Xiaomi	discussion

- R2-2406377	Required functions for A-IoT	Intel Corporation	discussion

- R2-2406390	Discussion on functionality aspects for Ambient IoT	vivo	discussion

- R2-2406453	Considerations for functionality aspects	Semtech Neuchatel SA	discussion

- R2-2406459	A-IoT Functionality	ZTE Corporation, Sanechips	discussion

- R2-2406482	Discussion on the Functionality Aspects for Ambient IoT	CATT	discussion

- R2-2406505	Ambient-IoT Functionality Aspects	NEC	discussion

- R2-2406540	Discussions on Functionality Aspect of Ambient IoT	Fujitsu	discussion

- R2-2406582	Inventory procedure without permanent device ID	VODAFONE Group Plc	discussion

- R2-2406590	Energy Status report	Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom	discussion

- R2-2406608	Discussions on functionalities required for AIoT	Samsung	discussion

- R2-2406614	Considerations on functionality aspects for Ambient IoT	Sony	discussion

- R2-2406654	Ambient IoT transport block size and MAC layer segmentation	MediaTek Inc., Samsung, vivo, ZTE Corporation, Sanechips, Qualcomm Incorporated	discussion

- R2-2406656	Ambient IoT data transmission after initial access	MediaTek Inc.	discussion

- R2-2406666	Funcitonal Aspects of A-IoT devices	Apple	discussion

- R2-2406709	Functionality for Ambient IOT	InterDigital	discussion

- R2-2406737	Discussion on functionality aspects of Ambient IoT	China Telecom 	discussion

- R2-2406751	Discussion on the functionalities required for Ambient IoT	Spreadtrum Communications	discussion

- R2-2406769	Discussion on functionalities required for A-IoT	OPPO	discussion

- R2-2406812	Considerations on functionality aspects for Ambient IoT	Lenovo	discussion

- R2-2406818	Functionality aspects for A-IoT	Ericsson	discussion

- R2-2406941	A-IoT functionalities	Huawei, HiSilicon	discussion

- R2-2406942	Views on the segmentation of A-IoT	Huawei, HiSilicon, Wiliot Ltd., Quanray, Telit Communications S.p.A., Nordic Semiconductor ASA, China Unicom, China Telecom, China Southern Power Grid, NTT DOCOMO INC., LG Electronics Inc., Spreadtrum Communications, Xiaomi, ASUSTeK, Kyocera, ETRI, CAICT, NEC	discussion

- R2-2407567	Views on the segmentation of A-IoT	Huawei, HiSilicon, Wiliot Ltd., Quanray, Telit Communications S.p.A., Nordic Semiconductor ASA, China Unicom, China Telecom, China Southern Power Grid, NTT DOCOMO INC., CMCC, LG Electronics Inc., Spreadtrum Communications, Xiaomi, ASUSTeK, Kyocera, ETRI, CAICT, NEC, Apple, TCL, Google Inc.	discussion

- R2-2406977	Discussion on functionality for A-IoT	CMCC	discussion

- R2-2407063	Discussion on security aspects for Ambient IoT	NTT DOCOMO, INC.	discussion

- R2-2407126	Discussion on security related aspects of Ambient IoT devices	T-Mobile USA Inc., Ericsson	discussion	  	 	Revised

- R2-2407132	AIoT Functionality aspects	Nokia	discussion

- R2-2407231	Discussion on security related aspects of Ambient IoT devices	T-Mobile USA Inc., Ericsson	discussion	  	 	R2-2407126

- R2-2407290	Discussion on functionality aspects of ambient IoT	LG Electronics Inc.	discussion

- R2-2407319	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated	discussion

- R2-2407342	Discussions on functionalities required for A-IoT Communication	HONOR	discussion

- R2-2407445	Functionalities for Ambient IoT	Kyocera	discussion

- R2-2407504	Discussions on functionality aspects for AIoT	Futurewei	discussion

- R2-2406378	Consideration on initial trigger message	Intel Corporation	discussion

- R2-2406391	Discussion on AIoT Paging	vivo	discussion

- R2-2406426	Discussion on A-IOT paging procedure	Xiaomi	discussion

- R2-2406483	Discussion on Paging for Ambient IoT	CATT	discussion

- R2-2406501	Ambient IoT device paging	TCL	discussion

- R2-2406520	Discussion on Ambient IoT paging message	ASUSTeK	discussion

- R2-2406541	Discussions on AIoT paging	Fujitsu	discussion

- R2-2406609	Discussion on A-IoT paging	Samsung	discussion

- R2-2406615	Considerations on paging for Ambient IoT	Sony	discussion

- R2-2406651	Paging for Ambient IoT	Qualcomm Incorporated	discussion

- R2-2406655	Procedures and signalling for ambient IoT paging	MediaTek Inc.	discussion

- R2-2406681	Discussion on Ambient IoT Paging	Apple	discussion

- R2-2406710	Paging Related Aspects for Ambient IOT	InterDigital	discussion

- R2-2406718	Discussion on paging procedure for Ambient IoT	OPPO	discussion

- R2-2406738	Discussion on Ambient IoT Paging	China Telecom 	discussion

- R2-2406747	Discussion on paging procedure of A-IoT	Spreadtrum Communications	discussion

- R2-2406856	Ambient-IoT Paging	NEC	discussion

- R2-2406879	Discussion on paging procedure for Ambient IoT	Lenovo	discussion

- R2-2406960	Discussion on A-IoT paging	CMCC	discussion

- R2-2407021	Discussion on Paging for A-IoT	Transsion Holdings	discussion

- R2-2407136	Discussion on A-IoT paging message	NTT DOCOMO, INC.	discussion

- R2-2407148	Further consideration on paging procedure for AIoT	ZTE Corporation, Sanechips	discussion

- R2-2407203	Ambient IoT identifiers for A-IoT paging	Panasonic	discussion

- R2-2407212	Paging procedures for Ambient IoT	Nokia France	discussion

- R2-2407222	Discussion on Paging aspects for Ambient-IoT	Continental Automotive	discussion

- R2-2407244	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion

- R2-2407261	Discussion on A-IoT paging	Sharp	discussion

- R2-2407283	Discussion on A-IoT paging	LG Electronics Inc.	discussion

- R2-2407343	Discussion on A-IoT Paging	HONOR	discussion

- R2-2407444	Consideration of paging for Ambient IoT	Kyocera	discussion

- R2-2407508	Discussions on AIoT paging	Futurewei	discussion

- R2-2407533	Discussion on Paging procedure for Ambient IoT	Philips International B.V.	discussion

- R2-2407547	Discussion on A-IOT Paging related aspects	Rakuten Mobile, Inc	discussion

- R2-2406341	Random Access for Ambient IoT device	NEC	discussion

- R2-2406361	Discussion on access procedure for ambient IOT	Xiaomi	discussion

- R2-2406379	Consideration on A-IoT Random access	Intel Corporation	discussion

- R2-2406392	Random Access Procedure for A-IoT Device	vivo	discussion

- R2-2406454	Considerations for Random Access	Semtech Neuchatel SA	discussion

- R2-2406460	Unified random-access procedure for A-IoT	ZTE Corporation, Sanechips	discussion

- R2-2406484	Discussion on the Random Access for Ambient IoT	CATT	discussion

- R2-2406502	Random Access for Ambient IOT	TCL	discussion

- R2-2406521	Discussion on Ambient IoT access message	ASUSTeK	discussion

- R2-2406542	Discussions on AIoT Random Access	Fujitsu	discussion

- R2-2406616	Considerations on random access aspects for Ambient IoT	Sony	discussion

- R2-2406682	Discussion on Random Access for Ambient IoT	Apple	discussion

- R2-2406711	Random Access Procedure for Ambient IOT	InterDigital	discussion

- R2-2406716	A-IoT random access procedure	Huawei, HiSilicon	discussion

- R2-2406752	Discussion on random access of Ambient IoT	Spreadtrum Communications	discussion

- R2-2406764	Further discussions on A-IoT random access	ETRI	discussion

- R2-2406770	Discussion on random access for A-IoT	OPPO	discussion

- R2-2406786	Discussion on UL multiple access	Ericsson	discussion

- R2-2406880	Discussion on random access for Ambient IoT	Lenovo	discussion

- R2-2406899	Random access procedure for Ambient IoT	China Telecom	discussion

- R2-2406987	Further consideration on Ambient IoT random access	CMCC	discussion

- R2-2407022	Discussion on Random Access for A-IoT	Transsion Holdings	discussion

- R2-2407207	Discussion on A-IoT random access procedure	NTT DOCOMO, INC.	discussion

- R2-2407220	Random access for Ambient IoT	Nokia France	discussion

- R2-2407262	Discussion on A-IoT random access	Sharp	discussion

- R2-2407265	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion

- R2-2407317	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated	discussion

- R2-2407344	Discussion on A-IoT random access	HONOR	discussion

- R2-2407385	Discussion on random access for ambient IoT	Google Ireland Limited	discussion

- R2-2407443	Consideration of random access of Ambient IoT	Kyocera	discussion

- R2-2407458	Further discussion on Ambient IoT random access	Samsung Electronics Czech	discussion

- R2-2407509	Discussions on AIoT random access	Futurewei	discussion

- R2-2407536	Discussion on Random Access procedure for Ambient IoT	Philips International B.V.	discussion

- R2-2407542	Discussion on Failure Handling 	Rakuten Mobile, Inc	discussion

- R2-2406380	Topology 2 considerations	Intel Corporation	discussion

- R2-2406393	Discussion on Topology 2 related aspects	vivo	discussion

- R2-2406406	Considerations on TP2 related aspects between BS and UE reader	Xiaomi	discussion

- R2-2406485	Discussion on Topology2 for Ambient IoT	CATT	discussion

- R2-2406522	Consideration on resource and scenario in topology 2	ASUSTeK	discussion

- R2-2406543	Discussions on topology 2 related aspects	Fujitsu	discussion

- R2-2406565	Discussion on topology 2 related aspects between gNB and reader	NEC	discussion

- R2-2406610	Discussion on A-IoT Topology 2	Samsung	discussion

- R2-2406652	Topology 2 related aspects	Qualcomm Incorporated	discussion

- R2-2406657	A-IoT Architecture	MediaTek Inc.	discussion

- R2-2406667	Discussion on Topology 2 for A-IoT	Apple	discussion

- R2-2406712	Topology 2 for Ambient IOT	InterDigital	discussion

- R2-2406720	Discussion on topology 2 for Ambient IOT	OPPO	discussion

- R2-2406748	Discussion on topology 2 issues	Spreadtrum Communications	discussion

- R2-2406785	Topology 2 and DO-A for Ambient IoT	Ericsson	discussion

- R2-2406881	Considerations on Topology 2 for Ambient IoT	Lenovo	discussion

- R2-2406943	A-IoT Topology 2 issues	Huawei, HiSilicon	discussion

- R2-2406961	Discussions on topology 2 for A-IoT	CMCC	discussion

- R2-2407074	RAN2 aspects for Topology 2	Nokia	discussion

- R2-2407112	Discussion on Topology 2 related aspects for Ambient IoT	China Telecom	discussion

- R2-2407151	Consideration on TP2 for AIoT	ZTE Corporation, Sanechips	discussion

- R2-2407291	Discussion on topology 2 considerations of ambient IoT	LG Electronics Inc.	discussion

- R2-2407442	Topology 2 aspects of Ambient IoT	Kyocera	discussion

- R2-2407450	Discussion on topology 2 specific issues for Ambient IoT	NTT DOCOMO, INC.	discussion

RAN3#125

- R3-244014	LS on RAN2 agreements and assumptions for Ambient IoT	RAN2 (OPPO)

- R3-244018	LS on Clarification of requirements for Ambient IoT	SA2 (Ericsson)

- R3-244030	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC

- R3-244058	(TP for TR38.769) RAN architecture for Ambient-IoT	ZTE Corporation

- R3-244059	(TP for TR38.769) NG interface impact for Ambient-IoT	ZTE Corporation

- R3-244060	Discussion on unified procedure for AIoT topology 1 and 2	ZTE Corporation

- R3-244061	(TP for TR38.769) Locating an AIoT device	ZTE Corporation

- R3-244102	(TP for TR 38.769) RAN architecture aspects	Huawei

- R3-244103	(TP for TR 38.769) Ambient IoT device locating	Huawei

- R3-244104	(TP for TR 38.769) Signalling and Procedures for Topology 1	Huawei

- R3-244105	(TP for TR 38.769) Signalling and Procedures for Topology 2	Huawei

- R3-244139	[TP for TR 38.769] RAN Architecture for AIoT	InterDigital Communications

- R3-244148	A-IoT Architecture	MediaTek Inc.

- R3-244157	RAN architecture discussion on ambient IoT	NEC

- R3-244158	RAN-CN interface impact on ambient IoT	NEC

- R3-244159	Further discussion on ambient IoT locating	NEC

- R3-244183	(TP to TR 38.769) A-IoT architecture aspects	CATT

- R3-244184	(TP to TR 38.769) Network signalling aspects for Topology 1	CATT

- R3-244185	(TP to TR 38.769) Network signalling aspects for Topology 2	CATT

- R3-244186	(TP to TR 38.879) Locating of A-IoT device	CATT

- R3-244191	(TP for TR 38.769) AIoT RAN architecture	Xiaomi

- R3-244192	(TP for TR 38.769) AIoT interface impacts between RAN and CN	Xiaomi

- R3-244193	(TP for TR 38.769) Locating AIoT device	Xiaomi

- R3-244248	(TP for TR 38.769) RAN architecture for Ambient-IoT	China Telecom

- R3-244327	RAN architecture considerations of ambient IoT	Lenovo

- R3-244328	Inventory procedure of ambient IoT	Lenovo

- R3-244329	Context Management and Data Transport of Ambient IOT	Lenovo

- R3-244387	On AIoT RAN functional and logical architecture	Ericsson

- R3-244388	[TP for TR 38.769] on AIoT Architecture aspects	Ericsson

- R3-244389	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling	Ericsson

- R3-244390	[TP for TR 38.769] On locating AIoT devices	Ericsson

- R3-244420	[TP for TR 38.769] AIoT Architecture for Topology 1 and 2	Nokia

- R3-244421	Inventory Signalling Reusing Paging procedures for AIoT Topology 2	Nokia

- R3-244422	[TP for TR 38.769] Inventory Signalling reusing Paging procedures for AIoT Topology 2	Nokia

- R3-244423	[TP for TR 38.769] Data Transport and Context Management for AIoT	Nokia

- R3-244427	Architecture, interfaces and transport for Ambient IoT	Qualcomm Incorporated

- R3-244428	Inventory, Command and device context management for Ambient IoT	Qualcomm Incorporated

- R3-244429	Methods for locating Ambient IoT devices	Qualcomm Incorporated

- R3-244555	Discussion on RAN architecture for Ambient IoT	Samsung

- R3-244556	Discussion on RAN-CN interface impact for Ambient IoT	Samsung

- R3-244564	Discussion on RAN-CN interface for Ambient IoT	China Telecom

- R3-244592	Work Plan for Ambient IoT SI	CMCC, Huawei, T-Mobile USA

- R3-244593	Discussion on RAN Architecture for Ambient IoT	CMCC

- R3-244594	Discussion on paging for Ambient IoT	CMCC

- R3-244595	Discussion on Device Context Management and Data Transfer	CMCC

- R3-244596	Discussion on locating Ambient IoT device	CMCC

- R3-244607	Discussion on NG impact for Ambient IoT	LG Electronics

- R3-244702	CB:#AIoT1_ArchitectureProtocol	Huawei

- R3-244703	CB:#AIoT2_CNRANSignalling	ZTE

- R3-244705	CB:#AIoT_Location	CMCC

- R3-244716	(TP for TR 38.769) RAN architecture aspects	Huawei

- R3-244792	(TP for TR38.769) NG interface impact for Ambient-IoT	ZTE Corporation

- R3-244811	(TP for TR38.769) Locating Ambient-IoT device	CMCC

- R3-244812	(TP for TR38.769) NG interface impact for Ambient-IoT	ZTE Corporation

- R3-244837	(TP for TR 38.769) RAN architecture aspects	Huawei

- R3-244838	(TP for TR38.769) NG interface impact for Ambient-IoT	ZTE Corporation, Xiaomi

- R3-244840	(TP for TR 38.769) Locating Ambient-IoT device	CMCC

- R3-244850	(pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC

RAN4#112

- R4-2411071	Discussion on the AIoT LLS for passive devices in RAN4    CATT

- R4-2412970	A-IoT general overview   Ericsson

- R4-2414306	draft TP to TR 38.769 for Co-existence evaluation assumptions   Huawei

- R4-2411770	collection of calibration data for A-IoT    CMCC

- R4-2411536	AIoT deployment scenario for D1T1    Sony

- R4-2411606	Discussion on the deployment scenarios and spectrum usage of Ambient IoT and NR	Xiaomi

- R4-2411767	Deployment scenario and spectrum usage	CMCC

- R4-2411865	Discussion on deployment scenarios and spectrum usage for ambient IoT    Spreadtrum

- R4-2412015	Discussion on deployment scenarios and spectrum usage for AIoT	  China Telecom

- R4-2412063	Discussion on the deployment and spectrum usage of AIoT     vivo

- R4-2412562	Discussion on Ambient IoT deployment scenarios and spectrum usage    Samsung

- R4-2412676	Discussion on Ambient IoT deployment scenarios for D1T1    NTT DOCOMO

- R4-2412696	Discussion on deployment scenarios and spectrum usage     ZTE

- R4-2412727	on deployment scenarios and spectrum usage for A-IoT    OPPO

- R4-2412880	Discussion on A-IoT deployment scenarios and spectrum usage    Huawei

- R4-2412917	On AIoT deployment scenarios and spectrum usage    Qualcomm

- R4-2412969	A-IoT deployment scenario and spectrum usage    Ericsson

- R4-2411123	Discussion co-existence evaluations for Ambient IoT in NR    CATT

- R4-2411124	Co-existence calibration results for Ambient IoT in NR    CATT

- R4-2411607	Discussion on the coexistence study of Ambient IoT and NR    Xiaomi

- R4-2411765	Discussion on A-IoT co-existence evaluation    CMCC

- R4-2411866	Discussion on co-existence evaluation for ambient IoT and NR-LTE    Spreadtrum

- R4-2412064	Discussion on the co-existence of AIoT    vivo

- R4-2412563	Discussion on coex evaluation assumptions    Samsung

- R4-2412697	Discussion on Co-existence evaluations    ZTE

- R4-2412881	A-IoT co-existence evaluations    Huawei

- R4-2412918	On Ambient IoT Coexistence Evaluation    Qualcomm

- R4-2412973	Coexisting study simulation assumptions and initial results    Ericsson

- R4-2411084	A-IoT BS feasibility and requirements    CATT

- R4-2411766	Discussion on A-IoT BS RF requirements    CMCC

- R4-2412065	Discussion on the RF requirement of AIoT BS    vivo

- R4-2412698	Discussion on RF requirement of Ambient IoT BS    ZTE

- R4-2412968	A-IoT BS RF overview    Ericsson

- R4-2413282	RF requirements for Ambient IoT BS    Huawei

- R4-2411072	Discussion on AIoT device RF requirement    CATT

- R4-2411537	Further considerations on the ambient IoT device implementation and RF aspect    Sony

- R4-2411768	Discussion on A-IoT device RF requirements    CMCC

- R4-2411867	Discussion on RF requirements impact for ambient IoT device    Spreadtrum

- R4-2412066	Discussion on the RF requirement of AIoT device    vivo

- R4-2412699	Discussion on RF requirement of Ambient IoT device    ZTE

- R4-2412972	A-IoT UE RF overview    Ericsson

- R4-2413030	On the RF requirements for Ambient IoT Device    Huawei

- R4-2413321	Energy harvesting considerations    Qualcomm

- R4-2413455	Discussion on Ambient IoT Device RF requirement impact    LGE

- R4-2411085	A-IoT intermediate UE feasibility and requirements    CATT

- R4-2411769	Discussion on A-IoT intermediate UE RF requirements    CMCC

- R4-2411868	Discussion on RF requirements impact for intermediate node (UE)    Spreadtrum

- R4-2412067	Discussion on the RF requirement of intermediate UE    vivo

- R4-2412700	Discussion on RF requirement of Intermediate node (UE)    ZTE

- R4-2412971	A-IoT UE as intermediate node RF overview    Ericsson

- R4-2413322	CW cancellation capability    Qualcomm

- R4-2412833	Topic summary for [112][131] FS_Ambient_IoT_solutions_part1    Moderator (CMCC)

- R4-2414280	Ad hoc minutes on A-IoT    CMCC

- R4-2414304	WF on Co-existence study for ambient IoT    CMCC

- R4-2412834	Topic summary for [112][132] FS_Ambient_IoT_solutions_part2    Moderator (Huawei)

- R4-2414305	WF on RF requirements for A-IoT    Huawei.

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template