3GPP TSG RAN Meeting #106							RP-242576

Madrid, Spain, December 9-12, 2024

Source:	Qualcomm Incorporated

Title:	Views on LP-WUS phase 2 WI in Rel-20

Agenda Item:	15.2

Document for: 	Discussion and Decision

# Background

Low-power wake-up signal (LP-WUS) has been studied in Rel-18 and then specified in Rel-19 [1]. UE is allowed to implement/use either OOK receiver or OFDM receiver as a low-power wake-up receiver (LP-WUR) for the LP-WUS reception. LP-WUS offers significant UE power saving for both RRC IDLE/INACTIVE mode and RRC CONNECTED mode.

In Rel-19, a significant amount of time of the work item has been spent on LP-WUS physical signal design, as it was the first release of LP-WUS introduction. In Rel-20, 3GPP can focus on some Rel-19 leftovers and enhancements for RRC CONNECTED mode procedure.  In this contribution we provide our views on LP-WUS phase 2 work item in Rel-20.

# Rel-19 leftover of LP-WUS

This section lists potential Rel-19 leftovers. We suggest that RAN considers working on these items in Rel-20, if not supported in Rel-19.

## 2.1	LP-WUS procedure option 1-3 for RRC CONNECTED mode

RAN1 defined following three LP-WUS procedures for RRC CONNECTED mode:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the start of the drx-onDurationTimer

  - The same functionality as Rel-16 DCP

- Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger a new timer for PDCCH monitoring

  - PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer

- Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring

In Rel-19, RAN1/RAN2 have focused on LP-WUS procedure option 1-1 and 1-2. However, both option 1-1 and option 1-2 do not enable the use of LP-WUR for UE power saving inside C-DRX active time. Since drx-InactivityTimer is typically set long, C-DRX active time continues once PDCCH monitoring is triggered by LP-WUS, regardless of the amount of traffic being served within the active time. For further UE power saving, it is beneficial to enable a UE to switch from PDCCH monitoring to LP-WUS monitoring even when C-DRX active time.

One may say the purpose can be achieved by using the legacy Long DRX / DRX command MAC CEs. These MAC CEs can indicate stopping of drx-onDurationTimer and drx-InactivityTimer for the DRX group. With these, for both LP-WUS options 1-1 and 1-2, the MAC CEs are effectively able to terminate C-DRX active time of the UE so that the UE can switch back to LP-WUS monitoring for power saving. However, this solution causes large overhead and processing latency for the switch.

During Rel-19, the Case 2 LP-WUS procedure was also discussed. In Case 2, C-DRX is not configured and LP-WUS monitoring can be configured anytime to trigger PDCCH monitoring. This looks an advanced scheme compared to the options above, but essentially, the procedure can be realized by Option 1-2. If Case 2 is considered, study is necessary to identify the benefit compared to the options above.

# Rel-20 enhancements of LP-WUS for RRC CONNECTED mode

This section lists potential enhancements of LP-WUS for RRC CONNECTED mode.

## 3.1	Measurement using LP-WUR in RRC CONNECTED mode

In Rel-19, it has been a common understanding that LP-SS is specified for a UE monitoring LP-WUS in RRC IDLE/INACTIVE mode. For LP-WUS in RRC CONNECTED mode, it is stated in the Rel-19 WID that RRM/RLM/BFD/CSI measurements are performed by MR. With this, RAN1 agreed that there is no measurement impact from LP-WUS monitoring.

However, this limits the UE power saving benefit achievable by LP-WUS since the UE has to wake-up MR for measurements regardless of the LP-WUS indication. Further UE power saving is expected if the UE is allowed to perform measurement using LP-WUR instead of MR. LP-WUR using OFDM receiver can measure existing RS such as PSS/SSS of SSB and therefore, LP-SS does not need to be introduced for this. We suggest studying the benefit and RAN4 specification impact of measurement using LP-WUR in RRC CONNECTED mode. The study/work can be limited to LP-WUR with OFDM receiver.

## 3.2	Multi-bits indication by a LP-WUS in RRC CONNECTED mode

Considering Rel-19 will be limited to a basic support of LP-WUS in RRC CONNECTED mode, binary indication (trigger or not trigger PDCCH monitoring) for a UE by a LP-WUS would be a simple and straightforward approach. If it is possible to indicate multiple bit information by a LP-WUS, it can control UE power saving behavior more flexibly. For example, it is possible to consider indicating sleep-state/level control by a LP-WUS to the UE.

# Conclusion

In this contribution, we provided our views on LP-WUS phase 2 work item for Rel-20. The proposal is summarized as follows.

Proposal:

- Consider following as the potential scope of the Rel-20 WI for LP-WUS phase 2

| Area | Features |
| --- | --- |
| Rel-19 leftover(s) | - LP-WUS Option 1-3, if not supported in R19 |
| Rel-20 enhancements | - Measurement using LP-WUR in RRC CONNNECTED mode - Multi-bits indication by a LP-WUS for a UE |

# Reference

- RP-234056, New WID on low-power Wake-up Signal and Receiver for NR