3GPP TSG RAN Meeting #106								            RP-242619

Madrid, Spain, December 9-12, 2024

Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | Low-power wake-up signal and receiver for NR (LP-WUS/WUR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LPWUS |  |  |  |  |
| Unique ID | 1020094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240801 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: n/a | Core part: 55% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Xueming Pan |
|  | Company | vivo |
|  | Email | panxueming@vivo.com |
| Rapporteur | Name | Shinya Kumagai |
|  | Company | NTT DOCOMO |
|  | Email | shinya.kumagai.yw@nttdocomo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

LP-WUS and LP-SS design

RAN1#118bis

Working Assumption

For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1

- Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1.

  - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation

    - It doesn’t preclude additional operation for OOK-4 with M>1.

    - DFT size is 2^n

Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

Agreement

In case of overlaid OFDM sequence carrying information, support option 2:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

- Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols.

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

Agreement

To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values:

- M=1, L= {4,6,8}

- M=2, L= {8,12,16,24}

- M=4, L= {16,24,32,56}

- FFS whether one or more than one L value (s) applied to each of M values M=1,2,4

- FFS the L values applied to other applicable SCS(s)

- FFS the L values if other M values in addition to M=1,2,4 are supported.

- FFS whether the time estimation averaged across multiple LP-SS occasions is applied

- Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS)

- Note 2: This doesn’t preclude any of three agreed sequence types

Above applies at least for both 15kHz and 30kHz

Agreement

For LP-SS periodicity, support at least 320ms

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement

For overlaid OFDM sequence at least for LP-WUS,

- For OOK-4 with M>1, support ZC sequence for overlaid sequence.

  - FFS details

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

Agreement

For the SCS used for LP-WUS and LP-SS, further discuss the following options:

- The single SCS is configured by gNB

- The single SCS is determined by pre-defined rule

Agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement

Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below:

- 4

- 8

- 16

- 32

- 64 for M not larger than 2

- FFS whether the same or different set of candidate overlaid sequences are used for different cells

Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority))

  - Assume the sliding window size is at least [-25us, 25us].

- RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s

  - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison

- Cross correlation: maximum cross-cor value/ maximum auto-cor value

  - FFS how to use cross-correlation for comparison

- Note: refer to RAN4 RRM assumption for the inter-cell interference

Agreement

Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements:

- adding CRC

- channel coding other than Manchester coding, including rate matching if any

- mapped to a binary sequence/codeword, include truncation if any

- repetition, including rate matching if any

- Note: combination of options above is not precluded

- Note: combination of Manchester coding with above is assumed

- FFS how LP-WUS is transmitted within a MO/MOs

Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1#119

Agreement

Update the existing agreement as shown below:

In case of overlaid OFDM sequence carrying information, support option 2:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

  - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols.

  - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

    - FFS the how the information bits are carried by the overlaid OFDM sequence(s)

Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Agreement

For the overlaid OFDM sequence in time domain, the sequence is generated based on

,

- M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length.

  - The base overlaid sequence is generated by extension of

,

  - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,,

- M=2, 4, is down-selected from the following:

  - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by extension of

,

    - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

    - Note it doesn’t preclude any pulse shaping scheme if any.

  - Alt2:is given by the largest prime number such that,  is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is.

For example,,

    - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s)

For example,,

        - where,

    - FFS the value of G, with G>0.

- FFS value(s) of root q and/or CS  for generating the candidate sequences if applied

- FFS on whether changes are needed to handle intra-/inter-cell interference

- Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above.

  - Above overrides any previous RAN1 agreement / working assumption

Agreement

At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following:

- Alt1:, with

  - FFS how to map the generated overlaid OFDM sequence to the frequency domain

- Alt2:

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Agreement

CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling:

- FFS: the set of values of CS and root for configuration

- FFS: details of the RRC signaling

Agreement

For the length L of LP-SS binary sequence, limit the selection to the following:

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

Agreement

To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy.

Agreement

For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Agreement

Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 4 candidates overlaid sequences for M=4

Agreement

Regarding whether to support additional sync signal to LP-SS, down-selection from the following:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms]

    - FFS: whether different values can be applicable for different M values

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

Agreement

For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier

- The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB

- FFS: Case where associated CD-SSB and initial DL BWP have different SCSs

- FFS: Which initial BWP for RedCap

Agreement

For collecting LP-SS binary sequence results

- Endorse the template for collecting LP-SS binary sequence results in R1-2410925

- Email discussion for collecting LP-SS binary sequences by 10th Jan.

- Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

LP-WUS operation in IDLE/INACTIVE modes

RAN1#118bis

Agreement

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Agreement

From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if

- it does not detect a LP-WUS on the monitored LO

- or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

- FFS the values

- Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

RAN1#119

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

LP-WUS operation in CONNECTED modes

RAN1#118bis

Agreement

For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

- LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

- UE is configured with legacy C-DRX configurations as Rel-18

- UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

- No impact on RRM/RLM/BFD measurement requirements is assumed

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

  - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

    - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

Agreement

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

- FFS: X

- FFS: definition of reported candidate value

- UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional.

- FFS: A single time offset is configured for a UE at a given time

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

  - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

  - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Agreement

- For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1

  - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Agreement

- Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective,

  - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions

  - Introducing monitoring window is not precluded

RAN1#119

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable

Agreement

LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication.

Note: LP-WUS indication does not impact SCell dormancy behavior

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

Agreement

For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

- Use existing DRX Command MAC CE and Long DRX Command MAC CE

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Conclusion

For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Agreement

For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

- If the parameter is NOT configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- If the parameter is configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- FFS: Whether RAN1 spec impact is required.

2.1.2	Remaining Open issues

- Remaining details of an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes

  - Remaining details of OOK (OOK-1 and OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

    - Remaining details of LP-SS based on OOK-1 and OOK-4 waveform with or without overlaid OFDM sequences.

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.2	RAN2

2.2.1	Agreements

Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

RAN2#127bis

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

RAN2 inform this conclusion to SA2/CT1/RAN3. This LS is approved in R2-2409225

RAN2#128

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- - Option 3: The formula for UE_ID based PEI subgrouping is reused.

We will use the following to draft the LS. The LS is not CC to RP.

State of discussions in RAN WG2:

During RAN WG2 discussions, many companies raised concerns that if LP-Receivers support only subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. This will impact user performance and decrease the utilisation of higher frequency bands. In order to ensure the LP-WUS capable UEs are camping on the band LR implemented, the NW would need to offload LP capable UEs to LR band.

On the other hand if the NW operator does not offload the UEs to the band LP receiver supports, only a subset of UEs would profit from LP-WUS feature.

Some companies do not see that overload problem would occur as the NW implementation can control the offload of the UEs to the lower bands in dedicated way.

In regards to the overload problem, RAN WG2 has not finished evaluations and would need to study the solutions more.

Actions:

RAN2 kindly asks RAN WG1 and WG4 to take the information above into account once deciding on the support of the use case mentioned above.

The LS is approved (unseen) in R2-2410957.

RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RAN2#127bis

- Working assumption

For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

RAN2#128

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

Procedures for LP-WUS in RRC_CONNECTED

RAN2#127bis

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

RAN2#128

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

2.2.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

  - Remaining details of RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

- For CONNECTED mode, remaining details of procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring

2.3	RAN3

2.3.1	Agreements

RAN3#125bis

The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

RAN3#126

agree with the BLCR assignment below.

BLCR to 38.300	Vivo/Docomo

BLCR to 38.413	ZTE

BLCR to 38.423	Ericsson

BLCR to 38.473	Huawei

BLCR to 38.410	CATT (TS38.410 is not included in WID yet)

BLCR to 38.470	Nokia (TS38.470 is not included in WID yet)

BLCR to 38.420 (if any)	Qualcomm

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

2.3.2	Remaining Open issues

- For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

2.4	RAN4

2.4.1	Agreements

LP-WUS UE RRM

RAN4#112bis

- Sub-topic 1-1 General aspects

Issue 1-1-1-1: For case 3, whether to combine MR and LP-WUR samples when serving cell measurement when results from both MR and LP-WUR are available

Agreement:

From RAN4 requirement perspective, combine measurement results from different receivers shall not be considered

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

Agreement

RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement:

RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-5: Nubmer of Rx for LP-WUR requirements

Agreement

Single Rx is the baseline for LR-WUR RRM measurement.

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

- Sub-topic 2-1 On simulation assumptions and parameters

Issue 2-1-3: Time/frequency error

Agreement:

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T= 5us for OOK-1 T= 2us for OOK-4 with M=2 T= 1us for OOK-4 with M=4 |  |

Issue 2-1-4: Accuracy baseline for simulation

Agreement:

Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only:

For FR1:

For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ

1st priority: 3.5dB, 2nd priority: 2.5dB

For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ

1st priority: 3.5dB, 2nd priority: 2.5dB

Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session

Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

RAN4#113

- Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

based on LO periodicity

FFS whether to define an upper bound

Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Agreement:

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

- Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

Scaling factor(s) is/are >= 4. Further down-select from

Option 1:  1 fixed scaling factor

Option 2: multiple values configured by NW

Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | TBD |
| 0.64 | TBD |
| 1.28 | TBD |
| 2.56 | TBD |

LP-WUS UE RF

RAN4#112bis

- Sub-topic 1-1 General

Issue 1-1-1: draft skeleton for LP-WUS RF TR 38.774

Agreements:

The draft skeleton for TR 38.774 in R4-2415778 is endorsed.

Below is the TR drafting work splitting among interested companies:

| Section in TR | Title | Owner(s) |
| --- | --- | --- |
| General part | (scope, reference, definition, background) | vivo |
| 5 | System parameters | Ericsson, CATT |
| 6 | Simulation assumptions and evaluation | CATT, CMCC, vivo |
| 7.1 | UE RF | Huawei, Nokia, CMCC |
| 7.2 | BS RF | ZTE, Ericsson |
| 8 | Testability | Nokia, Samsung |
| Annex | Other aspects (if needed) | vivo |

Issue 1-1-2: Consideration on RAN1 LS R4-2414909

Agreements:

no specific RF work related to RAN1 LS.

Issue 1-1-3: MDR value for RF requirements

Agreements:

1% MDR. Same for requirements and conformance testing.

Issue 1-1-4: FAR assumption for MDR evaluation

Agreements:

Use </=1% FAR as evaluation assumption for RF

- Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not.

Issue 1-2-2: considerations on LP-WUS allocation

Agreements:

RAN4 to assume PRB grid alignment between LP-WUS and NR.

RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier.

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS

Agreements:

Follow RAN1 agreement for number of RBs for LP-WUS with 15KHz SCS, i.e., 11RBs.

- Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Alignment of FR1 SNR calculation/definition in simulation

Agreements:

SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise per sub carrier, over all the LP-WUS RBs.

Consider the sub-carriers allocated to LP-WUS excluding Guard RBs.

Issue 2-1-2: FR1 SNR reference point

Agreements:

same SNR reference point as MR

Issue 2-1-3: Target FR1 SNR value for LP-WUS/WUR

Agreements:

RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers.

Issue 2-1-5: Link level simulation assumption for FR2 LP-WUR

Agreements:

RAN4 can further discuss simulation assumption for FR2 LP-WUS next meeting.

- Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS

Agreements:

use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR

Agreements:

Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed.

Encourage companies to also input the justification for each element.

Issue 2-2-10: Baseline architecture for FR2 OOK-based and OFDM-based LP-WUR

Agreements:

Companies to study baseline architecture for FR2 LP-WUS.

- Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: ASCS requirements value

Agreements:

Confirm ASCS RF requirements is needed for LP-WUS/WUR.

Issue 2-3-2: Required number of guard RB for ASCS

Agreements:

RAN4 further discuss required guard RBs for ASCS requirements.

Issue 2-3-3: Whether Test case for ASCS should be defined in RAN4

Agreements:

Detailed test case for ASCS should be defined.

- Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Phase noise for simulation assumption

Agreements:

Phase noise can be considered in LLS simulation analysis.

Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded.

Number of guard RBs will be further discussed until Feb meeting. Encourage companies to provide simulation results with phase noise modelling.

Issue 2-4-4: ACS requirements value

Agreements:

FFS.

Issue 2-4-5: Required guard RB for ACS requirements

Agreements:

RAN4 further discuss the required number of guard RBs for ACS requirements

Issue 2-4-7: Test parameters for LP-WUR ACS case

Agreements:

Further discuss test parameters for LP-WUS ACS

Issue 2-4-8: whether limit ADC bit for ACS requirements

Agreements:

No limiting to a specific ADC bit for ACS requirements.

Issue 3-2-2: Maximum input level requirements

Agreements:

The same maximum input power of [-25 dBm] could be starting point for LP-WUR.

FFS whether this value should be scaled-down based on the number of LP-WUS RBs.

Issue 3-2-3: Reference channel for LP-WUR requirements

Agreements:

RAN4 should define reference measurement channel for LP-WUR RF requirements.

FFS details.

- Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: General framework on LP-WUS testing

Way forward: FFS

Issue 4-1-2: Separate RF test case for idle and connection mode

Way forward: FFS

Issue 4-1-3: How to get feedback from LP-WUR during the test

Way forward: FFS

Issue 4-1-4: detailed Test procedure

Way forward: FFS

Issue 4-1-5: UE test mode for LP-WUR

Agreements:

FFS A UE test mode for LP-WUR is needed.

Issue 4-1-6: whether a LS to RAN5 on test issue

Way forward: FFS

RAN4#113

- Sub-topic 1-1 General

Issue 1-1-3: General issue on how to introduce LP-WUS feature into core specs 38.101-x

Agreements:

Add a new suffix “M” as “LP-WUS/WUR”

Issue 1-1-4: views on which requirements should be updated in core spec

Agreements:

The part should be updated for LP-WUS/WUR in spec.

The additional requirements are not precluded, if needed.

| Sections or requirements | Needed or not? |
| --- | --- |
| Operation bands | FFS |
| UE channel bandwidth | FFS |
| Channel raster | FFS |
| REFSENS | YES |
| Maximum input level | YES |
| ACS | YES |
| ASCS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | FFS |
| Wide band Intermodulation | FFS |
| Spurious emissions | FFS |
| Annex A Measurement channels | YES |

Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

No new channel raster is introduced dedicated for LP-WUS/WUR.

FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

- Sub-topic 2-1 SNR simulation and values

Issue 2-1-1: Target FR1 SNR value for LP-WUS/WUR

Agreements:

Use worst case waveform of OOK-4 M=4 to derive SNR.

Some detailed aspects should be also confirmed, e.g., CRC bit (8), sequence length (8, 16), codepoint length of known part of the sequence)

Issue 2-1-3: Link level simulation assumption for FR2 LP-WUR

Agreements:

Further discussions and alignment on FR2 RF simulation assumption not precluded.

Use the following as starting point

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | up to UE implementation |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 |  |  |  |
| WUS RB | Fixed 11RB |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | Center for ASCS, edge for ACS [assume no ASCS impact] |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | For ASCS: 0 or 1RB on each side of LP-WUS bandwidth For ACS: 1/2/3/4 RB |  |  |  |
| Filter | 3th/5th Order lowpass Butterworth Other order lowpass filter is not precluded |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | TBD |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | FFS |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

- Sub-topic 2-2 NF and REFSENS requirements

Issue 2-2-1: General views on REFSENS for LP-WUS

Agreements:

Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR

Agreements:

Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported.

The total value of NF+IM can be considered.

On how to handle the NF values:

Option 1: For NF whether average value can be adopted.

Option 2: two sets NFs for different LP-WUR types with different power consumption target

Option 3: also consider coverage

Capture the table in WF as background information

Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR

Agreements:

Further discuss next meeting

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR

Agreements:

FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details

Multiple frequency conversion in RF domain is not precluded

For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features:

It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized

FFS on whether and how to combine two Rx chains

Antenna gain is that of a single element

Coverage target should be considered

- Sub-topic 2-3 ASCS simulation and requirements

Issue 2-3-1: Required number of guard RB for ASCS

Agreements:

[0RB] as guard RB for ASCS.

Issue 2-3-2: Detailed Test case for ASCS

WF:

RAN4 further discuss the test case next meeting

- Sub-topic 2-4 ACS simulation and requirements

Issue 2-4-1: Phase noise for simulation assumption

Agreements:

Companies can use different phase noise assumptions, the phase noise assumption should be reported

Issue 2-4-2: General ACS requirements value and guard RB

Agreements:

For ACS evaluation, companies are encouraged to provide the simulation results by assuming

Baseline: the same interference experienced by LP-WUR and main radio.

Optional: the different interference level for LP-WUR and main radio

Issue 2-4-4: Test parameters for LP-WUR ACS case

WF:

RAN4 further discuss detailed test parameters table and how to modify spec.

- Sub-topic 3-1 IBB, OBB and Intermodulation requirements for UE RF

Issue 3-1-1: IBB, OBB requirements

WF:

Further discuss IBB and OBB requirements

Issue 3-1-3: detailed table for test parameters of IBB and OBB test case

WF:

FFS

- Sub-topic 3-2 Other Rx requirements

Issue 3-2-1: spurious response requirements

WF:

FFS whether spurious response can be relaxed

Issue 3-2-2: Maximum input level requirements

Agreements:

For maximum input power level, further discuss

Option 1: use legacy -25dBm

Option 2: scaled to -26.5dBm.

Option 3: same as MR, FFS test signal

Sub-topic 4-1 Testability for UE RF requirements

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing

Agreements:

The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance

Agreements:

Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point:

Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance.

FFS if the MR should only be woken up when data transfer the system simulator is needed.

Other options are not precluded.

LP-WUS BS RF

RAN4#112bis

Issue 1-2: Concept of LP-WUS power boosting

WF

Compare the Pros and Cons of the two definitions

At least the following aspects should be considered

Convenience for manufacturer’s declaration

Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc.

Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided

Comparison of power boosting with NB-IoT with the same definition should be provided

Test aspects of power boosting should be considered

Other aspects are not precluded

Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW

Issue 1-3: On minimum power boosting level for LP-WUS

WF

FFS

Issue 1-4: Whether a cap for LP-WUS power boosting should be considered

WF

[Leave the maximum power dynamic range to manufacturer’s declaration]

Issue 1-5: Whether to preclude small CBW for consideration of LP-WUS power boosting

WF

Consider exclusion of small CBW together with LP-WUS power boosting definition, manufacturer’s declaration as a package for further discussion

Issue 1-6: On applicable BS type for LP-WUS feature

WF

Non-AAS, AAS could be considered for LP-WUS feature

Issue 1-7: Whether to define the LP-WUS requirements only for supported BS types

WF

FFS.

Issue 1-8: General consideration on other requirements

WF

FFS.

Issue 1-9: Unwanted emissions

WF

At least current regulation relevant spectrum requirements, e.g. SEM, SE should also be applied for LP-WUS.

Issue 1-10: Transmitted signal quality

Recommended WF

FFS.

Issue 1-11: Multi-band operation

Proposals

Proposal 1: The same LP-WUS/LP-SS power boosting requirement and declaration can be applicable to multi-band BS. (Nokia, CMCC)

Proposal 2: Postpone the discussion on multi-band operation for LP-WUS until the single band operation is finalized. (CATT)

WF

Proposal 2.

RAN4#113

- Sub-topic 1-1 On dynamic range for LP-WUS

Issue 1-1: Concept of LP-WUS power boosting

Adhoc WF

“The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

FFS whether other cases can be additionally considered. E.g., single value declaration.

Issue 1-2: How to comprehend the de-boost on the NR transmission power

WF

Consider the de-boosting behaviour together with other relevant issues

Companies are encouraged to think about the draft spec language to reflect corresponding proposals

Issue 1-3: Core requirement for LP-WUS power boosting

WF

FFS the following options

Option 1: Leave the power boosting level to manufacturer declaration

Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW

Other options are not precluded

Issue 1-4: On applicable BS type for LP-WUS feature

WF

Drop the discussion, there was an agreement in last meeting that:

Non-AAS, AAS could be considered for LP-WUS feature.

Issue 1-5: Requirement type(s) for LP-WUS

WF

Stop further discussion of the issue, leave it to contribution driven

Sub-topic 1-2 Other considerations apart from power boosting for LP-WUS

Issue 1-6: General consideration on other requirements

Adhoc WF

Discuss on whether transmit signal quality should be defined

FFS whether equivalent EVM requirement could be considered,

E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier

FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

2.4.2	Remaining Open issues

- RRM：For IDLE/INACTIVE modes

  - Remaining details of procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

  - Remaining details of LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell.

  - Remaining details of requirements for further RRM relaxation of UE MR for both serving and neighbor cell measurements, and requirements for UE serving cell RRM measurement offloaded from MR to LP-WUR.

- RF：

  - UE RF requirements. Including TR editing. At least the following aspects:

    - REFSENS

    - Maximum input level

    - ACS

    - ASCS

    - In-band blocking

    - Out-of-band blocking

    - Narrow band blocking

    - Spurious response

    - Wide band Intermodulation

    - Spurious emissions

  - BS RF requirements.

- Demod：

  - Not started

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### RAN1#118bis

- R1-2407614	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2407634	Discussion on LP-WUS and LP-SS Design	TCL

- R1-2407664	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2407714	Discussion on LP-WUS and LP-SS design	Spreadtrum Communications

- R1-2407869	Discussion on LP-WUS and LP-SS design	vivo

- R1-2407913	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2407977	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2408055	Design of LP-WUS and LP-SS	CATT

- R1-2408074	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2408129	Signal design for LP-WUS and LP-SS	OPPO

- R1-2408217	Discussion on LP-WUS and LP-SS design	NEC

- R1-2408236	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2408253	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2408271	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2408280	Discussion on LP-WUS and LP-SS design framework for Low power WUS	InterDigital, Inc.

- R1-2408344	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2408399	LP-WUS and LP-SS Design	Everactive

- R1-2408416	LP-WUS and LP-SS design	Sony

- R1-2408476	LP-WUS and LP-SS design	Apple

- R1-2408537	LP-WUS and LP-SS design	Nokia

- R1-2408654	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2408679	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2408691	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2408794	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2408820	LP-WUS and LP-SS design	Ericsson

- R1-2408858	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2408918	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2408968	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2409089	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2409090	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2409091	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2409242	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2409243	Summary #5 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2409308   Final summary of discussions on LP-WUS and LP-SS design  Moderator (vivo)

- R1-2407615	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2407635	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2407665	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2407715	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum Communications

- R1-2407870	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2407914	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2407978	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2408056	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2408075	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2408130	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2408218	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2408254	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2408281	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2408345	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2408417	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2408477	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2408479	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2408480	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2408481	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2408500	Discussion on LP-WUS operation in idle/inactive mode	Fujitsu

- R1-2408538	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2408575	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2408592	On LR and MR operating frequencies	Vodafone

- R1-2408655	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2408680	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2408692	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2408795	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2408821	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2408859	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2408917	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2408986	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2408479	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2408480	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2408481	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple)

- R1-2409239	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2409240	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2407666	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2407716	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum Communications

- R1-2407871	Discussion on LP-WUS operation in CONNECTED modes	vivo

- R1-2407915	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2407979	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2408057	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2408076	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2408131	Further consideration on LP-WUS operation in connected mode	OPPO

- R1-2408219	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2408255	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2408282	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2408322	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

- R1-2408418	LP-WUS operation in CONNECTED mode	Sony

- R1-2408478	LP-WUS operation in CONNECTED modes	Apple

- R1-2408495	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2408539	LP-WUS operation in CONNECTED mode	Nokia

- R1-2408576	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2408656	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2408681	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2408693	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2408796	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2408822	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2408860	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2408921	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2409010	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2409197	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2409237	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2409265	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2409266	FL summary #5 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

### RAN1#119

- R1-2409423	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

- R1-2409444	LP-WUS and LP-SS design	Nokia

- R1-2409456	Discussion on LP-WUS and LP-SS design for low power WUS	InterDigital, Inc.

- R1-2409476	Discussion on LP-WUS and LP-SS Design 	TCL

- R1-2409520	Discussion on LP-WUS and LP-SS design	CMCC

- R1-2409534	Discussion on LP-WUS and LP-SS Design	EURECOM

- R1-2409546	Discussion on the LP-WUS and LP-SS design	Panasonic

- R1-2409559	Discussion on LP-WUS design	ZTE Corporation, Sanechips

- R1-2409605	Discussion on LP-WUS and LP-SS design	Samsung

- R1-2409644	Discussion on LP-WUS and LP-SS design	Spreadtrum, UNISOC

- R1-2409689	Discussion on LP-WUS and LP-SS design	vivo

- R1-2409811	LP-WUS and LP-SS design	Apple

- R1-2409860	Discussion on LP-WUS and LP-SS design	NEC

- R1-2409904	Discussion on LP-WUS and LP-SS design	Xiaomi

- R1-2409949	Design of LP-WUS and LP-SS	CATT

- R1-2410036	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

- R1-2410071	Signal design for LP-WUS and LP-SS	OPPO

- R1-2410135	Discussion on LP-WUS and LP-SS design	Fujitsu

- R1-2410181	Discussion on LP-WUS and LP-SS design	HONOR

- R1-2410231	LP-WUS and LP-SS design	Sony

- R1-2410294	Discussion on LP-WUS and LP-SS design	LG Electronics

- R1-2410310	LP-SS Code Sets and Comparison	Everactive

- R1-2410397	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

- R1-2410419	Discussion on LP-WUS and LP-SS design	Sharp

- R1-2410444	LP-WUS and LP-SS design	Ericsson

- R1-2410486	LP-WUS and LP-SS design	Qualcomm Incorporated

- R1-2410510	LP-WUS and LP-SS design	MediaTek Inc.

- R1-2410556	Discussion on LP-WUS and LP-SS design	Lenovo

- R1-2410569	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

- R1-2410747	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2410748	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2410837	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2410838	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2410883	Summary #5 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

- R1-2410884 	Final summary of discussions on LP-WUS and LP-SS design  Moderator (vivo)

- R1-2410925	Template for collecting LP-SS binary sequence results  Moderator (vivo)

- R1-2409424	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2409445	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2409457	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2409477	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

- R1-2409521	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2409547	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2409560	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2409606	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2409645	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2409690	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2409812	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2409861	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2409905	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2409950	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2410037	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2410072	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2410232	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2410274	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2410295	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2410300	On LR and MR operating frequencies	Vodafone, Deutsche Telekom

- R1-2410398	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2410420	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2410445	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2410487	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2410511	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2410557	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2410568	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

- R1-2409814	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2409815	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2409816	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2410861	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2410862	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2410863 	Summary #6 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

- R1-2409425	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

- R1-2409446	LP-WUS operation in CONNECTED mode	Nokia

- R1-2409458	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

- R1-2409522	Discussion on LP-WUS operation in CONNECTED mode	CMCC

- R1-2409561	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

- R1-2409607	Discussion on LP-WUS operation in CONNECTED modes	Samsung

- R1-2409646	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum, UNISOC

- R1-2409691	Discussion on LP-WUS operation in CONNECTED modes	vivo

- R1-2409813	LP-WUS operation in CONNECTED modes	Apple

- R1-2409862	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

- R1-2409875	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

- R1-2409906	Discussion on LP-WUS operation in Connected mode	Xiaomi

- R1-2409951	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

- R1-2410073	Further consideration on LP-WUS operation in connected mode	OPPO

- R1-2410233	LP-WUS operation in CONNECTED mode	Sony

- R1-2410275	Discussion on LP-WUS operation in CONNECTED modes	ETRI

- R1-2410296	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

- R1-2410358	Discussion on LP-WUS procedures in Connected mode	TCL

- R1-2410363	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

- R1-2410399	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

- R1-2410421	Discussion on LP-WUS operation in CONNECTED modes	Sharp

- R1-2410446	LP-WUS operation in CONNECTED mode	Ericsson

- R1-2410488	LP-WUR operation in connected mode	Qualcomm Incorporated

- R1-2410512	LP-WUS operation in CONNECTED modes	MediaTek Inc.

- R1-2410699	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator(NTT DOCOMO)

- R1-2410804	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2410859	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2410906	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2410907	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

- R1-2410908	Draft LS on LP-WUS operation in CONNECTED modes	Moderator (NTT DOCOMO)

- R1-2410909	LS on LP-WUS operation in CONNECTED mode	RAN1, NTT DOCOMO

### RAN2#127bis

- R2-2407921	LS on LP-WUS operation in IDLE/INACTIVE mode (R1-2407559; contact: Apple)	RAN1	LS in	Rel-19	NR_LPWUS	To:RAN2, RAN4

- R2-2408447	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

- R2-2409058	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core 	R2-2407396

- R2-2408114	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2409222 [Draft]LS to on LP-WUS subgrouping vivo

- R2-2409225 LS to on LP-WUS subgrouping vivo

- R2-2409226 LS on LP-WUS subgrouping vivo

- R2-2409157	On LR and MR operating frequencies (related to LS in R1-2407559)	Vodafone	discussion	Rel-19

- R2-2408572	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2408007	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2408043	LP-WUS in RRC_IDLE and INACTIVE	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2408072	LP-WUS operation in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408168	Discussion on LP-WUS operation in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

- R2-2408182	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2408239	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

- R2-2408289	Procedure of LP-WUS in RRC_IDLE and INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2408415	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408428	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2408450	Discussion on procedure and configuration of LP-WUS in RRC_IDLEINACTIVE	OPPO	discussion

- R2-2408489	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo	discussion	Rel-19	NR_LPWUS-Core

- R2-2408709	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2408741	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2408763	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2408768	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2408949	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2409005	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2408769	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2408159	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2408008	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2408071	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408111	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2408115	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2408169	Discussion on RRM measurement relaxation in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

- R2-2408183	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2408240	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

- R2-2408306	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

- R2-2408416	Discussion on LP-WUS RRM 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408429	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2408573	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2408710	Discussion on RRM aspects for  LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2408742	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2408765	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2408849	RRM Measurement Relaxation and Offloading in RRC_IDLE /INACTIVE Mode	China Telecom 	discussion

- R2-2408950	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2409006	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2409059	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2407397

- R2-2409076	LP-WUS in RRC_CONNECTED	Nokia	discussion

- R2-2408184	Analysis on LP-WUS for RRC_CONNECTED Mode	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2409007	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

- R2-2408116	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2408009	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2408030	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2408044	Procedures for LP-WUS in RRC_CONNECTED	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2408084	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408290	Procedure of LP-WUS in RRC_CONNECTED	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2408417	Discussion on LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2408430	Discussing on LP-WUS monitoring for RRC_Connected	Xiaomi Communications	discussion

- R2-2408451	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion

- R2-2408490	LP-WUS in RRC Connected Mode	Lenovo	discussion	NR_LPWUS-Core

- R2-2408574	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2408605	LP-WUS operation in RRC_CONNECTED Mode	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

  - Withdrawn

- R2-2408692	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

- R2-2408711	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2408764	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2409052	Discussion on LP-WUS for CONNECTED state	NTT DOCOMO INC.	discussion	Rel-19

- R2-2409060	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2407398

- R2-2409160	LP-WUS operation in RRC_CONNECTED Mode	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

### RAN2#128

- R2-2410841	Discussion on LP-WUS for IDLE/INACTIVE state	NTT DOCOMO, INC.	discussion	Rel-19

- R2-2410085	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409058

- R2-2410606	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2409718	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2409989	On LR and MR operating frequencies and the answer to RAN1 LS 	VODAFONE,VIVO, Deutsche Telekom	discussion	Rel-19

- R2-2409990	DRAFT Reply LS to R2-2409157/R1-2407559	VODAFONE Group Plc	LS out	Rel-19	To:RAN Plenary, RAN1	Cc:RAN4

- R2-2410670	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2409949	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2410955	Minutes of offline discussion: [AT128][205][LPWUS] R2 impact analysis and draft reply LS on LP-WUS supporting in different bands	VODAFONE	discussion	Rel-19	NR_LPWUS-Core

- R2-2410956	DRAFT Reply LS to R2-2409157/R1-2407559	VODAFONE	discussion	Rel-19	NR_LPWUS-Core

- R2-2410957	Reply LS to R2-2409157/R1-2407559	VODAFONE	discussion	Rel-19	NR_LPWUS-Core

- R2-2409718	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2409761	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2409871	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2409902	LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2409921	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

- R2-2409924	LP-WUS operation in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2409949	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2410085	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409058

- R2-2410119	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2410166	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2410377	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2410412	Discussion on IDLE/INACTIVE procedures for LP-WUS	Tejas Network Limited	discussion	Rel-19

- R2-2410509	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2410555	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2410606	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2410632	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

- R2-2410670	Further considerations on LP-WUS operation in IDLE INACTIVE mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2410683	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2410730	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2410798	Procedure and Configuration of LP-WUS in RRC Idle/ Inactive	Lenovo	discussion	NR_LPWUS-Core

- R2-2410841	Discussion on LP-WUS for IDLE/INACTIVE state	NTT DOCOMO, INC.	discussion	Rel-19

- R2-2410858	On LP-WUS paging monitoring considerations	Nordic Semiconductor	discussion	Rel-19	Late

- R2-2409925	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2410732	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2409592	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2409592	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2409719	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2409762	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2409872	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

- R2-2409903	LP-WUS measurement relaxation and offloading 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2409925	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2409950	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2410086	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409059

- R2-2410120	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2410167	RRM measurement relaxation and offloading in RRC_IDLE and RRC_INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2410273	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

- R2-2410341	Discussion on RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2410378	Discussion on RRM aspects for  LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2410510	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2410556	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2410607	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2410633	Discussion on RRM measurement relaxation and offloading	Sharp	discussion	Rel-19

- R2-2410694	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	China Telecom	discussion

- R2-2410732	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2410405	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

- R2-2409883	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion

- R2-2409588	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

- R2-2409904	LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2410634	Discussion on LP-WUS in RRC_CONNECTED	Sharp	discussion	Rel-19

- R2-2409763	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2410168	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2409588	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

- R2-2409713	LP-WUS operation for RRC_CONNECTED Mode	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2409720	Analysis on LP-WUS for RRC_CONNECTED Mode	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2409763	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2409883	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion

- R2-2409904	LP-WUS in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2409951	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2410087	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2409060

- R2-2410099	LP-WUS in CONNECTED mode	China Telecom	discussion	Rel-19	NR_LPWUS-Core

- R2-2410121	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2410168	Procedures for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2410319	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2410352	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

- R2-2410379	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2410405	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

- R2-2410413	Discussion on CONNECTED mode procedures for LP-WUS	Tejas Network Limited	discussion	Rel-19

- R2-2410608	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

- R2-2410634	Discussion on LP-WUS in RRC_CONNECTED	Sharp	discussion	Rel-19

- R2-2410731	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2410814	LP-WUS in RRC Connected Mode	Lenovo	discussion	NR_LPWUS-Core

### RAN3#125bis

- R3-245166	Discussion on signaling design for LP-WUS subgrouping (vivo, NTT DOCOMO INC.)

- R3-245250	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE (Qualcomm Inc.)

- R3-245200	Discussion on Low-power wake-up signal and receiver for NR (CMCC)

- R3-245475	Discussion of LPWUS impacts on NG-RAN architecture (Ericsson)

- R3-245220	(TP for TS 38.413 and TS 38.423) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-245633	Discussion on LP-WUS subgrouping (Samsung)

- R3-245285	(TP to 38.413, 38.423, 38.473 on LP-WUS) Discussion on the support of LP-WUS indicating paging monitoring (NEC)

- R3-245167	BLCR for TS 38.300 on support of LP-WUS (vivo, NTT DOCOMO INC.)

- R3-245221	Introduction of low-power wake-up signal and receiver for NR (Huawei)

- R3-245234	Support of LP-WUS (Nokia)

- R3-245235	Support of LP-WUS (Nokia )

- R3-245338	Introduction of LP-WUS paging monitoring (Ericsson)

- R3-245302	(TP to TS 38.413, 38.473, 38.423) Discussion on RAN3 impacts for LP-WUS (ZTE Corporation)

- R3-245518	(TP to BL CR for TS 38.423) support of LP-WUS (CMCC)

- R3-245635	(TP to BLCR for TS 38.413, 38.423, 38.473) LP-WUS Assistance Information to support subgrouping (Samsung)

- R3-245303	[draft LS] to SA2/CT1 on support of LP-WUS subgrouping (ZTE Corporation)

- R3-245275	Potential RAN3 impacts of LP-WUS (CATT)

- R3-245276	TP on TS 38.300 for LP-WUS (CATT)

### RAN3#126

- R3-247256	Work plan for Rel-19 WI on LP-WUSWUR (vivo)

- R3-247007	LS on LP-WUS subgrouping (RAN2(vivo))

- R3-247257	Discussion on LP-WUS for NR (vivo, NTT DOCOMO INC.)

- R3-247669	Discussion on supporting LP-WUS Indicating Paging Monitoring (CMCC)

- R3-247579	Discussion of LPWUS impacts on NG-RAN architecture (Ericsson)

- R3-247145	Discussion on RAN3 impacts for LP-WUS (ZTE Corporation)

- R3-247636	Further discussion on LP-WUS subgrouping (NTT DOCOMO INC..)

- R3-247231	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE (Qualcomm Inc.)

- R3-247404	(TP for TS 38.413 and TS 38.423) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-247473	Potential RAN3 impacts of LP-WUS (CATT)

- R3-247474	[Draft] Reply to LS on LP-WUS subgrouping (CATT)

- R3-247493	Support of LP-WUS (Nokia )

- R3-247258	BLCR for TS 38.300 on support of LP-WUS (vivo, NTT DOCOMO INC.)

- R3-247501	Support of LP-WUS (Nokia )

- R3-247569	Introduction of LP-WUS paging monitoring (Ericsson)

- R3-247719	(TP to BLCR for TS 38.413, 38.423, 38.473) Discussion on LP-WUS subgrouping (Samsung)

- R3-247220	(TP to 38.413, 38.423, 38.473 on LP-WUS) LP-WUS RAN3 related aspect (NEC)

- R3-247146	(TP to TS 38.413, 38.473, 38.423) on support of LP-WUS (ZTE Corporation)

- R3-247637	(TP to TS 38.413 and 38.473) Further discussion on LP-WUS subgrouping (NTT DOCOMO INC..)

- R3-247670	(TP to BL CR for 38.473) support of LP-WUS (CMCC)

- R3-247405	Introduction of low-power wake-up signal and receiver for NR (Huawei)

### RAN4#112bis

- R4-2415007	(NR_LPWUS-Core) Simulation assumption and results for LP-WUR	CMCC

- R4-2415008	(NR_LPWUS-Core) Discussion on RRM impact of LP-WUR	CMCC

- R4-2415017	Discussion on LP-WUS BS RF requirements	CMCC

- R4-2415018	Discussion on LP-WUS UE system parameters requirements	CMCC

- R4-2415019	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2415020	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2415065	Simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2415066	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2415130	Further discussion on system parameters for LP-WUS	CATT

- R4-2415131	On Rx requirements of REFSENS, ASCS and ACS for LP-WUS	CATT

- R4-2415132	On Rx requirements of IBB, OBB, intermodulation for LP-WUS	CATT

- R4-2415133	Testability for UE RF requirements for LP-WUS	CATT

- R4-2415134	On BS RF requirements for LP-WUS	CATT

- R4-2415143	On simulation assumptions and results for LP-WUR	vivo

- R4-2415144	Further consideration on RRM requirements for LP-WUR	vivo

- R4-2415185	Discussion on RRM requirements for LP-WUS/WUR	CATT

- R4-2415201	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2415202	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2415203	Further consideration on UE RF testability issues for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2415204	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2415235	Topic summary for [112bis][131] NR_LPWUS_UERF	Moderator(Vivo)

- R4-2415363	Discussion on simulation assumptions and results for LP-WUS/WUR	LG Electronics Inc.

- R4-2415364	Discussion on RRM core requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2415454	BS RF requirement overview for LP-WUS	Ericsson

- R4-2415455	On general issues for WUR	Ericsson

- R4-2415456	On system paramter for WUR	Ericsson

- R4-2415457	On WUR RF requirement testability	Ericsson

- R4-2415458	WUR RF requirement other than REFSENS	Ericsson

- R4-2415459	WUR RF requirement REFSESN ASC ASCS	Ericsson

- R4-2415486	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2415524	On Low-power Wake-up Receiver for NR	Apple

- R4-2415541	On RRM core requirements for LP-WUR	Apple

- R4-2415548	On Simulation Assumptions and Results for LP-WUR RRM Core Requirements	Apple

- R4-2415549	Reply LS on LP-WUS operation in IDLE/INACTIVE mode	Apple

- R4-2415558	Discussion on UE RF requirements for LP-WUR	Samsung

- R4-2415559	Discussion on testability for LP-WUR	Samsung

- R4-2415582	Further views on requirements of the low-power wake-up receiver	Sony

- R4-2415610	Discussion on RX requirements of REFSENS for LP-WUR	Spreadtrum Communications

- R4-2415611	Discussion on systems parameters for LP-WUR	Spreadtrum Communications

- R4-2415664	Topic summary for [112bis][217] NR_LPWUS	Moderator (vivo)

- R4-2415692	RRM impact for Rel-19 LP-WUS/WUR	SAMSUNG R&D INSTITUTE JAPAN

- R4-2415706	System parameters for LP-WUR	Nokia

- R4-2415707	Fundamental RX requirements for LP-WUR	Nokia

- R4-2415708	Other RX requirements for LP-WUR	Nokia

- R4-2415709	Testability aspects of LP-WUR	Nokia

- R4-2415723	On UE Rx requirements for the LPWUR	Qualcomm Incorporated

- R4-2415738	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2415778	Skeleton for LP-WUS TR 38.774	vivo

- R4-2415779	Discussions on LP-WUS system parameters	vivo

- R4-2415780	Further discussions on LP-WUS REFSENS, ASCS and ACS	vivo

- R4-2415781	Discussions on LP-WUS IBB and OBB requirements	vivo

- R4-2415782	Discussions on LP-WUS Testability	vivo

- R4-2415783	Further discussions on LP-WUS BS RF requirements	vivo

- R4-2415803	Discussions on LP-WUS RAN1 LS	vivo

- R4-2415864	Simulation assumption and initial simulation results for LP-WUR measurement	Huawei, HiSilicon

- R4-2415865	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2415934	Discussions on RRM core requirements for LP-WUSWUR	NTT DOCOMO, INC.

- R4-2415937	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2415938	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2415939	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2415940	Discussion on testability of LP-WUR	ZTE Corporation, Sanechips

- R4-2416020	Discussion on LP-WUR RRM requirement	Ericsson

- R4-2416021	Simulation assumption on LP-WUR RRM measurement	Ericsson

- R4-2416022	Simulation results on LP-WUR RRM measurement	Ericsson

- R4-2416123	Discussion on RRM requirements for LP-WUS WUR	OPPO

- R4-2416196	Discussion on REFSENS and ASCS for the LP-WUS/WUR	LG Electronics France

- R4-2416290	Discussion on LP-WUS Simulation assumptions and results	Nokia

- R4-2416291	Discussion on LP-WUS RRM core requirements	Nokia

- R4-2416367	Discussion on LP-WUS for core part	ZTECorporation,Sanechips

- R4-2416378	LP-WUS ACS	OPPO

- R4-2416469	RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2416509	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2416510	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2416529	Topic summary for [112bis][308] NR_LPWUS	Moderator (Huawei)

- R4-2416560	Draft LS on NR NTN and GNSS concurrency	 Qualcomm

- R4-2416566	Way Forward for [112bis][308] NR_LPWUS	Huawei

- R4-2416589	Way Forward for [112bis][308] NR_LPWUS	Huawei

- R4-2416861	WF on RRM requirements for NR_LPWUS	Vivo

- R4-2416883	Coffee break discussion minutes for NR_LPWUS	Vivo

- R4-2417112	WF on UE RF requirements for LP-WUS	Vivo

### RAN4#113

- R4-2417543	BS RF requirements for low-power wake-up signal for NR	Nokia

- R4-2417630	On UE Rx requirements for the LPWUR	Qualcomm Incorporated

- R4-2417631	On test method for LPWUS	Qualcomm Incorporated

- R4-2417728	Discussion on RRM requirements for LP-WUS/WUR	CATT

- R4-2417759	On LP-WUR NF and REFSENS	Apple

- R4-2417860	Further discussion on BS RF requirements for LP-WUS	CATT

- R4-2417861	Further discussion on system parameters for LP-WUS	CATT

- R4-2417862	draftCR to TS 38.101-1 on system parameters for LP-WUS	CATT

- R4-2417863	draftCR to TS 38.104 on system parameters for LP-WUS	CATT

- R4-2417864	Further discussion on REFSENS, ASCS and ACS for LP-WUS	CATT

- R4-2417865	Further discussion on other RF requirements for LP-WUS	CATT

- R4-2417866	Further discussion on Testability for UE RF requirements for LP-WUS	CATT

- R4-2417867	TP to TR 38.774 on Clause 5 (System parameters)	CATT

- R4-2417868	TP to TR 38.774 on Clause 6 (Simulation assumptions and evaluation)	CATT

- R4-2417885	Further consideration on general aspects of LP-WUS	Huawei, HiSilicon

- R4-2417886	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2417887	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2417888	Further consideration on UE RF testability issues for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2417889	Further consideration on BS RF for Rel-19 LP-WUS	Huawei, HiSilicon

- R4-2417942	Simulation assumptions and results for LP-WUS/WUR	Xiaomi

- R4-2417943	Discussion on RRM core requirements for LP-WUS/WUR	Xiaomi

- R4-2418025	On LR and MR operating frequencies and the answers to RAN1 LS	Vodafone, vivo

- R4-2418036	Further views on requirements of the low-power wake-up receiver	Sony

- R4-2418107	Discussion on RRM core requirements for LP-WUS/WUR	LG Electronics Inc.

- R4-2418156	Topic summary for [113][134] NR_LPWUS_UERF	Moderator (vivo)

- R4-2418215	(Skeleton) TR 38.774 v0.0.1 for LP-WUS RF	vivo

- R4-2418216	TP to TR 38.774 on LP-WUS	vivo

- R4-2418217	Further discussions on LP-WUS REFSENS, ASCS and ACS	vivo

- R4-2418218	Discussions on LP-WUS Testability	vivo

- R4-2418219	Further discussions on LP-WUS BS RF requirements	vivo

- R4-2418234	TR 38.774 v0.1.0 LP-WUS	vivo

- R4-2418284	Topic summary for [113][226] NR_LPWUS	Moderator (vivo)

- R4-2418338	Discussion on UE RF requirements for LP-WUR	Samsung

- R4-2418339	Discussion on testability for LP-WUR	Samsung

- R4-2418391	Discussion for LP-WUR REFSENS requirement	LG Electronics France

- R4-2418412	Discussion on RX requirements of REFSENS for LP-WUR	Spreadtrum Communications

- R4-2418413	Discussion on systems parameters for LP-WUR	Spreadtrum Communications

- R4-2418432	Simulation assumption and results for LP-WUR	CMCC

- R4-2418433	Discussion on RRM impact of LP-WUR	CMCC

- R4-2418473	Discussion on RRM core requirements for LP-WUS/WUR	China Telecom

- R4-2418479	Discussion on LP-WUS for core part	ZTECorporation,Sanechips

- R4-2418483	Discussion on OOK based LP-WUR simulation	ZTECorporation,Sanechips

- R4-2418567	On RRM core requirements for LP-WUR	Apple

- R4-2418575	Reply LS on LP-WUS operation in IDLE/INACTIVE mode	Apple

- R4-2418596	On Simulation Assumptions for LP-WUR RRM Core Requirements	Apple

- R4-2418605	Views on LP-WUS BS RF requirements	Qualcomm Germany

- R4-2418734	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS	CMCC

- R4-2418735	Discussion on LP-WUS BS RF requirements	CMCC

- R4-2418736	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	CMCC

- R4-2418737	Discussion on LP-WUS UE system parameters requirements	CMCC

- R4-2418773	Simulation results for NR_LPWUS	OPPO

- R4-2418774	On RRM requirements for LP-WUS WUR	OPPO

- R4-2418827	Discussion on BS RF requirements for LP-WUS/WUR	ZTE Corporation, Sanechips

- R4-2418828	Discussion on REFSENS, ASCS, ACS for LP-WUR	ZTE Corporation, Sanechips

- R4-2418829	Discussion on receiver characteristics for LP-WUR	ZTE Corporation, Sanechips

- R4-2418830	Discussion on testability of LP-WUR	ZTE Corporation, Sanechips

- R4-2418838	Discussion on simulation assumptions and results for LP-WUS/WUR	LG Electronics Inc.

- R4-2418892	On RRM for LP-WUR	vivo

- R4-2418893	Consideration on simulation assumptions and results for LP-WUR	vivo

- R4-2418981	System parameters for LP-WUR	Nokia

- R4-2418982	Fundamental RX requirements for LP-WUR	Nokia

- R4-2418983	Other RX requirements for LP-WUR	Nokia

- R4-2418984	Testability aspects of LP-WUR	Nokia

- R4-2419005	Discussion on LP-WUS RRM requirement	Ericsson

- R4-2419006	Simulation assumption on LP-WUS RRM measurement	Ericsson

- R4-2419007	Simulation on LP-WUS RRM measurement	Ericsson

- R4-2419145	Simulation assumption and additional simulation results for LP-WUR measurement	Huawei, HiSilicon

- R4-2419146	Discussion on RRM requirements for LP-WUR	Huawei, HiSilicon

- R4-2419482	BS RF requirement overview for LP-WUS	Ericsson

- R4-2419483	On general issues for WUR	Ericsson

- R4-2419484	Tp for TR 38.774: System paramter for WUR	Ericsson

- R4-2419485	On WUR RF requirement testability	Ericsson

- R4-2419486	WUR RF requirement other than REFSENS	Ericsson

- R4-2419487	WUR RF requirement REFSESN ACS ASCS	Ericsson

- R4-2419595	Topic summary for [113][308] NR_LPWUS	Moderator (Huawei)

- R4-2419624	RRM impact for Rel-19 LP-WUS/WUR	SAMSUNG R&D INSTITUTE JAPAN

- R4-2419732	Discussion on LP-WUS-WUR Simulation assumptions and results	Nokia

- R4-2419733	Discussion on LP-WUS RRM core requirements	Nokia

- R4-2419740	Simulation assumptions for R19 LP-WUS	MediaTek inc.

- R4-2419741	Discussion on the RRM core requirements for LP-WUS	MediaTek inc.

- R4-2419764	RRM requirements for LP-WUR	Qualcomm Incorporated

- R4-2419799	Adhoc minutes for [113][308] NR_LPWUS	Moderator (Huawei)

- R4-2419802	Way Forward for [113][308] NR_LPWUS	Huawei

- R4-2420100	WF on RRM requirements for NR_LPWUS	Vivo

- R4-2420115	Reply LS on LP-WUS operation in IDLE/INACTIVE mode	Vodafone

- R4-2420117	Ad-hoc minutes for NR_LPWUS	Vivo

- R4-2420392	TP to TR 38.774 on Clause 5 (System parameters)	CATT

- R4-2420393	TP to TR 38.774 on LP-WUS	vivo

- R4-2420394	WF on UE RF requirements for LP-WUS	Vivo

- R4-2420395	Ad hoc minutes on UE RF requirements for LP-WUS	Vivo

- R4-2420528	WF on UE RF requirements for LP-WUS	Vivo

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template