3GPP TSG RAN Meeting #106																RP-242630

Madrid, Spain, December 9-12, 2024

Source:	vivo

Title:	Views on MIMO phase 6 in 5G-A Rel-20

Agenda Item:	15.2

Document for:	Discussion and decision

Introduction

MIMO technology has evolved in each release of NR specifications since Rel-15. Each release introduced some enhancement to improve overall throughput of UL and DL MIMO transmission. Rel-20 being almost the last release for NR 5G-A, few very focused objectives relevant to practical deployment can be considered. Robust UL transmission is an important issue, especially in mid to high speed scenario. The user experience is pretty bad in these scenarios due to unreliable UL performance. With multiple transmission antennas at the UE side, techniques to exploit them in order to enhance UL performance is beneficial for both the UE and the network. Another, potential enhancement is related to foldable phones, which is getting popular and widely available in the market. Due to restriction on firm factor design, the RF chains cannot cross the hinge. In order to support SRS antenna switching for asymmetric antenna design on foldable phones, SRS configuration for antenna switching can be enhanced.

In this contribution we discuss two potential small topics for MIMO phase 6 in Rel-20.

- Support of UL robust transmission

- Support of enhanced SRS antenna switching configuration

Support of UL Robust Transmission

To proliferate UL MIMO implementation in real deployment, there should be strong motivation for device vendors to put more components in the device, which definitely increases cost and complexity. Let’s take an example of 2Tx UL MIMO with non-coherent codebook, where there are 2 rank=1 precoders and 1 rank=2 precoder. In the non-coherent codebook subset, the rank=1 precoders are antenna selection precoders, i.e., [1  0] and [0  1], which means only 1 antenna will be used for UL transmission when gNB schedules PUSCH with rank=1. Rank=2 scheduling most likely happens in high SNR/low mobility scenario where the wireless channel is in better condition, however in real network we observed that rank=2 scheduling also happens in high speed scenario. Rank=2 transmission increases peak data rate, however from overall user experience point of view it is important to boost performance for rank=1 transmission especially in highspeed scenario. For example, UL intensive services like video conference, live streaming etc. robust transmission is important than peak throughput.

In the sub sections below, we provide system and link simulations for open loop precoder cycling in UL for 2Tx/4Tx UE, we observe significant gain at 5 percentile performance in both UMa and UMi with UE speed of 30/60/120 Km/hr compared to closed loop precoder indication with Rel-15 non-coherent codebook and Rel-16 non-coherent codebook with full power mode 0. Furthermore, the performance gain is not sensitive to SRS periodicity, which means less overall RS overhead. Evaluations on PAPR and comparison between PUSCH frequency hopping are also provided, we observe that due to large DMRS overhead PUSCH with intra-slot frequency hopping performs worst among the simulated schemes. For single layer transmission, it can be observed that performance of precoder cycling and small delay CDD are similar, however to support dynamic rank adaptation there will be additional SRS overhead for small delay CDD.

Whether there will be interference fluctuation introduced by open-loop precoder cycling is of concern, intuitively the interference in the cell is already random even without applying open-loop precoder cycling. The UEs in neighboring cells are randomly scheduled and number of occupied PRBs and position in frequency domain are random as well, hence the inter-cell interference is unpredictable in nature. Further applying open-loop precoder cycling per PRG will not change the interference condition in the cell drastically.

Evaluation on Performance gain

Simulations assumptions:

- UL: 2Tx non-coherent UE

- Open loop precoder cycling for Rank=1 transmission using 4 full-coherent precoders in 38.211

  - Baseline 0: Rel-15 non-coherent codebook, where the max output power is 3dB less than UE declared power class

  - Baseline 1: Rel-16 non-coherent codebook, full power mode 0, where the UE can deliver max output power as declared for given power class

| Parameter | Value |
| --- | --- |
| Frequency Range | 3GHz |
| Antenna setup and port layouts at gNB | (2,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 2TX: (1,1,2,1,1,1,2), (dH, dV) = (0.5, 0.5)λ |
| Receiver noise figure | 5dB |
| Modulation | Up to 64QAM |
| Numerology | 14 OFDM symbol slot |
|  | SCS 15KHz, 50 PRB |
| MIMO scheme | SU-MIMO, rank=1 |
| SRS | 2-port SRS, 10 and 80ms periodicity |
| PRG size | 2 PRBs |
| UE speed | 30/60/120 Km/h |

Simulation results

Figure 1, Precoder cycling performance gain against baseline 0 (Rel-15 non-coherent codebook)

Precoder cycling gain against baseline 1

Figure 2, Precoder cycling performance gain against baseline 1 (Rel-16 non-coherent codebook with full power mode 0)

Observation 1: from the figures 1 and 2, significant performance gain can be observed with open-loop precoder cycling, especially at 5 percentile UE SE, against closed-loop precoding with non-coherent assumption at the UE in high speed scenario. The performance gain is not sensitive to SRS periodicity; hence SRS overhead can be significantly reduced. Furthermore, SRS and DMRS are used as specified in the current specification without enhancement, MCS is determined based on received SRS and gNB applies precoder cycling for MCS determination.

Evaluation comparison with PUSCH frequency hopping

Simulation assumptions:

| Parameter | Value |
| --- | --- |
| Frequency Range, Waveform, SCS | Fc=4GHz， CP-OFDM, SCS=15KHz |
| Wireless Channel | TDL-C, low correlation, DS=250ns, 30km/h, UE 2/4Tx and gNB 4Rx |
| PUSCH code rate | 32PRB，64QAM and code rate=0.6 (w/o FH), No Retransmission |
| Type A DMRS | [3,11] without Intra-slot Frequency Hopping [2, 6] and [0, 4] with Intra-slot Frequency Hopping |
| Precoding scheme | FullPowerMode0; Small Delay-CDD; Precoding cycle with PRG=4 PRB |
| Receiver | Channel estimation: MMSE filter Equalizer: MMSE-based |

PRG= 4PRBs

slot

32 PRBs

16 PRBs

16 PRBs

Slot，intra-slot FH

Figure 3, DMRS configuration for with and without Frequency Hopping

Figure 3 shows DMRS configuration for both with and without intra-slot frequency hopping PUSCH for high speed scenario where additional DMRS is configured within a slot. For intra-slot frequency hopping PUSCH, there are 2 DMRS symbols on each hop within a slot, hence PUSCH code rate is higher compared to without frequency hopping for the same TB size. In the simulations below, the code rate for PUSCH without frequency hopping is 0.6 with 64QAM modulation.

Simulation results

- UL 2Tx, single layer PUSCH                                            b) UL 4Tx, single layer PUSCH

Figure 4, performance comparison with PUSCH intra-slot FH,

Figure 4 shows the simulation results for different schemes with single layer PUSCH transmission. In the figures, FullPowerMode0 is Rel-16 non-coherent codebook with full power mode 0, where the precoders for rank=1 are [1   0] and [0   1], however each antenna can deliver full power according to power class. For small delay CDD, 2Tx with [0, CP/6] delay and 4Tx with [0, CP/8, CP/4, 3CP/8] delays are applied on different antennas. For precoder cycling, length 4 DFT vectors are applied per PRG, where PRG size is 4 PRBs. For single layer transmission, it can be noticed that performance of precoder cycling and small delay CDD are similar, however to support dynamic rank adaptation there will be additional SRS overhead for small delay CDD. Furthermore, in the case of 4Tx, how to apply small delay CDD for rank=2 is not clear.

Observation 2: Since there is additional DMRS overhead for intra-slot frequency hopping, BLER performance is worst compared to Rel-16 UL full power mode 0, small delay CDD and precoder cycling. To support dynamic rank adaptation there will be additional SRS overhead for small delay CDD.

Evaluation on PAPR

Below we compare PAPR for the case with and without UL precoder cycling.

Figure 5, PAPR comparison between with and without open-loop UL Precoder cycling

Observation 3: figure 5 shows comparison of PAPR between the cases of with and without UL precoder cycling for CP-OFDM waveform, it can be observed that there is no PAPR degradation.

Evaluation on interference fluctuation

Figure 6, comparison of interference across all cells between precoder cycling and wideband scheduling

Observation 4: figure 6 shows comparison of interference fluctuation across all cells between precoder cycling and wideband scheduling, it can be noticed that the CDF curves are similar, hence there is no drastic change in interference due to precoder cycling in UL.

Proposal 1:

- It is proposed to support sub-band precoder with precoder cycling for UL transmission, specify necessary enhanced including definition of PRG (Precoding RB Granularity).

Support of Enhanced SRS antenna switching configuration

Various configurations of SRS antenna switching are supported in the NR specifications, however the SRS configuration is symmetric, that is, multiple SRS resources are configured with same number of ports and Y is divisible by X for XTYR antenna switching.

Foldable phones are getting popular in the market, however there are certain restrictions on the design, for example the RF chains cannot cross the hinge. The figure 7 shows the case of 4T8R SRS antenna switching where 2 SRS resources each with 4 ports are configured on different symbols according to current specification, where 4 ports of first SRS resource are mapped to antenna 1, 2 and 5, 6, and in next occasion 4 ports of second SRS resource will be switched to antenna 3, 4 and 7, 8 without RF chains crossing the hinge.

Figure 7, 4T8R antenna switching

However, 2T6R is already supported and 3T6R SRS antenna switching is introduced in Rel-19. In the case of foldable phone, as mentioned above, due to the restriction of hinge the SRS antenna switching is asymmetric. In current spec, 3 SRS resources each with 2 ports are configured for 2T6R antenna switching, however such configuration cannot work for the foldable phone antenna architecture as shown in figure 8 below. The reason for placing 4 antennas on one side is to support 4Rx when the phone is folded, meaning that antenna 5 and 6 are disabled when the phone is folded. In this case 2 SRS resources with 2 ports and 2 SRS resources with 1 port are needed for completing the full cycle of antenna switching. 2 SRS resources with 2 ports are mapped to antenna 1, 5 and 2, 6, and 2 SRS resources with 1 port are mapped to antenna 3 and 4 respectively.

2T6R

Figure 8, asymmetric SRS antenna switching in foldable phone

Proposal 2:

- It is proposed to support flexible configuration of SRS resources in order to support SRS antenna switching for foldable phones

Conclusions

In this contribution, we discuss two potential enhancements for NR MIMO Phase 6, which are closely related to real deployment and already available smartphones. The commercial values of these proposals are not only relevant to the UEs but also benefits the overall performance of the network.

Proposal 1:

- It is proposed to support sub-band precoder with precoder cycling for UL transmission, specify necessary enhanced including definition of PRG (Precoding RB Granularity).

Proposal 2:

- It is proposed to support flexible configuration of SRS resources in order to support SRS antenna switching for foldable phones