# 3GPP TSG RAN Meeting #106     	                          RP-2402806
Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.1.5

Source:	Apple

Title:	View on open issues of OD-SIB1 for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1 Introduction

In this contribution, we share our view on the following 2 open issues on on-demand SIB1:

- Whether Cell A can transmit its own UL WUS config

- Whether support OD-SIB1 for RRC_CONNECED UE

# 2 Discussion

## 2.1 Whether Cell A can transmit its own UL WUS config

This issue was first discussed in RAN1#118b (issue in FL summary R1-2409014 [1]), but there was no conclusion.

### FL Proposal 3-1 (in FL summary R1-2409014) RAN1 to discuss whether Cell A can also transmit the UL WUS configuration for itself. - E.g. For the case where Cell A may become a NES cell later. Omit table of company view OPPO: There may be a blocking issue out there. Not support. Xiaomi: If cell A becomes NES cell, the WUS configuration should be provided by another cell A. Nokia: Cell A is a coverage cell only; allowing Cell A transmit the UL WUS configuration for itself will complicate the scenario. Not support. ZTE: Out of scope of Case 2. Not support. Spreadtrum: The WID has the following definition for Cell A: A cell that is periodically transmitting at least its own SIB1. CMCC/ETRI/LGE/Fraunhofer/DCM/Lenovo: Left to RAN2 discussion.

Then, the same proposal was discussed in RAN2#128 [2], but still no conclusion can be made:

R2-2410842	Considerations on OD-SIB1 feature	Deutsche Telekom, Vodafone, Lenovo, Fraunhofer IIS   	discussion	Rel-19

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

Noted.

Discussion:

[IDC]: Specification will not specify cell A or NES cell, but since we already agreed NES cell can include its own WUS configuration, in principle agree with proposal 1. [Huawei]: Agree with IDC. [NTTDCM]: Having the clarification is good to other WGs since there are some confusion there.

[Qualcomm]: We should make sure the proposal 1 does not mean standalone case is supported. [Ericsson]: With the proposal, cell A needs to provide its own WUS configuration. [Nokia]: In addition, NES cell also needs to provide any neighboring cells’ WUS configuration (no restriction to neighboring cell’s NES cell). [Huawei]: We do not need to agree any further thing except what we already agreed. [Qualcomm]: Concerns on agreeing with the proposal. We already finished SI based on the terms and now we’re changing the terms. [Apple]: It was discussed in RAN1 and no conclusion was made. Would like to suggest to discuss in RAN.

We think the root cause of this issue is because it may be misunderstood as case 1 (i.e. standalone) which was precluded in study phase. Specifically, the highlighted wording (“it intends to”) may mean that Cell A and NES cell can be the same Node during some duration.

Proposal 1: Clarify that Cell A can also transmit the WUS configuration for itself in case it intends to become a SIB1-less cell.

We think it seems to be a misunderstanding on the intention of the proposal. Our understanding is:

- It is not case 1 (standalone) because Cell A and NES cell can’t be the same Node at the same time.

- As RAN2#127b [3] has agreed that NES cell can include its own and neighbor cells’ UL WUS config, no further specification work is required because the role switching between Cell A and NES cell is NW implementation and specification will not specify cell A or NES cell.

Agreements on OD-SIB1 (RAN2#127b [3])

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

If above clarifications can be agreed, we think it is unnecessary to update WID. Thus, we propose:

Proposal 1: On whether Cell A can transmit its own UL WUS config, no need to update WID if below clarifications can be agreed:

- It is not case 1 (standalone) because Cell A and NES cell can’t be the same Node at the same time.

- As RAN2#127b has agreed that NES cell can include its own and neighbor cells’ UL WUS config, no further specification work is required because the role switching between Cell A and NES cell is NW implementation and specification will not specify cell A or NES cell.

## 2.2 Whether support OD-SIB1 for RRC_CONNECED UE

R2-2409695	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to consider supporting SIB1 request in RRC_CONNECTED.

R2-2410303	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: Support RRC_CONNECTED UEs to trigger on-demand SIB1 acquisition by enhancing the legacy DedicatedSIBRequest or a new RRC message.

Discussion:

[Fujitsu, LG, ZTE]: It is not clear whether handling of RRC connected UEs is included in WID. [LG]: Understand the RRC connected UE can acquire SIB1 only when search space for SIB1 is configured. [CATT]: We can rely on option 2 in Samsung contribution, i.e. UE does not send SIB1 request. UE rely on network to provide the latest SIB1 via RRC message. Then we don’t need any spec change. [Huawei]: NW does not have any idea when the UE will trigger OD-SIB request or when the UE enters RLF. If we follow option2, unnecessary signalling overhead will be brought.

[LG]: Clarify previous LG’s comment was not to block any handling on RRC connected UEs. For SIB1 request, WID clearly indicates it is only for RRC idle/active, so SIB1 request is not applicable to RRC connected UE. However other options are still can be discussed/decided in RAN2. [Fujitsu]: Agree with LG.

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

This issue was discussed in RAN2#128 [2], but still no conclusion can be made:

We understand that there are two aspects of CONNECTED UE with OD-SIB1:

- RRC re-establishment after RLF: whether the UE can trigger OD-SIB1 acquisition in NES cells when T311 is running.

- According to section 5.3.7.1 of 38.331, the UE is in CONNECTED state when T311 is running during RRC re-establishment, although the procedure is similar to IDLE/INACTIVE UE.

- OD-SIB1 acquisition for CONNECTED UE in normal operation (i.e. not RLF case): whether/how the CONNECTED UE can acquire OD-SIB1 (e.g. similar to existing CONNECTED OD-SIB procedure via dedicated RRC message request).

- One solution is to extend existing DedicatedSIBRequest message (for OD-SIB in CONNECTED) to be used for OD-SIB1 request.

We tend to not extend WID scope of OD-SIB1 to RRC_CONNECTED UE because of below reasons:

- We think that the two issues are not essential issue, and Rel-19 NES TU is already limited.

  - For issue 1), if no specification change, the UE will first select Cell A during RRC re-establishment and then reselects to NES cell. Thus, the system can still work.

  - For issue 2), it can be addressed by NW implementation (e.g. by sending OD-SIB1via dedicated RRC message every 3 hours to the UE).

- The proposal is not aligned with agreement made in RAN2#127 [4]:

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

Thus, we propose:

Proposal 2: Keep the current WID scope of OD-SIB1 for UEs in idle/inactive mode.

# 3 Conclusion

In this contribution, we share our view on open issues of OD-SIB1 in Rel-19 NES. Our proposals are:

Proposal 1: On whether Cell A can transmit its own UL WUS config, no need to update WID if below clarifications can be agreed:

- It is not case 1 (standalone) because Cell A and NES cell can’t be the same Node at the same time.

- As RAN2#127b has agreed that NES cell can include its own and neighbor cells’ UL WUS config, no further specification work is required because the role switching between Cell A and NES cell is NW implementation and specification will not specify cell A or NES cell.

Proposal 2: Keep the current WID scope of OD-SIB1 for UEs in idle/inactive mode.

# 4 References

[1] R1-2409014	, FL summary 3 for on-demand SIB1 in idle/inactive mode, Moderator (MediaTek).

[2] RAN2#128, Chair Notes.

[3] RAN2#127b, Chair Notes.

[4] RAN2#127, Chair Notes.