3GPP TSG RAN WG1 Meeting #106								RP-242828

Madrid, Spain, December 9-12, 2024

Agenda item:		10.1.1

Source: 			Shanghai Jiao Tong University, NERC-DTV, PengCheng Laboratory

Title:	   Time Frequency Interleaver for LTE-based 5G Broadcast

Document for:		Discussion and Decision

1.Introduction

Time interleaving and frequency interleaving can be exploited to achieve performance gain to support diverse channel conditions. In this contribution we reference time interleaving from previous proposals and introduce frequency interleaving scheme based on cross-row block interleaver (CRBI) with well-designed cyclic shift[1]. The numerical results validate that our designed frequency interleaving scheme has better performance.

2.Time interleaving

Time interleaving is important to provides protection against rapid fading and impulsive noise. All modern broadcasting standards (such as DVB-T2, DTMB, ISDB-T, DAB/DAB+) include time interleaving as an important feature [2]. The benefits of time interleaving have been verified under 3GPP system in several contributions: [3][4][5][6].

Time interleaving in the high-speed case can normally obtain about 3dB BLER gain in TDL channels, and the gain will increase with the increase of interleaving depth and with the severity of the channel conditions. By introducing time interleaving, the performance of broadcast data receiving in mobile scenarios can be enhanced.

3.Frequency interleaving

The performance of system can be further improved by frequency interleaver. The intuitive way to apply the traditional row-column block interleaver is not a good choice from the perspective of minimum span (which determines performance). In this part we introduce a novel interleaver scheme named CRBI which has originally been proposed in R1-1913290 [7].

Figure 1 illustrates the basic idea of the CRBI. First, the input data are written column-wise into the interleaving memory with  rows and  columns. Second, the row indexes are reordered by

,.

where , , and the interval . Finally, the REs are read out by row-wise according to the new order of rows. The row reordering operation in the CRBI is based on two processes including equidistant increment (when ) and initiation reselection (when ).

Given an -length input data, the rationale behind the CRBI is to increase the minimum span [2] within every  cells as large as possible. As we all known, the minimum span of an  traditional row-column block interleaver (BI) is. For a CRBI, its minimum span can be given by

,

where denotes the minimum operation and  represents the minimum times of equidistant increment before initiation reselection. It can be seen that  decreases as  increases. Therefore the design of  aims to maximize the minimum between  and . Intuitively, we can observe that the CRBI largely improves the minimum span property of the interleaver, especially for big . This potentially offers significant performance gain.

Figure 1: Example demonstrating the CRBI.

The CRBI-based frequency interleaver (CRBI-FI) interleaves the REs from an OFDM symbol using the CRBI. In particular, the column number of the CRBI-FI can be given by , where  denotes the number of codeblock per OFDM symbol. If the number of effective subcarriers used for data cells transmission within an OFDM symbol, denoted by, is not an integer multiple of a codeblock,dummy cells are inserted to the end of the data vector before CRBI and deleted after CRBI, where . It is worth noting that the proposed CRBI-FI can perform not only inter-codeblock interleaving but intra-codeblock interleaving, even interleaving a fraction part of a codeblock.

Recently, the art of contemporary broadcasting has successfully migrated into single frequency networks (SFNs). The cellular communication sector has introduced SFN features into its multicast mode architecture, namely, multimedia broadcast/multicast service (MBMS) [8]. Such a concept known as multicast-broadcast SFN (MBSFN) has been adopted in the 3rd Generation Partnership Project (3GPP) standards since Rel-7 [9]. The 3GPP MBSFNs have long sought a low-power low-tower (LPLT)-based architecture for the sake of compatibility with legacy cells, but lately, the enablers are envisaging an integration of high towers as well to meet emerging requirements [10]. For SFN networks, the tapped delay line (TDL) channel is a commonly used descriptive method. For areas influenced by two SFN towers, it can be abstracted as a more extreme case, namely a two-tap channel, as shown in Figure 2.

Figure 2: Model of two-tap channel.

Figure 3: Signal of two-tap channel.

The signal at specific frequencies for the two-tap channel is shown in Figure 3. Due to the mutual cancellation of the signals from the two towers, fading occurs at specific frequencies. Depending on the distance between the two towers, this fading manifests as periodic deep fades.

Considering the periodic deep fades in frequency, a method of cyclically shifting the elements of each row has been introduced in R1-1913470[1]. Before the cross-row readout (after column-by-column writing), we shift the elements in recorded rows by certain shift offset S(Ri), where S(Ri) can be given by

SRi=modi2×-1i-1,N，i=1,2,3,…,M

where N is the number of columns and Ri is the recorded row indexes from section 2.1.

In this way, we can avoid the influence of periodic fading in frequency domain and still keep the minimum span within a large range. The minimum span after cyclic shifting is given by

​Smin_CRBI_shift≥min(d+1,bN-1)

For random cyclic shifts, when modSRi+SRi-1, N=N-1, its minimum span will definitely decrease by N-1. In the case of 1-(N-1N)M≈1, its probability can be considered as certain. Therefore, the proposed design of cyclic shifts in [1] has a significant advantage over purely random cyclic shifts.

The frequency interleaving method is described formally below in the following steps:

S1: The number of columns of the sub-block interleaver is N satisfying N=TBS6144-24;

S2: The number of rows of the sub-block interleaver is M=NscDL N, where NscDL is the number of data tones that are used for transmission of PMCH symbols;

S3: When NscDL is not an integer multiple of a codeblock, Ndummy dummy cells are inserted to the end of the data vector before CRBI and deleted after CRBI;

S4: The elements are then written into the M×N matrix column-by-column;

S5: After column-by-column writing, cyclic shifting with specific offset S(Ri) is added to each row based on the recorded row indexes Ri;

S6: Obtain the interleaved data after reading the elements of the above M×N matrix by cross-row way as mentioned in the CRBI.

Performance Evaluation:

The performance of frequency interleaving are evaluated for PMCH rooftop reception in MPMT scenarios. The simulation parameters are the same as those of PMCH frequency interleaving for rooftop reception in [11]. TABLE 1 shows the interleaver size and minimum span  of the row-column BI and CRBI-FI. Different from the row-column BI which has 9 and 15 columns [11], the CRBI-FI has 8 and 13 columns for TBS sizes of 45352 bits and 76208 bits respectively. That is, the column number of the CRBI-FI corresponds to the number of codeblock per TBS. It can be seen that the CRBI-FI has a much larger minimum span than the row-column BI in [11].

Figure 4 and Figure 5 show the BLER performance versus SNR for PMCH reception with and without frequency interleaving, where the row-column BI, CRBI and CRBI with cyclic shift are included for comparison of frequency interleaving performance [12]. The results verify the performance improvement by the use of frequency interleaving. Furthermore, the proposed CRBI-FI achieves a better BLER performance than the traditional row-column BI in [11].

TABLE 1: Minimum span for BIs with frequency interleaving

| TBS | Interleaver size M*N |  | Minimum span |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Row-col BI | CRBI-FI | Row-col BI | CRBI-FI | CRBI-SHIFT-FI |
| 45352 | 2475*9 | 2785*8 | 10 | 153 | ≥147 |
| 76208 | 1485*15 | 1714*13 | 16 | 155 | ≥144 |

Figure 4: BLER vs. SNR for PMCH numerology in tow-tap delay channel with 35µs of delay spread in a 10 MHz system bandwidth (TBS=76208).

Figure 5: BLER vs. SNR for PMCH numerology in tow-tap delay channel with 35µs of delay spread in a 10 MHz system bandwidth (TBS=45352).

Observation 1: The CRBI- FI exhibits much larger minimum span and achieves a better BLER performance than the traditional row-column interleaver.

Observation 2: The CRBI- FI with well-designed cyclic shit exhibits further performance gain than CRBI-FI without shift under two-tap delay channel.

Observation 3: The CRBI- FI with well-designed cyclic shift has an obvious advantage in minimum span over purely random cyclic shifts

Proposal 1: Time and Frequency Interleaving should be introduced to enhance the system performance.

Proposal 2: The CRBI-FI with well-designed cyclic shift should be specified to improve the performance under two-tap delay channel while ensuring a large minimum span.

4. Summary

Observation 1: The CRBI- FI exhibits much larger minimum span and achieves a better BLER performance than the traditional row-column interleaver.

Observation 2: The CRBI- FI with well-designed cyclic shit exhibits further performance gain than CRBI-FI without shift under two-tap delay channel.

Observation 3: The CRBI- FI with well-designed cyclic shift has an obvious advantage in minimum span over purely random cyclic shifts.

we would like to propose as follows:

Proposal 1: Time and Frequency Interleaving should be introduced to enhance the system performance.

Proposal 2: The CRBI-FI with well-designed cyclic shift should be specified to improve the performance under two-tap delay channel while ensuring a large minimum span.

References

- R1-1913470, “CRBI-based interleaver for 5G terrestrial broadcasting”, ABS, Shanghai Jiao Tong University, 3GPP RAN WG1, #99, Reno, USA, November 2019.

- R1-1913095 Information on Time Interleaving in State-of-the-Art Terrestrial Broadcast Systems, EBU/BBC/IRT/ABS/NHK

- R1-1912692 Numerology with 100us CP, Qualcomm Incorporated

- R1-1912331 CRBI-based interleaver for 5G terrestrial broadcasting, ABS,SARI-CAS,Shanghai Jiao Tong University

- R1-1913343 On the performance evaluation of HARQ-based time-interleaving for LTE-based 5G terrestrial broadcast, EBU, BBC, IRT

- R1-1910979 Time and Frequency Interleaver for LTE-based 5G Terrestrial Broadcast, EBU, BBC, IRT

- R1-1913290 CRBI-based interleaver for 5G terrestrial broadcasting, ABS, Shanghai Jiao Tong University

- Y. Zhang, D. He, Y. Xu, Y. Guan, and W. Zhang, “MBSFN or SC-PTM: How to efficiently multicast/broadcast,” IEEE Trans. Broadcast., vol. 67, no. 3, pp. 582–592, Sep. 2021.

- Introduction of the Multimedia Broadcast Multicast Service (MBMS) in the Radio Access Network (RAN); Stage 2 (Release 7), 3GPP Standard TS 25.346 V7.6.0, Dec. 2007.

- “Overall description of LTE-based 5G broadcast (release 16),” 3GPP , Sophia Antipolis, France, Rep. TR 36.976 V1.0.0, Dec. 2019.

- R1-1911355, “Time and frequency interleaver for LTE-based 5G terrestrial broadcast”, EBU, BBC, and IRT, 3GPP RAN WG1, #98bis, Chongqing, China, October 2019.

- H. Ju, Y. Xu, D. He and W. Zhang, "A Frequency Interleaver Scheme with Cyclic Shift for LTE-based 5G Terrestrial Broadcasting," 2021 IEEE International Symposium on Broadband Multimedia Systems and Broadcasting (BMSB), Chengdu, China, 2021.