3GPP TSG RAN Meeting #106	RP-242840

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.3.1.3

Source:	Apple

Title:	On AI/ML UE-sided Data Collection

Document for:	Discussion

# Introduction

In this contribution we provide our views about how to proceed with the UE-sided data collection objective for AI/ML.

# Discussion

The issue of UE-sided data collection was extensively discussed for many meetings in RAN2, with no consensus about which of the following options under discussion to proceed with:

- 1a: OTT 3GPP transparent

- 1b: based on EVEX

- 2: First termination entity in CN

- 3: First termination entity in OAM

The pros and cons, standards impact, security concerns, etc of all the options have been extensively analyzed (see, for instance, [1]) and we do not intend to repeat this here. For reference, the table summarizing all the options is provided in Appendix A.

Without repeating all the argumentation, we provided in [1], we’d like to reiterate that option 1a (i.e. OTT 3GPP transparent) works for all the use cases of Rel-19 AI/ML, it has been widely deployed in the industry, has no standardization impact and as such can (and in our view should) be the option to go forward with, especially if there is no consensus about whether the other options can satisfy the requirements in terms of privacy, etc.

Eventually, the discussion was escalated to RAN, and since no consensus was reached in RAN either, RAN#105 endorsed the following way forward (important parts highlighted in yellow) [6]:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

In summary, RAN#105:

- Confirmed that option 1a, which has no standards impacts, is not precluded

- Asked SA2 inputs on options 1b, 2, and 3 in order to proceed (as these solutions, if selected, need to meet certain agreed requirements)

Observation 1: RAN#105:

- Confirmed that option 1a, which has no standards impacts, is not precluded

- Identified a list of requirements options 1b, 2 and 3 must satisfy

- Asked SA WGs to provide feedback on whether options 1b, 2, and 3 satisfy the requirements identified

When analyzing the SA2 response (see below) and deciding how to proceed it is important to keep in mind the requirements agreed in RAN#105 and listed in [6].

In this meeting we’ve received the reply from SA2 [5] indicating:

- SA2 did not reach consensus on the feasibility, for any of the options outlined above, to meet RAN requirements specified in RP-242389 (S2-2411455) - A full evaluation of potential solutions for all UE side data collection options would require further study in SA2, and coordination with SA5, SA3, and RAN2

As there is no consensus in SA2, the question of whether options 1b, 2 and 3 satisfy the requirements agreed is still pending. This answer clearly doesn’t allow RAN2 to proceed with the UE-sided data collection objective, as there is no response from SA2 on the feasibility to meet the requirements agreed in RAN, let alone a date when such analysis would be done.

Observation 2: as of RAN#106 it is still very much unclear whether and how options 1b, 2 and 3 can satisfy the requirements agreed in RAN#105.

We further note that the reply from SA2 does not indicate that the study of such feasibility is ongoing or is about to be started. In fact, considering the load situation across all the multiple WGs that would need to be involved (SA2, SA5, SA3, and RAN2) it is very unlikely that such study can be accommodated in the current release, let alone concluded in time.

Observation 3: SA2 answer doesn’t allow RAN2 to proceed with the UE-sided data collection objective, as there is no indication from SA2 on the feasibility to meet the requirements agreed in RAN.

Additionally, SA3 provided their reply in [7], as follows:

After reviewing the initial RAN requirements on UE-side data collection for UE-side model sharing, below are SA3 inputs, including request for further clarifications: - SA3 needs clarification on the RAN security requirement “The data collected is secured and data integrity and confidentiality for that data is ensured”. SA3 would like to understand whether the collected data is to be secured at the time of collection (i.e. within the UE (storage)), and/or during communication. - SA3 will require more inputs on type of data to be collected, to better assess the sensitivity of the information conveyed by the data and to study appropriate solutions for the RAN (security) requirement on “User data privacy, anonymity and user consent is respected”. Once the final architecture decision/solution is made in RAN2/SA2, SA3 will perform a detailed security assessment and study on the security mechanisms for this feature.

Here we observe that, similarly to the reply from SA2, there are no answers from SA3 on whether the solutions 1b, 2 and 3 can meet the requirements – just follow up questions and in indication that more studies in SA are needed.

Observation 4: similarly to the reply from SA2, there are no answers from SA3 on whether the solutions 1b, 2 and 3 can meet the requirements.

Finally, and perhaps more importantly, it is clear that as of now:

- RAN and RAN2 cannot proceed with objective of UE-sided data collection

- There is no indication from SA and SA WGs whether such study is planned, let alone a collusion time

Considering the status in RAN2, the status in RAN and the replies from SA2, and SA3 we are left with the only two options:

- De-scope UE-sided data collection from the WID [3], or

- Proceed with option 1a

Proposal 1: RAN to select between the following two options:

- De-scope UE-sided data collection from the WID [3], or

- Proceed with option 1a

# Conclusion and Proposals

Observation 1: RAN#105:

- Confirmed that option 1a, which has no standards impacts, is not precluded

- Identified a list of requirements options 1b, 2 and 3 must satisfy

- Asked SA WGs to provide feedback on whether options 1b, 2, and 3 satisfy the requirements identified

Observation 2: as of RAN#106 it is still very much unclear whether and how options 1b, 2 and 3 can satisfy the requirements agreed in RAN#105.

Observation 3: SA2 answer doesn’t allow RAN2 to proceed with the UE-sided data collection objective, as there is no indication from SA2 on the feasibility to meet the requirements agreed in RAN.

Observation 4: similarly to the reply from SA2, there are no answers from SA3 on whether the solutions 1b, 2 and 3 can meet the requirements.

Proposal 1: RAN to select between the following two options:

- De-scope UE-sided data collection from the WID [3], or

- Proceed with option 1a

# Appendix A - Analysis of different data collection options for UE-side model training

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

# Appendix B – proposed revision of WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

### 4.1	Objective of SI or Core part WI or Testing part WI

Provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

- CSI feedback enhancement, encompassing [RAN1/RAN2]:

  - CSI prediction (UE-sided model):

    - Functionality-based LCM leveraged from other use cases, when necessary and applicable,

    - Study, and if necessary, specify consistency of training/inference,

    - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case.

  - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Study objectives:

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

NOTE: offline training is assumed for the purpose of this project.

NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference.

NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

# References

[1]	RP-241918, View on Rel-19 UE-sided data collection, Apple Inc.

[2]	RP-242389, LS on AIML data collection

[3]	RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[4]	RP-242351, Moderator's summary on offline discussion about RAN2 aspects on 	REL-19 WI NR_AIML_air

[5]	S2-2412726, LS RP-242389 (S2-2411455) LS on AIML data collection

[6]	RAN#105 meeting minutes

[7]	RP-243195, Reply LS to RP-242389 on AIML data collection