3GPP TSG RAN Meeting #106								RP-242850

Madrid, Spain, December 9-12th, 2024

Agenda Item:	9.3.1.3

Source:	Ericsson

Title:	Data collection for UE-side model training

Document for:	Discussion, Decision

# 1	Background

Last time, RAN plenary sent the LS in RP-242389 about UE side data collection for AIML.

In S2-2412726, SA2 says they need to further evaluate:

And in RP-243198, SA5 say they also need more evaluation:

Taking the above replies into account, we propose that plenary down-selects to UP-based solutions to speed up progress.

# 2	Discussion

The solutions on the table can be grouped into CP and UP solutions: Option 1b is only UP-based, while options 2 and 3 can be either CP and UP-based.

As shown above, SA2/SA5 claimed that they need to further study the feasibility of the options but, below we further analyse the solutions to do a down-selection to solutions requiring less specification efforts and network updates, while fulfilling the requirements.

First, it is important to note that for UE-sided AIML model generation, different devices with different hardware/software characteristic (e.g. devices of different vendors, or even devices of the same vendors but of different versions/releases) may need to carry out independent data collection sessions to generate their models. This is an important aspect to consider when deciding between a UP and CP based approach.

Solution 2 based on CP: According to the CR to TR38.843 (R2-2407807), NAS is considered the default CP protocol.  NAS signalling is designed to carry (small messages with) control data for connection management, UP sessions handling, etc. It is not designed to carry large amount of data that instead should be considered for the data collection procedure. This would not only require specification effort to extend the NAS, but also significant update at RAN to ensure coexistence with the legacy NAS signalling. RAN is responsible to transfer NAS messages between AMF and UE, and must stay in-sync between the UE and the CN, e.g. any NAS message that the RAN may not be able to deliver (e.g. due to overload) need to be handled by the RAN. However, today there is no mechanism for the RAN to notify the CN about undelivered UL NAS PDUs (only for the DL NAS PDUs it is possible for the NG-RAN node to notify the CN with non-delivery indication). This could be a problem because, considering the potentially high data rate and payload that training data for UE -side models may generate, the NG-RAN may not be unable to deliver all the UL NAS PDUs generated by the UE. Hence, mechanisms to enable the NG-RAN to notify the AMF of undelivered UL NAS PDUs may be needed.

Hence, we see that carrying AIML data over NAS is not suitable and would result in additional complexity which is not well justified.

Option 2 based on CP would require extending the NAS signalling which is not designed to carry large amount of payloads. Further, NG-RAN would need to be updated to cope with the scenario in which the UL NAS PDUs cannot be delivered due to NAS traffic overload.

Solution 3 based on CP: According to the CR to TR38.843 (R2-2407807) and to the SA5 LS reply, solution 3 based on CP may use the MDT framework to carry the collected data from the UE to the OAM. At RAN level, MDT leverages on the RRC signalling. Similar to NAS signalling, RRC is also not designed to carry large amount of data as required by the AIML use case. Hence, the burden in RRC layer will be large, and similar coexistence issues exist as for NAS (i.e. AIML RRC signalling vs. legacy RRC signalling need to share the UL).

Further, with option 3, all gNBs and RAN interfaces would need to be updated with the AIML-enhanced RRC protocol. Otherwise, the UE would not be able to perform data collection when the UE connects to those gNBs that do not support such enhanced RRC protocol. This would create an unjustified bottleneck in the data collection flexibility, especially considering that the data collected for the purpose of the UE-side model training are intended to be consumed by the training entity which trains the UE-side model, i.e. not by the gNB. To further exacerbate the problem, whenever 3GPP will introduce a new AIML use case in future releases and will enhance the specifications to carry new AIML data over RRC, all the gNBs would need again to be updated, otherwise the UE would not be able to carry out the data collection for the new AIML use case.

Hence, we see that carrying AIML data over RRC is not suitable and would result in additional complexity which is not well justified.

Option 3 based on CP would require extending the RRC signalling which is not designed to carry large amount of payloads (MDT was designed to collect data from few selected UEs only for observability purposes). Further, it presents non-negligible scalability issues that may hinder the feasibility of this option in a real-world deployment.

UP-based solutions (i.e. Option 1b and Option 2 UP): On the other hand, a UP solution does not present the above-mentioned issues, and the requirements of visibility and controllability can be preserved as for any CP solution.

Regarding controllability, the MNO can control the UP data collection process, as it would do for any other PDU sessions injected in the 3GPP system, e.g. the UE can trigger a UE initiated service request indicating that it needs to establish a PDU session for the purpose of the AIML data collection. The MNO can decide to allow/disallow the data transfer based load, policies, etc. Note: this traffic management task is not so straightforward in the CP-based solutions.

Regarding visibility, this can be achieved by SA WGs by standardizing that data should be visible and that should be included by the UE in the data collection. There are already examples in SA specifications in which specific UP data (e.g. packet loss rate, packet delay budget, UL/DL throughput, etc) can be collected by a Network Data Analytics Function (NWDAF) from UE Application(s) via an intermediary Application Function, such as the data collection application function (see TS 23.288 or TS 23.532 for further details). Hence, visibility is feasible for a UP-based solution.

Note: unlike the CP based solutions, UP based solutions not have the scalability issues since it would be just a function in the MNO network that would need to be updated.

The CP solutions do not offer any advantage compared with the UP solutions, and SA2 should focus on UP-based solutions (1b and the UP-version of 2) which definitely sound more appealing deployment-wise.

We propose:

RAN to ask SA2 prioritizing the study of UP-based solutions (i.e. 1b and UP-version of 2).

# 4	Conclusion

We made the following observations:

Observation 1	Option 2 based on CP would require extending the NAS signalling which is not designed to carry large amount of payloads. Further, NG-RAN would need to be updated to cope with the scenario in which the UL NAS PDUs cannot be delivered due to NAS traffic overload.

Observation 2	Option 3 based on CP would require extending the RRC signalling which is not designed to carry large amount of payloads (MDT was designed to collect data from few selected UEs only for observability purposes). Further, it presents non-negligible scalability issues that may hinder the feasibility of this option in a real-world deployment.

Given the observations, we propose the following:

Proposal 1	RAN to ask SA2 prioritizing the study of UP-based solutions (i.e. 1b and UP-version of 2).

In legacy, the MDT is only meant to collect data from few selected UEs (selected by the network) only for observability purposes, which is very different from the use case of data collection for UE-side model training. 
RAN2 has agreed to reuse (at least to a large extent) the MDT and the RRC for the NW-side data collection (for the gNB- and OAM-centric data collection respectively). However, in the NW-side data collection, the intention is to collect data from a limited number of UEs, just in order to generate a NW-side model that is able to operate under different NW configurations, or different radio conditions. The type of UE (e.g. the vendor) from which data are collected is not relevant in the context of NW-side data collection. That is instead very relevant in the context of UE-side data collection.