3GPP TSG RAN Plenary #106			RP-242856

Madrid, Spain, December 9-12, 2024

Agenda item:		9.1.1

Source:	China Telecom

Title:	Views on Ambient IoT WI for Rel-19

Document for:		Discussion

Introduction

In the last RAN working group meetings, the study on solutions for Ambient IoT (Internet of Things) in NR was completed, and the following was recommended in the TP of TR 38.769 [1-4].

- RAN1 RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2 - RAN2 From RAN2 perspective, RAN2 has studied Topology 1 and Topology 2, and has concluded they are feasible. In details: - General AS procedures, A-IoT paging, A-IoT random access, A-IoT data transmission and Topology 2 aspects have been studied: - See sub-clause 6.3.1 for the overall AS procedure and the information useful to be visible to the reader from CN, etc. - See sub-clause 6.3.3 for the A-IoT paging function, including paging message content, paging monitoring behavior, multiple (subsequent) A-IoT paging messages for the same service, etc. - See sub-clause 6.3.4) for the A-IoT random access, including the contention-free access and contention-based random access procedure, the message contents, the re-access solutions, etc. - See sub-clause 6.3.2 and 6.3.5 for protocol stack for A-IoT radio interface between A-IoT device and reader, required functionalities, and A-IoT data transmission functionalities, etc. - See sub-clause 6.3.6 for the Topology 2 specific aspects, including the architecture options and management of radio resources in A-IoT radio interface, etc. - From RAN2 perspective, the descriptions under clause 6.3 and 6.10 are considered as the baseline conclusions. - Following is further concluded: - At least following features are recommended for the normative phase in the A-IoT MAC layer: - A-IoT paging - A-IoT random access: - For contention-based random access, further down-selection between Solution 1 (3step-only) and Solution 3 (unified solution) in sub-clause 6.3.4. - A-IoT data transmission - For Topology 2, at least the following aspects are considered: - Forwarding of A-IoT upper layer data and A-IoT radio resource management. - No further down-selection for Topology 2 architecture options (which are feasible) from RAN2 perspective (the final decision depends on TSGs/WGs outcome). - RAN3 Logical system architectures have been studied, as documented in clause 6.4. Impacts on CN-RAN interface, signaling and procedures have been studied to support A-IoT Inventory operation and Command operation for topology 1 and topology 2 respectively, as documented in clause 6.5. Solutions for locating an Ambient IoT device with minimal specification impact are identified, as documented in clause 6.9. No evaluation and down selection for T2 solutions have been performed which requires coordination among WGs. - RAN4 RAN4 has performed adjacent channel co-existence studies between ambient IOT and NR for the scenario that NR macro BS outdoor and ambient IOT system indoor. RAN4 evaluated a total of 9 deployment scenarios as described in Table 6.6.2-1. For each deployment scenario, a total of 4 cases were performed as described in Table 6.6.2-1. The co-existence cases are described in Table 6.6.2-2. The performance metrics for NR system impacted by ambient IOT are throughput loss at the cell edge and cell average. The performance metric for ambient IOT system impacted by NR system is SINR degradation for inter-system interference impact analysis. Meanwhile, in addition to SINR degradation, ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which can be further analyzed in WI phase. The summary of co-existence evaluation can be found in section 6.6.5. RAN4 has studied the RF requirements impact for ambient IOT, including system parameters, ambient IoT BS, intermediate UE, devices (device 1, 2a and 2b) and CW node, which can be found in section 6.7.1, 6.7.2, 6.7.3 and 6.7.4. RAN4 has also studied the feasibility of CW interference cancellation for both ambient IoT BS and intermediate UE, and the feasible interference cancellation values can be found in section 6.7.5. Moreover, RAN4 also studied the testability issue for ambient IOT devices, which can be found in section 6.7.4.4.

In this contribution, we will provide our view on the potential next step work as well as the detailed scope based on the TR guidance.

Views on the checkpoint of Ambient IoT

Till now, the study for Rel-19 Ambient IoT has been completed, and a comprehensive technique report has been accomplished, which can provide guidance for the potential design method of Ambient IoT system. Based on the guidance in RAN#102, a checkpoint was set for RAN #106 meeting whether to convert Ambient IoT to WI or continue the study.

Based on the current market development situation, we think a subsequent WI is necessary for the next step. Firstly, the traditional passive communication technology such as RFID is still explosively growing in 2024. In China, the Cainiao Smart Logistics company is the largest company that will process tens of billions of packages yearly. At the same time, it cooperates with RFID vendors on pilot validation to improve the performance of package RFID tags. Based on the newspaper Beijing Business Today, they are attempting to inventory RFID identification rates from 70% to over 90%, which will cause a potential market loss for the ambient IoT market. Secondly, a potential competitor of IEEE is emerging. The normative work of IEEE Ambient Power Communication is in full swing, and the corresponding MAC and physical modifications based on 802.11 are discussed for battery-free devices that are powered by energy harvesting in sub-1 GHz and 2.4 GHz. It can be foreseen that the ambient IoT system will face a sharply competitive marketplace with competitors from multiple types of devices and standards organizations as time passes, thus establishing an earlier standard to capture the current market is of significant urgency.

Thus, we support to start a work item for Rel-19 Ambient IoT in NR.

Proposal 1: Support to start a work item for Rel-19 Ambient IoT in NR.

Potential scope of the Rel-19

General aspects of scenario and deployment

- Deployment

In the SI stage, RAN focuses on D1T1 and D2T2 to study their communication process and evaluate their coverage performance. The evaluation results show that D2T2 has a worse coverage value compared with D1T1, and the signaling overhead is also high due to the interaction process between gNB and the intermediate node, meanwhile the network architecture based on D2T2 will be more complex since the interface definition between the gNB and intermediate node may be different for the different intermediate node type, thus the D1T1 scenario has a better performance in the coverage performance and lower complexity in the system design principle compared with D2T2. Considering the workload and the potential limit TU for the potential work item arrangement, D1T1 should be prioritized, while D2T2 can be the second priority.

- Carrier frequency

Meanwhile, FR1 licensed spectrum in FDD is proposed in the SID general scope, and both 900MHz and 2GHz carrier frequencies are evaluated in the SI stage. According to the simulation results, 2GHz has a similar link level simulation performance in both R2D and D2R, at most 1dB performance loss can be observed and such a gap can be compensated by some enhancement solutions such as repetition or a high overhead midamble configuration. However, the coverage range of 2GHz is worse due to the natural transmission pathloss for the higher carrier frequencies, causing the result only half of the 900MHz coverage value. Therefore, for a better coverage performance guarantee, the target carrier frequency should be 900MHz.

- CW characteristic

In addition, the CW characteristic has also been studied in the SI phase, two CW waveform generation methods and some potential CW transmission cases are discussed based on performance and deployment characteristics. Based on the D2R reception performance at TDL-A 30ns delay spread, almost no performance gain can be observed in the 10% BLER target for the two unmodulated single-tones waveforms with a gap between [75KHz, 900KHz], and at most 2.5dB can be observed between [1.08MHz, 4.2MHz]. From this perspective, the added unmodulated tones in the CW waveform generation will not have a large performance gain for the D2R reception, but a larger deployment bandwidth and a higher complexity will be caused. Considering the allocated bandwidth for Ambient IoT systems may be not so large, for simplicity and low network implementation complexity, we think it’s better to focus on a single unmodulated tone in the CW generation method for Rel-19.

Besides, for the CW transmission cases, case 1-1 shows the best coverage performance among all the potential candidate solutions, the CW transmitted on the D spectrum has at least 10dB gain compared with the U spectrum and around 10m coverage gap can be observed. However, since a large frequency shift is not agreed to by device 1 and device 2a, the utilization of case 1-1 means all the R2D and D2R communication will happen on the D spectrum, resulting in a waste of U spectrum and adding the interference level for the D2R reception. From operator’s perspective, such spectrum waste is not acceptable for the deployment method. Therefore, we think the CW transmission case 1-1 is not supported in the potential work item scope.

- Device type

The device type shall be confirmed for the potential work item scope. Though both device 1 and device 2 are studied in the SI, the studied traffic type for those devices focuses on DT and DO-DTT, which means the backscatter communication is the main studied communication scenario. Therefore, even though device 2b has been studied, its communication method is assumed to be a backscatter method instead of active communication. Hence, device 1 and 2a can be regarded as the potential device type in the work item stage for simplicity and low workload.

Our view on the deployment scenario for the potential work item phase can be summarized in the following table:

Table 1 Potential WI scope for different characteristics

| use cases | Characteristics | Proposed WI scope | TR 38.769 in Rel-19 |
| --- | --- | --- | --- |
| Indoor inventory Indoor command | Deployment and topology | D1T1 with high priority, D2T2 with second priority | D1T1, D2T2 |
|  | Spectrum and carrier frequency | Licensed FDD, 900MHz | Licensed FDD |
|  | CW generation method | Single unmodulated single-tone | Single unmodulated single-tone or multiple unmodulated single-tones |
|  | CW transmission case | Case 1-2 or case 1-4 | Case 1-1, case 1-2 or case 1-4 |
|  | Device characteristic | Device 1 and Device 2a | Device 1 and Device 2 |

Proposal 2: Considering the following WI scope of scenario and deployment for Rel-19:

| use cases | Characteristics | Proposed WI scope | TR 38.769 in Rel-19 |
| --- | --- | --- | --- |
| Indoor inventory Indoor command | Deployment and topology | D1T1 with high priority, D2T2 with second priority | D1T1, D2T2 |
|  | Spectrum and carrier frequency | Licensed FDD, 900MHz | Licensed FDD |
|  | CW generation method | Single unmodulated single-tone | Single unmodulated single-tone or multiple unmodulated single-tones |
|  | CW transmission case | Case 1-2 or case 1-4 | Case 1-1, case 1-2 or case 1-4 |
|  | Device characteristic | Device 1 and Device 2a | Device 1 and Device 2 |

Views on the WI scope for Rel-19 Ambient IoT

In this section, we will provide our view on the potential work item scope related to different RAN groups.

- Bandwidth and numerologies

In the SI stage, the system parameters such as bandwidth size, sub-carrier space and the CP related content of Ambient IoT systems have been discussed. With respect to the evaluation assumptions, the system bandwidth is assumed as 180kHz in the downlink, the sub-carrier space in the downlink is 15KHz, transmission bandwidth is assumed as 15KHz in the uplink and other values are not precluded in the SI discussion. Since system design is fundamental to the Ambient IoT normative work, system design parameters shall be determined first for the subsequent work. Therefore, the bandwidth and numerologies parameter need to be specified in the Rel-19 WI stage.

- Physical channel signal design

In the SI stage, the generation process of PRDCH and PDRCH has been studied, and a rough generation process based on PRDCH or PDRCH signal is also generated. In the PRDCH channel, R2D signals are regarded to be generated based on OFDM-based OOK waveform for a normal R2D control or data information, and a generation figure is also agreed in the SI stage, but the detailed generation process and generation steps have not been determined yet. In the work item stage, such steps shall be clarified and specified in the potential protocols, such as the modulation process, line coding, CRC attachment, etc. Considering that the OOK-1 or OOK-4 generation process has been specified for LP-WUS, it can provide reference for Ambient IoT PRDCH generation process. Similarly, in the PDRCH channel, the potential modulation method is OOK, BPSK, or MSK. Considering the MSK modulation will have a larger bandwidth and is more affected by the SFO, such a modulation method can be a low priority in the first work item stage. For the BPSK or OOK modulation, since only the generation figure and the bandwidth figure are illustrated in the SI stage, the detailed normative work shall be done in the work item stage, which may include modulation process, line coding, FEC coding, or CRC attachment.

- Synchronization signal design

In the SI stage, the R2D or D2R synchronization signals are also studied. Based on the SI study, preamble is necessary and beneficial for the potential R2D or D2R transmission, and the potential preamble structure is confirmed different for R2D and D2R. In the R2D transmission, the preamble may consist of 3 potential parts, the first one is the start indicator part to indicate the start of the R2D transmission, then a clock acquisition part is followed to provide at least chip synchronization for the subsequent PRDCH transmission. Meanwhile, considering R2D calibration function is also considered in the device architecture, a potential clock calibration may also be considered in the preamble part. Since the components and preliminary design are already agreed upon in the SI stage, they can be further refined and specified in the potential WI stage. In the D2R transmission, the preamble part is designed as a binary sequence due to the device capability limitation, and the detailed sequence design or generation method can be further specified in the WI stage.

Besides, midamble and postamble are also studied in the SI stage, and at least it has been observed that midamble can provide performance benefits on the D2R reception, especially for the large payload size. Therefore, at least the midamble can be further discussed and specified in the WI stage, such as the generation method, sequence design or the overhead in the D2R transmission. For the postamble, if it is confirmed necessary in the D2R transmission, it can also be specified in the potential WI stage.

- Channel structure design.

In the SI stage, the information carried in the PRDCH and PDRCH has also been discussed. In the PRDCH, higher layer information or L1 control information may be carried in that channel. Considering they are different types of information bits, the PRDCH channel may use separate fields to distinguish them. Moreover, considering different control information may have different functionalities, the ‘field’ concept may be more detailed for those functionalities, such as the ‘length field’, and ‘command type field’, and the whole PRDCH channel may be composed of several fields. Hence, the PRDCH channel structure needs to be specified in the WI stage, like the DCI format design in the NR system.

For the PDRCH channel, considering the feasibility of D2R control information is not confirmed, the content in the PDRCH channel can be just regarded as data information, and no more channel design is needed. If D2R control information can be agreed upon in the WI stage, a specific field can be reserved in the PDRCH for such information.

- Procedures, protocol and resource allocation related design.

During the SI stage, several timeline-related parameters have been discussed to define the time interval between R2D transmission and D2R transmission, and the related processes such as paging, access are also studied. Considering the timeline value is the basic element for the transmission and reception process of the paging, access, or other procedure, the detailed timeline value and its timeline definition shall be specified.

Besides, the specific functionalities for the Ambient IoT (A-IoT) paging function, paging message content, paging monitoring behaviours, multiple (subsequent) A-IoT paging messages for the same service, etc. have been discussed in the SI stage as well. For the A-IoT random access, the CFRA and CBRA procedure, the message contents, the resource allocation method, and the re-access solutions are the key points of discussion. Considering the multiple access process may be more complex compared with other aspects, limited by the workload and TU, we may focus on the TDMA method on the WI stage first, and further study or specify the other multiple access method in the subsequent release.

In addition, aspects of the overall AS procedure, visible information to the reader from CN, the protocol stack for the A-IoT radio interface between the A-IoT device and reader, required functionalities, A-IoT data transmission functionalities, etc. have also been discussed in RAN2. Since such contents are necessary for the devices to receive the Reader’s scheduling, understand the scheduling, and behave based on the scheduling, they shall be specified in the WI stage as well. For the Topology 2 specific aspects, RAN2 focuses on the architecture options and management of radio resources in the A-IoT radio interface. If Topology 2 is in the work item scope, forwarding of A-IoT upper layer data and A-IoT radio resource management should be specified, and any additional impacts from other WGs will also be discussed in the WI phase.

- RAN Architecture

Logical system architectures are also studied in the SI stage, including indoor deployment scenarios with Topology 1 and outdoor deployment scenarios with Topology 2, as shown below.

Figure 1 Logical system architecture for topology 1

Figure 2 Logical system architecture for topology 2

If TU allows, both T1 and T2 can be considered in the R19 WI, but T1 shall be a first priority. In addition, whether “XXAP” is realized by including AIoTF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision in the RAN #106 meeting.

- RAN-CN Interface Impact

RAN3 has studied the impact on RAN-CN interface, including necessary parameters and procedures to support A-IoT Inventory operation and Command operation for T1 and T2 respectively. Detailed signaling design needs to be completed in the WI phase.

- IoT Device Locating

Solutions for locating an Ambient IoT device with "Reader ID granularity" positioning accuracy is studied in SI. The A-IoT RAN node may report the RAN node ID or Reader ID to the A-IoT CN in the Inventory Report or Command Response messages. For T2, the location of the A-IoT enabled UE can be obtained via existing procedures such as ULI and MCS mechanisms.

Consequently, our view on the potential work item phase for Ambient IoT can be summarized as follows:

RAN1-led:

- Define the parameter and value for the bandwidth, numerologies and other aspects (if needed).

- Specify the design of PRDCH and PDRCH channel

  - Specify the OOK modulation, line coding, and CRC attachment for the PRDCH channel

  - Specify the modulation method (OOK or BPSK), line coding, FEC coding, CRC attachment, and repetition process for the PDRCH channel

  - Control information related structure design

- Specify the signal for R2D transmission and D2R transmission

  - Design the generation and functionalities of preamble, midamble (if needed) and postamble (if needed)

RAN2-led:

- Specify signaling and procedures for A-IoT paging, random access, and data transmission.

- Specify the A-IoT MAC layer protocol.

- Specify signaling and resource allocation for Topology 2, if needed.

RAN3-led:

- Specify RAN architecture, signaling, and procedures for the NG interface to support A-IoT operation.

- Specify A-IoT-enabled UE authorization and solutions for Topology 2, if needed.

  - Note: Coordination with SA2 is needed on the detailed requirements, architecture, and solutions.

Hence, we have the following proposal.

Proposal 3: Considering the following scope for the Rel-19 Ambient IoT work item:

RAN1-led:

- Define the parameter and value for the bandwidth, numerologies and other aspects (if needed).

- Specify the design of PRDCH and PDRCH channel

  - Specify the OOK modulation, line coding, and CRC attachment for the PRDCH channel

  - Specify the modulation method (OOK or BPSK), line coding, FEC coding, CRC attachment, and repetition process for the PDRCH channel

  - Control information related structure design

- Specify the signal for R2D transmission and D2R transmission

  - Design the generation and functionalities of preamble, midamble (if needed) and postamble (if needed)

RAN2-led:

- Specify signaling and procedures for A-IoT paging, random access, and data transmission.

- Specify the A-IoT MAC layer protocol.

- Specify signaling and resource allocation for Topology 2, if needed.

RAN3-led:

- Specify RAN architecture, signaling, and procedures for the NG interface to support A-IoT operation.

- Specify A-IoT-enabled UE authorization and solutions for Topology 2, if needed.

  - Note: Coordination with SA2 is needed on the detailed requirements, architecture, and solutions.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: Support to start a work item for Rel-19 Ambient IoT in NR.

Proposal 2: Considering the following WI scope of scenario and deployment for Rel-19:

| use cases | Characteristics | Proposed WI scope | TR 38.769 in Rel-19 |
| --- | --- | --- | --- |
| Indoor inventory Indoor command | Deployment and topology | D1T1 with high priority, D2T2 with second priority | D1T1, D2T2 |
|  | Spectrum and carrier frequency | Licensed FDD, 900MHz | Licensed FDD |
|  | CW generation method | Single unmodulated single-tone | Single unmodulated single-tone or multiple unmodulated single-tones |
|  | CW transmission case | Case 1-2 or case 1-4 | Case 1-1, case 1-2 or case 1-4 |
|  | Device characteristic | Device 1 and Device 2a | Device 1 and Device 2 |

Proposal 3: Considering the following scope for the Rel-19 Ambient IoT work item:

RAN1-led:

- Define the parameter and value for the bandwidth, numerologies and other aspects (if needed).

- Specify the design of PRDCH and PDRCH channel

  - Specify the OOK modulation, line coding, and CRC attachment for the PRDCH channel

  - Specify the modulation method (OOK or BPSK), line coding, FEC coding, CRC attachment, and repetition process for the PDRCH channel

  - Control information related structure design

- Specify the signal for R2D transmission and D2R transmission

  - Design the generation and functionalities of preamble, midamble (if needed) and postamble (if needed)

RAN2-led:

- Specify signaling and procedures for A-IoT paging, random access, and data transmission.

- Specify the A-IoT MAC layer protocol.

- Specify signaling and resource allocation for Topology 2, if needed.

RAN3-led:

- Specify RAN architecture, signaling, and procedures for the NG interface to support A-IoT operation.

- Specify A-IoT-enabled UE authorization and solutions for Topology 2, if needed.

  - Note: Coordination with SA2 is needed on the detailed requirements, architecture, and solutions.

- References

- R1-2410704, Feature Lead Summary #2 for Clause 8 of TR 38.769, 3GPP TSG RAN-WG1 Meeting #119, Orlando, USA, November 18-22, 2024.

- R2-2411221, TP for TR 38.769 update, 3GPP TSG-RAN WG2 Meeting #128, Orlando, USA, 18th – 22nd, November, 2024.

- R3-247924, (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, 3GPP TSG-RAN WG3 Meeting #126, Orlando, US, 18 - 22 Nov, 2024.

- R4-2420372, TP to TR38.769: RAN4 part for Ambient-IoT, 3GPP TSG-RAN4 Meeting #113, Orlando, USA, 18th – 22nd November, 2024.