3GPP TSG-RAN WG1 Meeting # 118bis	R1-2409322

# Hefei, China,  October 14th - October 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1446 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to higher-layers parameters for IoT (Internet of Things) NTN (non-terrestrial network) enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Align higher-layer parameter names to match the ASN.1 according to RAN2 LS R1-2407592(R2-2407827) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the following higher-layer parameter names: - downlinkHARQ-FeedbackDisabled-Bitmap to downlinkHARQ-FeedbackDisabledBitmap - downlinkHARQ-FeedbackDisabled-DCI to downlinkHARQ-FeedbackDisabledDCI - downlinkHARQ-FeedbackDisabled-Bitmap-NB to downlinkHARQ-FeedbackDisabledBitmap-NB - downlinkHARQ-FeedbackDisabled-DCI-NB to downlinkHARQ-FeedbackDisabledDCI-NB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsitency on higher-layer parameter names between RAN1 TS 36.213 and RAN2 specifcations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.3, 10.2, 16.4.2, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.1	UE procedure for receiving the physical downlink shared channel

<Unchanged parts are omitted>

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A, 6-1B, 6-2 intended for the UE, decode the corresponding PDSCH in one more BL/CE DL subframes as described in Clause 7.1.11, with the restriction of the number of transport blocks defined in the higher layers.

For a BL/CE UE in a NTN FDD serving cell with a PDSCH ending in subframe n, and the UE configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap or higher layer parameter downlinkHARQ-FeedbackDisabledDCI, if the UE shall not provide HARQ-ACK for a HARQ process associated with a transport block in the PDSCH, the UE is not expected to receive a MPDCCH or a PDSCH without a corresponding MPDCCH for the same HARQ process as the PDSCH ending in subframe n in any BL/CE DL subframe starting from subframe n+1 to subframe n+3.

For the purpose of decoding PDSCH containing SystemInformationBlockType2, a BL/CE UE shall assume that subframes in which SystemInformationBlockType2 is scheduled are non-MBSFN subframes.

<Unchanged parts are omitted>

## 7.3	UE procedure for reporting HARQ-ACK

If the UE is not configured with shortTTI, the term 'subframe/slot' refers to a subframe in this clause.

<Unchanged parts are omitted>

For a BL/CE UE in a NTN FDD serving cell, and the UE not configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating enabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, the UE shall provide HARQ-ACK for the HARQ process associated with the transport block.

For a BL/CE UE in a NTN FDD serving cell, and the UE configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, the UE shall provide HARQ-ACK for a HARQ process associated with a transport block in a detected PDSCH

-	if the UE is configured with CEModeA, and configured with higher layer parameter harq-FeedbackEnablingforSPSactive = 'enabled', and the detected PDSCH is the first SPS PDSCH after SPS activation.

For a BL/CE UE in a NTN FDD serving cell, and the UE configured with CEModeB and higher layer parameter downlinkHARQ-FeedbackDisabledDCI, the UE shall provide HARQ-ACK for a HARQ process associated with a transport block in a detected PDSCH

-	if the HARQ-ACK Resource offset field does not function as HARQ feedback disabled indicator as specified in [4] in DCI format 6-1B in the MPDCCH corresponding to the PDSCH.

For a BL/CE UE in half-duplex FDD operation in a NTN serving cell, if the UE is configured with CEModeA, and configured with higher layer parameter ce-HARQ-AckBundling, and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, the UE is not expected to receive the corresponding DCI with HARQ-ACK bundling flag set to 1.

For a BL/CE UE, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,

-	for the UE in a NTN FDD serving cell, if the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block of the multiple TB, and if the UE shall provide HARQ-ACK for at least one TB of the multiple TB, the UE shall generate an ACK for HARQ-ACK corresponding to the transport block associated with the HARQ process with disabled HARQ-ACK information;

-	for HARQ-ACK transmission associated with the corresponding DCI, the UE shall generate M HARQ-ACK bits by performing a logical AND operation of HARQ-ACKs across all TBs in each TB bundle Ab where b = 1, …, M;

-	the set of TBs that belong to TB bundle Ab and the number of TB bundles M are given by Table 7.3-1;

-	the value of is the number of scheduled TB determined in the corresponding DCI.

<Unchanged parts are omitted>

## 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, "UL/DL configuration" of the serving cell in Clause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

<Unchanged parts are omitted>

For FDD, if a BL/CE UE is not configured with higher layer parameter harq-AckBundling in ce-PDSCH-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Clause 10.1.2.1 in subframe(s) sb+ki+Koffset with b=0,1,⋯NTB-1,  i =0,1, …, N-1, where

-	if the UE is in a NTN serving cell and the UE is configured with CEModeA and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH, or

-	if the UE is in a NTN serving cell and the UE is configured with CEModeB and not configured with higher layer parameter downlinkHARQ-FeedbackDisabled-DCI and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the PDSCH,

-	 is the number of scheduled TB associated with HARQ processes with enabled HARQ-ACK information and with TB indices in increasing order denoted by t0,t1,t2, …tNTB-1;

-	otherwise

-	is the number of scheduled TB determined in the corresponding DCI, and tb=b;

-	if the UE is not configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config and the UE is not in half-duplex FDD operation

-	s0=n0+4, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	otherwise

-	s0=max n0+4,  nL+2, sb=max {nb+4,  sb-1+Nb-1}, b≠0

-	nb is the last subframe in which the PDSCH containing TB tb is transmitted;

-	subframe nL is the last subframe in which the PDSCH is transmitted;

-	Nb denotes the number of consecutive subframes including non-BL/CE subframes where the PUCCH with HARQ ACK for TB tb with repetition number of N is transmitted;

and

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) sb+ki+Koffset with i=0,1,…,N-1 for TB tb are N consecutive BL/CE UL subframe(s) immediately after subframe sb-1+Koffset, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

<Unchanged parts are omitted>

### 16.4.2	UE procedure for reporting ACK/NACK

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	 DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, or if the UE is in a NTN serving cell and multiple TB are scheduled in the NPDCCH corresponding to the NPDSCH and the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI-NB - and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap-NB indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the NPDSCH, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

-	for TDD, .

-	For

-	if the UE is configured with higher layer parameter harq-AckBundling in npdsch-MultiTB-Config, and the NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI,

-	if the UE is in a NTN serving cell and if the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI-NB and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap-NB indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in the NPDSCH, the UE shall generate an ACK for HARQ-ACK corresponding to the transport block

-	the ACK/NACK response is generated by performing a logical AND operation of HARQ-ACKs corresponding to the TBr+1 ,

-	otherwise,

-	if

-	the ACK/NACK response is the HARQ-ACK corresponding to the transport block associated with the HARQ process with enabled HARQ-ACK information

-	otherwise

-	NB-IoT UL slots  with  of the NPUSCH carry ACK/NACK response for TBr+1 ,

except if the UE is in a NTN serving cell, and the UE is not configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI-NB and configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap-NB indicating disabled HARQ-ACK information for all HARQ process(es) associated with transport block(s) in the NPDSCH, or the HARQ-ACK Resource field functions as HARQ feedback disabled indicator in DCI format N1 as specified in [4] in the NPDCCH corresponding to the NPDSCH.

<Unchanged parts are omitted>

## 16.6	Narrowband physical downlink control channel related procedures

Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0.

<Unchanged parts are omitted>

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3, or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’ for the same HARQ process ID, or if the NPUSCH transmission carries ACK/NACK response, as determined in clause 16.4.2, for the same HARQ process ID associated with a transport block scheduled in a NPDCCH scheduling a single transport block, and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap-NB indicating disabled HARQ-ACK information for the same HARQ process ID and configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI-NB;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3] for FDD; and

-	the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 except if the UE is configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’, or if the NPUSCH transmission carries ACK/NACK response as determined in clause 16.4.2 and the UE is configured with higher layer parameter downlinkHARQ-FeedbackDisabledBitmap-NB indicating disabled HARQ-ACK information and configured with higher layer parameter downlinkHARQ-FeedbackDisabledDCI-NB.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

<Unchanged parts are omitted>