3GPP TSG RAN Meeting #106								       RP-242965

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name | New SID: Study on solutions for Ambient IoT (Internet of Things) in NR |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: | Performance part: | Testing part: |
| Acronym | FS_Ambient_IoT_solutions |  |  |  |
| Unique ID | 1020085 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240826 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2024 (No change) | Core part: | Performance part: | Testing part: |
| Overall Completion level | Study Item: 100 % | Core part: | Performance Part: | Testing part: |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Shen |
|  | Company | CMCC |
|  | Email | shenxiaodong@chinamobile.com |
|  | Name | Matthew Webb |
|  | Company | Huawei |
|  | Email | matthew.webb@huawei.com |
|  | Name | John Humbert |
|  | Company | T-Mobile USA |
|  | Email | John.Humbert2@T-Mobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#118bis

Agreement

From RAN1 perspective, there is no lower bound on the minimum TB size for R2D and D2R directions.

Agreement

From RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported.

- How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.

Agreement

The draft LS in R1-2409249 is endorsed. Final LS in R1-2409250.

RAN1#119

Agreement

TP#1 to TP#5 (and sub-TPs) below are endorsed for the conclusion section of TR38.769

TP#1

It is feasible to support a harmonized air interface design, with minimized differences (where necessary) for Ambient IoT, to enable the devices described in Clause 3 for addressing the objectives stated in [2]. The extent to which maximum distance targets, differentiated per device and deployment/topology scenario, as described in Clause 4.1, can be met under different sets of evaluation assumptions is reported in Clause 7.1. A latency target for inventory of a single device in TR 38.848, can be met under different evaluation assumptions according to evaluations reported in Clause 7.2.1. The addressed subset of design targets described in TR 38.848 can be met or taken into account in feasible designs, and the design targets left to WGs in Clause 5 of TR 38.848 have been refined. Evaluations have also reported the inventory completion time for multiple devices. Corresponding evaluation scenarios, assumptions, and methodologies have been developed.

TP#2

The basic blocks of A-IoT device architectures based on RF-ED receiver for device 1, 2a, and 2b have been identified, and also for receiver architectures based on IF-ED or ZIF for device 2b. For device 2a, the characteristics of blocks representing reflection amplifier(s) and large frequency shifter have been studied. Aspects to consider further regarding whether the large frequency shifter block is feasible, and its necessity or effectiveness have also been identified.

Purposes for which clocks(s) may be used by the different A-IoT devices 1/2a/2b have been identified, together with descriptions of the corresponding power consumption, and clock accuracy characteristics.

Sub-TP #3.1 v2

Regarding the potential impact of device (un)availability due to energy harvesting, two 'Directions' were found, depending on whether the reader does not, or can, provide information to a device based on which the device may become available/unavailable. Company solutions are reported in Clause 6.2, grouped according to the Direction they apply to.

Sub-TP #3.2 v2

The physical channels studied are PRDCH in R2D, and PDRCH in D2R, for both of which the generation process has been described at high level. Feasible candidates for the methods involved in the processes have also been documented, together with evaluations and pros/cons reported by companies. Information related to R2D reception and D2R scheduling have been listed together with ways they could be conveyed.

The physical signals studied have potential purposes involving start indication, timing acquisition/tracking, calibration/frequency synchronization at the device and the reader, channel or interference estimation at the reader, and indicating the end of a R2D/D2R transmission. Some of those purposes could be provided by information in a PRDCH.

Sub-TP #3.3 v2

Pros and cons of multiplexing/multiple access options for R2D and D2R links have been studied by company reports. Options and relationships among the resource allocations for messages of the random access procedures have been identified. The needed timing relationships between corresponding transmissions have been captured, for example the minimum time between a R2D transmission and the corresponding D2R transmission following it; and others.

TP #4

Carrier-wave waveforms and their characteristics have been identified, together with cases for how CW transmission could be deployed in terms of the CW node and the spectrum used. Two CW waveforms were studied: a single-tone unmodulated sinusoid; or two unmodulated single-tone sinusoids. Their characteristics were studied and compared in terms of D2R backscattering reception performance and CW interference suppression at the reader, spectrum utilization of the backscattered signal, and relative complexity of CW generation. The effects of frequency hopping and antenna hopping were included.

TP#5

For Topology 2, RAN1 studied resource allocation and/or control of the intermediate UE via the Uu interface via higher-layer signaling, or L1 and higher-layer signaling.

The harmonized air interface design focuses on DT and DO-DTT use cases. The study also assessed whether the harmonized air interface design can address the DO-A use case, and concluded that it cannot. The part(s) which is/are not sufficient for the DO-A use case are summarized in Clause 6.10.

Agreement

The TP below is endorsed for the conclusion of TR38.769

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR.

(1) The following techniques are recommended in the first normative phase:

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-1 and OOK-4 in R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R and no FEC in R2D

- R2D and D2R signal(s)

(2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release:

- Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

- D1T1 and/or D2T2

  - Scenario A1 and/or A2 and/or B and/or C

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D

- OOK and/or BPSK and/or MSK modulation for D2R

- Carrier-wave waveform 1 and/or 2, and transmission case(s)

- Device (un)availability direction 1 and/or 2

- Proximity determination solution 1 and/or 2

##### 2.1.1.1	Evaluation assumptions and results

RAN1#118bis

Agreement

For single-device latency, the text proposal in R1-2409122 is endorsed in principle for the section 7.2.1 and Annex X of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- The values of the assumptions and results in Tables are further checked by companies and targeted for completion in RAN1#119.

- Other per-source evaluation assumptions that were used are to be provided by company in Annex X (if any) and targeted for completion in RAN1#119.

Note: final agreement on the updated values is to be done at a later time.

Agreement

For multi-device inventory completion time, the text proposal in R1-2409123 is endorsed in principle for the section 7.2.2 and Annex Y of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- The values of the assumptions and results in Tables are further checked by companies and targeted to be completion in RAN1#119.

- Other per-source evaluation assumptions that were used are to be provided by company in Annex Y (if any) and targeted to be completion in RAN1#119.

Note: final agreement on the updated values is to be done at a later time.

Agreement

The receiver sensitives for device 1/2a in tdoc R1-2409125 as Annex [Z] of TR38.769 is endorsed in principle with the following changes to be made,

- Replace company names with ‘[ref]’

- Companies are encouraged to provided reference of the receiver sensitivity (if any) in the column ‘Reference’ for Table Z.1 and Z.2 and targeted to be completion in RAN1#119.

Note: a table will be added for device 2b with RF-ED

Agreement

Update the D2R spectrum and R2D spectrum columns in Table 4.2.1-1 of TR38.769 as follows,

| Scenario | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- |
| D1T1-A1 | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW | At least DL |
| D1T1-A2 | Same as D1T1-A1 | Same as CW | At least DL |
| D1T1-B | Case 1-4 (outside topology, UL) | Same as CW | At least DL |
| D1T1-C | N/A | UL | At least DL |
| D2T2-A1 | Case 2-2 (inside topology, UL) | Same as CW | UL |
| D2T2-A2 | Same as D2T2-A1 | Same as CW | UL |
| D2T2-B | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW | UL |
| D2T2-C | N/A | At least UL | UL |

Agreement

Update [2K1] in link budget table as follows,

| [2K1] | Remaining CW interference (dBm) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| --- | --- | --- | --- |

Agreement

Update [1F] in link budget table as follows,

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [1a] Refer to LLS table [2a1] |

Agreement

Update [2B] in link budget table as follows,

| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a] [receiver bandwidth?] [2a3] |
| --- | --- | --- | --- |

Agreement

The drift rate of CFO for device 2b for evaluation is 0.1 or 1 ppm/s. Companies to report which value is used in their evaluations.

Agreement

For coverage evaluation, the text proposal in R1-2409124 is endorsed in principle for the section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- The values of the assumptions and results in Tables are further checked by companies and targeted to be completion in RAN1#119.

- Remove [] for [7.1.4.1 Device 1].

- Remove the ‘The minimum achievable coverage’ part for each observation sub-section.

- Add a column ‘carrier frequency’ in the table in each ‘collection of results’ sub-section.

- Add descriptions to figures in each ‘collection of results’ sub-section to indicate the meaning of blue bars and red bars.

Note: final agreement on the updated values is to be done at a later time.

Note: further discuss how to explain the ranges of values shown in the observations

- R2D Tx power 33dBm/23dBm/24dBm, R2D Rx sensitivity for device 1/2a/2b

- D2R BLER 1%/10%, D2R data rate ~1kbps/~5-7kbps

- D2R coherent receiver/non-coherent receiver

- For Device 2b, separate observation for budget-Alt1 and Alt2

- For device 1/2a, CW cancellation capability (range1, range2,...), range is TBD

- For device 1/2a, backscatter loss and modulation schemes (OOK, BPSK, BFSK)

- EIRP value (xx, xx), R2D Rx sensitivity for device 1/2a/2b

- Channel coding schemes for D2R

Note: observations are capturing results based on mandatory evaluation parameter values

Note: R2D RF-ED LLS results may not be used for coverage evaluation (as agreed)

Note: for D2T2, kept the InH-Office results only in the observation part

Note: companies are encouraged provide information on whether they used one way or two way channel for their evaluations and details of their relevant assumption, according to earlier agreements on evaluation assumptions

Agreement

For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows,

- For device 1 in D1T1:

  - a value within [10 - 20]m

- For device 2a in D1T1:

  - a value within [20 - 30]m

- For device 2b in D1T1:

  - a value within [30 - 50]m

- For device 1 in D2T2:

  - 10 m

- For device 2a in D2T2:

  - a value within [10 - 20]m

- For device 2b in D2T2:

  - a value within [30 - 50]m

The distance target is compared to the results which assumes mandatory assumptions (as agreed).

RAN1#119

Agreement

For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows,

- For device 1 in D1T1:

  - 15m

- For device 2a in D1T1:

  - 25m

- For device 2b in D1T1:

  - 50m

- For device 2a in D2T2:

  - 15m

- For device 2b in D2T2:

  - 40m

Agreement

Capture the TP for single device latency in R1-2410827 for updating section 7.2.1 of TR 38.769.

- Replace company names with ‘[ref]’

- Update the last observation as shown below:

  - 5 sources provide results not including Step D D2R transmission

- Assuming Msg3 size (Device ID) of 96 bits and Step C R2D message size small than or equal to 400 bits,

- For data rate of 1 kbps, 5 sources [R1-9411-10], [R1-9411-11], Samsung, NTT DOCOMO, Spreadtrum provide results, and the single device latency is [196.28 ms ~ 612 ms].

- For data rate of 7 kbps, 5 sources [R1-9411-5], [R1-9411-10], [R1-9411-11], Samsung, NTT DOCOMO provide results, the single device latency is [28.66 ms ~ 87.90 ms].

Agreement

Capture the TP for multiple device inventory completion time in R1-2410828 for updating section 7.2.2 of TR 38.769.

- Replace company names with ‘[ref]’

Agreement

For coverage evaluation, the text proposal in R1-2410826 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- In the observation subsection, change the CW cancellation capability range for D1T1 into (-inf, 130dB), [130-140), [140-150), [150, +inf) dB

- Update Figure 7.1.1.2.1.2-1 to make it clear to show the companie’s name

- Update Figures in non-coherent D2R LLS (section 7.1.3) to make it clear to show the companies’ name.

- In Tables for link budget for section 7.1.1 and 7.1.2, change Note 15 into “Row indices in the spreadsheet (row 1 corresponds to row 6 in spreadsheet)”.

Agreement

Capture the TP for receiver sensitivity in R1-2410854 for updating annex A of TR 38.769.

- Replace company names with ‘[ref]’

- Replace company names in the first column in each table with the reference to the RAN1#119 Tdoc

Agreement

Update the [2D] in link budget table as follows,

| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, Device 2 - FFS other values For IF/ZIF receiver - 15dB, Device 2 Other values can be optionally used and reported by companies | For BS as reader - 5dB For intermediate UE as reader - 7dB |
| --- | --- | --- | --- |

Update the note D in link budget table as follows,

Note D: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB]  assuming a noise figure in [2D] from the table, exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

Agreement

Update the [1E3] in link budget table as follows,

| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (See note 1) D2T2-B: - 5m, - 10m, FFS other values For scenarios ‘A1’ and ‘A2’: Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note 1: Only applicable for device 1/2a. Note 2: Companies to report which value(s) are evaluated. |
| --- | --- | --- | --- |

Agreement

Update the [1M] in link budget table as follows,

| [1M] | EIRP (dBm) | Calculated (see Note 1) FFS: any limitation of the EIRP subject to future discussion | Calculated (see Note 1) |
| --- | --- | --- | --- |

Agreement

Update the [2K] in link budget table as follows,

| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Notes: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS The value for other cases (e.g., multi-tone CW) can be reported by companies. |
| --- | --- | --- | --- |

Agreement

Update the [3C] in link budget table as follows,

| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 |
| --- | --- | --- | --- |

Agreement (updated on Thursday)

Update the [0q] in link budget table as follows,

| [0q] | Sampling frequency | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| --- | --- | --- |

Agreement

Update the [0q] in link budget table as follows,

| [2a1] | Transmission bandwidth | [2a1]-Alt1 (M): DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz [2a1]-Alt2: SSB X kHz is considered for D2R transmission bandwidth. The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. Not applicable for at least device 1 For device 2b only, FFS for device 2a. or device 2a with large frequency shift. X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies |
| --- | --- | --- |

Agreement

Update Table 4.2.2-2 in TR 38.769 as follows,

Table 4.2.2-2: Assumptions on layout for D1T1 and D2T2

| Parameter | Assumptions for D1T1 | Assumptions for D2T2 |  |
| --- | --- | --- | --- |
| Scenario | InF-DH | InH-office | InF-DL |
| Hall size | 120x60 m | 120 x50 m | 300x150 m |
| Room height | 10 m | 3m | 10 m |
| Sectorization | None |  |  |
| BS deployment / Intermediate UE dropping | 18 BSs on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m BS height = 8 m | L=120m x W=50m; Intermediate UE height = 1.5 m FFS: Intermediate UE dropping Option 1: - Intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: - Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. | L=300m x W=150m; Intermediate UE height = 1.5 m FFS: Intermediate UE dropping Option 1: - The intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: - Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. | Device Height= 1.5m A-IoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. |
| Device mobility (horizontal plane only) | 3 kph | 3 kph | 3 kph |
| Note: The CW distribution in D1T1-B or D2T2-B is reported by companies |  |  |  |

Agreement

For coverage evaluation, the text proposal in R1-2410855 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769.

Note: this is an update of the earlier agreement copied below

Agreement

For coverage evaluation, the text proposal in R1-2410826 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- In the observation subsection, change the CW cancellation capability range for D1T1 into (-inf, 130dB), [130-140), [140-150), [150, +inf) dB

- Update Figure 7.1.1.2.1.2-1 to make it clear to show the companie’s name

- Update Figures in non-coherent D2R LLS (section 7.1.3) to make it clear to show the companies’ name.

- In Tables for link budget for section 7.1.1 and 7.1.2, change Note 15 into “Row indices in the spreadsheet (row 1 corresponds to row 6 in spreadsheet)”.

Agreement

Include the spreadsheet of the coverage evaluation results (R1-2410875) as an attachment of the TR38.769.

Agreement

Capture the following as part of the conclusion sub-section 7.1

RAN1 has evaluated the coverage for InF-DH NLOS scenarios for D1T1 and InH-Office LOS/InF-DL NLOS scenarios for D2T2, across all devices 1/2a/2b. Coverage evaluation results comparing various R2D Tx power/EIRP value, R2D Rx sensitivity, BLER 1%/10% target, data rate ~1kbps/~5-7kbps, D2R coherent receiver/non-coherent receiver, R2D budget-Alt1 and Alt2, CW cancellation, backscatter loss and modulation schemes, D2R with/without channel coding schemes, etc, have been provided. The observations for coverage evaluation results can be found in Clause 7.1.1.1.2/7.1.1.2.2/7.1.1.3.2/7.1.2.1.2/7.1.2.2.2/7.1.2.3.2.

##### 2.1.1.2	Ambient IoT device architectures

RAN1#118bis

Agreement

RAN1 deprioritize IF receiver for device 2a.

Agreement

RAN1 deprioritize ZIF receiver for device 2a.

Agreement

- Adopt following updated table format.

- One or more rows under a single purpose may exist to capture different options, if any.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |

Agreement

TR captures following descriptions for large frequency shift for device 2a.

- Large frequency shift may be technically feasible in terms of power consumption, and device complexity.

- Remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Additional complexity and power consumption may be needed to achieve image suppression and harmonics suppression.

- The necessity or effectiveness of large frequency shift may depend on spectrum for R2D/CW2D/D2R, full duplex capability of reader, whether device 1 and 2a are supported by a single reader, spectrum regulation/usage, frequency error, coverage, etc.

- Agreement

- For clock purpose #3, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / Calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| [ #3 ] | [ Time counting (if supported) ] | e.g. tens kHz | 1 (if applicable) 2a, 2b | <0.1uW | [10^4 - 10^5] | Calibration may not always be feasible for this clock purpose |
|  |  |  | 2a, 2b | <0.1uW | [10^3-10^4] | FFS N/A |

- Agreement

- For clock purpose #5, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^23 ~ 10^4] | Option 1: 10s (Note2) Option 2: 100s (Note2) |
| Note 2: Either internal clock counter for calibration clock counting based on at least R2D signals or clock counting based on transmission from reader Calibration is based on existing or new signal from reader. FFS how to calibrate (9.4.2.2). No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

- Further study above options and strive to down select considering its implication on performance, system design, necessity, etc.

- Note: study of whether additional signal is necessary or not for calibration is discussed under agenda 9.4.2.2.

- Agreement

- For clock purpose #4, adopt following update.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #4 | Large frequency shift (if supported for device 2a) | 10s MHz (e.g., 50MHz) | 2a | [10s] uW | [10^3-10^4] | [10^3] (Note 2) |
| Note2: Digital clock counter can be assumed for calibration clock counting based on R2D signals how to calibrate. No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

RAN1#119

Agreement

Update clock purpose #3 as follows.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #3 | Time counting (if supported) | e.g. tens kHz | 1 (if applicable), 2a, 2b | <0.1uW | [10^4 - 10^5] | Calibration may not always be feasible for this clock purpose |

Agreement

Update clock purpose #4 as follows.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #4 | Large frequency shift (if supported for device 2a) | 10s MHz (e.g., 50MHz) | 2a | [10s] uW | [10^3-10^4] | 10^3 (Note 2). Whether better accuracy can be achieved has not been determined. |
| Note2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

Agreement

Adopt following table to update clock purpose #5.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^3 ~ 10^4] | Option 4: within 10s – 200 (Note 2), there is no need to determine a single value for the study |
| Note 2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

Agreement

For clock purpose #1, adopt following table.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Sampling | a few MHz | 1 (Note 4) | < 1uW |  | Option 1: 10^5 (Note 2) |
|  |  |  |  |  | [10^4 ~ 10^5] | Option 2: 10^4 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1uW power consumption at least for device 1 |
|  |  |  | 2a, 2b | < 1uW | 10^4 ~ 10^5 | 10^4 (Note 2) |
|  |  |  |  | < 10uW | 10^3 ~ 10^4 | 10^3 (Note 2) |
| Note 2: Calibration is based on signal from reader. No drastic temperature change is assumed during calibration. Note 3: Accuracy after clock sync / calibration is better than initial clock accuracy. Note 4: Device 2a, 2b could also use similar implementation |  |  |  |  |  |  |

Observation

Several sources [Samsung, Docomo, Oppo, InterDigital, Qualcomm, ZTE] think that for clock purpose #1 (sampling), accuracy after clock sync / calibration at device side of 104 ppm is achievable with < 1uW power consumption for device 1, and that 105 ppm may not be sufficient for certain designs.

Agreement

Update the earlier observation as below

Observation

Several sources [Samsung R1-2409597, Docomo R1-2410389, Oppo R1-2410091, InterDigital R1-2410313, Qualcomm R1-2410478, ZTE R1-2409551] think that for clock purpose #1 (sampling), accuracy after clock sync / calibration at device side of 104 ppm is achievable with < 1uW power consumption for device 1, and that 105 ppm may not be sufficient for certain designs.

- Source [Samsung] R1-2409597 mentioned that device 1 can support basic calibration operation within a reasonable power consumption or complexity, achieving accuracy of 10^4 PPM.

- Source [DCM] R1-2410389 mentioned that w/ clock calibration based on CAP design, 10^4 could be achieved after clock calibration.

- Source [Oppo] R1-2410091 reported that clock purpose #1 could be applied to all device types. But, considering complexity and power, device 1 and 2 could have different power, accuracy. For Device1, the power consumption should be < 1 uW; the timing accuracy should be 10^4 ~ 10^5; the calibration can reach to 10^4.

- Source [InterDigital] R1-2410313 proposed device 1 with accuracy of 10^4 after clock sync/calibration.

- Source [QC] R1-2410478] reported that using simple clock counting based calibration method, 1% could be achieved with power consumption <1uW. It was reported that 1% is the sweet spot of target accuracy considering performance, complexity, signal design, etc.

- Source [ZTE] R1-2409551 mentioned that device can use recommended clock calibration method to get high accuracy of 103~104 ppm for device 1 and 102~103 ppm for device 2.

Agreement

For clock purpose #2, adopt following table.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #2 | Small frequency shift | a few MHz | 1 (Note 4) | < 1uW |  | Option 1: 10^5 (Note 2) |
|  |  |  |  |  |  | Option 2: 10^4 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1uW power consumption at least for device 1 |
|  |  |  | 2a, 2b (if applicable) | < 10uW |  | 10^3 (Note 2) |
| Note 2: Calibration is based on signal from reader. No drastic temperature change is assumed during calibration. Note 4: Device 2a, 2b (if applicable) could also use similar implementation |  |  |  |  |  |  |

Observation

For A-IoT device clock, 9 sources [TCL] R1-2409358, [ZTE] R1-2409551, [HW] R1-2409417, [QC] R1-2410478, [Samsung] R1-2409597, [CATT] R1-2409941, [Vivo] R1-2409681, [DCM] R1-2410389, [MTK] R1-2410514 have reported that clock calibration based on at least clock counting for a known duration is feasible for A-IoT devices.

##### 2.1.1.3	General aspects of physical layer design

RAN1#118bis

Agreement

For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 2SB transmission, +FSx and -FSx parts are included.

- For 1SB transmission, FFS which part is included.

Agreement

In D2R, a chip corresponds to one modulated symbol at least for OOK and BPSK.

- FFS: the definition for MSK.

Agreement

2SB modulation is feasible for D2R transmission for all devices.

- Feasibility and necessity of 1SB would depend on the impacts due to issues including: device-side filtering requirements (i.e. image suppression), RF resource usage / spectral efficiency, etc.

Agreement

Capture the following observation in the TR:

- Sources [Ericsson][CATT][Qualcomm][Lekha] report that MSK is feasible in some way:

  - [Ericsson][CATT] say it is feasible for all devices, for example when it is implemented with multiple impedances switching

  - [Qualcomm] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b

    - For device 1 and 2a this type of MSK does not have continuous phase

  - [Ericsson] say that benefits include lower sidelobes than OOK and BPSK, and lower BER than OOK and same BER as BPSK

- Sources [Huawei][TCL][CMCC][CTC][vivo][Xiaomi][Comba] report that MSK is either infeasible or should be deprioritized for all devices.

  - [Huawei][CMCC][CTC][vivo][TCL][Xiaomi][Comba] say that MSK is less spectrally efficient than OOK and BPSK because there are issues due to poor phase accuracy in the device

  - [Huawei][CTC][TCL][vivo][Xiaomi] say that MSK would increase reader and device complexity

  - [vivo] say that MSK performance for device 2b would materially degrade due to CFO

Agreement

Agree the following text proposal for the TR:

#### 6.1.1.x R2D bandwidths …(unchanged parts omitted)… Table 6.1.1.x-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.x-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3. …(unchanged parts omitted)…

Conclusion

For R2D, whether single sideband or double sideband modulation is used by the reader does not need to be studied by RAN1.

Agreement

For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR:

Agreement

For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

Agreement

For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where:

- Fc is the carrier-wave frequency, at least when externally generated

- FSx depict examples of amount of small frequency shift

- For 1SB transmission, one of the following figures applies as appropriate.

Agreement

State in the TR that:

- Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave would be impacted by at least the following aspects:

  - A possible guard band Bguard,D2R due to at least SFO

  - A possible guard band Bguard,D2R due to CFO for Device 2b

Agreement

Capture in the TR that for R2D:

- Time-domain multiplexing is the baseline.

- Code-domain multiplexing is not considered for device 1/2a/2b.

- Frequency-domain multiplexing is not considered for the devices with RF-ED receiver, and studied for Device 2b with IF-ED or ZIF receiver considering the technical aspects in Table 6.1.1.x-1.

Table 6.1.1.x-1: Observations on the feasibility and necessity of FDM for Device 2b

| Aspects to be considered for feasibility/benefit | Observations |
| --- | --- |
| Inventory completion time | Sources [Ericsson, LGE, Nokia, OPPO] state that FDM is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. |
| Device implementation | Sources [Apple, DOCOMO, Ericsson, LGE, OPPO] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Sources [ITL, MTK, Spreadtrum, TCL, Huawei] thinks that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Sources [Ericsson] state that the larger R2D responses are harder for the devices to process in the case of TDM+FDM/TDM only for D2R/R2D, respectively. |
| Spectrum utilization | Sources [ITL] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. |
| Coverage (in the case of single reader) | Sources [Huawei] state the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Sources [Ericsson] think the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture). |
| Reader implementation (in the case of single reader) | Sources [Huawei] state that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions. |
| Harmonized design for all devices | Sources [ASUSTek, CMCC, ETRI] state that it is not appropriate to include FDM only for Device 2b, while Device 1 and 2a cannot support it. |

Note: further updates to the table can be agreed at RAN1#119

Agreement

Capture in the TR:

- For R2D OFDM-based OOK waveform generation, CP-OFDM and DFT-s-OFDM are both feasible

  - For M=1, i.e. use of OOK-1 or OOK-4 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible

  - For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible.

Agreement

Adopt the TPs below.

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE][Lenovo] [Spreadtrum][Samsung][Docomo] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location - Some sources [vivo][Huawei][CMCC][Futurewei] report the device implementation may depend on M values For CP handling Alt M1-1, the potential impacts are discussed as follows: - Some sources [Futurewei][Huawei][Spreadtrum][vivo][InterDigital][Qualcomm] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4, while [vivo] reports that for large M values Alt M1-1 is used in combination with Alt M1-2. - Some sources [Ericsson][ZTE] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [vivo][Apple] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [Huawei] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible at 10% BLER even for large M values (e.g. M=24) under large SFO (e.g. 10^4-10^5 ppm). Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ) while [ZTE] shows [1~2 dB] loss compare to no CP case for M<24, and an error floor at BLER=10% for M=24. - Some source [CMCC][Apple] report that the device needs additional complexity to handle CP, while other sources [Huawei][InterDigital] reports that it is feasible in terms of implementation complexity based on transition edge detection. For CP handling Alt M1-2, the potential impacts are discussed as follows: - Some sources [Futurewei][Nokia][CMCC][Ericsson][Huawei][Spreadtrum][vivo][ZTE][Apple][InterDigital][Docomo] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8, while [vivo] reports that for large M values Alt M1-2 is used in combination with Alt M1-1. - One source [LGE] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [vivo] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 10^4 ppm). For Method 2, two approaches regarding subcarrier orthogonality are studied: …(unchanged parts omitted)…

#### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. …(unchanged parts omitted)… For Method 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For CP handling Method Type 2, depends on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by, - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant. - Some source [Qualcomm] report that non-constant OOK chip duration may impact performance, while some other source [ZTE] report that non-constant OOK chip duration does not impact performance. For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. For CP handling Alt M2-2, the solutions and potential impacts are discussed as follows, - [vivo][OPPO][CATT][Samsung] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC][LGE][DOCOMO] report that CP handling of Alt M2-2 would cause interference to NR, while [vivo] reports single PRB guard band would be sufficient to handle interference. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC][LGE][DOCOMO] report that CP handling of Alt M2-2 would increase the transmitter complexity.

Agreement

For R2D repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [ETRI], [QC] and [IITK, IITM] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported:

  - Bit-level repetition

Positive observations

    - Source [Ericsson] state that bit level repetition can be studied if coverage enhancement of the R2D link is required.

    - Source [OPPO] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power.

    - Source [QC] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices.

Negative observations

    - Source [CMCC] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception.

    - Source [Vivo] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination.

  - Block-level repetition

Positive observations

    - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain.

Negative observations

    - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded.

  - Chip-level repetition

Positive observations

    - Source [CMCC] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps.

    - Source [IITK, IITM] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions.

Negative observations

    - Sources [Ericsson][vivo] and [CATT] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option.

Agreement

For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [Huawei], [Nokia], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] that block level repetitions should be supported.

General observations

  - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition

  - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block.

  - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission.

  - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum.

  - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation.

Performance comparisons

  - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding.

  - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1.

  - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same.

  - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping.

  - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

Agreement

For D2R repetitions, the following observations are captured in TR 38.769:

- It is reported by sources [ZTE], and [Samsung] that bit level repetitions should be supported.

  - Source [Huawei] state that for bit level type 1, the repetition data bit is decoded after FEC detection, and hence, the digital data bit cannot be combined with combining gain.

  - Source [ZTE] state that the performance of bit level type 1 repetition is degraded by ~3.5dB @ 10% BLER under the cases of AWGN and TDL-A channel when compared to bit level type 2.

  - Source [Huawei] state that since the bit level type 2 repetition is performed after FEC at the transmitter, at the receiver side, each bit would have a log-likelihood ratio (LLR) that can be combined and provided as the input for FEC decoding, thereby providing a better performance as compared to bit level type 1.

Agreement

For D2R FEC, include the following table in the TR:

| Option # | CC Design | Pros | Cons |
| --- | --- | --- | --- |
| Baseline | Constraint length 7 Code rate 1/3 | - [Vivo] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. - [Vivo] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. - [DCM] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. - [CATT] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. - [Lekha] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. - [Xiaomi] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 - [Xiaomi] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 |  |
| 1 | Constraint length 4 Code rate 1/2 – 1/4 | - [Ericsson] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes. | - [CMCC] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. |
| 2 | Constraint length 6 Code rate 1/2 – 1/6 | - [QC] Reduced reader and device complexity due to smaller number of shift registers. | - [QC] Code rate 1/2: Decoding performance is decreased by 0.6dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/2: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [QC] Code rate 1/3: Decoding performance is decreased by 0.4dB@1% BLER, when compared to baseline. - [QC] Code rate 1/4: Decoding performance is decreased by 0.2dB@1% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is decreased by ~0.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. - [QC] Code rate 1/6: Decoding performance is decreased by 0.15dB@1% BLER, when compared to baseline. |
| 3 | Constraint length 7 Code rate 1/2 – 1/4 | - [CATT] Code rate 1/2, TBCC: Decoding performance is increased by ~9dB@10% BLER, when compared to no CC but with Manchester line codes. - [CATT] Code rate 1/4: Decoding performance is increased by ~1.4dB@10% BLER, when compared to baseline. - [ZTE] Code rate 1/4: Decoding performance is increased by 0.5dB@10% BLER, when compared to CC with code rate 1/2. - [QC] Code rate 1/4: Decoding performance is increased by 0.2dB@1% BLER, when compared to baseline. |  |
| 4 | Constraint length 8 Code rate 1/2 – 1/6 | - [CMCC] Code rate 1/2: Decoding performance is increased by ~0.22dB@10% BLER, when compared to constraint length 7, code rate 1/2. - [ZTE] Mother code rate 1/4, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/4, code rate 1/4. - [ZTE] Mother code rate 1/3, code rate 1/6: Decoding performance is increased by ~1.8@10% BLER, when compared to constraint length 8, mother code rate 1/3, code rate 1/4. - [ZTE] Mother code rate 1/2, code rate 1/6: Decoding performance is increased by ~1.7@10% BLER, when compared to constraint length 8, mother code rate 1/2, code rate 1/4. |  |

Agreement

For D2R FEC, include the following observations in the TR, where k = constraint length and m = 1/code-rate:

- Performance of other {k, m} pairs decreased 0.15 – 1.4 dB when k<7, with reported benefit of reduced reader and device complexity due to shorter shift registers.

- Performance of other {k, m} pairs increased 0.22 – 1.8 dB when k>7, with an increase in the number of shift register resulting in increased device complexity.

- Performance of other {k, m} pairs decreased 0.3 – 2.5 dB when m<3.

- Performance of other {k, m} pairs increased 0.2 – 1.8 dB when m>3.

Agreement

For CRC, capture the following observation in the TR:

- Sources as follows recommend that the length of the CRC used for A-IoT depends on:

  - Block size or TBS-like size, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, CATT, ZTE, DCM, ITL. Furthermore:

    - Source [Huawei] states that specifically for short messages, the CRC length can directly impact the transmission efficiency due to the overhead caused by the CRC.

    - Source [ZTE] state that CRC length depends on the TBS due to its impact on the transmission overhead and FAR performance. When the TBS is too small, the overhead of CRC-6 becomes quite significant.

    - Source [ITL] states that CRC length is fundamentally proportional to the size of the message.

  - Target false alarm rate or error detection performance, according to sources Huawei, Spreadtrum, TCL, CMCC, CATT, MTK, ITL. Furthermore:

    - Source Huawei evaluated that the probability of undetected errors is higher for CRC-6 as compared to CRC-16, and transmissions with CRC-16 performs better, for a given TBS=24, and source MTK say that a longer CRC would mean a better error detection performance.

  - Type of message, such as a command/reply function, to determine when no CRC is used, according to sources Huawei, Spreadtrum, TCL, CMCC, Vivo, MTK, DCM. Furthermore:

    - Sources Huawei and Spreadtrum say that similar to RFID, only important messages, decided based on their functionality, such as configuration/control/command messages, are appended with CRC to increase their reliability, while other messages are not protected by CRC to save the overhead.

    - Source TCL proposes different CRC lengths for Paging-like/select/challenge messages that may or may not contain the device ID/group ID.

    - Source CMCC states that some messages such as a QueryRep-like command do not result in serious FAR issues if mis-detected, and hence the functionality of a message/command has to be considered.

  - Overhead caused by CRC, according to sources Huawei, Spreadtrum, CMCC, ZTE, MTK, Qualcomm

    - Sources Huawei, Spreadtrum, CMCC, MTK mention that longer CRCs have high overhead but enjoy better error detection performance, affecting the transmission efficiency. In order to balance the overhead and performance, Hence, short CRC can be used for short commands to reduce the false alarm rate and make the system more robust as compared to no CRC.

    - Source CMCC states that adding CRC to QueryRep-like commands which are sent repeatedly with a small payload would waste network resources, and hence the overhead should be considered.

    - Sources ZTE, Qualcomm states that the overhead of CRC decreases as the TBS increases, and that CRC-16 has higher overhead than CRC-6.

Agreement

For the CRC generator polynomials, capture the following observations in the TR:

- Sources China Telecom, OPPO, Xiaomi, CATT, Samsung recommend that the same polynomials from TS 38.212 are reused, giving justifications:

  - Sources CATT, OPPO state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials.

  - Source Xiaomi states that the device complexity is increased when different polynomials are introduced for the D2R transmission.

  - Source China Telecom states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used.

- On the other hand, sources [ZTE], [Qualcomm] recommend that the same polynomial for CRC-6 () from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, , incorporating the CRC-6 polynomial, giving justifications:

  - The proposed new CRC-16 polynomial nests the existing CRC-6 within its structure.

  - Using separate sets of circuits to implement the polynomials for CRC-6 and CRC-16 as per TS 38.212 requires a higher number of registers and XOR gates when compared to using the new CRC-16 polynomial.

  - FAR performance is the same for the legacy and proposed new polynomial for CRC-16, but the new polynomial reduces the complexity and power consumption of the device.

Agreement

Capture the following observations in the TR for R2D line codes:

- It is reported by sources [Huawei], [CMCC], [EURECOM], [CATT], [ZTE], [Lekha], [Samsung], [ETRI], [Lenovo], [Apple], [LG] that PIE has a higher ratio of high to low voltage, which enables it to provide stable instantaneous RF energy to the device –

  - Sources [CMCC], [Samsung], [LG], [EURECOM] and [Lenovo] consider this a justification for studying PIE

  - Sources [Huawei], [CATT], [ZTE], [ETRI] and [Apple] acknowledge this aspect of PIE, but question its necessity.

- It is stated by sources [Nokia], [Ericsson], [Vivo], [CATT], [ZTE], [Apple], [DCM, [IITK, IITM] that, due to variable codeword lengths in PIE, scheduling time domain resources would be different to transmit the same amount of data due to the number of 0 and 1, as compared to the constant codeword lengths in Manchester codes.

- It is stated by sources [Huawei], [Spreadtrum], [CATT], [ZTE], [Lekha], [Apple] that, due to variable codeword lengths in PIE, the error detection of one information bit can result in error propagation for the detection of subsequent bits and therefore negatively affecting the detection of the overall data length, as compared to Manchester codes.

- Source [Ericsson] says that variable codeword lengths in PIE complicates A-IoT resource management, especially in in-band deployments when a reader needs to multiplex both NR and A-IoT communication.

- Source [CMCC] says that equal codeword length can overcome the cons of variable codeword length in PIE.

- According to sources [Nokia], [Ericsson], [CATT], [ZTE], [Lekha], [Samsung], [Apple], [DCM], transitions during Manchester code transmissions can be utilized for recovering the signal’s clock.

- Sources [Nokia], [Ericsson] say that the signal does not lose information or distort when fed to a DC blocker because each Manchester encoded symbol maintains a constant DC level, i.e. always the same average power, unlike if there was a long sequence of 1s or 0s.

- Evaluations are reported showing that Manchester codes perform better than PIE by 1dB to 6dB @10% BLER – [Huawei], [CMCC], [CATT], [ZTE], [IITk, IITM].

  - Source CMCC claims only a 1dB gain when comparing Manchester over equal length codeword in PIE, stating that the gain is negligible since the downlink coverage bottleneck is the receiver sensitivity and not the decoding performance.

Agreement

For D2R line codes, the following observations are captured in TR 38.769:

- Sources [Huawei], [CMCC], [Vivo], [CATT], [ZTE], [Samsung] report that Manchester codes perform better than Miller codes by 0.5dB to 5.5dB @10% BLER.

- Sources [Huawei], [CMCC] [ZTE] report that Manchester codes perform better than FM0 by 0.2dB to 2.5dB @10% BLER.

  - According to sources [Vivo], [CATT], Manchester codes have a similar performance as FM0.

- Sources [Huawei], [Lekha] state that the power of Miller coding is more concentrated in the low-frequency region which is adjacent to the carrier wave frequency, as compared with FM0 code and Manchester code which leads to worse interference handling capability for Miller codes.

- Source [QC] state that performance of BPSK square wave is better than Miller/FM0 by ~3-4 dB @10%BLER with coherent demodulation.

Agreement

For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length).

Agreement

For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options:

- One option is that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

- Another option is that the multiplication operation is an XNOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

Agreement

Regarding small frequency shift factor, capture in the TR that:

- For study purposes

  - For BPSK and OOK, the small-frequency-shift factor is defined as the ratio of bit length to two times the chip length

  - The value of small-frequency-shift factor depends on details of design, which according to proposals considered could be e.g.

    - The repetition number of Manchester codeword within one information bit duration

    - The M value of Miller in Figure 6-12 of UHF RFID standard

    - The number of square-wave periods within one information bit duration

Agreement

Add the following TP to the TR section on “D2R multiple access” for TDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [CMCC, CATT, Ericsson, FUTUREWEI] state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [DOCOMO, LGE, MTK] state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration (….)

Agreement

Add the following TP to the TR section on “D2R multiple access” for FDMA:

6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. (FL : Would insert here the TP from Proposal 3.6a) … NOTE: For the purposes of this section, X refers to the device’s SFO expressed as 10X ppm. According to sources [Ericsson, CMCC, CATT, CEWiT, DOCOMO, FUTUREWEI, Huawei, InterDigital], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [CMCC], and potentially increased power consumption for devices according to sources [CATT, Sony, OPPO]. For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes Option 1: By repetition of the codewords within the same time duration corresponding to an information bit. FFS how to define this repetition. Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift. Companies to report how they perform multiplying for option 2. - For applying with Miller line codes, according to Figure 6-13 of [6]. - For FM0, small frequency shift is not defined - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift. - Potential purposes include: - FDMA of D2R, if supported - CW interference avoidance, if supported Note: Small frequency shifts for D2R are studied for the same potential purposes for MSK. It is observed that the performance of FDMA may be impacted by the following aspects: - The large SFO of device - Sources [Qualcomm, ZTE] state that large SFO (e.g. up to 105 ppm) produces higher BLER degradation due to inter-device interference than a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO. Source [ZTE] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - Source [Huawei] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤ 1dB) on the performance of FDMA between multiple devices. - Sources [vivo] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [Sony] think that the required guard band size increases for higher switching frequencies for passive devices. - The timing offset between devices - Sources [InterDigital] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - The maximum range of small frequency shift - Sources [MTK] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - The harmonics in the backscattered signal - Sources [vivo] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - The potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision - The number of multiplexed devices - Source [ZTE] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5~3dB performance loss from 6 FDMA-ed devices compared to single device. (….)

Agreement

Add the following TP to the TR section on “D2R multiple access” for CDMA:

6.1.2.x D2R multiple access (…) According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Source [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Source [vivo] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Source [FUTUREWEI] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Source [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. - Source [Qualcomm] state that quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Source [Huawei] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Source [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. - In the case of relatively smaller SFO (e.g., X = 4), - Source [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. The impact of CFO on the performance of CDMA is studied for Device 2b as follows. - Source [Huawei] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. The impact of timing offset between CDMed D2R transmissions on the performance of CDMA is studied for Device 2b as follows. - Source [Lenovo] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Source [OPPO] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Source [MTK] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Source [Samsung] think that the different propagation delays from devices may also degrades decoding performance. - Source [ZTE] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Source [Qualcomm] state that the larger power variation severely degrades the capacity of CDMA for msg-1. - Source [ZTE] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Source [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Source [Ericsson] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Source [MTK] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Source [TCL] think that CDMA leads to higher power consumption and lower data rate. - Source [vivo] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Source [OPPO] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Source [OPPO] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. - Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. - Source [ZTE] state that BPSK modulation for the CDMA improve the D2R transmission performance and multiplexing capacity compared with OOK based modulation.

Agreement

The D2R baseband signal (as distinct from the internal or external carrier wave) is non-OFDM.

Agreement

For CRC, capture the following evaluation results in the TR:

- NOTE 1: Pud = Probability of undetected error according to [J. K. Wolf and R. D. Blakeney, “An exact evaluation of the probability of undetected error for certain shortened binary CRC codes,” Proc. Milcom ’88, vol. 1, pp. 287-292, 1988]

- NOTE 2: Overhead = CRC length / (TBS + CRC length).

| TBS | CRC-6 |  |  | CRC-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source | CRC overhead | PFA or Pud | Source | CRC overhead | PFA or Pud |
| 8 bits | HW | 43% |  | HW | 67% |  |
| 12 bits | E/// QC | 33% 33% | ~10^(-2) PFA | E/// QC | 57% 57% | ~10^(-5) PFA ~10^(-11) Pud @10-3 BLER |
| 16 bits | HW | 27% |  | HW | 50% |  |
|  | ZTE QC | 27% 27% |  | ZTE QC | 50% 50% |  |
| 24 bits | HW | 20% | ~10^(-6) Pud @ 10% BLER and 2.7dB SNR | HW | 40% | ~10^(-9) Pud @10% BLER and 2.8dB SNR |
|  | E/// ZTE QC | 20% 20% 20% | ~10^(-2) PFA | E/// QC | 40% 40% | ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 32 bits | HW ZTE QC | 16% 16% 16% | ~10^(-5) Pud @10-3 BLER | HW ZTE QC | 33% 33% 33% | ~10^(-10) Pud @10-3 BLER |
| 40 bits | HW ZTE QC | 13% 13% 13% | ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | HW ZTE QC | 29% 29% 29% | ~10^(-10) Pud @10-3 BLER |
| 48 bits | E/// ZTE QC | 11% 11% 11% | ~10^(-2) PFA ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER | E/// ZTE QC | 25% 25% 25% | ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER |
| 96 bits | HW E/// ZTE QC | 6% 5% 6% 6% | ~10^(-2) PFA ~10^(-4) Pud @10-3 BLER | HW E/// ZTE QC | 14% 14% 14% 14% | ~10^(-5) PFA ~10^(-9) Pud @10-3 BLER |
| 128 bits | ZTE QC | 4% 4% | ~10^(-3) Pud @10-3 BLER | ZTE QC | 11% 11% | ~10^(-9) Pud @10-3 BLER |
| 256 bits | HW QC | 2% 2% | ~10^(-3) Pud @10-3 BLER | HW QC | 6% 6% | ~10^(-9) Pud @10-3 BLER |

Agreement

For small frequency shifts in D2R, the following observations are captured in TR 38.769:

- Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding.

- Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration.

- Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is equivalent to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit.

- Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worst performance compared with small frequency shift (Option 1) and small frequency shift (Option 2).

RAN1#119

Agreement

Capture in the TR clause on “D2R waveform and modulation”:

- Device 2b internally generates carrier wave with only a single tone. Device 2b does not use a ‘2 single tone’ carrier wave.

Agreement

For device 2b, FDMA in D2R can be achieved by direct modulation of its internally generated carrier wave at the desired frequency.

Agreement

Capture the following TP update into TR 38.769, where source IITM will be added where relevant.

…(unchanged parts omitted)… For Alt M1-1, the potential impacts are discussed as follows: - Some sources [R1-9421-1], [R1-9421-5], [R1-9421-6], [R1-9421-8], [R1-9421-25], [R1-9421-28], [IITM] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4, while [R1-9421-8] reports that for large M values Alt M1-1 is used in combination with Alt M1-2. - Some sources [R1-9421-3], [R1-9421-32] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [R1-9421-8], [R1-9421-18] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [R1-9421-5] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible at 10% BLER even for large M values (e.g. M=24) under large SFO (e.g. 104-105 ppm). Sources [R1-9421-8], [R1-9421-32] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 104-105 ppm ) while [R1-9421-32] shows [1~2 dB] loss compare to no CP case for M<24, and an error floor at BLER=10% for M=24. - Some sources [R1-9421-9], [R1-9421-18] report that the device needs additional complexity to handle CP, while other sources [R1-9421-5], [R1-9421-25] reports that it is feasible in terms of implementation complexity based on transition edge detection. - One source [CATT] report that the device might remove the wrong portion of the CP part of the OFDM symbol due to timing error, which could introduce the false rising/falling edge for the subsequent OOK demodulation. For Alt M1-2, the potential impacts are discussed as follows: - Some sources [R1-9421-1], [R1-9421-4], [R1-9421-9], [R1-9421-3], [R1-9421-5], [R1-9421-6], [R1-9421-8], [R1-9421-32], [R1-9421-18], [R1-9421-25], [R1-9421-27], [CATT] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8, while [R1-9421-8] reports that for large M values Alt M1-2 is used in combination with Alt M1-1. - One source [R1-9421-22] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [R1-9421-8] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 104 ppm). For Method Type 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For Method Type 2, depending on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by: - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant. - One Some sources [R1-9421-28][Huawei] report that non-constant OOK chip duration may impact performance, while some other source [R1-9421-32] report that non-constant OOK chip duration does not impact performance. For Alt M2-1, the potential impacts are discussed as follows, - Some sources [R1-9421-5], [R1-9421-9], [R1-9421-8], [R1-9421-33], [R1-9421-21], [R1-9421-11], [R1-9421-18], [R1-9421-3], [Sony] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). Source [Ericsson] report that for M>8, the CP size becomes comparable to that of the normal OOK chip, and hence it would be challenging to identify the invalid transition caused by CP. Sources [CATT] report that if chip duration is comparable to CP duration, CP could not be identified as the invalid chip by the A-IoT device, e.g., M>8. - Some sources [R1-9421-1], [R1-9421-6], [R1-9421-28], [R1-9421-32] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [R1-9421-6], [R1-9421-32] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 105 ppm). - Some sources [R1-9421-28], [R1-9421-9], [R1-9421-32] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. Source [Huawei] report that due non-constant OOK chip duration around CP, Alt M2-1 has ~1dB worse performance than Alt M1-1 at BLER 10% and BLER 1% when it used for small M value (e.g., M = 6). - Some sources [R1-9421-3], [R1-9421-5], [R1-9421-11], [R1-9421-32], [R1-9421-22], [R1-9421-4], [R1-9421-27] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [R1-9421-25], [R1-9421-1], [R1-9421-9] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. - Source [CATT] report that if chip duration is significantly different from CP length, M2-1-2 would be complicated to be used for removing false transition edge occurring at the end of the CP. And M2-1-2 would require high complexity of A-IoT device implementation if it is used for the R2D preamble. For Alt M2-2, the solutions and potential impacts are discussed as follows, - [R1-9421-8], [R1-9421-12], [R1-9421-11], [R1-9421-21], [Panasonic] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [R1-9421-3], [R1-9421-5], [R1-9421-6], [R1-9421-10], [R1-9421-32], [R1-9421-25], [R1-9421-28], [R1-9421-4], [R1-9421-9], [R1-9421-22], [R1-9421-27] report that CP handling of Alt M2-2 would cause interference to NR, while [R1-9421-8] reports single PRB guard band would be sufficient to handle interference. [Panasonic] reports the guard band would anyway be needed when SCS is different between R2D and other NR signal. - Sources [R1-9421-5], [R1-9421-25], [R1-9421-28], [R1-9421-31], [R1-9421-9], [R1-9421-22], [R1-9421-27] report that CP handling of Alt M2-2 would increase the transmitter complexity. …(unchanged parts omitted)…

Agreement

Capture the following TP update into TR 38.769

…(unchanged parts omitted)… Table 6.1.1.x-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.x-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 23. 32; 34. …(unchanged parts omitted)…

Agreement

For R2D FEC, adopt the TP below in Section 6.1.1.x.1 of TR 38.769:

***unchanged parts omitted*** ##### 6.1.1.x.1 Channel coding and CRC PRDCH without FEC is studied as the baseline, with evaluations performed by comparison to this baseline. The study assumes PRDCH can attach a CRC, where the baseline design is using a 6-bit or 16-bit CRC with polynomials as per TS 38.212 [R1-3]. A baseline of no CRC attachment is also included. For the study of CRC designs, see Clause 6.1.0.2. Sources [Huawei], [TCL], [Vivo], [ZTE], [Samsung], [Futurewei][CMCC][Xiaomi][Fujitsu] and [Apple] provide justifications for not having R2D FEC beyond the baseline, with the following observations: - Sources [Huawei], [ZTE], [Futurewei], [Xiaomi] and [Fujitsu] state that FEC decoders require complicated arithmetic or logical operations which are too complicated to be implemented in device 1. - Sources [ZTE] and [Samsung] state that it would be difficult for a device to implement a FEC decoder due to its low power consumption. - Source [Huawei] and [Fujitsu] state that FEC decoder procedures such as the de-interleaving operation or route metric caching require volatile memory of a certain size with a certain reading/writing throughput, which cannot be supported by device 1. - They also mention that the received signal power at the device can be relatively high (e.g., >-60 dBm), making the receiver sensitivity not the bottleneck of the link budget for target coverage, even for device 2b, thus questioning the necessity of R2D FEC. Sources [Ericsson] and [Qualcomm] provide the following justifications for using FEC in R2D for device 2b: - Source [Ericsson] claims that CC with small constraint lengths (e.g., 3 or less) offer a substantial performance gain over uncoded transmission, especially in a fading environment, with reasonable complexity. CC with explicit tail-biting transmission to aid decoding may be suitable for R2D. - Source [QC] claim that even simple block code (e.g., Golay, RM) with hard decisions can significantly reduce the required SNR for achieving a target BLER e.g., 1%. ***unchanged parts omitted***

Agreement

For R2D repetitions, adopt the TP below in Section 6.1.1.x.2 of TR 38.769:

***unchanged parts omitted*** ##### 6.1.1.x.2 Repetition Regarding R2D repetitions, it is reported by sources [R1-9421-11] (only for R2D control, if supported), [R1-9421-12], [R1-9421-32], [R1-9421-13], [R1-9421-21], [R1-9421-19], [R1-9421-28] and [R1-9421-30] that R2D repetitions should be supported. The following are observations regarding the different types of repetition that should be supported. … … ***unchanged parts omitted*** On the other hand, it is reported by sources [Nokia], [Ericsson], [Huawei], [CMCC], [TCL] and [Vivo] that R2D repetitions should not be supported, giving justifications: - Source [Nokia] mention that the transmission power of a R2D transmission is typically much greater than its corresponding D2R transmissions, and if the R2D transmission has coverage issues, then the corresponding D2R transmission would not reach the reader. Hence it should be considered for D2R transmissions alone. - Source [Ericsson], [TCL] say that not supporting R2D repetition can be the baseline. - Source [CMCC], [LG] and [Xiaomi] include that the decision to support R2D repetitions can be based on whether the activation threshold is a bottleneck according to the coverage evaluations. - Source [CMCC][Xiaomi] and [Huawei] also comment that from a device perspective, especially device 1 with low complexity and memory storage, it is not possible to combine multiple repetitions. ***unchanged parts omitted***

Agreement

For R2D repetitions, adopt the TP below in Section 6.1.1.x.2 of TR 38.769:

***unchanged parts omitted*** Bit-level repetition Positive observations: - Source [R1-9421-3] state that bit level repetition can be studied if coverage enhancement of the R2D link is required. - Source [R1-9421-12] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power. - Source [R1-9421-28] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices. Negative observations: - Source [R1-9421-9] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception. - Source [R1-9421-8] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination. - Source [Fujitsu] state that repetition gain of a bit-level repetition, which requires additional standardization effort to define necessary control information, mainly comes from the energy accumulation of the signal, and should be similar with the achievable gain by directly lowering the chip rate/reducing the M value, which does not require this additional effort. Block-level repetition Positive observations: - Source [R1-9421-32] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. - Source [ZTE] further state that the device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost. - Source [Fujitsu] state that block-level repetition can obtain a bigger repetition gain than that achieved by bit- or chip-level repetition, and can enjoy both the time diversity gain and the gain of energy accumulation. Negative observations - Source [R1-9421-8] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded. - Source [Fujitsu] state that block-level repetition additionally requires a very large volatile memory to store all received repetitions of one block. Chip-level repetition Positive observations: - Source [R1-9421-9] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps. - Source [R1-9421-30] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions. Negative observations: - Sources [R1-9421-3], [R1-9421-8] and [R1-9421-11] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option. - Source [Fujitsu] state that repetition gain of a chip-level repetition, which requires additional standardization effort to define necessary control information, mainly comes from the energy accumulation of the signal, and should be similar with the achievable gain by directly lowering the chip rate/reducing the M value, which does not require this additional effort. ***unchanged parts omitted***

Agreement

Add the following TPs to the TR:

#### 6.1.1.x R2D multiplexing For R2D, time-domain multiplexing is the baseline. Code-domain multiplexing is not considered for device 1/2a/2b. Frequency-domain multiplexing is not considered for the devices with an RD-ED receiver (see Clause 5). For device 2b with IF-ED or ZIF receivers, the study considered the following technical aspects: Table 6.1.1.x-1: Observations on the feasibility and necessity of FDM for Device 2b Aspects to be considered for feasibility/benefit; Observations. Inventory completion time; Sources [R1-9421-3], [R1-9421-22], [R1-9421-4], [R1-9421-12] state that FDM is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. Source [vivo] state that inventory latency reduction would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Device implementation; Sources [R1-9421-18], [R1-9421-27], [R1-9421-3], [R1-9421-22], [R1-9421-12] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Source [Panasonic] states that narrowband RF filtering at device side to realize R2D FDM would be challenging considering reception performance and complexity, while such filtering would also limit the deployment scenario supported by device. Sources [R1-9421-34], [R1-9421-24], [R1-9421-6], [R1-9421-2], [R1-9421-5] state that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Source [R1-9421-3] states that the larger R2D responses are harder for the devices to process in the case of TDM+FDM/TDM only for D2R/R2D, respectively.. Spectrum utilization; Source [R1-9421-34] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. Source [Ericsson] state that the spectrum utilization can still be higher for non-RF-ED based devices if FDM is used, despite guard bands. Source [vivo] state that spectrum efficiency improvement would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Coverage (in the case of single reader); Source [R1-9421-5] states the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Source [R1-9421-3] states the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture).. Reader implementation (in the case of single reader); Source [R1-9421-5] states that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions.. Harmonized design for all devices; Sources [R1-9421-26], [R1-9421-9], [R1-9421-19], [Spreadtrum] state that it is not appropriate to include FDM only for Device 2b, while Device 1 and 2a cannot support it. Source [vivo] state that non-harmonized resource allocation for different device types complicates the system design. Source [Ericsson] state that a deployment supporting a combination of RF-ED devices and non-RF ED devices can be harmonized by TDMA’ing different types of R2D time slots, where some time slots can support only a single frequency occasion while other time slots support multiple frequency occasions..

Agreement

In R2D, a chip corresponds to one OOK symbol.

Conclusion

Since R2D chip duration is a consequence of CP handling design, it is not studied further in RAN1, and is left to a later phase.

Agreement

Capture the following TP update into TR38.769

…(unchanged parts omitted)… For all devices, the following D2R baseband modulations are studied: - OOK - Binary PSK - Binary FSK, as MSK (and not GMSK) OOK and BPSK for baseband modulation are feasible for D2R for all devices. - Sources [R1-9421-3], [R1-9421-11], [R1-9421-28], [R1-9421-16] report that MSK is feasible in some way: - [R1-9421-3], [R1-9421-11] say it is feasible for all devices, for example when it is implemented with multiple impedances switching - [R1-9421-28] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b - For device 1 and 2a this type of MSK does not have continuous phase - [R1-9421-3] say that benefits include lower sidelobes than OOK and BPSK, and lower BER than OOK and same BER as BPSK - Sources [R1-9421-5], [R1-9421-2], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-10], [R1-9421-23] report that MSK is either infeasible or should be deprioritized for all devices. - [R1-9421-5], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-2], [R1-9421-10], [R1-9421-23] say that MSK is less spectrally efficient than OOK and BPSK because there are issues due to poor phase accuracy in the device - [R1-9421-5], [R1-9421-7], [R1-9421-2], [R1-9421-8], [R1-9421-10] say that MSK would increase reader and device complexity - [R1-9421-8] say that MSK performance for device 2b would materially degrade due to CFO - [TCL] say that it is difficult to modulate MSK signal using impedance switching due to the implementation complexity, including frequency mapping and phase continuation. [Xiaomi] say that if multiple impedances switching are applied to maintain the phase continuity, it violates the principle of low device cost. …(unchanged parts omitted)…

Agreement

For D2R line codes, FM0 is deprioritized.

Agreement

For small frequency shifts in D2R, adopt the TP below in Section 6.1.2.x.1 of TR 38.769:

##### 6.1.2.x.1 Small frequency shifts ***unchanged parts omitted*** Sources [R1-9421-1], [R1-9421-8], [Huawei] and [R1-9421-28] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. Sources [CMCC] and [Vivo] state that option 1 has a more concentrated spectrum, and requires lesser bandwidth as compared to Option 2. Source [Vivo] further states that while Option 1 and option 2 show similar BLER performance for single device case, Option 1 outperforms Option 2 with FDMA, especially with presence of 105 ppm SFO. Option 1 can achieve additional gain for coverage evaluation due to lower effective noise power. Sources [R1-9421-8], [R1-9421-32] and [R1-9421-13] state that the output waveform for Manchester line codes by Option 2 introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Option 1. ***unchanged parts omitted***

Agreement

For small frequency shifts in D2R using Manchester line codes by multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes Rs number of square wave periods, where Rs = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is Rs/Tb = 1/(2 * chip length).

Agreement

For small frequency shifts in D2R using a square-wave corresponding to the small frequency-shift and no line codes, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

Agreement

For D2R block level repetitions, adopt the TP below in Section 6.1.2.x.3 of TR 38.769:

##### ***unchanged parts omitted*** ##### 6.1.2.x.3 Repetition For definitions of repetition types, see Clause 6.1.0. For D2R, at least block-level and bit-level repetition type 1 and type 2 are studied. Block-level repetition ##### ***unchanged parts omitted*** Performance comparisons - Source [R1-9421-5] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding. - Source [R1-9421-10] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1. - Source [R1-9421-32] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same. - Source [R1-9421-8] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping. - Sources [R1-9421-11] and [R1-9421-32] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel. - Source [Xiaomi] state that for the no FEC case, with 3 times repetition, block level repetition provides ~5dB gain at 1% BLER when compared with bit level type 1 repetition. ##### ***unchanged parts omitted***

Agreement

For D2R FEC, update Table 6.1.2.x.1-1 of Section 6.1.2.x.1 of TR 38.769 as follows:

***unchanged parts omitted*** Table 6.1.2.x.1-1: Summary of study on D2R FEC Option #; CC Design; Pros; Cons. Baseline; Constraint length 7 Code rate 1/3; [R1-9421-8] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. [R1-9421-8] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. [R1-9421-27] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. [R1-9421-27] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. [R1-9421-11] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. [R1-9421-16] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. [R1-9421-10] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 4, code rate 1/3 [R1-9421-10] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 [Nokia] Decoding performance is increased by 3 dB@ 10% BLER with 2 RX, when compared to no CC or repetitions. 1; Constraint length 4 Code rate 1/2 – 1/4; [R1-9421-3] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes.; [R1-9421-9] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. [R1-9421-32] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. [R1-9421-32] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. [CATT] Code rate 1/2, 1/3, TBCC: Decoding performance is decreased by ~1dB@10% BLER, when compared to baseline with TBCC.. ***unchanged parts omitted***

Agreement

Update section 6.1.2.x.1 of the TR on D2R multiple access:

#### D2R multiple access ##### 6.1.2.x.1 Multiple access schemes Time-domain multiple access, and frequency domain multiple access at least by using a small frequency shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [R1-9421-9], [R1-9421-11], [R1-9421-3], [R1-9421-1], [Nokia], state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [R1-9421-27], [R1-9421-22], [R1-9421-24], [CATT], [Nokia], [Qualcomm], state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration. According to sources [R1-9421-3], [R1-9421-9], [R1-9421-11], [R1-9421-35], [R1-9421-27], [R1-9421-1], [R1-9421-5], [R1-9421-25], [Nokia], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [R1-9421-9], and potentially increased power consumption for devices according to sources [R1-9421-11], [R1-9421-17], [R1-9421-12], [Nokia]. It is observed that the performance of FDMA may be impacted by the following aspects. - Large SFO of device - Sources [R1-9421-28], [R1-9421-32] state that large SFO (e.g. up to 105 ppm) produces higher BLER degradation due to inter-device interference than a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO. Source [Qualcomm] states that under the case of the large SFO (e.g. up to 105 ppm), two among four devices using small frequency shifts have BLER floor and cannot achieve BLER 1%. Source [R1-9421-32] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - Source [R1-9421-5] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤1dB) on the performance of FDMA between multiple devices. - Sources [R1-9421-8] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [R1-9421-17] think that the required guard band size increases for higher switching frequencies for passive devices. - Source [CMCC] observed that the performance gap among 10% SFO and 1% SFO (residual) is similar compared FDMA with no FDMA, e.g., about 0.5dB @ 10% BLER, i.e., the performance gap among 10% SFO and 1% SFO (residual) is irrelevant to whether FDMA scheme is used or not. - Source [Huawei] state that large SFO (e.g. up to 105 ppm) has little impact on the link performance of FDMA, as it is observed that the performance gap between 10% and 1% SFO is negligible in the case of FDMA among 4 devices. - Timing offset between devices - Sources [R1-9421-25] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - Maximum range of small frequency shift - Sources [R1-9421-24] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - Harmonics in the backscattered signal - Sources [R1-9421-8] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - Potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision - Source [Sony] thinks that if the guard band size between D2R transmissions is fixed, allocating passive devices with large SFO to frequency shifts closer to the A-IoT carrier frequency and either (1) passive devices with smaller SFO or (2) active devices to frequency shifts further from the A-IoT carrier frequency reduces frequency resource collision. - Number of multiplexed devices - Source [R1-9421-32] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5 - 3dB performance loss from 6 FDMA-ed devices compared to single device. - Source [Sony] thinks that the potential number of multiplexed devices depends in the maximum rate of frequency switching. - Source [Sony] thinks that if the guard band size depends on the SFO capability of the device, the number of multiplexed devices can be increased if passive devices with large SFO are allocated frequency shifts closer to the A-IoT carrier frequency and passive / active devices with smaller SFO are allocated frequency shifts further from the A-IoT carrier frequency. According to sources [R1-9421-11], [IIT], [R1-9421-12], [R1-9421-32], [Nokia], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [R1-9421-27] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [R1-9421-12] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. Source [CATT] thinks that the CDMA scheme is mostly used for the signals without carrying information. However, sources [R1-9421-18], [R1-9421-26], [R1-9421-3], [R1-9421-1], [R1-9421-5], [R1-9421-6], [R1-9421-8], [Nokia] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. Impact of SFO on the performance of CDMA In the case of large SFO (e.g., 105 ppm): - Source [R1-9421-6] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Source [R1-9421-8] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Source [R1-9421-1] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Source [R1-9421-32] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [R1-9421-32] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. - Source [ZTE] state that convolutional codes and BPSK modulation for the CDMA improve the D2R transmission performance with multiple multiplexing devices. Source [ZTE] state that for D2R transmission with TBS16+CRC0, the performance difference between the CDMA scheme using 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is less than 0.5dB, and the CDMA scheme has 1~2dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers. While for D2R transmission with TBS96+CRC16, the performance difference between the CDMA scheme with 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is also less than 0.5dB, and the CDMA scheme has 1~2.5dB SNR gain at 10%BLER compared with FDMA scheme at the same multiplexing device numbers. - Source [Qualcomm R1-9421-28] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences even when power variation is [-9dB, +9dB], while quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Source [R1-9421-5] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Source [R1-9421-12] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. In the case of relatively smaller SFO (e.g., 104 ppm), - Source [Qualcomm R1-9421-28] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences even when power variation is [-12dB, +12dB] when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. Impact of CFO on the performance of CDMA for Device 2b - Source [R1-9421-5] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. Impact of timing offset between CDMed D2R transmissions on the performance of CDMA for Device 2b - Source [R1-9421-31] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Source [R1-9421-12] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Source [R1-9421-24] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Source [R1-9421-21] think that the different propagation delays from devices may also degrades decoding performance. - Source [R1-9421-32] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Source [Qualcomm R1-9421-28] state that the larger power variation of at least up to [-9dB, +9dB] can be addressed by reader receiver and CDMA can achieve significant throughput gain for msg-1 severely degrades the capacity of CDMA for msg-1. - Source [R1-9421-32] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Source [R1-9421-3] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Source [R1-9421-3] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Source [R1-9421-24] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Source [R1-9421-2] think that CDMA leads to higher power consumption and lower data rate. - Source [R1-9421-8] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Source [R1-9421-12] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Source [R1-9421-12] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. - Source [R1-9421-32] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5, the performance difference between the CDMA scheme with 64-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is about 1dB, and the CDMA scheme has about 2.5dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers. - Source [R1-9421-32] state that BPSK modulation and convolutional code for the CDMA further improve the D2R transmission performance with multiple multiplexing devices and multiplexing capacity compared with OOK based modulation. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5 and using convolutional codes and BPSK modulation, the performance difference between the CDMA scheme using 16-length or 64-length orthogonal code for 6 devices and the repetition scheme for single device at 1%BLER is less than 1dB, and the CDMA scheme has about 9dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers.

Conclusion

No further discussion is needed in the SI for the FFS on “the definition of D2R chip duration for MSK” from RAN1#118bis.

Agreement

Capture in the TR:

- “Since device 2b has internal carrier-wave generation, the 2SB and/or 1SB Btx,D2R and Bocc,D2R can be affected by how it performs D2R modulation and baseband waveform generation.”

Agreement

For CRC, adopt the TP below in Section 6.1.1.x.1 of TR 38.769:

#### 6.1.0.2 CRC ***unchanged parts omitted*** For the CRC generator polynomials: Sources [R1-9421-7], [R1-9421-12], [R1-9421-10], [R1-9421-11], [R1-9421-21] recommend that the same polynomials from TS 38.212 are reused, giving justifications: - Sources [R1-9421-11], [R1-9421-12] state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials. - Source [R1-9421-10] states that the device complexity is increased when different polynomials are introduced for the D2R transmission. - Source [R1-9421-7] states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used. On the other hand, sources [R1-9421-32], [R1-9421-28], [TCL] and [Panasonic] recommend that the same polynomial for CRC-6 () from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, , incorporating the CRC-6 polynomial, giving justifications: ***unchanged parts omitted***

Agreement

For CRC, adopt the TP below in Table 6.1.0.2-1 of Section 6.1.1.x.1 of TR 38.769:

***unchanged parts omitted*** Table 6.1.0.2-1: CRC evaluations CRC-6; CRC-16. TBS; Source; CRC overhead; PFA or Pud; Source; CRC overhead; PFA or Pud. 8 bits; [R1-9421-5] [QUALCOMM119]; 43%; ~10^(-8) Pud @10-3 BER; [R1-9421-5] [QUALCOMM119]; 67%; ~10^(-11) Pud @10-3 BER. 12 bits; [R1-9421-3] [QUALCOMM119]; 33% 33%; ~10^(-2) PFA ~10^(-8) Pud @10-3 BER; [R1-9421-3] [QUALCOMM119]; 57% 57%; ~10^(-5) PFA ~10^(-11) Pud @10-3 BLER. 16 bits; [R1-9421-5]; 27%; [R1-9421-5]; 50%. [R1-9421-32] [QUALCOMM119]; 27% 27%; ~10^(-7) Pud @10-3 BER; [R1-9421-32] [QUALCOMM119]; 50% 50%; ~10^(-10) Pud @10-3 BER. 24 bits; [R1-9421-5]; 20%; ~10^(-6) Pud @ 10% BLER and 2.7dB SNR; [R1-9421-5]; 40%; ~10^(-9) Pud @10% BLER and 2.8dB SNR. [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 20% 20% 20%; ~10^(-2) PFA ~10^(-7) Pud @10-3 BER; [R1-9421-3] [QUALCOMM119]; 40% 40%; ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER. 32 bits; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 16% 16% 16%; ~10^(-5) Pud @10-3 BLER; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 33% 33% 33%; ~10^(-10) Pud @10-3 BLER. 40 bits; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 13% 13% 13%; ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 29% 29% 29%; ~10^(-10) Pud @10-3 BLER. 48 bits; [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 11% 11% 11%; ~10^(-2) PFA ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER; [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 25% 25% 25%; ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER. 96 bits; [R1-9421-5] [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 6% 5% 6% 6%; ~10^(-2) PFA ~10^(-4) Pud @10-3 BLER; [R1-9421-5] [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 14% 14% 14% 14%; ~10^(-5) PFA ~10^(-9) Pud @10-3 BLER. 128 bits; [R1-9421-32] [QUALCOMM119]; 4% 4%; ~10^(-3) Pud @10-3 BLER; [R1-9421-32] [QUALCOMM119]; 11% 11%; ~10^(-9) Pud @10-3 BLER. 256 bits; [R1-9421-5] [QUALCOMM119]; 2% 2%; ~10^(-3) Pud @10-3 BLER; [R1-9421-5] [QUALCOMM119]; 6% 6%; ~10^(-9) Pud @10-3 BLER. ***unchanged parts omitted***

Agreement

Capture the following TP update into TR 38.769

OOK and BPSK for baseband modulation are feasible for D2R for all devices. - Sources [R1-9421-3], [R1-9421-11], [R1-9421-28], [R1-9421-16] report that MSK is feasible in some way: - [R1-9421-3], [R1-9421-11], [Wiliot R1-2402720] say it is feasible for all devices, for example when it is implemented with multiple impedances switching. - [R1-9421-28] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b - For device 1 and 2a this type of MSK does not have continuous phase - [R1-9421-3] say that benefits include lower sidelobes than OOK and BPSK, and lower BER than OOK and same BER as BPSK - Sources [R1-9421-5], [R1-9421-2], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-10], [R1-9421-23] report that MSK is either infeasible or should be deprioritized for all devices. - [R1-9421-5], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-2], [R1-9421-10], [R1-9421-23] say that MSK is less spectrally efficient than OOK and BPSK because there are issues due to poor phase accuracy in the device - [R1-9421-5], [R1-9421-7], [R1-9421-2], [R1-9421-8], [R1-9421-10] say that MSK would increase reader and device complexity - [R1-9421-8] say that MSK performance for device 2b would materially degrade due to CFO - Source [MTK] reports that BPSK performance with coherent receiver highly depends on the CFO value and D2R transmission duration after a D2R amble. 2SB modulation is feasible for D2R transmission for all devices. Feasibility and necessity of 1SB modulation would depend on impacts due to issues including: device-side filtering requirements (i.e. image suppression), RF resource usage / spectral efficiency, etc.

Agreement

For small frequency shifts in D2R, update the previously-agreed TP, shown in red text, with the additional text shown in green.

##### 6.1.2.x.1 Small frequency shifts ***unchanged parts omitted*** Sources [R1-9421-1], [R1-9421-8], [Huawei] and [R1-9421-28] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. Sources [CMCC] and [Vivo] state that option 1 has a more concentrated spectrum, and requires lesser bandwidth as compared to Option 2. Source [Vivo] further states that while Option 1 and option 2 show similar BLER performance for single device case, Option 1 outperforms Option 2 with FDMA, especially with presence of 105 ppm SFO. Option 1 can achieve additional gain for coverage evaluation due to lower effective noise power. Source [InterDigital] observed that while Option 1 has a narrower main lobe, it has higher sidelobes than Option 2. Sources [R1-9421-8], [R1-9421-32] and [R1-9421-13] state that the output waveform for Manchester line codes by Option 2 introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Option 1. ***unchanged parts omitted***

##### 2.1.1.4	Frame structure and timing aspects

RAN1#118bis

Agreement

TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions.

- The contents in the Tables are further checked by companies and targeted for completion in RAN1#119.

Agreement

The start indicator part of the R2D time acquisition signal is not included in TD2R_min.

Agreement

For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code.

Agreement

- The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options:

  - Option 1: D2R preamble only

  - Option 2: D2R preamble + X midamble(s), where X 1

  - Option 3: D2R preamble + postamble

  - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1

- For the above options, companies are encouraged to report at least the following:

- Purpose(s) of the preamble, midamble and postamble

- Whether companies assume multiple options can be supported

Agreement

RAN1 studies following:

- A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

- The study includes

  - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

  - Size(s) for resource allocation in the time domain

  - Determination of the X time domain resource(s) by the device

  - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

Agreement

Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following

- How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

Agreement

From RAN1 perspective, capture in the TR that at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type.

- For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic.

Conclusion

For the purpose of the Rel-19 study, the study on DO-A in RAN1 is considered completed based on the above agreement.

Agreement

RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access.

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

Agreement

RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception.

Agreement

For analysing the trade-offs among the D2R amble(s) options, companies can refer to the Table 3.2.4 in section 3.2.4 of R1-2408993 for information.

Agreement

For R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used.

RAN1#119

Agreement

Capture following observations in the TR 38.769, where CFO is assumed to be zero or negligible.

For coherent detection of PDRCH with a payload of 16 bits or 20 bits with 6-bit or 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code:

- Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, when the same amble(s) overhead is maintained, Option 3 provides comparable performance results to Option 1.

- Source [7, Samsung] observed that with up to 10% SFO, ~5kbps data rate, for device 1 and with up to 1% SFO for device 2, the decoding performance with/without midamble are similar

- Source [9, vivo] observed that Option 1 is sufficient to achieve 10% and 1% BLER, with no more than 8 SFO hypotheses tested at the reader side.

  - With up to 10% SFO, ~ 5kbps data rate, the SNR needed to achieve 10% and 1% BLER is similar (~ -2dB and 4 dB) for Option 1, Option 2 of D2R preamble+1midamble and Option 3.

  - With up to 1% SFO, ~ 5kbps data rate, the SNR needed to achieve 10% and 1% BLER is similar (~ -2.8dB and 3.3dB) for Option 1, Option 2 of D2R preamble+1 midamble and Option 3.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <5*10^3 ppm

  - With up to 10% SFO, achieving the required accuracy necessitates more than 20 SFO hypotheses at the reader side for Option 1 and 10 SFO hypotheses are sufficient for Option 3 of D2R preamble + postamble. But for Option 3 reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

  - With up to 1% SFO, 4 SFO hypotheses are sufficient for Option 1 to achieve the required accuracy.

For coherent detection of PDRCH with a payload of 96bits with 16-bit CRC (or 6-bit CRC [14, Xiaomi]), using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- Sources [3, Huawei], [5, CMCC] and [14, xiaomi] observed that Option 1 cannot achieve 10% BLER.

- Sources [6, ZTE], [7, Samsung], [9, vivo], [20, OPPO] and [30, QC] observed that Option 1 can achieve 10% BLER.

- Sources [3, Huawei], [5, CMCC], [6, ZTE], [7, Samsung], [9, vivo], [14, xiaomi], [16, China Telecom] observed that adding additional amble improves the performance.

  - Source [3, Huawei] observed that with up to 10% SFO,

    - Option 2 of D2R preamble+ 1 midamble achieves 10% BLER at SNR around -3dB, but cannot achieve 1% BLER.

    - Option 3 of D2R preamble+ postamble achieves 10% BLER at SNR around -4dB, and can achieve 1% BLER at SNR around 4dB.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 3 allows reader to precisely search and detect the SFO with 0.03% residual SFO at -3dB SNR TDL-A channel, achieving 10% BLER -2.44dB SNR for ~1 kbps data rate and -2.17 dB for ~6 kbps data rate. Source [5, CMCC] further observed that when the reader adopts same number of SFO hypothesis based on preamble, with 1% SFO, Option 3 can achieve 10% BLER at -4.27 dB SNR for ~1 kbps and at -4.29 dB SNR for ~6 kbps, which provides 1~2 dB performance gain when compared to 10% SFO.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble + 1 midamble, option 3, and option 4 of D2R preamble + 1 midamble+postamble achieve basically the same performance, the SNR for 10% BLER is 5dB for 1.25 kbps data rate.

  - Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, and the same amble(s) overhead, Option 3 can provide 1~2 dB, 5dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 1. Additionally, Option 3 can provide ~1dB, 2dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 2 of D2R preamble +1 midamble.

  - Source [7, Samsung] observed that for ~5kbps data rate, compared to option 1,

    - For device 1 with up to 10% SFO, Option 2 of D2R preamble + 1 midamble provides ~0.5 dB SNR gain at 10% BLER with TDL-A channel and ~0.9 dB SNR gain with TDL-D channel.

    - For device 2 with up to 1% SFO, Option 2 of D2R pramble + 1midamble provides ~1 dB SNR gain at 10% BLER with TDL-A channel and ~1.4 dB SNR gain with TDL-D channel.

  - Source [9, vivo] observed that,

    - With up to 10% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~0.7dB and 10dB, respectively; Additionally, maintaining the same amble overhead, Option 2 (D2R preamble + 1 midamble) and Option 3 demonstrate similar performance, achieving 10% and 1% BLER at SNR around -1.7dB and 5.2dB, respectively.

    - With up to 1% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~ -1.3dB and 11dB, respectively. Additionally, with the same amble overhead, the SNR difference between Option 2 (D2R preamble+1midamble) and Option 3 for 10% and 1% BLER is less than 1dB, with SNRs ~ -3.1dB to -2.5dB for 10% BLER and ~3.6dB to 4.5dB for 1% BLER.

  - Source [16, China Telecom] observed that with up to 10% SFO, ~7.5kbps data rate, there is ~6~7dB performance gap at 10% BLER and ~10.5~11.5dB performance gap at 1% BLER between option 2 of D2R preamble+111 midambles and option 1. Note that Source [16, China Telecom] does not use any convolutional code.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <10^3 ppm. To achieve the required accuracy,

  - For Option 1, more than 50 SFO hypotheses at reader side are necessary for device with up to 10% SFO and 6 SFO hypotheses are sufficient at reader side for device with up to 1% SFO.

  - For Option 3, 10 SFO hypotheses are sufficient for device with up to 10% SFO, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

For coherent detection of PDRCH with a payload of 400bits with 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- For option 1 of D2R preamble only,

  - Sources [3, Huawei], [5, CMCC], [6, ZTE], [8, Spreadtrum], [9, vivo], [14, xiaomi] observed that with up to 10% SFO, 10% BLER cannot be achieved.

  - Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission.

- For other amble options,

  - Source [3, Huawei] observed that

    - With accurate SFO estimation, Option 2 of D2R preamble + 4 midambles can achieve 10% BLER at SNR ~ 2.7dB but cannot achieve 1% BLER.

    - With up to 10% SFO, Option 3 cannot achieve 10% BLER.

    - With up to 10% SFO, Option 4 of D2R preamble+2 midambles+postamble achieves 10% BLER at SNR of ~0.25dB; But it cannot achieve 1% BLER. Option 4 of D2R preamble+3 or 4 midambles+postamble, achieves a 10% BLER at an SNR of around -0.2 dB, and achieves 1% BLER at SNR around 9dB or 8dB, respectively.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 4 of D2R preamble combined with 1 to 4 midambles + postamble, achieves 10% BLER at SNR of 2.5 dB, 1 dB, 0.8 dB, or 0.5 dB, respectively, for a data rate of around 1 kbps.

  - Source [6, ZTE] observed that with up to 10% SFO,

    - Option 3 can provide ~5.5 dB performance gain compared to option 2 of D2R preamble+1midamble for 10% BLER, with the same amble(s) overhead for ~1kbps data rate.

    - Option 2 of D2R preamble+1midamble cannot achieve 1% BLER for ~1kbps data rate.

    - Option 4 of the D2R preamble+1 or 2 midamble(s)+postamble, has similar performance, it can achieves a 10% BLER at SNR of -1dB and achieves a 1% BLER at SNR of 6dB and 5dB respectively for ~1kbps data rate.

  - Source [8, Spreadtrum] observed that with up to 10% SFO,

    - Option 3 of D2R preamble+ postamble cannot achieve 10% BLER for ~7kpbs.

    - Option 4 of D2R preamble + 1 midamble + postamble can achieve 10% BLER and 1% BLER at SNR around -6dB and 0 dB, respectively for ~7kpbs data rate.

  - Source [9, vivo] observed that

    - With up to 10% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1 postamble demonstrate similar performance, achieving 10% BLER at SNR ~0.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~ 9.2dB and 12.8dB, respectively for ~5.5kpbs data rate.

    - With up to 1% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1postamble demonstrate similar performance, achieving 10% BLER at SNR around -1.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~7.8dB and 9.1dB, respectively for ~5.5kpbs data rate.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble+3 midambles and Option 4 of D2R preamble+3 midambles+postamble can achieve 10% BLER when the SNR is within the range of [15, 25] dB for 1.25 kbps data rate.

  - Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is much smaller than 10^3 ppm. To achieve the required accuracy,

    - For Option 2 of D2R preamble+X midamble(s) where midamble inserted per every certain number of PDRCH bits (e.g., 192 bits),

      - For SFO estimation using each amble for the subsequent PDRCH bits (e.g., 192 bits), with up to 10% SFO, more than 50 SFO hypotheses are necessary at the reader side and with up to 1% SFO, 6 SFO hypotheses are sufficient at the reader side.

      - For SFO estimation based on the time gap between preamble and midamble, with up to 10% SFO, 10 SFO hypotheses are used, but reader has to store the received samples and wait for the midamble to start SFO/channel/interference estimation, demodulation, decoding, etc.

    - For Option 3 of D2R preamble+postamble, SFO estimation is based on the time gap between preamble and postamble, with up to 10% device SFO, 10 SFO hypotheses are used for reader, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc

For the synchronization and timing tracking of D2R transmission,

- Source [5, CMCC] report that with up to 10% SFO, option 1 is not sufficient for D2R reception since the residual SFO at reader side is larger than 1%. While with option 3, the reader can precisely search and detect the SFO with a residual SFO of 0.03% at -3dB SNR TDL-A channel.

- Source [14, xiaomi] report that

  - For packet size of 96bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO over 100ppm decreases to 6% for Option 2, 3 and 4, but remains at 95% for Option 1.

  - For packet size of 400bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO larger than 10ppm decreases to 5% for Option 2, 3, and 4, but is still 99.6% for Option 1.

- Sources [9, vivo], [15, CATT] report that SFO estimation based on D2R preamble can achieve accurate estimation without additional ambles (midamble or postamble).

  - Source [9, vivo][7 Samsung] observed that for non-coherent detection of PDRCH, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for option 2, 3 and 4 compared to the option 1. Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and prevents pipelined processing of the reception.

  - Source [15, CATT] observed that

    - The coarse estimation of SFO based on the D2R preamble indicates that the SFO estimation error is less than 1% with a probability of 99.3%, and less than 0.1% with a probability of 49.9%.

    - The fine estimation of SFO based on the D2R preamble shows that the SFO estimation error is less than 1% with a probability of 99.5%, and less than 0.1% with a probability of 90.8%.

    - Reader/gNB can achieve a probability of not less than 99.5% for SFO estimation error below 1%, and 90.8% for SFO estimation error below 0.1% by receiving D2R preamble signals.

- Source [30, Qualcomm] report that for D2R with coherent demodulation at reader, the reader needs to estimate the device clock frequency with the accuracy of 0.5% (5 * 10^3 ppm) or lower for a short message (e.g., 72 bits after CRC/coding) and 0.1% (10^3 ppm) or lower for a long message (e.g., 224 bits after CRC/coding). The source further reports that design of D2R amble(s) (e.g., overhead) and the correspondingly required number of SFO hypothesis for the estimation depend on the sampling clock accuracy that the device uses for D2R.

Note: in the observations above where coherent detection is used, sources that evaluated option 3 and option 4 assumed that the postamble is used at least for time/frequency tracking and for channel estimation.

Agreement

For the CFO calibration signal, which is required only for device 2b to reduce the frequency offset range and the guard-bandwidth of D2R transmission, the following observations are captured in TR 38.769:

- Source [3, Huawei] report that a single-tone RF signal is used as the CFO calibration signal, it is not a part of time acquisition signal and can be transmitted as an optional R2D signal after the PRDCH transmission.

- Sources [2, Ericsson], [19, Panasonic] and [20, OPPO] report that additional synchronization signal is needed.

  - [OPPO] state the R2D timing acquisition signal may not be sufficient or may not be usable for CFO calibration since a reference frequency is needed when separate LOs are used for Tx and Rx in device 2b.

- Sources [7, Samsung], [9, vivo], [30, Qualcomm], [36, Apple] report that additional synchronization signal is needed if the synchronization for carrier frequency using R2D signal/channel does not provide required functionalities for device 2b.

- Source [5, CMCC][31, MTK] report that it may not be possible to achieve enough frequency accuracy (0.01 ppm) even after CFO calibration based on R2D time acquisition signals for coherent detection at reader especially when the D2R data rate is low.

Agreement

For device 2b, a signal for CFO calibration should be provided to synchronize / calibrate the device clock for LO for carrier frequency (Clock purpose #5) to achieve the accuracy after clock sync / calibration at device side captured in Table 5.2.3-1.

- Frequency calibration at device 2b is beneficial at least to reduce the guard-bandwidth of D2R transmission.

Agreement

Adopt the updates documented in R1-2410653 for section 6.2 of the TR38.769.

Agreement

Adopt following update to the TP agreed on Monday

Capture following observations in the TR 38.769, where CFO is assumed to be zero or negligible.

[omit unchanged part]

For coherent detection of PDRCH with a payload of 96bits with 16-bit CRC (or 6-bit CRC [14, Xiaomi]), using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- Sources [3, Huawei], [5, CMCC] and [14, xiaomi] observed that Option 1 cannot achieve 10% BLER.

- Sources [6, ZTE], [7, Samsung], [9, vivo], [20, OPPO] and [30, QC] observed that Option 1 can achieve 10% BLER.

- Sources [3, Huawei], [5, CMCC], [6, ZTE], [7, Samsung], [9, vivo], [14, xiaomi], [16, China Telecom] observed that adding additional amble improves the performance.

  - Source [3, Huawei] observed that with up to 10% SFO,

    - Option 2 of D2R preamble+ 1 midamble achieves 10% BLER at SNR around -3dB, but cannot achieve 1% BLER.

    - Option 3 of D2R preamble+ postamble achieves 10% BLER at SNR around -4dB, and can achieve 1% BLER at SNR around 4dB.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 3 allows reader to precisely search and detect the SFO with 0.03% residual SFO at -3dB SNR TDL-A channel, achieving 10% BLER -2.44dB SNR for ~1 kbps data rate and -2.17 dB for ~6 kbps data rate. Source [5, CMCC] further observed that when the reader adopts same number of SFO hypothesis based on preamble, with 1% SFO, Option 3 can achieve 10% BLER at -4.27 dB SNR for ~1 kbps and at -4.29 dB SNR for ~6 kbps, which provides 1~2 dB performance gain when compared to 10% SFO.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble + 1 midamble, option 3, and option 4 of D2R preamble + 1 midamble+postamble achieve basically the same performance, the SNR for 10% BLER is 5dB for 1.25 kbps data rate.

  - Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, and the same amble(s) overhead, Option 3 can provide 1~2 dB, 5dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 1. Additionally, Option 3 can provide ~1dB, 2dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 2 of D2R preamble +1 midamble.

  - Source [7, Samsung] observed that for ~5kbps data rate, compared to option 1,

    - For device 1 with up to 10% SFO, Option 2 of D2R preamble + 1 midamble provides ~0.5 dB SNR gain at 10% BLER with TDL-A channel and ~0.9 dB SNR gain with TDL-D channel.

    - For device 2 with up to 1% SFO, Option 2 of D2R pramble + 1midamble provides ~1 dB SNR gain at 10% BLER with TDL-A channel and ~1.4 dB SNR gain with TDL-D channel.

    - Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevent pipelined processing of the reception.

  - Source [9, vivo] observed that,

    - With up to 10% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~0.7dB and 10dB, respectively; Additionally, maintaining the same amble overhead, Option 2 (D2R preamble + 1 midamble) and Option 3 demonstrate similar performance, achieving 10% and 1% BLER at SNR around -1.7dB and 5.2dB, respectively.

    - With up to 1% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~ -1.3dB and 11dB, respectively. Additionally, with the same amble overhead, the SNR difference between Option 2 (D2R preamble+1midamble) and Option 3 for 10% and 1% BLER is less than 1dB, with SNRs ~ -3.1dB to -2.5dB for 10% BLER and ~3.6dB to 4.5dB for 1% BLER.

  - Source [16, China Telecom] observed that with up to 10% SFO, ~7.5kbps data rate, there is ~6~7dB performance gap at 10% BLER and ~10.5~11.5dB performance gap at 1% BLER between option 2 of D2R preamble+111 midambles and option 1. Note that Source [16, China Telecom] does not use any convolutional code.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <10^3 ppm. To achieve the required accuracy,

  - For Option 1, more than 50 SFO hypotheses at reader side are necessary for device with up to 10% SFO and 6 SFO hypotheses are sufficient at reader side for device with up to 1% SFO.

  - For Option 3, 10 SFO hypotheses are sufficient for device with up to 10% SFO, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

For coherent detection of PDRCH with a payload of 400bits with 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- For option 1 of D2R preamble only,

  - Sources [3, Huawei], [5, CMCC], [6, ZTE], [8, Spreadtrum], [9, vivo], [14, xiaomi] observed that with up to 10% SFO, 10% BLER cannot be achieved.

  - Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission.

- For other amble options,

  - Source [3, Huawei] observed that

    - With accurate SFO estimation, Option 2 of D2R preamble + 4 midambles can achieve 10% BLER at SNR ~ 2.7dB but cannot achieve 1% BLER.

    - With up to 10% SFO, Option 3 cannot achieve 10% BLER.

    - With up to 10% SFO, Option 4 of D2R preamble+2 midambles+postamble achieves 10% BLER at SNR of ~0.25dB; But it cannot achieve 1% BLER. Option 4 of D2R preamble+3 or 4 midambles+postamble, achieves a 10% BLER at an SNR of around -0.2 dB, and achieves 1% BLER at SNR around 9dB or 8dB, respectively.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 4 of D2R preamble combined with 1 to 4 midambles + postamble, achieves 10% BLER at SNR of 2.5 dB, 1 dB, 0.8 dB, or 0.5 dB, respectively, for a data rate of around 1 kbps.

  - Source [6, ZTE] observed that with up to 10% SFO,

    - Option 3 can provide ~5.5 dB performance gain compared to option 2 of D2R preamble+1midamble for 10% BLER, with the same amble(s) overhead for ~1kbps data rate.

    - Option 2 of D2R preamble+1midamble cannot achieve 1% BLER for ~1kbps data rate.

    - Option 4 of the D2R preamble+1 or 2 midamble(s)+postamble, has similar performance, it can achieve a 10% BLER at SNR of -1dB and achieves a 1% BLER at SNR of 6dB and 5dB respectively for ~1kbps data rate.

  - Source [8, Spreadtrum] observed that with up to 10% SFO,

    - Option 3 of D2R preamble+ postamble cannot achieve 10% BLER for ~7kpbs.

    - Option 4 of D2R preamble + 1 midamble + postamble can achieve 10% BLER and 1% BLER at SNR around -6dB and 0 dB, respectively for ~7kpbs data rate.

  - Source [9, vivo] observed that

    - With up to 10% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1 postamble demonstrate similar performance, achieving 10% BLER at SNR ~0.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~ 9.2dB and 12.8dB, respectively for ~5.5kpbs data rate.

    - With up to 1% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1postamble demonstrate similar performance, achieving 10% BLER at SNR around -1.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~7.8dB and 9.1dB, respectively for ~5.5kpbs data rate.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble+3 midambles and Option 4 of D2R preamble+3 midambles+postamble can achieve 10% BLER when the SNR is within the range of [15, 25] dB for 1.25 kbps data rate.

  - Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is much smaller than 10^3 ppm. To achieve the required accuracy,

    - For Option 2 of D2R preamble+X midamble(s) where midamble inserted per every certain number of PDRCH bits (e.g., 192 bits),

      - For SFO estimation using each amble for the subsequent PDRCH bits (e.g., 192 bits), with up to 10% SFO, more than 50 SFO hypotheses are necessary at the reader side and with up to 1% SFO, 6 SFO hypotheses are sufficient at the reader side.

      - For SFO estimation based on the time gap between preamble and midamble, with up to 10% SFO, 10 SFO hypotheses are used, but reader has to store the received samples and wait for the midamble to start SFO/channel/interference estimation, demodulation, decoding, etc.

    - For Option 3 of D2R preamble+postamble, SFO estimation is based on the time gap between preamble and postamble, with up to 10% device SFO, 10 SFO hypotheses are used for reader, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc

  - Source [7, Samsung] observes that the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevent pipelined processing of the reception.

For the synchronization and timing tracking of D2R transmission,

- Source [5, CMCC] report that with up to 10% SFO, option 1 is not sufficient for D2R reception since the residual SFO at reader side is larger than 1%. While with option 3, the reader can precisely search and detect the SFO with a residual SFO of 0.03% at -3dB SNR TDL-A channel.

- Source [14, xiaomi] report that

  - For packet size of 96bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO over 100ppm decreases to 6% for Option 2, 3 and 4, but remains at 95% for Option 1.

  - For packet size of 400bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO larger than 10ppm decreases to 5% for Option 2, 3, and 4, but is still 99.6% for Option 1.

- Sources [9, vivo], [15, CATT] report that SFO estimation based on D2R preamble can achieve accurate estimation without additional ambles (midamble or postamble).

  - Source [9, vivo][7 Samsung] observed that for non-coherent detection of PDRCH, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for option 2, 3 and 4 compared to the option 1. Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevents pipelined processing of the reception.

  - Source [15, CATT] observed that

    - The coarse estimation of SFO based on the D2R preamble indicates that the SFO estimation error is less than 1% with a probability of 99.3%, and less than 0.1% with a probability of 49.9%.

    - The fine estimation of SFO based on the D2R preamble shows that the SFO estimation error is less than 1% with a probability of 99.5%, and less than 0.1% with a probability of 90.8%.

    - Reader/gNB can achieve a probability of not less than 99.5% for SFO estimation error below 1%, and 90.8% for SFO estimation error below 0.1% by receiving D2R preamble signals.

- Source [30, Qualcomm] report that for D2R with coherent demodulation at reader, the reader needs to estimate the device clock frequency with the accuracy of 0.5% (5 * 10^3 ppm) or lower for a short message (e.g., 72 bits after CRC/coding) and 0.1% (10^3 ppm) or lower for a long message (e.g., 224 bits after CRC/coding). The source further reports that design of D2R amble(s) (e.g., overhead) and the correspondingly required number of SFO hypothesis for the estimation depend on the sampling clock accuracy that the device uses for D2R.

- Source [37, MediaTek] reports that transmitting 96-bit packet size with 16-bit CRC requires residue SFO after reader compensation to be 1000 ppm, and transmitting 1000-bit packet size with 16-bit CRC requires residue SFO after reader compensation to be 100 ppm.

Note: in the observations above where coherent detection is used, sources that evaluated option 3 and option 4 assumed that the postamble is used at least for time/frequency tracking and for channel estimation.

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring

- For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device,

  - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource.

  - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources.

- For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring

- Option 1: Predefined

- Option 2: Indicated in the R2D transmission triggering random access

- Note: Option 1 and Option 2 may not be mutually exclusive.

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options for a device to determine the reference time for the starting time for Msg2 monitoring

  - Option 1: End of time domain resource where the device transmitted the Msg1

  - Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s)

  - Option 3: End of the R2D transmission triggering random access

  - Option 4: End of a R2D transmission carrying Msg2

  - Note the reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

##### 2.1.1.5	Downlink and uplink channel/signal aspects

RAN1#118bis

Agreement

For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769:

- Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration

Agreement

For the PRDCH design details, following is captured in the TR 38.769:

Following design options have been studied for PRDCH:

- Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered

  - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling

- Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied

  - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data

  - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively

Agreement

For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769:

- Following options have been studied for the start-indicator part of the R2D time acquisition signal:

  - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including

    - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations

    - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration

  - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation

  - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions.

- Miss-detection ratio (MDR), false-alarm ratio (FAR) and detection complexity have been considered for the design of the R2D start indicator part by following companies

  - It is observed by 1 source [Huawei] that for an FAR of ~0%, the MDR of less than 1% can be achieved with Alt 2 of option 1 (considering 2 ON-OFF transmissions with different durations) and it is also observed that low-complexity and reduced power consumption can be achieved

  - 1 source [ZTE] evaluated Alt 1 of option 1 (considering same duration for ON and OFF) and Alt 2 of option 1 (considering multiple ON-OFF transmissions with same duration) and observed that for an FAR of ~0%, the MDR of less than 1% can be achieved and Alt 1 of option 1 performs better than Alt 2 of option 1.

  - 1 source [CATT] observed with ON-OFF pattern, that for an FAR of ~0%, the MDR of less than 1% can be achieved with a duration of at least 1 OFDM symbol

  - 1 source [Qualcomm] compares the performance between option 1 and option 2. It shows almost similar coverage range (SNR requirement) for target MDR of 1%. For MDR of 10%, it shows that sequence-based design provides better performance, and it is observed that during the available time, it is feasible for all devices to detect the start-indicator sequence within the power budget. It is further observed that the FAR with sequence-based design can be improved in case of interference scenarios when compared with pattern-based design.

- For both the options, it may be beneficial that the start-indicator part is distinguishable at least from other parts of the R2D transmissions

Agreement

For single PDRCH generation at the device, following options are captured in TR 38.769:

- Option 1: With line coding

  - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1

  - Note: Presence of the block “small frequency shift with square wave generation” depends on the agreed options for small frequency shift under agenda 9.4.2.1

  - Note: the figure below is for the case where FEC and repetition are present

- Option 2: Without line coding

  - Note: Location of repetition block depends on the repetition types agreed to be studied under agenda 9.4.2.1

  - Note: the figure below is for the case where FEC and repetition are present

Note: Further updates to PDRCH generation can be considered, depending on the progress

Agreement

For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied:

- Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M

- Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant

- FFS: Whether/what restriction on M values for the clock-acquisition part

Note: Other functionalities of clock-acquisition part is a separate discussion

Agreement

For the D2R preamble, binary signal is considered.

Agreement

For the topology 2, following is captured in the TR 38.769:

For resource allocation and/or controlling of intermediate UE in topology 2 for R2D and D2R transmissions via the Uu interface between the BS and intermediate UE, following two options have been studied:

    - Option 1: Higher-layer signaling is used

    - Option 2: L1 and higher-layer signaling are used

From RAN1 perspective, detailed aspects on the two options are left up to work item phase.

Following observations related to the two options have been made by following companies:

    - Four sources [Ericsson, Continental, Vivo, Mediatek] observed that option 1 can save overhead compared to option 2.

    - One source [Huawei] observed that the timing gap between R2D message and D2R message is not extended, and inventory rate is comparable with UHF RFID and furthermore, it is observed that with option 1, the standard impact includes no or less impact on L1 signaling for resource allocation and controlling of intermediate UE

    - Six sources [Ericsson, Qualcomm, Oppo, Futurewei, IIT-K, Cewit] observed that option 2 with L1 signaling provides better resource utilization compared to option 1

For the purpose of the Rel-19 study, the study on resource allocation and/or controlling of intermediate UE in topology 2 in RAN1 is considered completed based on two options above. Additional observations can be captured at RAN1#119.

RAN1#119

Agreement

Following observations on R2D clock-acquisition part are captured in TR 38.769:

- On impact/restriction of M values for the clock-acquisition part

  - 9 sources [TCL, Nokia, Huawei, CMCC, ZTE, Apple, CATT, Mediatek, Qualcomm] provided observations on the impact/restriction of M values for the clock-acquisition part design requirements:

- 1 source [Nokia] observed that increasing value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length and use of  M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

- 2 sources [TCL, Huawei] observed for option 1 of the clock-acquisition part design that no restriction is required to be placed on the M values. Furthermore, 1 source [Huawei] observed that the same 2 ON-OFF voltage (with the same duration) satisfies the FDR performance metric of less than 1% for different M values, e.g., M = 2, 6 and 24, where FDR is the False detection ratio (FDR), i.e. incorrectly calculating M, is the performance metric.

- 1 source [CMCC] observed that pattern of the clock-acquisition part is related to M chips per OFDM symbol and when M is small, the clock-acquisition part may cross multiple OFDM symbols, and the CP insertion may degrade the timing acquisition performance.

- 1 source [ZTE] observed that with option 2, the duration of the clock-acquisition part remains consistent across all M values, at least three OFDM symbols maybe required for clock-acquisition part and it maybe not as efficient as option1

- 1 source [Apple] observed that among the two options studied for the clock-acquisition part, option 2 provides increased robustness, especially in case of large value of M, when compared to option 1 and potentially increase the detection performance of the clock-acquisition part.

- 1 source [CATT] observed that if the chip duration is variable based on the M value used for OOK-4 waveform, the detection performance would be limited by the received SINR of the CAP with clear transition of the rising and falling edges.

- 1 source [Qualcomm] observed that the option 1 with M>1 has shorter duration of clock acquisition part than M=1 and worse timing acquisition accuracy. At least part of PRDCH following the clock acquisition part may need to be used to improve the timing acquisition. Furthermore, the larger M (e.g., M>4) with small chip duration is more sensitive to the SFO accuracy and the restriction of M for the clock acquisition part may be needed.

- 1 source [Mediatek] further observed that different M values may impact the chip accuracy obtained by the clock acquisition part.

- On impact of CP insertion/handling on the clock-acquisition part

  - 10 sources [TCL, CMCC, ZTE, Samsung, Vivo, CATT, NTT Docomo, Qualcomm, Mediatek, Spreadtrum] observed that the CP insertion/handling may impact the design requirements of the clock-acquisition part:

  - 1 source [CMCC] further observed that when the clock-acquisition part occupies more than one OFDM symbol, ON-OFF state transition around CP can avoid the error rising or falling edges due to the CP insertion.

  - 1 source [ZTE] further observed that to mitigate the impact of the CP in the clock-acquisition part for large M values, it can reuse the CP handling method for PRDCH

  - 1 source [Samsung] further observed that CP insertion/handling on the clock-acquisition part can cause false rising/falling transition and, therefore, the clock acquisition part should be designed such that it does not incur a false rising or falling edges due to CP insertion when CP-OFDM is used for OOK signal generation.

  - 1 source [vivo] further observed that CP insertion/handling on the clock acquisition part will impact the chip duration estimation accuracy. It is further observed that for CP handling, device may not be able to count the clock and estimate OFDM symbol duration accurately until the clock acquisition part if the start indicator only includes a single ON-OFF transmission.

  - 1 source [CATT] further observed that the SER will be degraded due to uneven chip interval when the CP is inserted within an OFDM symbol, where SER refers to the number of samples which is mismatched for comparing to the total number of samples in a chip.

    - 1 source [NTT Docomo] further observed if CP insertion would cause false rising/falling edges, accuracy of timing acquisition may be impacted.

  - 1 source [Mediatek] further observed that the issues of chip extension, false raising/falling transition, and additional raising/falling transition caused by CP insertion/handling considering different M values will impact the chip accuracy obtained by the clock acquisition part.

  - 1 source [Spreadtrum] further observed that the design of clock acquisition part should consider that CP insertion does not cause a false rising or falling edges and does not cause different length of multiple high / low voltages within the clock acquisition part when the clock acquisition spans multiple OFDM symbols.

  - 1 source [Huawei] observed CP insertion/handling may not impact the design requirements of the clock-acquisition part

Agreement

For the D2R preamble design, following aspects have been studied and can be captured in the TR 38.769:

- Autocorrelation Property

  - 10 sources [Nokia, Huawei, CMCC, Xiaomi, CATT, Oppo, Ericsson, NTT Docomo, Qualcomm, ZTE] observed that the signal should have good autocorrelation properties for accurate peak detection based on the signal correlation at the reader

- Cross-correlation Property

  - 7 sources [Nokia, CMCC, Oppo, Ericsson, Qualcomm, ZTE, CATT] observed that the signal should have good cross-correlation properties if multiple D2R preamble sequences are considered (e.g. for multiple access schemes (if supported) for D2R transmissions).

- Line coding

  - 1 source [Nokia] observed that line coding may impact the autocorrelation property of the sequence.

  - 1 source [Huawei] observed that for D2R preamble, to apply backscattering, line coding can help improve the detection performance based on shifting the D2R signal’s frequency location away from the carrier wave

- Sequence Types (not limited to below types only)

  - M-sequence

    - 3 sources [Nokia, Vivo, Xiaomi] observed that m-sequence can be considered for D2R preamble mainly owing to good correlation properties.

  - Golay sequence

    - 4 sources [CMCC, Vivo, Xiaomi, Samsung] observed that Golay sequence can be considered for D2R preamble mainly owing to good correlation properties and availability of large number of distinct sequences and complementary pairs.

  - Walsh sequence

    - 1 source [Oppo] observed that Walsh sequence can be considered as a candidate for D2R preamble thanks to its good auto/cross-correlation property and flexible length

- General Observations

  - 1 source [Huawei] observed can achieve 0.97% residual SFO with 98% probability under -2.5dB SNR and 0.1% MDR with [-1/8, 1/8] chip timing error with 99.05% probability under -2.5dB SNR with D2R preamble including 2-parts with clock-like sampling frequency signal and timing-acquisition signal, having 32-length ‘1’ sequence (encoded to 64-chip Manchester code) and 32-length sequence (encoded to 64-chip Manchester code), respectively.

  - 4 sources [TCL, CMCC, ZTE, Vivo] observed that for D2R preamble with binary signal, the timing synchronization performance is highly related to the sequence length of the preamble. Furthermore, 1 source [CMCC] observed that to achieve a BLER performance at 10%, the timing synchronization error should be less than 10%. Furthermore, 1 source [ZTE] observed that the channel estimation performance is also highly related to sequence length. 1 source [ZTE] observed that using a 32 bits preamble provides ~8 dB, ~5 dB performance gain than using 8 bits, 16 bits preamble, respectively. And using a 64 bits preamble provides ~2.5dB performance gain than using a 32 bits preamble.

  - 1 source [Ericsson] observed that for D2R preamble with binary signal, normalized SFO estimation error of less than 10% can be achieved with a training sequence length 64 or longer. The simulated D2R preamble consisting of a Golay complementary pair can tolerate SFO up to 1% (AWGN) with up to 1 dB loss in performance for a sufficiently long preamble sequence length (32 or greater).

Agreement

For determining the end of PRDCH at the device, following two options are studied and captured in the TR 38.769:

- Option 1: TBS information (via implicit/explicit L1 R2D control information)

- Option 2: Postamble (at the end of PRDCH)

14 sources [Nokia, Huawei, ZTE, CMCC, Samsung, Ericsson, Oppo, LGE, Qualcomm, Spreadtrum, Mediatek, Cewit, Ericsson, vivo] provided following observations on the above two options for determining the end of PRDCH:

- 3 sources [Nokia, Huawei, ZTE] observed that option 2 provide two benefits, namely, the variable payload length and to provide timing acquisition before the subsequent transmission of either PDRCH or PRDCH, thus improving the detectability at both reader and the device, respectively. Furthermore, 1 source [Huawei] observed that R2D postamble indicates the TBS with high efficiency for small packets by avoiding a large padding overhead, unlike option 1, which may require devices to perform blind detection of different PRDCH formats (if supported) and the overhead caused by the inclusion of a R2D postamble does not exceed 20% for even the smallest of message sizes and may be less than the signaling overhead caused by using a dedicated TBS indicator

- 1 source [CMCC] observed for option 2, that for small payload size with only a few bits, the presence of long postamble generates large resource overhead, while for large payload size with more bits, the resource overhead of postamble is smaller.

- 1 source [vivo] observed for option 2, that for small payload size with only a few bits, the presence of long postamble generates large resource overhead.

- 1 source [Samsung] observed option 2 is not strictly required, however, given the possible clock drift at a device, it may be still beneficial to also attach postamble at least for the determination of the end of PRDCH at a device.

- 3 sources [Oppo, Spreadtrum, CEWiT] observed that with option 2, the false detection may be higher for shorter postamble. Source [OPPO[ observed that in contrast to option 2, it is more reliable and efficient to indicate TBS with control information in option 1

- 2 sources [LGE, vivo] observed that if a message type or a command ID is included in L1 control information and implicitly indicates a known size of a fixed TB, then there is no need for either option 1 or option 2

- 2 sources [Qualcomm, vivo] observed that option 1 has the advantages of avoiding blind detection of postamble and providing the power saving for non-target devices to skip the R2D detection.

- 1 source [MediaTek] observed that option 1 is feasible for the device to avoid the unnecessary reception of a TB with a specific size and thus enable power saving, e.g., when the TB has a size exceeding the allowance of the device remaining power.

- 1 source [Ericsson] observed option 2 is not strictly required if the end of PRDCH can be explicitly indicated by R2D control information, and it is subject to the miss-detection rate. It may be beneficial if a PRDCH postamble can serve as an additional timing acquisition signal prior to a PDRCH transmission.

Agreement

For transmission of R2D control information for R2D reception and D2R scheduling, following two options are studied and captured in the TR 38.769:

- Option 1: L1-control signaling

- Option 2: Higher-layer signaling

12 sources [Huawei, CMCC, ZTE, Samsung, Oppo, Qualcomm, Futurewei, Spreadtrum, NTT Docomo, Cewit, Ericsson, vivo] provided following observations on the above two options for signaling R2D control information:

- 1 source [Huawei] observed that option 2 avoids physical layer design, while at the same time offering flexibility in terms of the size of the grant. On the other hand, option 1 may require the device to perform blind detection of different PRDCH formats (if supported), while at the same time, reducing the flexibility and resource utilization efficiency. Option 1 would also require a separate CRC, which would result in breaking the pipelining of the CRC decoding due to the added latency and increases overhead.

- 3 sources [CMCC, ZTE, Futurewei] observed that for D2R scheduling information, as the PDRCH transmission only occurs when the device can successfully decode the preceding PRDCH transmission, therefore no benefit is observed for option 1 compared to option 2. Furthermore, one source [CMCC] also observed that for R2D reception, if TBS of PRDCH is indicated by R2D control information, such R2D control information should be carried via L1 signaling as the device needs to understand the packet size of PRDCH before decoding.

- 1 source [Samsung] observed that for D2R scheduling information indication, option 1 has an advantage as the decoding of D2R scheduling information is decoupled from that of R2D payload and the device can early start the preparation of the PDRCH. For R2D scheduling information indication, Option 1 has an advantage as the device can early detect whether the corresponding R2D transmission is addressed to the device or not, which also saves device’s power consumption.

- 1 source [Oppo] observed that D2R scheduling information can be transmitted as physical layer signalling or higher layer signalling as there is no fundamental difference between the 2 options.

- 1 source [Qualcomm] observed that option 1 allows the decoding of R2D and D2R scheduling information to occur before the decoding of the R2D payload. The D2R scheduling in L1 control enables the device to provide feedback on the (un)successful detection of the R2D payload, e.g., due to insufficient energy/power or channel/interference variation. Additionally, 6 sources [ZTE, Qualcomm, CMCC, Samsung, OPPO, vivo] observed that the R2D scheduling information in L1 control enables early identification of target devices and power saving for non-target devices by allowing them to skip the following R2D detection.

- 1 source [Spreadtrum] observed that device can decode L1 control information first which can allow device to obtain the scheduling information (e.g. TBS rated information) before data part reception in option 1 and if device knows from control information that the following data is not for it, the data reception can be skipped.

- 1 source [NTT Docomo] observed that option 1 allows the decoding of R2D control information separately from the subsequent R2D reception, which enables devices to feedback when subsequent R2D reception is failed and also enables identification of target devices before the subsequent R2D reception.

- 1 source [CEWiT] observed that Option 1 enables the separate decoding of R2D control information before R2D data reception, which supports feedback if R2D reception fails. Additionally, L1 control can carry D2R scheduling info, allowing devices to avoid the non-intended R2D reception.

- 1 source [Ericsson] observed that with Option 1, L1 R2D control information may include important information needed to decode the data portion of the PRDCH signal. This requires L1 R2D control information part to have a fixed payload size and self-decodable with separate CRC.

Agreement

For D2R scheduling, midamble (if supported) related information can be explicitly/implicitly indicated via corresponding PRDCH.

Agreement

Following observations related to the agreed options on resource allocation and/or controlling of intermediate UE in topology 2 are additionally captured in TR 38.769:

  - 1 source [Vivo] observed that if UE determines the resource used for A-IoT transmission within A-IoT resources configured by the network, option 1 provides better resource efficiency and option 2 enables faster A-IoT resource adaptation compared to option 1. And it is further observed that if the resource allocation from NW is for each R2D and/or D2R transmission and is indicated by a L1 signaling of option2, option 2 may affect the A-IoT communication timing relations and causes substantial overhead between NW and UE reader and may lead to resource inefficiency as NW lacks direct information of channel condition between A-IoT device and UE reader.

  - 1 source [Xiaomi] observed that option 1 can save signaling overhead b/w gNB and intermediate UE compared to option 2 but may cause resource waste issue. For resource allocation of intermediate UE, Option 2 can provide better resource efficiency compared to Option 1 but requires more signaling overhead b/w gNB and intermediate UE.

  - 1 source [CATT] compared option 1 and option 2 in terms of signaling overhead, signaling latency, flexibility and resource utilization. It is observed that signaling overhead is higher with option 2, but latency is higher with option 1. Option 2 may provide higher flexibility and better resource utilization compared to option 1.

  - 1 source [Fujitsu] observed that option 1 only requires the standardization workload for adding new higher layer signaling and does not impact the current DCI design for UEs, but the resource utilization efficiency of option 1 may be lower than that of option 2. Option 2 can provide better resource usage efficiency than that of option 1, but Option 2 requires standardization effort on renewing DCI design for intermedia nodes.

  - 1 source [Qualcomm] observed that option 2 is too costly in terms of overhead and latency if using DCI-based scheme to trigger each R2D or D2R, but it is beneficial for flexible resource sharing between AIoT communications as well as the coexistence with the legacy transmission.

  - 1 source [Mediatek] observed that Option 2 would increase the latency of A-IoT transmission. Furthermore, it is observed that the two options are not necessarily exclusive.

  - 1 source [Spreadtrum] observed that the signaling overhead of option 2 is higher and the spec impact in RAN1 is lager. If each R2D or D2R transmission resource is scheduled by DCI, option 2 will bring high transmission latency and low transmission efficiency.

  - 1 source [ ZTE] observed that Option 2 has larger standardization impacts, for example the DCI design. And Option 2 may affect the A-IoT timing definitions.

Agreement

For D2R control information, following is captured in TR 38.769:

14 sources [Huawei, Samsung, Spreadtrum, Fujitsu, Oppo, Ericsson, NTT Docomo, Qualcomm, Futurewei, ZTE, Cewit, Xiaomi, CATT, ETRI] studied feedback information regarding the (un)successful reception of the R2D command at the device and provided following observations:

- 5 sources [Huawei, Futurewei, ZTE, Xiaomi, ETRI] observed that L1-based ACK/NACK is unnecessary or infeasible for ambient IoT, since all D2R transmission are controlled and activated by the reader. Additionally, 1 source [ZTE] observed that it is infeasible for devices to report NACK since it cannot distinguish whether the R2D transmission is dedicated for it and the resource allocation for D2R NACK if the R2D transmission is not successfully detected. And ACK can be transmitted in higher layer signaling.

- 11 sources [Samsung, Spreadtrum, Oppo, Ericsson, NTT Docomo, Qualcomm, Nokia, Mediatek, Cewit, Xiaomi, Apple] observed that ACK/NACK feedback improves transmission reliability, when the BS or intermediate node sends R2D transmission to the device in the command procedure and if the device does not respond ACK/NACK, the BS or intermediate node cannot determine whether the transmission is successful. 1 source [Fujitsu] observed that in the use cases of command where some device actions are required and no D2R message needs to be fed back, only D2R ACK feedback in response to the PRDCH is beneficial for the reader to know that the reception of the PRDCH is successful and the required actions have been done by the device.

Agreement

Following observations on R2D clock-acquisition part are additionally captured in TR 38.769:

- On purpose of SFO estimation/correction based on the clock-acquisition part

  - 3 sources [Nokia, CATT, Qualcomm] provided observations on the applicability of clock-acquisition part for frequency synchronization:

- 1 source [Nokia] observed that the length of preamble sequence may need to consider also the robustness against SFO

- 1 source [CATT] observed that device 2a/2b may require higher synchronization accuracy for signal transmission or backscattering and therefore, the design of CAP may be required to accommodate the requirement of additional frequency synchronization and clock calibration for Device 2a/2b.

- 1 source [Qualcomm] further observed for Option 1, as the CAP duration with high M is decreased, only CAP may not be sufficient for SFO correction and for Option 2, as the CAP duration is fixed and independent from M, the CAP with long enough duration can support SFO correction.

- On purpose of CFO estimation/correction based on the clock-acquisition part

  - 2 sources [Ericsson, Qualcomm] provided observations on the applicability of clock-acquisition part for frequency synchronization:

- 1 source [Ericsson] observes that the clock-acquisition part can be utilized to solve the frequency synchronization problem without impacting the time-domain sequence, for example by transmitting in some frequency resources and it can be a harmonized solution for both chip duration indication and device frequency synchronization. However, it is further observed that if the time interval between an R2D transmission and the corresponding D2R transmission and if the device loses the timing obtained from the R2D timing acquisition signal due to timing drift at the time for the D2R transmission, then an additional synchronization signal is needed

- 1 source [Qualcomm] further observed for Option 1, as the CAP duration with high M is decreased, only CAP may not be sufficient for CFO correction and for Option 2, as the CAP duration is fixed and independent from M, the CAP with long enough duration can support CFO correction.

##### 2.1.1.6	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

RAN1#118bis

Agreement

Adopt the following update for TR 38.769 table 6.8.2-1.

Waveform 2 provides [0, 8] dB frequency diversity gain compared to waveform 1 at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel's coherence bandwidth. In a TDL-A fading channel with 30ns delay spread - For the gap between [75KHz, 900KHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [0, 1.5] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB. - For the gap between [1.08MHz, 4.2MHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [3, 5.8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [0.4, 2.5] dB. - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 5 sources are within [5, 8] dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within [1.3, 4] dB. In a TDL-D fading channel with 30ns delay spread - For 10MHz gap, 1 source [11] observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain. (Note: loss due to the power split in TDL-D); and 1 source [R1-2408854] observed 1dB@1%BLER and 0.5dB@10%BLER frequency diversity gain In a TDL-A fading channel with 150ns delay spread - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [1, 3] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [0, 2.5] dB. - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8] dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 5.5] dB.

Agreement

Single-tone unmodulated sinusoid waveform with frequency hopping (2-hops) is studied, and the following table is captured.

| CW waveform characteristics | Waveform 1 with frequency hopping (2-hops) compared to waveform 1 without frequency hopping |
| --- | --- |
| D2R reception performance | Observations on frequency diversity: - 6 sources observed that waveform 1 with frequency hopping should be used together with bit-level repetition or TB-level repetition. - 3 sources further observed that the unaligned timing (due to SFO at A-IoT device) between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission. - 2 sources observed that for milli-second level CW duration for each frequency hop/block repetition, the diversity gain loss caused by miss-alignment of a few micro-seconds level is marginal, TB level repetition can tolerate some time mis-alignment. - 1 source [R1-2408851] observed that waveform 1 with frequency hopping achieves frequency diversity gain by using polynomial sweeping-based CC encoding (proposed by [R1-2408851]) instead of repetitions When bit-level repetition, TB-level repetition or polynomial sweeping-based CC encoding is not used, - Waveform 1 with frequency hopping provides [0, 0.5] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter. - In a TDL-A fading channel with 30ns delay spread - For 10MHz gap between two hops, 1 source observed 0 dB@10%BLER frequency diversity gain. - In a TDL-A fading channel with 150ns delay spread - For 1.65MHz gap between two hops, 1 source observed 0.5 dB@10%BLER and 0 dB@1%BLER frequency diversity gain. When bit-level repetition, TB-level repetition is used, - Waveform 1 with frequency hopping provides [0.5, 8] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter, at least depending on the gap between the two hops and the channel's coherence bandwidth. - In a TDL-A fading channel with 30ns delay spread - For the gap of 2.16MHz, 1 source observed 4dB@1% BLER and 2dB@10% BLER frequency diversity gains. - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.8, 8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [1.2, 6] dB. - In a TDL-D fading channel with 30ns delay spread - For 10MHz gap, 1 source observed 1 dB@1%BLER and 0.5dB@10%BLER frequency diversity gain. - In a TDL-A fading channel with 150ns delay spread - For the gap between [1.65MHz, 2.16MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.5, 8] dB, and the frequency diversity gain at 10% BLER target observed by 2 sources is 5 dB. When polynomial sweeping-based CC encoding [R1-2408851] is used or assumed - In a TDL-A fading channel with 30ns delay spread, and for 10MHz gap between two hops, 1 source [R1-2408851] observed that waveform 1 with frequency hopping provides 4 dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% BLER for a 1Rx receiver and a 1Tx CW transmitter. Note: the above evaluations assume the same time domain resources overhead for waveform 1 with or without frequency hopping. |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | 4 sources observed that for the D2R transmission BW corresponding to the CW waveforms, compared to waveform 1 without frequency hopping, waveform 1 with frequency hopping utilizes the same amount of frequency domain resources for D2R transmission at a certain time. |
| CW interference suppression at D2R receiver | 5 sources observed that waveform 1 with frequency hopping requires additional complexity for CW interference suppression if RF interference cancellation is used. 4 sources observed that waveform 1 with frequency hopping requires individual cancellation for each hop in different time, e.g. one tunable RF or IF narrow-band bandpass filter, or two RF or IF narrow-band bandpass filters. 2 sources observed that for the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires fast re-detection of the parameters of the received carrier-wave interference, which at least doubled the implementation complexity of D2R receiver and may not be supported by the intermediate UE in D2T2. Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver. |
| Relative complexity of CW generation | 2 sources observed that waveform 1 with frequency hopping requires additional complexity to construct the single-tone unmodulated sinusoid waveform at different center frequencies in a TDMed manner. 2 sources observed that waveform 1 with frequency hopping doesn’t lead to higher PAPR of the generated CW. 1 source observed that waveform 1 with frequency hopping does not require additional complexity of CW generate compared to waveform 1 without frequency hopping. |

Agreement

Capture the following observations for D2R reception performance for 2Tx cases in the TR table 6.8.2-1.

One source [R1-2408854] observed that waveform 1 with antenna hopping (AH) provides [1, 5.75] dB spatial diversity gain compared to Waveform 1 with no AH at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 2Tx CW transmitter. - In a TDL-D fading channel with 30ns delay spread - One source observed that the spatial diversity gain is 1.6 dB at 1% BLER and 1 dB at 10% BLER. - In a TDL-A fading channel with 30ns delay spread - For the gap of 2.16MHz, one source observed that the spatial diversity gain is 5.75dB at 1% BLER target and 2.5dB at 10% BLER target. - For the gap of 10MHz, one source observed that the spatial diversity gain is 2.2dB at 1% BLER target and 1.5dB at 10% BLER target. One source [R1-2408854] observed that waveform 2 with AH provides [1.4, 11.5] dB spatial diversity and frequency diversity gain compared to Waveform 1 with no AH at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 2Tx CW transmitter, at least depending on the gap between the two tones and the channel's coherence bandwidth. - In a TDL-D fading channel with 30ns delay spread - For the gap of 10MHz, one source observed that the spatial diversity and frequency diversity gain is 2.9 dB at 1% BLER and 1.4 dB at 10% BLER. - In a TDL-A fading channel with 30ns delay spread - For the gap of 2.16MHz, one source observed that the spatial diversity and frequency diversity gain is 8.25dB at 1% BLER and 5dB at 10% BLER. - For the gap of 10MHz, one source observed that the spatial diversity and frequency diversity gain is 11.5dB at 1% BLER and 7dB at 10% BLER.

Agreement

For the observation of the CW characteristics which would need control of the CW node(s), remove the sub-bullet of “FFS other characteristics”, and adopt the following TP for TR 38.769

The following CW waveform characteristics which would need control of the CW node(s) are identified: - When CW is transmitted or not transmitted - Transmission Power - Frequency resources, e.g., frequency location(s) for waveform (e.g., waveform 1, waveform 2) Other CW waveform characteristics which would need control of the CW node(s), if any, can be further identified in a later stage.

RAN1#119

Agreement

For CW transmission with large frequency shift, capture the following observation in TR 38.769.

If large frequency shift is supported by device 2a, the D2R backscattering can be transmitted in a different carrier as CW for D2R backscattering (e.g., CW in DL spectrum and D2R in UL spectrum, or CW in UL spectrum and D2R in DL spectrum), and the followings are observed. - In-band full-duplex capability is not needed for CW transmission and D2R reception at D2R receiver side, if the D2R receiver only supports device 2a with large frequency shift capability. - No interference from CW to the corresponding D2R reception at D2R receiver side. - For CW in DL spectrum and D2R in UL spectrum, higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.

Agreement

Adopt the following TP for the gap between two frequency hops in section 6.8.2 of TR 38.769:

<Unchanged parts are omitted>

For the gap between two tones or two frequency hops of a single tone to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

<Unchanged parts are omitted>

Agreement

Capture the D2R reception performance comparison as the TP below in section 6.8.2 of TR 38.769:

<Unchanged parts are omitted>

Table 6.8.2-1: Observations and/or comparisons of CW waveform candidates

| CW waveform characteristics | Waveform 1 compared to Waveform 2 NOTE 1: Waveform 1 without frequency hopping NOTE 2: Waveform 2 with both tones from the same CW node | Waveform 1 with frequency hopping (2-hops) compared to waveform 1 without frequency hopping |
| --- | --- | --- |
| D2R reception performance | <Unchanged parts are omitted> | Observations on frequency diversity: - 6 sources observed that waveform 1 with frequency hopping should be used together with bit-level repetition or TB-level repetition. - 3 sources further observed that the unaligned timing (due to SFO at A-IoT device) between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission. - 2 sources observed that for milli-second level CW duration for each frequency hop/block repetition, the diversity gain loss caused by miss-alignment of a few micro-seconds level is marginal, TB level repetition can tolerate some time mis-alignment. - 1 source [R1-2408851] observed that waveform 1 with frequency hopping achieves frequency diversity gain by using polynomial sweeping-based CC encoding (proposed by [R1-2408851]) instead of repetitions When bit-level repetition, TB-level repetition or polynomial sweeping-based CC encoding is not used, - Waveform 1 with frequency hopping provides [0, 0.5] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter. - In a TDL-A fading channel with 30ns delay spread - For 10MHz gap between two hops, 1 source observed 0 dB@10%BLER frequency diversity gain. - In a TDL-A fading channel with 150ns delay spread - For 1.65MHz gap between two hops, 1 source observed 0.5 dB@10%BLER and 0 dB@1%BLER frequency diversity gain. When bit-level repetition, TB-level repetition is used, - Waveform 1 with frequency hopping provides [0.5, 8] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter, at least depending on the gap between the two hops and the channel's coherence bandwidth. - In a TDL-A fading channel with 30ns delay spread - For the gap of 2.16MHz, 1 source observed 4dB@1% BLER and 2dB@10% BLER frequency diversity gains. - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.8, 8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [1.2, 6] dB. The 8 dB gain is achieved under the assumption of ideal channel estimation. - In a TDL-D fading channel with 30ns delay spread - For 10MHz gap, 1 source observed 1 dB@1%BLER and 0.5dB@10%BLER frequency diversity gain - In a TDL-A fading channel with 150ns delay spread - For the gap between [1.65MHz, 2.16MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.5, 8] dB, and the frequency diversity gain at 10% BLER target observed by 2 sources is 5 dB. The 8 dB gain is achieved under the assumption of ideal channel estimation. When polynomial sweeping-based CC encoding [R1-2408851] is used or assumed - In a TDL-A fading channel with 30ns delay spread, and for 10MHz gap between two hops, 1 source [R1-2408851] observed that waveform 1 with frequency hopping provides 4 dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% BLER for a 1Rx receiver and a 1Tx CW transmitter. The following observations were made by one source ([R1-2408854]): - Waveform 1 with frequency hopping and antenna hopping provides [1, 9] dB spatial diversity and frequency diversity compared to Waveform 1 with no frequency hopping and no antenna hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 2Tx CW transmitter, at least depending on the gap between the two tones and the channel's coherence bandwidth and assuming the ideal channel estimation. - In a TDL-D fading channel with 30ns delay spread, for the gap of 10MHz, the gain is 2.6 dB at 1% BLER and 1 dB at 10% BLER. - In a TDL-A fading channel with 30ns delay spread, for the gap of 2.16MHz, the gain is 7dB at 1% BLER target and 3.5dB at 10% BLER target. For the gap of 10MHz, the gain is 9dB at 1% BLER target and 5dB at 10% BLER target. Note: The total transmission power is assumed the same for both waveforms. Note: The above evaluations assume the same time domain resources overhead for waveform 1 with or without frequency hopping. |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | <Unchanged parts are omitted> | <Unchanged parts are omitted> |
| CW interference suppression at D2R receiver | <Unchanged parts are omitted> | <Unchanged parts are omitted> |
| Relative complexity of CW generation | <Unchanged parts are omitted> | <Unchanged parts are omitted> |

<Unchanged parts are omitted>

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.1.1	Protocol and Functionality aspects

RAN2#127bis

Agreements on AS ID

1	RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception.  Up to RAN1 to decide whether a “AS ID” is defined.

2	RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID.  FFS on the length.   FFS if AS ID can be based on partial upper layer device ID.

3	From RAN2 perspective, following options are possible for “AS ID”:

Option 1: a random ID if used in Msg1 can be reused;

Option 2: reader assigns this “AS ID”. FFS via which R2D message.

4	RAN2 will aim to define one common design for all RA procedures (if technically possible)

Agreements on energy bit indication

1	Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

Agreements

1	From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

2	The D2R message size would be beneficial but it is not essential.

3	Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

Agreements on segmentations

1	RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

2	Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

3	Do not support sequence number for A-IoT segmentation functionalities.

4	Do not support segment number and the number of segments for A-IoT segmentation functionalities.

5	Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

6	RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

Agreements on DoA

1	From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

2	From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

RAN2#128

Agreements

1	RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on AIoT interfaces.

2	Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID.   FFS what message is used

3	Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision.

4	Segmentation indication will be discussed in WI phase.

5	If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment.  FFS how this is done.

6	The reader needs to know whether a response is expected and expected D2R message size.   FFS how the reader gets this information.  Wait for SA2 LS response on expected D2R message size to resolve the FFS.    FFS if command type is explicit, if it can be inferred from expected D2R message size if available.

7	Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for the reader and ask whether they can be provided (e.g. service type).     Ask SA2 if there are requirements from SA2 perspective (e.g. on latency of completion time of procedure)

8	We will not capture a new use case in the TR.   RAN2 confirms that this use case can be done via reader implementation, as per previous agreement that it is up to the reader whether MSG2 is transmitted.  FFS if the device needs to be aware whether msg2 will be transmitted.

##### 2.2.1.2	Paging

RAN2#127bis

Agreements

1   The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

2	Based on this information the device determines whether to skip sending the response to paging.

3	From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

4	If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

5	For all use cases, device determines the random access type from the paging message.   FFS whether it is explicitly or implicitly.   FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

RAN2#128

Agreements

1	An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

2	Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

3	Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

##### 2.2.1.3	Random Access

RAN2#127bis

Agreements

1	In case of D2R data transmission failure, device follows the reader instruction:

- it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

- reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

RAN2#128

Agreements on 3-step CBRA and 2-step CBRA

1	Capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

2	FFS recommendation on whether we have 3step only or we have unified solution.

Agreements on CBRA and CFRA

1	Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1 is defined.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

Agreements on Msg3

1	RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

2	If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

Agreements on feedback indication

1	The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose.

Agreements on reaccess

2	Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

Agreements

1	For 2 step and 3step CBRA, capture the unified solution in the TR and capture advantage and disadvantage of the solutions.  This requires that devices would support both forms.

2	FFS recommendation on whether we have 3step only or we have unified solution for 2step and 3step CBRA.

3	Capture in TR as one option for unification for CBRA and CFRA.  For this option random ID is always included in msg1.  Capture disadvantage of reader doesn’t know whether the intended device is responding on the given resources.

4	RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID

5	If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

6	The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

7	Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

##### 2.2.1.4	Topology 2 considerations

RAN2#127bis

Agreements

1	Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

2	The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

3	The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

RAN2#128

Agreements

1	Resource validity across multiple cells is not supported in the initial release.

2	Include in the TR the resource validity is under network control.  CB which option we include.

3	We will capture the following options in the TR for resource validity in different cases when the resources are allocated semi-statically by the network:

Reader UE performing AIOT operation in RRC_CONNECTED (downselection or support of both options will be decided in the WI phase):

- Option 1: UE receives resources in dedicated signalling.  The resources remain valid until the network releases them explicitly (understanding is this would be supported by RRC reconfiguration)

- Option 2: UE receives resources in dedicated signalling along with a time period in which they can be used.  The resources remain valid for that time period.

Reader UE performing AIOT operation in RRC_INACTIVE (downselection or support of both options will be decided in the WI phase):

- Option 1: UE receives resources in dedicated signalling before moving to RRC_INACTIVE.  The resources remain valid until the network releases them explicitly.

- Option 2: UE receives resources and a time period in dedicated signalling before moving to RRC_INACTIVE.  The resources remain valid for that period of time.

Reader UE performing AIOT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation. (downselection between the following options will be performed in the WI phase)

- Option 1: Resources remain valid at the reader for a time period (FFS whether the same time period as in RRC_CONNECTED options or a different time period.  FFS whether the time period is related to the ongoing AIOT operation).

- Option 2: Resources are temporarily invalid at the reader during the “temporary out of connection” condition but may become valid again if the UE recovers from the condition.

If reader UE supports AIOT operation in RRC_IDLE, options of RRC_INACTIVE are candidates.

-	Support of RRC_IDLE (other than in temporary out of connection) is down-prioritized.   During WI can consider it if feasible with minimum spec impact

For all options

The resources remain valid only in the same cell for which the resources were configured.

4       From RAN2 perspective, at least for RRC based architecture, there may be radio related reasons for gNB to be involved in reader selection.

5       Whether the gNB or CN selects the reader, and/or what information is shared between gNB and CN is up to RAN3/SA2

6       Capture the following options:

- An AIoT resource allocation from the gNB is sent in response to a request from the UE reader and consists of a set of radio resources and validity criteria (if applicable)

- BS can directly configure a set of radio resources and validity criteria (if applicable) based on service request from CN.

- These two options can co-exist

##### 2.2.1.5	Organizational

RAN2#128

Agreements

- From RAN2 point of view, the SI is considered complete

- The TR will capture in corresponding subsection any impact and/or dependencies from other WGs when applicable.

- The TR will recommend that RAN2 will have to support paging, RACH, data transmission functionality in MAC.  RAN2 has discussed topology 1 and topology 2 and has concluded it is feasible, from RAN2 perspective.   RAN2 will discuss in WI phase any additional impacts from other WGs.

#### 2.2.2	Remaining Open issues

None.

## 2.3	RAN3

#### 2.3.1	Agreements

##### 2.3.1.1	RAN Architecture

RAN3#123bis

RAN3 considers both Topologies at the same time looking whether commonalities are applicable.

[Topo1] AIoT RAN node:

Corresponds to the basestation in Figure 4.2.1.1-1 in TR 38.848;

A RAN node providing AIoT radio, and connecting with an AIoT-aware CN node via the XX interface. Details of the AIoT-aware CN node is subject to SA2.

[Topo2] UE Reader:

A UE (corresponds to the intermediate node in Figure 4.2.1.2-1 in TR 38.848);

Providing AIoT radio, and connecting with a gNB (may be an AIoT enhanced gNB, corresponds to the basestation in Figure 4.2.1.2-1 in TR 38.848) via NR Uu interface.

For Topology 1, RAN3 starts with AIoT RAN node being aggregated.

RAN3#124

Agreements of RAN Architecture in RAN3#124 meeting are captured in the following document:

- R3-243962	[TP for TR 38.769] CB:#AIoT1_Architecture (Ericsson, Huawei)

RAN3#125

Agreements of RAN Architecture in RAN3#125 meeting are captured in the following document:

- R3-244837	(TP for TR 38.769) RAN architecture aspects (Huawei)

RAN3#125bis

Agreements of RAN Architecture in RAN3#125bis meeting are captured in the following document:

- R3-245846	(TP to TR 38.769) A-IoT RAN Architecture aspects (Huawei)

- R3-245847	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions (Huawei, CMCC, ZTE, CATT, NEC, Xiaomi)

RAN3#126

Agreements of RAN Architecture in RAN3#126 meeting are captured in the following document:

- R3-247830	(TP to TR 38.769) A-IoT RAN Architecture aspects (Huawei)

- R3-247918	(TP to TR 38.769) Protocol Stacks for Topology 2 NAS/UP based solutions (Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm)

##### 2.3.1.2	RAN-CN Interface Impact

RAN3#123bis

AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command).

RAN3#124

Agreements of signaling and procedures for CN-RAN interface in RAN3#124 meeting are captured in the following document:

- R3-243963	Inventory and Command between AIoT CN and AIoT RAS (Huawei, CMCC, Nokia, Ericsson, ZTE, Xiaomi, Qualcomm Incorporated, Samsung, CATT, Lenovo, LG Electronics, NEC)

RAN3#125

Agreements of signalling and procedures for CN-RAN interface in RAN3#125 meeting are captured in the following document:

-	R3-244838	(TP for TR38.769) NG interface impact for Ambient-IoT (ZTE)

RAN3#125bis

Agreements of signalling and procedures for CN-RAN interface in RAN3#125bis meeting are captured in the following document:

-	R3-244838 	[TP for TR 38.769] CB:#AIoT2_RANCNinterface (Ericsson)

RAN3#126

Agreements of signalling and procedures for CN-RAN interface in RAN3#126 meeting are captured in the following document:

- R3-247919 	[TP for TR 38.769] CB:#AIoT2_SignallingFlow (Ericsson)

##### 2.3.1.3	Others

RAN3#123bis

Use cases for locating an AIoT device:

-	Find an appropriate “reader” close to the A-IoT device;

-	Find where the A-IoT device is.

Support locating the A-IoT device at “reader” granularity.

RAN3#124

Agreements of locating an Ambient IoT device in RAN3#124 meeting are captured in the following document:

- R3-243964	(TP for TR38.769) Locating Ambient-IoT device (CMCC, ZTE, CATT, Xiaomi, LGE, Huawei, Lenovo, NEC)

RAN3#125

Agreements of locating an Ambient IoT device in RAN3#125 meeting are captured in the following document:

-	R3-244840	(TP for TR 38.769) Locating Ambient-IoT device (CMCC)

RAN3#125bis

The following principle applies to Topology1 and Topology2:

For both purpose a) and b), identifier of the reader may be reported to AIoT CN in the Inventory Report message.

While for purpose b), whether the reader ID based granularity of the AIoT device location is sufficient can be further checked.

RAN3#126

Agreements of locating an Ambient IoT device in RAN3#126 meeting are captured in the following document:

-	R3-247920	 [TP for TR 38.769] Locating AIoT Devices (Nokia)

##### 2.3.1.4	General

RAN3#126

Agreements of general parts in RAN3#126 meeting are captured in the following document:

- R3-247921	(TP to TR 38.769) Uncritical Editor's Note Removal (CMCC, Huawei, Samsung, CATT, Lenovo, Xiaomi, ZTE, NEC, China Telecom)

- R3-247922	(TP to TR 38.769) RAN3 Conclusions and Recommendations (CMCC, T-Mobile USA, Huawei)

- R3-247923	[DRAFT] LS on RAN3 outcome of Ambient IoT study (Huawei)

- R3-247924	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR (Huawei, CMCC, Ericsson, ZTE, CATT, NEC, Xiaomi, Lenovo)

Impacts and dependencies on other WGs

No evaluation and down selection for T2 solutions have been performed which requires coordination among WGs.

Editor's note: Whether and which of the following options on CN triggered A-IoT session resource allocation and UE triggered A-IoT session resource allocation will be finally considered needs further discussion involving other WGs as needed.

#### 2.3.2	Remaining Open issues

  - None

## 2.4	RAN4

#### 2.4.1	Agreements

Agreements in RAN4#112bis meeting are captured in the following document:

- R4-2417196	WF on Co-existence study for A-IoT and NR/LTE, CMCC

- R4-2417088	WF on RF requirement impact of A-IoT, Huawei

- R4-2417193	A-IoT general overview, Ericsson

- R4-2416356	TP to TR38.769: Co-existence simulation, Huawei, HiSilicon

- R4-2417195	TP for CW interference cancelation, Huawei, HiSilicon, Ericsson

Agreements in RAN4#113 meeting are captured in the following documents:

- R4-2420359	Draft TP to TR38.769 on conclusion for A-IOT	CMCC

- R4-2420373	LS of RAN4 input on Ambient-IoT to TR 38.769	CMCC

- R4-2420470	WF on UE RF requirements for A-IoT	Huawei

#### 2.4.2	Remaining Open issues

None.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#118bis

- R1-2407593	LS on data block sizes for Ambient IoT	RAN2, MediaTek

- R1-2407600	Reply LS on Clarification of requirements for Ambient IoT	SA1, Ericsson

- R1-2407608	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

- R1-2407609	Discussion on Rel-19 Ambient IoT device architecture	FUTUREWEI

- R1-2407610	Discussion on physical layer design for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2407611	Discussion on Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

- R1-2407612	Discussion on D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

- R1-2407613	Discussion on External Carrier Waveform Characteristics for Rel-19 Ambient IoT devices	FUTUREWEI

- R1-2407627	Discussion on ambient IoT device architectures	TCL

- R1-2407628	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2407629	Discussion on downlink and uplink channel/signal aspects	TCL

- R1-2407630	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

- R1-2407636	Evaluation assumptions and results for Ambient IoT	Ericsson

- R1-2407637	Ambient IoT device architectures	Ericsson

- R1-2407638	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2407639	Frame structure and timing aspects for Ambient IoT	Ericsson

- R1-2407640	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2407641	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

- R1-2407642	On RAN2 LS on data block sizes for Ambient IoT	Ericsson

- R1-2407643	Evaluation assumptions and results for Ambient IoT	Nokia

- R1-2407644	Ambient IoT device architectures	Nokia

- R1-2407645	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2407646	Frame structure and timing aspects for Ambient IoT	Nokia

- R1-2407647	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

- R1-2407648	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

- R1-2407667	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

- R1-2407668	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

- R1-2407669	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2407670	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon

- R1-2407671	Physical channels and signals for Ambient IoT	Huawei, HiSilicon

- R1-2407672	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

- R1-2407673	Discussion on RAN2 LS on Clarification of data block sizes for Ambient IoT	Huawei, HiSilicon

- R1-2407705	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum Communications

- R1-2407706	Discussion on Ambient IoT device architectures	Spreadtrum Communications, SGITG

- R1-2407707	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum Communications

- R1-2407708	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum Communications

- R1-2407709	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum Communications

- R1-2407710	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum Communications

- R1-2407727	Discussion on RAN2 LS on data block sizes for Ambient IoT	China Telecom

- R1-2407732	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2407733	Discussion on Ambient IoT device architectures	China Telecom

- R1-2407734	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2407735	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2407736	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

- R1-2407737	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Telecom

- R1-2407760	Discussion on Link Level simulation of A-IoT	Tejas Network Limited

- R1-2407761	Discussion on receiver architecture of A-IoT	Tejas Network Limited

- R1-2407762	Resource allocation and frame structure of A-IoT	Tejas Network Limited

- R1-2407763	Study the downlink and uplink channels of A-IoT	Tejas Network Limited

- R1-2407764	Discussion on CW waveform characteristics of A-IoT	Tejas Network Limited

- R1-2407791	Discussion on LS on data block sizes for Ambient IoT	Spreadtrum Communications

- R1-2407818	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

- R1-2407819	Discussion on channel/signal aspects for Ambient IoT	Lenovo

- R1-2407820	Discussion on external carrier wave for Ambient IoT	Lenovo

- R1-2407827	Draft reply LS on data block sizes for Ambient IoT	vivo

- R1-2407860	Evaluation methodologies assumptions and results for Ambient IoT	vivo

- R1-2407861	Discussion on Ambient IoT Device architectures	vivo

- R1-2407862	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2407863	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT	vivo

- R1-2407864	Discussion on  Downlink and uplink channel/signal aspects	vivo

- R1-2407865	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

- R1-2407886	Discussion on RAN2 LS on data block size for Ambient IOT	CMCC

- R1-2407887	Draft reply LS on data block size for Ambient IoT	CMCC

- R1-2407904	Discussion on evaluation assumptions and results for Ambient IoT	CMCC

- R1-2407905	Discussion on Ambient IoT device architectures	CMCC

- R1-2407906	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2407907	Discussion on frame structure and timing aspects for A-IoT	CMCC

- R1-2407908	Discussion on downlink and uplink channel/signal aspects	CMCC

- R1-2407909	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2407942	Discussion on RAN2 LS on data block sizes for Ambient IoT	Xiaomi

- R1-2407943	Draft reply LS on RAN2 LS on data block sizes for Ambient IoT	Xiaomi

- R1-2407968	Evaluation assumptions and results for Ambient IoT	Xiaomi

- R1-2407969	Discussion on ambient IoT device architectures	Xiaomi

- R1-2407970	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2407971	Discussion on frame structure and timing aspects for Ambient IoT	Xiaomi

- R1-2407972	Discussion on downlink and uplink channel and signal aspects for Ambient IoT	Xiaomi

- R1-2407973	Discussion on waveform characteristics of carrier-wave	Xiaomi

- R1-2408046	The evaluation methodology and preliminary results of Ambient IoT	CATT

- R1-2408047	Study of the Ambient IoT devices architecture	CATT

- R1-2408048	Discussion on general aspects of physical layer design	CATT

- R1-2408049	Study of Frame structure and timing aspects for Ambient IoT	CATT

- R1-2408050	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

- R1-2408051	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

- R1-2408063	Discussion on data block sizes for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408064	Draft reply to LS on data block sizes for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408065	Discussion on Ambient IoT evaluations	ZTE Corporation, Sanechips

- R1-2408066	Discussion on Ambient IoT device architectures	ZTE Corporation, Sanechips

- R1-2408067	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408068	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408069	Discussion on channel and signal  for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408070	Discussion on carrier wave for Ambient IoT	ZTE Corporation, Sanechips

- R1-2408113	Discussion on frame structure and timing aspects	Fujitsu

- R1-2408114	Discussion on downlink and uplink channel/signal aspects	Fujitsu

- R1-2408145	Discussion on evaluation assumptions and results for A-IoT	OPPO

- R1-2408146	Discussion on device architecture for A-IoT device	OPPO

- R1-2408147	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2408148	Discussion on frame structure and timing aspects of A-IoT communication	OPPO

- R1-2408149	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

- R1-2408150	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

- R1-2408151	Discussion on LS from RAN2 on data block sizes for Ambient IoT	OPPO

- R1-2408152	Draft reply LS to RAN2 on data block sizes for Ambient IoT	OPPO

- R1-2408206	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2408207	Discussion on frame structure and timing for ambient IoT	NEC

- R1-2408208	Discussion on downlink and uplink channel for ambient IoT	NEC

- R1-2408209	Discussion on the waveform of carrier-wave for the Ambient IoT	NEC

- R1-2408233	Discussion on evaluation assumptions and results for Ambient IoT	HONOR

- R1-2408234	Discussion on frame structure and timing aspects for Ambient IoT	HONOR

- R1-2408250	Discussion on general aspects of physical layer design	Sharp

- R1-2408251	Discussion on frame structure and timing aspects	Sharp

- R1-2408252	Discussion on downlink and uplink channel/signal aspects	Sharp

- R1-2408264	Discussion on Frame structure and timing aspects for A-IoT	China Unicom

- R1-2408265	Discussion on downlink and uplink channel aspects for A-IoT	China Unicom

- R1-2408266	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	China Unicom

- R1-2408272	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2408308	On General Physical Layer Design Considerations for Ambient IoT Applications	Lekha Wireless Solutions

- R1-2408355	Analysis for Inventory Completion Time for Multiple A-IoT Devices	Wiliot Ltd.

- R1-2408370	Discussion on frame structure and timing aspects	Google

- R1-2408371	Discussion on downlink and uplink transmission aspects	Google

- R1-2408374	Discussion on ambient IoT evaluation framework	NEC

- R1-2408375	Device architecture requirements for ambient IoT	NEC

- R1-2408410	General aspects of Ambient IoT physical layer design	Sony

- R1-2408411	Frame structure and timing aspects for Ambient IoT	Sony

- R1-2408412	Downlink and uplink channel / signal aspects for Ambient IoT	Sony

- R1-2408434	Frame structure and timing aspects of Ambient IoT	InterDigital, Inc.

- R1-2408435	Downlink and uplink channels aspects of Ambient IoT	InterDigital, Inc.

- R1-2408445	Discussion on  RAN2 LS on data block sizes for AIoT	Apple

- R1-2408446	Draft reply LS to RAN2 on data block sizes for AIoT	Apple

- R1-2408464	On remaining evaluation assumptions and results for AIoT	Apple

- R1-2408465	On remaining details of device architecture for AIoT	Apple

- R1-2408466	On remaining general physical layer design aspects for AIoT	Apple

- R1-2408467	On remaining frame structure and timing aspects for AIoT	Apple

- R1-2408468	On remaining details of physical channels/signals for AIoT	Apple

- R1-2408469	On remaining details of carrier waveform and interference handling for AIoT	Apple

- R1-2408470	FL summary#1 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2408471	FL summary#2 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2408472	FL summary#3 on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2408532	Considerations on Intermediate UE in A-IoT	Continental Automotive

- R1-2408536	Discussion on A-IoT Frame Structure and Timing Aspects	Panasonic

- R1-2408568	Discussion on general aspects of physical layer design	ETRI

- R1-2408569	Discussion on frame structure and timing aspects	ETRI

- R1-2408570	Discussion on downlink and uplink channel/signal aspects for A-IoT	ETRI

- R1-2408571	Discussion on carrier-wave for Ambient IoT	ETRI

- R1-2408596	Frame Structure and Timing Aspects for Ambient IoT	IIT, Kharagpur

- R1-2408598	Link level simulation for Ambient IoT R2D	Panasonic

- R1-2408599	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2408601	Discussion on downlink and uplink channels and signals for A-IoT	Panasonic

- R1-2408602	Discussion on waveform characteristics of carrier-wave for Ambient IoT device	Panasonic

- R1-2408645	Considerations for evaluation assumptions and results	Samsung

- R1-2408646	Remaining issues for Ambient-IoT device architectures	Samsung

- R1-2408647	Considerations for general aspects of Ambient IoT	Samsung

- R1-2408648	Considerations for frame structure and timing aspects	Samsung

- R1-2408649	Considerations for downlink and uplink channel/signal aspect	Samsung

- R1-2408650	Considerations for Waveform characteristics of carrier-wave	Samsung

- R1-2408670	Discussion on Ambient IoT evaluation	LG Electronics

- R1-2408671	Discussion on Ambient IoT device architectures	LG Electronics

- R1-2408672	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2408673	Frame structure and timing aspects for Ambient IoT	LG Electronics

- R1-2408674	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

- R1-2408675	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

- R1-2408685	Discussion on physical layer design for Ambient IoT	Comba

- R1-2408687	Discussion on frame structre and timing aspects for Ambient IoT	BUPT

- R1-2408696	Discussion on RAN2 LS on data block sizes for Ambient IoT	MediaTek Inc.

- R1-2408697	Draft reply LS to RAN2 on data block sizes for Ambient IoT	MediaTek Inc.

- R1-2408699	Evaluation assumptions and results for A-IoT	MediaTek Inc.

- R1-2408700	Ambient IoT device architectures	MediaTek Inc.

- R1-2408701	General aspects of physical layer design	MediaTek Inc.

- R1-2408702	Frame structure and timing aspects	MediaTek Inc.

- R1-2408703	Downlink and uplink channel/signal aspects	MediaTek Inc.

- R1-2408725	Discussion on RAN2 LS on data block sizes for Ambient IoT	InterDigital, Inc.

- R1-2408730	On the general aspects of physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2408731	Evaluation results for Ambient IoT	InterDigital, Inc.

- R1-2408732	Carrier-wave for Ambient IoT	InterDigital, Inc.

- R1-2408733	Device architectures for Ambient IoT	InterDigital, Inc.

- R1-2408742	Considerations for frame structure and timing aspects	Semtech Neuchatel SA

- R1-2408743	Considerations for downlink and uplink channel/signal aspects	Semtech Neuchatel SA

- R1-2408763	Discussion on frame structure and timing aspect	ASUSTeK

- R1-2408764	Discussion on downlink and uplink channel/signal aspect	ASUSTeK

- R1-2408765	Discussion on A-IoT physical layer design	ASUSTeK

- R1-2408770	Draft reply LS on data block sizes for Ambient IoT	NTT DOCOMO, INC.

- R1-2408785	Study on evaluation assumptions and results for Ambient IoT	NTT DOCOMO, INC.

- R1-2408786	Study on device architectures for Ambient IoT	NTT DOCOMO, INC.

- R1-2408787	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2408788	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2408789	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2408790	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

- R1-2408831	Draft reply LS on data block sizes for Ambient IoT	Qualcomm Incorporated

- R1-2408849	Evaluation Assumptions and Results	Qualcomm Incorporated

- R1-2408850	Ambient IoT Device Architecture	Qualcomm Incorporated

- R1-2408851	General aspects of physical layer design	Qualcomm Incorporated

- R1-2408852	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2408853	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

- R1-2408854	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

- R1-2408875	Discussion on multiple access for D2R	LG Uplus

- R1-2408901	Discussion on frame structure and timing aspects for Ambient IoT	WILUS Inc.

- R1-2408920	Discussion on frame structure and timing aspects for Ambient IoT	TCL

- R1-2408930	Discussion on Evaluation assumptions and results	CEWiT

- R1-2408931	Discussion on Frame structure and timing aspects	CEWiT

- R1-2408932	Discussion on Downlink and Uplink channel/signal aspects	CEWiT

- R1-2408933	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

- R1-2408938	Evaluation assumption and results for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408939	Views on Architecture of Ambient IoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408941	Discussion on General aspects of physical layer design of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408942	Frame structure and timing aspects of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408943	Discussion on Downlink and Uplink channel signal aspects for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2408966	Discussion on the evaluation assumptions for Ambient IoT devices	Lenovo

- R1-2408967	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2408990	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2408991	FL summary #1 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2408992	FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2408993	FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2409005	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2409006	Discussion on evaluation assumptions and results for Ambient IoT	CMCC

- R1-2409008	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT	vivo

- R1-2409022	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2409023	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2409024	On RAN2 LS on data block sizes for Ambient IoT	Ericsson

- R1-2409026	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

- R1-2409027	Considerations for evaluation assumptions and results	Samsung

- R1-2409057	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2409066	RAN1#118-bis FL Summary #1 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2409067	RAN1#118-bis FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2409068	RAN1#118-bis FL Summary #3 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2409069	RAN1#118-bis FL Summary #4 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2409070	Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2409071	Feature Lead Summary #2 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2409072	Feature Lead Summary #3 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2409119	FL summary #1 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2409120	FL summary #2 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2409121	FL summary #3 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2409122	Draft TP for Ambient IoT evaluation results for TR38.769 – single device latency	Moderator (CMCC)

- R1-2409123	Draft TP for Ambient IoT evaluation results for TR38.769 – multiple-device inventory completion time	Moderator (CMCC)

- R1-2409124	Draft TP for Ambient IoT evaluation results for TR38.769 – coverage evaluation	Moderator (CMCC)

- R1-2409125	Draft TP for Ambient IoT receiver sensitivities for TR38.769	Moderator (CMCC)

- R1-2409126	[TP for Annex Z of TR38.769 for receiver sensitivity]	Moderator (CMCC)

- R1-2409127	[TP for section 7.1.1 of TR38.769 for D1T1 coverage evaluations]	Moderator (CMCC)

- R1-2409128	[TP for section 7.1.2 of TR38.769 for D2T2 coverage evaluations]	Moderator (CMCC)

- R1-2409129	[TP for section 7.1.3 and 7.1.4 of TR38.769 for R2D and D2R link performance evaluations]	Moderator (CMCC)

- R1-2409149	FL summary#1 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2409167	Summary#1 for Reply LS on data block sizes for Ambient IoT	Moderator (MediaTek)

- R1-2409187	TP on section 6.2 Device (un)availability for TR 38.769	Moderator (vivo)

- R1-2409204	Summary#2 for Reply LS on data block sizes for Ambient IoT	Moderator (MediaTek)

- R1-2409223	Session notes for 9.4 (Study on solutions for Ambient IoT in NR)	Ad-Hoc Chair (Huawei)

- R1-2409238	FL summary#2 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2409244	Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2409249	[Draft] Reply LS to RAN2 on data block sizes for Ambient IoT	Moderator (MediaTek)

- R1-2409250	Reply LS to RAN2 on data block sizes for Ambient IoT	RAN1, MediaTek

- R1-2409259	Feature Lead Summary #4 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2409297	Final FL summary on CW waveform characteristics for A-IoT	Moderator (Spreadtrum Communications)

- R1-2409304	Final FL summary on downlink and uplink channel/signal aspects	Moderator (Apple)

- R1-2409311	Feature Lead Summary #5 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2409319	FL summary (final) for Ambient IoT evaluation	Moderator (CMCC)

RAN1#119

- R1-2409358	Discussion on ambient IoT device architectures	TCL

- R1-2409359	Discussion on general aspects of physical layer design for Ambient IoT	TCL

- R1-2409360	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	TCL

- R1-2409361	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

- R1-2409362	Evaluation assumptions and results for Ambient IoT	Nokia

- R1-2409363	Ambient IoT device architectures	Nokia

- R1-2409364	General aspects of physical layer design for Ambient IoT	Nokia

- R1-2409365	Frame structure and timing aspects for Ambient IoT	Nokia

- R1-2409366	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

- R1-2409367	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

- R1-2409388	General aspects of physical layer design for Ambient IoT	Ericsson

- R1-2409389	Frame structure and timing aspects for Ambient IoT	Ericsson

- R1-2409390	Evaluation assumptions and results for Ambient IoT	Ericsson

- R1-2409391	Ambient IoT device architectures	Ericsson

- R1-2409416	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

- R1-2409417	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

- R1-2409418	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

- R1-2409419	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon, CBN, China Broadnet

- R1-2409420	Physical channels and signals for Ambient IoT	Huawei, HiSilicon, CBN, China Broadnet

- R1-2409421	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

- R1-2409485	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

- R1-2409486	Discussion on channel/signal aspects for Ambient IoT	Lenovo

- R1-2409487	Discussion on external carrier wave for Ambient IoT	Lenovo

- R1-2409511	Discussion on evaluation assumptions and results for Ambient IoT	CMCC

- R1-2409512	Discussion on Ambient IoT device architectures	CMCC

- R1-2409513	Discussion on general aspects of A-IoT physical layer design	CMCC

- R1-2409514	Discussion on frame structure and timing aspects for A-IoT	CMCC

- R1-2409515	Discussion on downlink and uplink channel/signal aspects	CMCC

- R1-2409516	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

- R1-2409535	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

- R1-2409550	Discussion on Ambient IoT evaluations	ZTE Corporation, Sanechips

- R1-2409551	Discussion on Ambient IoT device architectures	ZTE Corporation, Sanechips

- R1-2409552	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

- R1-2409553	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE Corporation, Sanechips

- R1-2409554	Discussion on channel and signal  for Ambient IoT	ZTE Corporation, Sanechips

- R1-2409555	Discussion on carrier wave for Ambient IoT	ZTE Corporation, Sanechips

- R1-2409596	Considerations for evaluation assumptions and results	Samsung

- R1-2409597	Remaining issues for Ambient-IoT device architectures	Samsung

- R1-2409598	Considerations for general aspects of Ambient IoT	Samsung

- R1-2409599	Considerations for frame structure and timing aspects	Samsung

- R1-2409600	Considerations for downlink and uplink channel/signal aspect	Samsung

- R1-2409601	Considerations for Waveform characteristics of carrier-wave	Samsung

- R1-2409635	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

- R1-2409636	Discussion on Ambient IoT device architectures	Spreadtrum, UNISOC, SGITG

- R1-2409637	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum, UNISOC

- R1-2409638	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum, UNISOC

- R1-2409639	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum, UNISOC

- R1-2409640	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum, UNISOC

- R1-2409680	Evaluation methodologies assumptions and results for Ambient IoT	vivo

- R1-2409681	Discussion on Ambient IoT Device architectures	vivo

- R1-2409682	Discussion on General Aspects of Physical Layer Design	vivo

- R1-2409683	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT	vivo

- R1-2409684	Discussion on  Downlink and uplink channel/signal aspects	vivo

- R1-2409685	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

- R1-2409733	Discussion on frame structure and timing aspects	InterDigital, Inc.

- R1-2409734	Discussion on downlink and uplink channel/signal aspects	InterDigital, Inc.

- R1-2409757	Discussion on Link Level simulation of A-IoT	Tejas Networks Limited

- R1-2409758	Discussion on receiver architecture of A-IoT	Tejas Networks Limited

- R1-2409759	Resource allocation and frame structure of A-IoT	Tejas Networks Limited

- R1-2409760	Study the downlink and uplink signal aspects of A-IoT	Tejas Networks Limited

- R1-2409799	On remaining evaluation assumptions and results for AIoT	Apple

- R1-2409800	On remaining details of device architecture for AIoT	Apple

- R1-2409801	On remaining general physical layer design aspects for AIoT	Apple

- R1-2409802	On remaining frame structure and timing aspects for AIoT	Apple

- R1-2409803	On remaining details of physical channels/signals for AIoT	Apple

- R1-2409804	On remaining details of carrier waveform and interference handling for AIoT	Apple

- R1-2409805	FL summary#1 on downlink and uplink channel/signal aspects	Apple

- R1-2409806	FL summary#2 on downlink and uplink channel/signal aspects	Apple

- R1-2409807	FL summary#3 on downlink and uplink channel/signal aspects	Apple

- R1-2409864	Discussion on general aspects of ambient IoT physical layer design	NEC

- R1-2409865	Discussion on frame structure and timing for ambient IoT	NEC

- R1-2409866	Discussion on downlink and uplink channel for ambient IoT	NEC

- R1-2409867	Discussion on the waveform of carrier-wave for the Ambient IoT	NEC

- R1-2409895	Evaluation assumptions and results for Ambient IoT	Xiaomi

- R1-2409896	Discussion on Ambient IoT device architectures	Xiaomi

- R1-2409897	Discussion on physical layer design of Ambient IoT	Xiaomi

- R1-2409898	Discussion on frame structure and timing aspects for Ambient IoT	Xiaomi

- R1-2409899	Discussion on downlink and uplink channel and signal aspects for Ambient IoT	Xiaomi

- R1-2409900	Discussion on waveform characteristics of carrier-wave	Xiaomi

- R1-2409940	The evaluation methodology and preliminary results of Ambient IoT	CATT

- R1-2409941	Study of the Ambient IoT devices architecture	CATT

- R1-2409942	Discussion on general aspects of physical layer design	CATT

- R1-2409943	Study of Frame structure and timing aspects for Ambient IoT	CATT

- R1-2409944	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

- R1-2409945	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

- R1-2409976	On General Physical Layer Design Considerations for Ambient IoT Applications	Lekha Wireless Solutions

- R1-2409993	TR 38.769 v1.1.0 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei (TR editor)

- R1-2410000	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

- R1-2410001	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

- R1-2410002	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2410003	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

- R1-2410024	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

- R1-2410025	On remaining open issues in Rel-19 Ambient IoT device architecture	FUTUREWEI

- R1-2410026	On remaining open issues in Rel-19 Ambient IoT physical layer design	FUTUREWEI

- R1-2410027	Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

- R1-2410028	D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

- R1-2410059	Discussions on FEC/repetition in R2D and D2R	Fujitsu

- R1-2410060	Discussion on frame structure and timing aspects	Fujitsu

- R1-2410061	Discussion on downlink and uplink channel/signal aspects	Fujitsu

- R1-2410063	Discussion on A-IoT Frame Structure and Timing Aspects	Panasonic

- R1-2410090	Discussion on evaluation assumptions and results for A-IoT	OPPO

- R1-2410091	Discussion on device architecture for A-IoT device	OPPO

- R1-2410092	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

- R1-2410093	Discussion on frame structure and timing aspects of A-IoT communication	OPPO

- R1-2410094	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

- R1-2410095	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

- R1-2410127	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

- R1-2410128	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

- R1-2410166	Discussion on frame structure and timing aspects	Google

- R1-2410167	Discussion on downlink and uplink transmission aspects	Google

- R1-2410178	Discussion on evaluation assumptions and results for Ambient IoT	HONOR

- R1-2410179	Discussion on frame structure and timing aspects for Ambient IoT	HONOR

- R1-2410199	Discussion on downlink and uplink channels and signals for A-IoT	Panasonic

- R1-2410207	Discussion on ambient IoT evaluation framework	NEC

- R1-2410208	Device architecture requirements for ambient IoT	NEC

- R1-2410225	Physical layer design of Ambient IoT	Sony

- R1-2410226	Frame structure and timing aspects of Ambient IoT	Sony

- R1-2410227	Downlink and uplink channel aspects of Ambient IoT	Sony

- R1-2410267	Discussion on general aspects of physical layer design	ETRI

- R1-2410268	Discussion on frame structure and timing aspects	ETRI

- R1-2410269	Discussion on downlink and uplink channel/signal aspects for A-IoT	ETRI

- R1-2410270	Discussion on carrier-wave for Ambient IoT	ETRI

- R1-2410285	Discussion on Ambient IoT evaluation	LG Electronics

- R1-2410286	Discussion on Ambient IoT device architectures	LG Electronics

- R1-2410287	General aspects of Ambient IoT physical layer design	LG Electronics

- R1-2410288	Frame structure and timing aspects for Ambient IoT	LG Electronics

- R1-2410289	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

- R1-2410290	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

- R1-2410311	Discussion on physical layer design for Ambient IoT	InterDigital, Inc.

- R1-2410312	Evaluation results for Ambient IoT	InterDigital, Inc.

- R1-2410313	Device architectures for Ambient IoT	InterDigital, Inc.

- R1-2410314	Carrier-wave for Ambient IoT	InterDigital, Inc.

- R1-2410330	TP for Conclusions in TR 38.769	Huawei, HiSilicon

- R1-2410351	Link level simulation for Ambient IoT R2D	Panasonic

- R1-2410352	General aspects of physical layer design for Ambient IoT	Panasonic

- R1-2410356	Discussion on frame structure and timing aspects for Ambient IoT	TCL

- R1-2410372	Discussion on A-IoT physical layer design	ASUSTeK

- R1-2410388	Study on evaluation assumptions and results for Ambient IoT	NTT DOCOMO, INC.

- R1-2410389	Study on device archtectures for Ambient IoT	NTT DOCOMO, INC.

- R1-2410390	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

- R1-2410391	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2410392	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

- R1-2410393	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

- R1-2410409	Discussion on frame structure and timing aspect	ASUSTeK

- R1-2410411	Discussion on downlink and uplink channel/signal aspect	ASUSTeK

- R1-2410416	Discussion on general aspects of physical layer design	Sharp

- R1-2410417	Discussion on frame structure and timing aspects	Sharp

- R1-2410418	Discussion on downlink and uplink channel/signal aspects	Sharp

- R1-2410422	Discussion on evaluation assumptions and results for Ambient IoT	Indian Institute of Tech (M)

- R1-2410423	Discussion on Ambient IoT Device Architectures	Indian Institute of Tech (M), IIT Kanpur

- R1-2410439	Considerations for downlink and uplink channel/signal aspects	Semtech Neuchatel SA

- R1-2410477	Evaluation Assumptions and Results	Qualcomm Incorporated

- R1-2410478	Ambient IoT Device Architecture	Qualcomm Incorporated

- R1-2410479	General aspects of physical layer design	Qualcomm Incorporated

- R1-2410480	Frame structure and timing aspects	Qualcomm Incorporated

- R1-2410481	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

- R1-2410482	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

- R1-2410502	Analysis for CFO calibration for device 2b	Wiliot Ltd.

- R1-2410513	Evaluation assumptions and results for A-IoT	MediaTek Inc.

- R1-2410514	Ambient IoT device architectures	MediaTek Inc.

- R1-2410515	General aspects of physical layer design	MediaTek Inc.

- R1-2410516	Frame structure and timing aspects	MediaTek Inc.

- R1-2410517	Downlink and uplink channel/signal aspects	MediaTek Inc.

- R1-2410518	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	MediaTek Inc.

- R1-2410551	Discussion on the evaluation assumptions for Ambient IoT devices	Lenovo

- R1-2410552	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

- R1-2410560	Discussion on frame structure and timing aspects for Ambient IoT	WILUS Inc.

- R1-2410577	Discussion on Evaluation assumptions and results	CEWiT

- R1-2410578	Discussion on Frame structure and timing aspects	CEWiT

- R1-2410579	Discussion on Downlink and Uplink channel/signal aspects	CEWiT

- R1-2410580	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

- R1-2410590	Evaluation assumption and results for AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2410591	Discussion on General aspects of physical layer design of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2410592	Frame structure and timing aspects of AIoT	IIT Kanpur, Indian Institute of Tech (M)

- R1-2410593	Discussion on Downlink and uplink channel/signal aspects	IIT Kanpur, Indian Institute of Tech (M)

- R1-2410629	Discussion on Ambient IoT evaluations	ZTE Corporation, Sanechips

- R1-2410630	Considerations for evaluation assumptions and results	Samsung

- R1-2410637	FL summary #1 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2410638	FL summary #2 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2410639	FL summary #3 for Ambient IoT evaluation	Moderator (CMCC)

- R1-2410643	FL summary #1 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2410644	FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2410645	FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2410647	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom

- R1-2410653	TP updates on section 6.2 Device (un)availability for TR 38.769	Moderator (vivo)

- R1-2410658	Discussion on Evaluation assumptions and results	CEWiT

- R1-2410700	Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2410701	Feature Lead Summary #2 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2410702	Feature Lead Summary #3 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

- R1-2410703	Feature Lead Summary #1 for 9.4: “Ambient IoT – TR Conclusions”	Moderator (Huawei)

- R1-2410704	Feature Lead Summary #2 for 9.4: “Ambient IoT – TR Conclusions”	Moderator (Huawei)

- R1-2410710	FL Summary #1 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2410711	FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2410712	FL Summary #3 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2410713	FL Summary #4 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

- R1-2410773	FL summary#1 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum)

- R1-2410825	TP for Annex Z of TR38.769 for receiver sensitivity	Moderator (CMCC)

- R1-2410826	TP for Clause 7.1.1-7.1.4 of TR38.769 for coverage results	Moderator (CMCC)

- R1-2410827	TP for Clause 7.2.1 of TR38.769 for single device latency	Moderator (CMCC)

- R1-2410828	TP for Clause 7.2.2 of TR38.769 for multi-device inventory completion time	Moderator (CMCC)

- R1-2410845	Session notes for 9.4 (Study on solutions for Ambient IoT in NR)	Ad-Hoc Chair (Huawei)

- R1-2410854	TP for Annex Z of TR38.769 for receiver sensitivity	Moderator (CMCC)

- R1-2410855	TP for Clause 7.1.1-7.1.4 of TR38.769 for coverage results	Moderator (CMCC)

- R1-2410866	Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1-2410875	TP for coverage results spreadsheet for TR38.769	Moderator (CMCC)

RAN2#127bis

- R2-2407907	Reply LS on Clarification of requirements for Ambient IoT (R1-2407364; contact: Ericsson)	RAN1

- R2-2407934	Reply LS on Clarification of requirements for Ambient IoT (S1-242527; contact: Ericsson)	SA1

- R2-2407946	Discussion on the Functionality Aspects for Ambient IoT	CATT

- R2-2407947	Discussion on Paging for Ambient IoT	CATT

- R2-2407948	Discussion on the Random Access for Ambient IoT	CATT

- R2-2407949	Discussion on Topology 2 for Ambient IoT	CATT

- R2-2407953	Discussion on access procedure for ambient IOT	Xiaomi

- R2-2407954	Discussion on A-IOT paging procedure	Xiaomi

- R2-2407984	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA

- R2-2407985	Report of [POST127][033][AIoT] Random Access	Huawei, HiSilicon

- R2-2408041	Discussion on A-IoT random access	China Telecom

- R2-2408042	Discussion on Topology 2 related aspects for Ambient IoT	China Telecom

- R2-2408058	Ambient-IoT Functionality Aspects	NEC  Corporation

- R2-2408059	Discussion on the paging for A-IoT	Transsion Holdings

- R2-2408060	Discussion on the random access for A-IoT	Transsion Holdings

- R2-2408068	Discussion on A-IoT paging	CMCC

- R2-2408069	Discussions on topology 2 for A-IoT	CMCC

- R2-2408093	Further consideration on random access for ambient IoT	CMCC

- R2-2408099	Discussion on functionality for A-IoT	CMCC

- R2-2408137	Discussion on paging procedure for Ambient IoT	OPPO

- R2-2408139	Discussion on topology 2 for Ambient IOT	OPPO

- R2-2408140	random access for AIoT	OPPO

- R2-2408145	Discussions on Functionality Aspect of Ambient IoT	Fujitsu

- R2-2408146	Discussions on AIoT paging	Fujitsu

- R2-2408147	Discussions on AIoT Random Access	Fujitsu

- R2-2408148	Discussions on topology 2 related aspects	Fujitsu

- R2-2408162	Ambient-IoT Paging	NEC

- R2-2408166	Discussion on the functionalities required for Ambient IoT	Spreadtrum Communications

- R2-2408167	Discussion on random access of Ambient IoT	Spreadtrum Communications

- R2-2408170	Discussion on paging procedure of A-IoT	Spreadtrum Communications

- R2-2408171	Discussion on Topology 2 issues	Spreadtrum Communications

- R2-2408186	Discussion on ambient IoT paging	LG Electronics Inc.

- R2-2408188	Considerations on TP2 related aspects between BS and UE reader	Beijing Xiaomi Software Tech

- R2-2408191	Discussion on A-IOT functionality aspects	Xiaomi

- R2-2408192	Study the A-IoT random access procedure	Tejas Network Limited

- R2-2408195	Discussion on Topology 2 for A-IoT	SHARP Corporation

- R2-2408226	Discussion on functionality aspects for Ambient IoT	vivo

- R2-2408227	Discussion on AIoT Paging	vivo

- R2-2408228	Random Access Procedure for A-IoT Device	vivo

- R2-2408229	Discussion on Topology 2 related aspects	vivo

- R2-2408243	A-IoT random access procedure	Huawei, HiSilicon

- R2-2408253	A-IoT functionalities	Huawei, HiSilicon

- R2-2408254	A-IoT paging	Huawei, HiSilicon

- R2-2408255	A-IoT Topology 2 aspects	Huawei, HiSilicon

- R2-2408275	Discussions on A-IoT functionality	HONOR

- R2-2408276	Consideration on resource allocation for Topology 2	HONOR

- R2-2408277	Discussion on A-IoT random access	HONOR

- R2-2408278	Discussion on A-IoT paging	HONOR

- R2-2408291	Stop-and-wait protocol for UP	ZTE Corporation, Sanechips, China Unicom, Nordic Semiconductor ASA

- R2-2408309	Discussion on paging procedure for Ambient IoT	Lenovo

- R2-2408310	Discussion on random access for Ambient IoT	Lenovo

- R2-2408311	Considerations on Topology 2 for Ambient IoT	Lenovo

- R2-2408331	Further consideration on the details of paging for AIoT	ZTE Corporation, Sanechips

- R2-2408332	Further consideration on TP2 for AIoT	ZTE Corporation, Sanechips

- R2-2408355	Discussion on energy status report and data status indication	ASUSTeK

- R2-2408356	Discussion on Ambient IoT paging message with multiple IDs	ASUSTeK

- R2-2408357	Consideration on information and scenario in Topology 2	ASUSTeK

- R2-2408383	Discussion on random access for AIoT	China Unicom

- R2-2408395	Functionality for Ambient IOT	InterDigital

- R2-2408396	Paging Related Aspects for Ambient IOT	InterDigital

- R2-2408397	Random Access Procedure for Ambient IOT	InterDigital

- R2-2408398	Topology 2 for Ambient IOT	InterDigital

- R2-2408439	Ambient IoT device paging	TCL

- R2-2408446	Energy Status report	Vodafone, Qualcomm, Nokia, Interdigital, Xiaomi, Samsung, Deutsche Telekom, ZTE, Sharp, Semtech, Asustek

- R2-2408448	Discussion on AIoT functionalities	OPPO

- R2-2408453	Inventory procedure without device ID	Vodafone

- R2-2408458	Discussion on Ambient IoT device AS ID and device energy status report	Panasonic

- R2-2408463	Discussion on A-IoT paging	Panasonic

- R2-2408482	Discussion on A-IoT functionality	ETRI

- R2-2408504	AIoT Functionality Aspects	Nokia France

- R2-2408512	Considerations for D2R failure/success indication in A-IoT	Panasonic

- R2-2408536	Unified random-access procedure for A-IoT	ZTE Corporation, Sanechips

- R2-2408542	Energy status report for Ambient-IoT	Quectel

- R2-2408546	Considerations on functionality aspects for Ambient IoT	Lenovo

- R2-2408549	Random Access for Ambient IoT device	NEC

- R2-2408580	Functional Aspects of Ambient IoT	Apple

- R2-2408581	Discussion on Ambient IoT Paging	Apple

- R2-2408582	Discussion on Random Access for Ambient IoT	Apple

- R2-2408583	Discussion on Ambient IoT Topology 2	Apple

- R2-2408609	Topology 2 related aspects	NEC

- R2-2408625	Ambient IoT Paging	Qualcomm Incorporated

- R2-2408626	Further aspects of Topology 2	Qualcomm Incorporated

- R2-2408668	Functionality aspects for A-IoT	Ericsson

- R2-2408671	Discussions on Energy Status Report in Ambient IoT Devices	ROBERT BOSCH GmbH

- R2-2408672	Discussions on Failure Detection/Recovery in Ambient IoT Devices	ROBERT BOSCH GmbH

- R2-2408687	On Topology 2 and DO-A	Ericsson

- R2-2408688	Discussion on UL multiple access	Ericsson

- R2-2408698	Paging procedures for Ambient IoT	Nokia

- R2-2408700	Discussion on random access for Ambient AIoT	Nokia

- R2-2408701	Discussion on A-IoT functionalities	NTT DOCOMO INC.

- R2-2408702	Discussion on A-IoT paging message	NTT DOCOMO INC.

- R2-2408703	Discussion on A-IoT random access	NTT DOCOMO INC.

- R2-2408706	Considerations on functionality aspects for Ambient IoT	Sony

- R2-2408707	Considerations on paging for Ambient IoT	Sony

- R2-2408708	Considerations on random access aspects for Ambient IoT	Sony

- R2-2408753	Toplogy 2 considerations	Nokia

- R2-2408829	Discussion on functionality for ambient IoT	Google Ireland Limited

- R2-2408834	Discussion on multiple access for ambient IoT	Google Ireland Limited

- R2-2408847	Ambient IoT Paging Procedure	China Telecom

- R2-2408850	Discussion on Functionalities Required for A-IoT	China Telecom

- R2-2408868	Considerations on Topology 2 for Ambient IoT	TCL

- R2-2408869	Discussion on A-IoT paging	Fraunhofer HHI, Fraunhofer IIS

- R2-2408906	Discussion on Paging aspects for Ambient-IoT	Continental Automotive

- R2-2408908	Discussion on RACH aspects for Ambient-IoT	Continental Automotive

- R2-2408927	Further discussions on A-IoT random access	ETRI

- R2-2408934	Message size reporting and segmentation	Nordic Semiconductor ASA

- R2-2408938	AS scheduling ID and AS short ID considerations	Nordic Semiconductor ASA

- R2-2408942	Considerations for energy status report	Semtech Neuchatel SA

- R2-2408951	Ambient IoT paging and unification of use cases	MediaTek Inc.

- R2-2408952	Ambient IoT data transmission after initial access	MediaTek Inc.

- R2-2408953	Ambient IoT topology 2 and out-of-coverage operation	MediaTek Inc.

- R2-2408954	Duplicate A-IoT paging message transmission and indication	SHARP Corporation

- R2-2408955	Use cases for energy status report in AIoT	SHARP Corporation

- R2-2408961	Discussion on DL messages for Ambient IoT UEs	Ericsson

- R2-2408992	Further consideration of A-IoT paging for Ambient IoT	Kyocera

- R2-2408993	Further consideration of functionality aspects for Ambient IoT	Kyocera

- R2-2408994	Discussion on Topology 2 aspects of Ambient IoT	Kyocera

- R2-2408995	Discussion on random access of Ambient IoT	Kyocera

- R2-2409013	Discussino on segmentation functionalities for Ambient IoT	Samsung Electronics Czech

- R2-2409019	Discussion on A-IoT paging	Samsung Electronics Czech

- R2-2409020	Discussion on A-IoT Topology 2	Samsung Electronics Czech

- R2-2409021	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated

- R2-2409022	Discussions on functionalities required for AIoT	Samsung Electronics Czech

- R2-2409028	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated

- R2-2409029	Discussion on functionality aspects of ambient IoT	LG Electronics Inc.

- R2-2409030	Discussion on topology 2 considerations of ambient IoT	LG Electronics Inc.

- R2-2409050	Discussion on Ambient IoT paging	LG Uplus

- R2-2409075	Discussion on Ambient IoT random access	KT Corp.

- R2-2409109	Discussion on A-IOT paging procedure	CEWiT

- R2-2409110	Discussion on random access for Ambient IoT	CEWiT

- R2-2409122	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.

- R2-2409167	Discussion on A-IOT RACH related aspects	Rakuten Mobile, Inc

- R2-2409169	Timing Relationship of Msg3 and R2D response	Philips International B.V.

- R2-2409172	Discussions on AS ID for Ambient IoT devices	Futurewei

- R2-2409173	Further discussions on Ambient IoT random access	Futurewei

- R2-2409176	Further discussions on Ambient IoT Paging	Futurewei

- R2-2409405	Reply LS to RAN2 on data block sizes for Ambient IoT (R1-2409250; contact: MediaTek)	RAN1

- R2-2409412	LS on assistance information from the CN to the Reader for A-IoT	RAN2

RAN2#128

- R2-2409511	Reply LS to RAN2 on data block sizes for Ambient IoT (R1-2409250; contact: MediaTek)	RAN1

- R2-2409526	LS on security aspects of Ambient IoT (S2-2411049; contact: OPPO)	SA2

- R2-2409577	Discussion on access procedure for ambient IOT	Xiaomi

- R2-2409578	Discussion on A-IOT paging procedure	Xiaomi

- R2-2409582	Discussion on access procedure for ambient IOT	Xiaomi

- R2-2409583	Discussion on A-IOT paging procedure	Xiaomi

- R2-2409621	Discussion on the Functionality Aspects for Ambient IoT	CATT

- R2-2409622	Discussion on Paging for Ambient IoT	CATT

- R2-2409623	Discussion on the Random Access for Ambient IoT	CATT

- R2-2409624	Discussion on Topology 2 for Ambient IoT	CATT

- R2-2409687	Remaining issues of AIoT Paging	ZTE Corporation, Sanechips

- R2-2409688	Remaining issues of AIoT TP2	ZTE Corporation, Sanechips

- R2-2409700	Device support of ambient IoT random access procedures	MediaTek Inc.

- R2-2409701	Ambient IoT resource allocation for topology 2	MediaTek Inc.

- R2-2409702	Ambient IoT coverage, energy, TB size, and all sorts of other things	MediaTek Inc.

- R2-2409703	Ambient IoT ID management and implications for energy status indication	MediaTek Inc.

- R2-2409707	Discussion on functionality aspects for Ambient IoT	vivo

- R2-2409708	Discussion on AIoT Paging	vivo

- R2-2409709	Random Access Procedure for AIoT Device	vivo

- R2-2409710	Discussion on Topology 2 related aspects	vivo

- R2-2409738	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.

- R2-2409739	Discussion on ambient IoT paging	LG Electronics Inc.

- R2-2409742	Considerations on TP2 related aspects between BS and UE reader	Xiaomi

- R2-2409783	A-IoT functionality	ZTE Corporation, Sanechips

- R2-2409784	Unified random-access procedure for A-IoT	ZTE Corporation, Sanechips

- R2-2409792	Random Access for Ambient IoT device	NEC

- R2-2409812	TP for TR 38.769 update	Huawei, CMCC, T-Mobile USA

- R2-2409837	Discussions on Functionality Aspect of Ambient IoT	Fujitsu

- R2-2409838	Discussions on AIoT paging	Fujitsu

- R2-2409839	Discussions on AIoT Random Access	Fujitsu

- R2-2409840	Discussions on topology 2 related aspects	Fujitsu

- R2-2409884	Discussion on Paging for A-IoT	Transsion Holdings

- R2-2409885	Discussion on Random Access for A-IoT	Transsion Holdings

- R2-2409890	Discussion on D2R failure/success indication in A-IoT	SHARP Corporation

- R2-2409892	Discussion on paging procedure for Ambient IoT	OPPO

- R2-2409894	Discussion on topology 2 for Ambient IOT	OPPO

- R2-2409895	random access for AIoT	OPPO

- R2-2409897	A-IoT functionalities	Huawei, HiSilicon

- R2-2409898	A-IoT paging	Huawei, HiSilicon

- R2-2409899	A-IoT Topology 2 aspects	Huawei, HiSilicon

- R2-2409963	Functional Aspects of Ambient IoT	Apple

- R2-2409964	Discussion on Ambient IoT Paging	Apple

- R2-2409965	Discussion on Random Access for Ambient IoT	Apple

- R2-2409966	Remaining open issues in Topology 2	Apple

- R2-2410002	Functionality for Ambient IOT	InterDigital

- R2-2410003	Paging Related Aspects for Ambient IOT	InterDigital

- R2-2410004	Random Access Procedure for Ambient IOT	InterDigital

- R2-2410005	Topology 2 for Ambient IOT	InterDigital

- R2-2410016	Discussion on A-IOT functionality aspects	Xiaomi

- R2-2410068	New and retransmitted A-IoT paging indication and device behaviors	Sharp

- R2-2410083	Validity of resources and reader behavior in Topology 2	NEC

- R2-2410096	Discussion on functionality for Ambient IoT	China Telecom

- R2-2410097	Random access procedure for A-IoT	China Telecom

- R2-2410098	Discussion on Topology 2 related aspects for Ambient IoT	China Telecom

- R2-2410124	Discussion on AIoT functionalities	OPPO

- R2-2410125	Ambient-IoT Paging	NEC

- R2-2410134	Considerations on functionality aspects for Ambient IoT	Lenovo

- R2-2410137	Discussion on the functionalities required for Ambient IoT	Spreadtrum, UNISOC

- R2-2410138	Discussion on random access of Ambient IoT	Spreadtrum, UNISOC

- R2-2410140	Discussion on paging procedure of AIoT	Spreadtrum, UNISOC

- R2-2410141	Discussion on Topology 2 issues	Spreadtrum, UNISOC

- R2-2410151	On Topology 2	Ericsson

- R2-2410152	Discussion on UL multiple access	Ericsson

- R2-2410178	Discussion on Ambient IoT paging message with multiple IDs	ASUSTeK

- R2-2410179	Consideration on information and scenario in Topology 2	ASUSTeK

- R2-2410193	Functionality aspects for A-IoT	Ericsson

- R2-2410251	Discussion on topology 2 for A-IoT	KT Corp.

- R2-2410261	Study the A-IoT Random Access Procedure	Tejas Network Limited

- R2-2410264	Discussion on paging procedure for Ambient IoT	Lenovo

- R2-2410265	Discussion on random access for Ambient IoT	Lenovo

- R2-2410266	Considerations on Topology 2 for Ambient IoT	Lenovo

- R2-2410295	Discussion on A-IoT paging	Panasonic

- R2-2410300	Discussion on Ambient IoT segmentation functionality	Panasonic

- R2-2410311	Remaining issues on study of AIoT functionalities	NTT DOCOMO, INC.

- R2-2410312	Discussion on RA type indication in AIoT paging	NTT DOCOMO, INC.

- R2-2410313	Remaining issues on study of AIoT random access	NTT DOCOMO, INC.

- R2-2410315	Further discussion on ambient IoT random access	CMCC

- R2-2410334	Discussion on A-IoT paging	CMCC

- R2-2410335	Discussions on topology 2 for A-IoT	CMCC

- R2-2410351	A-IoT random access procedure	Huawei, HiSilicon

- R2-2410353	Ambient-IoT Functionality Aspects	NEC

- R2-2410370	A-IoT functionalities	ETRI

- R2-2410371	Discussion on ‘temporary out of connection’ in Topology 2	ETRI

- R2-2410374	Considerations on functionality aspects for Ambient IoT	Sony

- R2-2410375	Considerations on paging for Ambient IoT	Sony

- R2-2410376	Considerations on random access aspects for Ambient IoT	Sony

- R2-2410403	CBRA type down-selection	Vodafone, Huawei, CMCC, Telit Communications S.p.A., CATT, Nokia

- R2-2410404	Paging procedures for Ambient IoT	Nokia

- R2-2410414	Open issues on random access for AIoT	Nokia

- R2-2410416	Lean Inventory procedure/counting of devices	VODAFONE

- R2-2410423	Functionality aspects of AIoT	Nokia France

- R2-2410467	RAN2 Aspects for AIoT Operation in Topology 2	Nokia

- R2-2410476	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated

- R2-2410477	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated

- R2-2410493	Discussion on remaining aspects of AS ID	Continental Automotive

- R2-2410551	Ambient IoT Paging	Qualcomm Incorporated

- R2-2410552	Further aspects of Topology 2	Qualcomm Incorporated

- R2-2410572	Discussions on Energy Status Indication for Ambient IoT	Futurewei

- R2-2410573	Further discussions on AS ID for Ambient IoT devices	Futurewei

- R2-2410575	Further discussions on Ambient IoT paging	Futurewei

- R2-2410576	Issues with previous agreement on Msg2 in 2-step CBRA	Futurewei

- R2-2410593	Discussion on Ambient IoT random access	KT Corp.

- R2-2410594	Discussion on functionality aspects of ambient IoT	LG Electronics Inc.

- R2-2410595	Discussion on topology 2 considerations of ambient IoT	LG Electronics Inc.

- R2-2410603	Use cases for one-bit energy status report in AIoT	SHARP Corporation

- R2-2410605	Discussion on DL messages for Ambient IoT UEs	Ericsson

- R2-2410616	Ambient IoT device paging	TCL

- R2-2410620	Discussion on functionality for ambient IoT	Google Ireland Limited

- R2-2410629	Discussion on random access for ambient IoT	Google Ireland Limited

- R2-2410645	Remaining issues of A-IoT paging for Ambient IoT	Kyocera

- R2-2410646	Remaining issues of functionalities for Ambient IoT	Kyocera

- R2-2410647	Remaining issues on Topology 2 aspects of Ambient IoT	Kyocera

- R2-2410672	Discussion on functionality for A-IoT	CMCC

- R2-2410679	Discussion on functionality aspects for A-IoT	HONOR

- R2-2410680	Discussion on A-IoT Paging	HONOR

- R2-2410681	Discussion on A-IoT random access	HONOR

- R2-2410682	Consideration on resource configuration for Topology 2	HONOR

- R2-2410693	Further discussions on Ambient IoT Paging	China Telecom

- R2-2410719	Further discussion on Ambient IoT random access	Samsung

- R2-2410753	Discussions on functionalities required for AIoT	Samsung

- R2-2410754	Discussion on A-IoT paging	Samsung

- R2-2410755	Discussion on A-IoT Topology 2	Samsung

- R2-2410760	Considerations on Topology 2 for Ambient IoT	TCL

- R2-2410766	Discussion on paging procedure for ambient IoT	Google Ireland Limited

- R2-2410779	Discussion on A-IOT paging procedure	CEWiT

- R2-2410780	Discussion on random access for Ambient IoT	CEWiT

- R2-2410805	Discussion on Topology 2 related aspects	Fraunhofer HHI, Fraunhofer IIS

- R2-2410868	Handling of Msg2 for 2step CBRA	Philips International B.V.

- R2-2410890	A-IoT Random Access - failure/success feedback indication for following D2R data	Wiliot Ltd.

- R2-2411078	Reply LS on security aspects of Ambient IoT (S3-245139; contact: OPPO)	SA3

- R2-2411179	Conclusion of  [AT128][016][AIoT] Resource validity (Interdigital)	InterDigital

- R2-2411213	TP for the A-IoT conclusion	Huawei, HiSilicon

RAN3#125bis

- R3-245005	Reply LS on Clarification of requirements for Ambient IoT	RAN1(Ericsson)

- R3-245015	Reply LS on Clarification of requirements for Ambient IoT	SA1(Ericsson)

- R3-245051	Discussion on Unified logical system architecture for topology 1 and 2	ZTE Corporation

- R3-245052	(TP for TR38.769) Unified logical system architecture for Topology 1 and Topology 2	ZTE Corporation

- R3-245054	(TP for TR38.769) content for Ambient-IoT Inventory procedure	ZTE Corporation

- R3-245055	(TP for TR38.769) Locating an AIoT device	ZTE Corporation

- R3-245058	(TP to TR 38.769) A-IoT architecture aspects	CATT

- R3-245059	(TP to TR 38 769) Procedures for AIoT Command	CATT

- R3-245060	(TP to TR 38 769) Information exchange between AIoT CN and AIoT RAN	CATT

- R3-245061	(TP to TR 38.879) Locating of A-IoT device	CATT

- R3-245069	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC

- R3-245070	(TP to TR 38.769) A-IoT RAN Architecture aspects	Huawei

- R3-245071	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi

- R3-245072	(TP to TR 38.769) A-IoT Signalling and Procedures for Topology 1	Huawei

- R3-245073	(TP to TR 38.769) A-IoT Signalling and Procedures for Topology 2	Huawei

- R3-245074	(TP to TR 38.769) A-IoT Device Locating	Huawei

- R3-245081	(TP to BL CR for TR 38.769) AIoT RAN architecture	Xiaomi

- R3-245082	(TP to BL CR for TR 38.769)) AIoT interface impacts between RAN and CN	Xiaomi

- R3-245083	(TP to BL CR for TR 38.769)) Locating AIoT device	Xiaomi

- R3-245127	Further discussion on RAN architecture for Ambient IoT	Samsung

- R3-245128	Further discussion on RAN-CN interface impact for Ambient IoT	Samsung

- R3-245141	(TP to TR 38.769) RAN architecture for Ambient-IoT	China Telecom

- R3-245289	(TP for TR 38.769) Discussion on Architecture of Ambient IoT	NEC

- R3-245290	Discussion on RAN-CN interface impact on Ambient IoT	NEC

- R3-245307	(TP for TR38.769) Text description for AIoT Signalling and Procedures	ZTE Corporation, Huawei, NEC, Nokia, CATT, China Telecom, CMCC

- R3-245331	Architecture, interfaces and transport for Ambient IoT	Qualcomm Incorporated

- R3-245332	Inventory and Command related procedures for Ambient IoT	Qualcomm Incorporated

- R3-245333	Methods for locating Ambient IoT devices	Qualcomm Incorporated

- R3-245452	On RAN architecture of ambient IoT	Lenovo

- R3-245453	On basic procedures of ambient IoT	Lenovo

- R3-245454	Context Management and Data Transport of Ambient IOT	Lenovo

- R3-245529	Work Plan for Ambient IoT SI	CMCC, Huawei, T-Mobile USA

- R3-245530	Discussion on RAN Architecture for Ambient IoT	CMCC

- R3-245531	Discussion on signalling procedure for topology1	CMCC

- R3-245532	Discussion on signalling procedure for topology2	CMCC

- R3-245533	Discussion on locating Ambient IoT device	CMCC

- R3-245573	[TP for TR 38.769] Protocol Stack Option for Topology 2	Nokia

- R3-245574	Inventory Signalling reusing Paging procedures for AIoT Topology 2	Nokia

- R3-245575	[TP for TR 38.769] Inventory Signalling reusing Paging procedures for AIoT Topology 2	Nokia

- R3-245576	[TP for TR 38.769] Data Transport and Context Management for AIoT	Nokia

- R3-245579	Further discussionis on AIoT RAN architecture	Ericsson

- R3-245580	[TP for TR 38.769] on AIoT Architecture aspects	Ericsson

- R3-245581	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling	Ericsson

- R3-245582	[TP for TR 38.769] On locating AIoT devices	Ericsson

- R3-245638	Discussion on RAN impact for Ambient IoT	LG Electronics

- R3-245644	Discussion on RAN-CN interface for Ambient IoT	China Telecom

- R3-245721	CB:#AIoT1_ProtocolStack	Huawei

- R3-245723	[TP for TR 38.769] CB:#AIoT2_RANCNinterface	Ericsson

- R3-245725	CB:#AIoT3_Location	Nokia

- R3-245827	(TP for TR 38.769] TP for Locating Ambient IoT devices	Nokia

- R3-245828	(TP to TR 38.769) A-IoT RAN Architecture aspects	Huawei

- R3-245829	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi

- R3-245846	(TP to TR 38.769) A-IoT RAN Architecture aspects	Huawei

- R3-245847	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo

- R3-245848	[TP for TR 38.769] CB:#AIoT2_RANCNinterface	Ericsson LM

- R3-245849	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC, Ericsson, ZTE, CATT, NEC, Xiaomi, Lenovo

RAN3#126

- R3-247036	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC, Ericsson, ZTE, CATT, NEC, Xiaomi, Lenovo

- R3-247147	(TP to 38.769) Conclusion for Topology 1 and updates to terminologies	Qualcomm Incorporated

- R3-247148	(TP to 38.769) Conclusion for Topology 2	Qualcomm Incorporated

- R3-247149	(TP to 38.769) Inventory and Command related procedures for Ambient IoT	Qualcomm Incorporated

- R3-247150	(TP to 38.769) Locating Ambient IoT devices	Qualcomm Incorporated

- R3-247184	(TP for TR38.769) some leftover issues on RAN architecture aspects	ZTE Corporation

- R3-247185	(TP to TR 38.769) Signaling and Procedures for NAS/UP based Topology 2	ZTE Corporation, Huawei, Xiaomi, NEC, Nokia, China Telecom, CATT, CMCC

- R3-247186	(TP to TR 38.769) Completion of Signaling and Procedures for Topology 2	ZTE Corporation

- R3-247187	(TP for TR38.769) Inventory content for Ambient-IoT	ZTE Corporation

- R3-247188	(TP for TR38.769) Locating an AIoT device	ZTE Corporation

- R3-247189	(TP to TR 38.769) A-IoT RAN Architecture aspects	Huawei

- R3-247190	(TP to TR 38.769) Protocol Stacks for Topology 2 NAS/UP based solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm

- R3-247191	(TP to TR 38.769) A-IoT Signalling and Procedures for Topology 1	Huawei

- R3-247192	[DRAFT] LS on RAN3 outcome of Ambient IoT study	Huawei

- R3-247193	Consideration on A-IoT Device Locating	Huawei

- R3-247216	(TP to TR38.769) Discussion on architecture of Ambient IoT	NEC

- R3-247217	(TP to TR38.769) Discussion on RAN-CN interface impact on Ambient IoT	NEC

- R3-247284	(TP to TR 38.769) A-IoT architecture aspects	CATT

- R3-247285	(TP to TR 38 769) A-IoT Procedures and Information exchange between A-IoT CN and RAN	CATT

- R3-247286	(TP to TR 38.879) A-IoT reader selection	CATT

- R3-247287	(TP to TR 38.879) Locating of A-IoT device	CATT

- R3-247312	Discussion on AIoT RAN architecture	Xiaomi

- R3-247313	AIoT interface impacts between RAN and CN	Xiaomi

- R3-247314	(TP for TR 38.769) Procedures for A-IoT command	Xiaomi, Huawei, ZTE, CATT, CMCC

- R3-247381	A-IoT architecture aspects	China Telecom

- R3-247437	Considerations on bistatic mode of operation	Lenovo

- R3-247438	(TP to TR 38.769) Protocol Stack of UP based Solution for Topology 2	Lenovo

- R3-247439	Radio resource allocation and UE reader selection for topology 2	Lenovo

- R3-247489	[TP for TR 38.769] Information exchanged between AIoT RAN and AIoT CN	Nokia

- R3-247490	[TP for TR 38.769] Reader Selection Principles in AIoT Topology 2	Nokia

- R3-247492	[TP for TR 38.769] Locating AIoT Devices	Nokia

- R3-247585	Further aspects on AIoT Architecture	Ericsson

- R3-247586	[TP for TR 38.769] on AIoT Architecture aspects	Ericsson

- R3-247587	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling	Ericsson

- R3-247588	[TP for TR 38.769] On locating AIoT devices	Ericsson

- R3-247610	Remaining issues on RAN architecture for Ambient IoT	Samsung

- R3-247611	Remaining issues on RAN-CN interface impact for Ambient IoT	Samsung

- R3-247643	Discussion on Signalling Procedure Based on Topology 1	China Telecom

- R3-247683	Work Plan for Ambient IoT SI	CMCC, Huawei, T-Mobile USA

- R3-247684	(TP to TR 38.769) RAN3 Conclusions and Recommendations	CMCC, T-Mobile USA, Huawei

- R3-247685	(TP to TR 38.769) Uncritical Editor's Note Removal	CMCC, Huawei, Samsung, CATT, Lenovo, Xiaomi, ZTE, NEC, China Telecom

- R3-247686	Discussion on RAN Architecture for Ambient IoT	CMCC

- R3-247687	Discussion on coordination between CN and RAN	CMCC

- R3-247688	Discussion on locating Ambient IoT device	CMCC

- R3-247689	(TP to TR 38.769) locating ambient IoT devices	CMCC, Huawei, ZTE, CATT, Xiaomi, Nokia, LG Electronics, China Telecom

- R3-247726	Discussion on RAN impact for Ambient IoT	LG Electronics

- R3-247727	Further discussion on locating A-IoT Device	LG Electronics

- R3-247761	Response to R3-247684	Nokia

- R3-247786	(TP to TR 38.769) Signaling and Procedures for NAS/UP based Topology 2	ZTE Corporation, Huawei, Xiaomi, NEC, Nokia, China Telecom, CATT, CMCC

- R3-247794	CB:#AIoT1_ProtocolStack	Huawei

- R3-247795	[TP for TR 38.769] CB:#AIoT2_SignallingFlow	Ericsson

- R3-247797	CB:AIoT3_LocationReport	Nokia

- R3-247830	(TP to TR 38.769) A-IoT RAN Architecture aspects	Huawei

- R3-247831	(TP to TR 38.769) Protocol Stacks for Topology 2 NAS/UP based solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm

- R3-247845	[TP for TR 38.769] Locating AIoT Devices	Nokia

- R3-247902	SoD of CB: # AIoT4_SIConclusionandTR	CMCC

- R3-247903	(TP to TR 38.769) RAN3 Conclusions and Recommendations	CMCC, Huawei, T-Mobile USA

- R3-247904	(TP to TR 38.769) Uncritical Editor's Note Removal	CMCC, Huawei, Samsung, CATT, Lenovo, Xiaomi, ZTE, NEC, China Telecom

- R3-247918	(TP to TR 38.769) Protocol Stacks for Topology 2 NAS/UP based solutions	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo, China Telecom, Samsung, Qualcomm

- R3-247919	[TP for TR 38.769] for signalling flow	Ericsson

- R3-247920	[TP for TR 38.769] Locating AIoT Devices	Nokia

- R3-247921	(TP to TR 38.769) Removal of Editor's Note	CMCC, Huawei, Samsung, CATT, Lenovo, Xiaomi, ZTE, NEC, China Telecom

- R3-247922	(TP to TR 38.769) RAN3 Conclusions and Recommendations	CMCC, Huawei, T-Mobile USA

- R3-247923	LS on RAN3 outcome of Ambient IoT study	RAN3(Huawei)

- R3-247924	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR	Huawei, CMCC, Ericsson, ZTE, CATT, NEC, Xiaomi, Lenovo

RAN4#112bis

- R4-2415001	Collection of calibration data for A-IoT, CMCC

- R4-2415097	Discuss on AIoT R2D LLS in RAN4, CATT

- R4-2415449	A-IoT general overview, Ericsson

- R4-2417193	A-IoT general overview, Ericsson

- R4-2414963	Discussion on A-IoT deployment scenarios and spectrum usage, Huawei HiSilicon

- R4-2415305	Discussion on AIoT deployment scenarios and spectrum usage, Qualcomm

- R4-2415448	A-IoT deployment scenario and spectrum usage, Ericsson

- R4-2415763	Discussion on the impact of CW node, vivo

- R4-2414996	Discussion on A-IoT co-existence evaluation, CMCC

- R4-2415098	Discussion on co-existence simulation parameters, CATT

- R4-2415306	Discussion on Ambient IoT Coexistence Evaluations, Qualcomm

- R4-2415453	Coexisting study simulation assumptions and initial results, Ericsson

- R4-2415580	Coexistence evaluation on D1T1 deployment scenario, Sony

- R4-2415596	on co-existence evaluations, OPPO

- R4-2415608	Discussion on co-existence evaluation results for Ambient IoT and NR, Spreadtrum

- R4-2415764	Discussion on the co-existence for D2T2, vivo

- R4-2416229	Discussion on Co-existence evaluations, ZTE

- R4-2416230	The calibration results for A-IoT evaluations, ZTE

- R4-2416301	On co-existence assumptions, Samsung

- R4-2415052	Simulation results for Ambient-IoT Co-existence study, Xiaomi

- R4-2415082	Co-existence evaluation results for Ambient IoT in NR, CATT

- R4-2416355	Collection of simulation results for Ambient-IoT co-existence study, Huawei

- R4-2416356	TP to TR38.769: Co-existence simulation, Huawei

- R4-2414997	Discussion on A-IoT BS RF requirements, CMCC

- R4-2415111	Discussion on A-IoT BS requirements, CATT

- R4-2415765	Discussion on the RF requirement for AIoT BS, vivo

- R4-2416231	Discussion on RF requirement of Ambient IoT BS, ZTE

- R4-2416282	RF requirements for Ambient IoT BS, Huawei

- R4-2415110	draft TP to TR 38.769 for A-IoT BS TX requirements, CATT

- R4-2415447	A-IoT BS RF overview, Ericsson

- R4-2414998	Discussion on A-IoT device RF requirements, CMCC

- R4-2415099	Discussion on A-IoT device RF requirements, CATT

- R4-2415466	On the RF requirements for Ambient IoT Device, Huawei

- R4-2415530	Energy harvesting considerations for A-IoT device, Qualcomm

- R4-2415581	Further considerations on the ambient IoT device implementation and RF aspect, Sony

- R4-2415588	Discussion on RF requirements impact for ambient IoT device, Spreadtrum

- R4-2415598	on device RF requirements, OPPO

- R4-2415766	Discussion on the RF requirement for AIoT device, vivo

- R4-2416232	Discussion on RF requirement of Ambient IoT device, ZTE

- R4-2416482	Discussion on Ambient IoT Device RF requirement impact, LGE

- R4-2415051	TP for TR 38.769 on RF requirements for device 2a, Xiaomi

- R4-2415451	A-IoT device OTA requirement and testing aspects, Ericsson

- R4-2415591	TP for TR 38.769 on device 1 RF requirements, Spreadtrum

- R4-2415595	TP to TR on device 2b RF requirements, OPPO

- R4-2414964	On the RF Requirements for Ambient IoT intermediate UE, Huawei

- R4-2414999	Discussion on A-IoT intermediate UE RF requirements, CMCC

- R4-2415112	Discussion on A-IoT intermediate UE requirements, CATT

- R4-2415589	Discussion on RF requirements impact for intermediate node (UE), Spreadtrum

- R4-2415599	on intermediate UE RF requirements, OPPO

- R4-2415767	Discussion on the RF requirement for intermediate UE, vivo

- R4-2416233	Discussion on RF requirement of Intermediate node (UE), ZTE

- R4-2416352	Simultaneous operation of Ambient IoT and Uu in intermediate UE, LGE

- R4-2415450	RF overview for UE as reader, Ericsson

- R4-2416360	TP to TR 38.769 for Intermediate node Rx requirement, LGE

- R4-2415000	Discussion on CW cancellation capability, CMCC

- R4-2415529	CW cancellation feasibility, Qualcomm

- R4-2415590	Discussion on methods and feasibility values of CW interference cancellation, Spreadtrum

- R4-2415597	on CW cancellation capability, OPPO

- R4-2415768	Dicussion on the CW cancellation capability, vivo

- R4-2416234	Discussion on the feasibility values of CW interference cancellation from BS perspective, ZTE

- R4-2416300	On Ambient IoT CW cancellation, Samsung

- R4-2415452	On CW cancellation capability, Ericsson

- R4-2416283	CW interference cancelation, Huawei

- R4-2417195	TP for CW interference cancelation, Huawei, HiSilicon, Ericsson

- R4-2415233	Topic summary for [112bis][129] FS_Ambient_IoT_solutions_part1, Moderator (CMCC)

- R4-2417196	WF on Co-existence study for A-IoT and NR/LTE, CMCC

- R4-2415234	Topic summary for [112bis][130] FS_Ambient_IoT_solutions_part2, Moderator (Huawei)

- R4-2417088	WF on RF requirement impact of A-IoT, Huawei

RAN4#113

- R4-2417831	Further co-existence evaluation results for Ambient IoT in NR	CATT

- R4-2417844	Discussion on A-IoT device RF requirements	CATT

- R4-2417855	Discussion on A-IoT BS requirements	CATT

- R4-2417856	TP to TR 38.769 for A-IoT BS TX requirements	CATT

- R4-2417857	Discussion on A-IoT intermediate UE requirements	CATT

- R4-2417936	TP for TR 38.769 on RF requirements for device 2a	Xiaomi

- R4-2417979	Simulation analysis for Ambient IoT co-existence	Huawei, HiSilicon

- R4-2417980	TP to TR38.769: Co-existence simulation	Huawei, HiSilicon

- R4-2417981	Collection of simulation results for Ambient-IoT co-existence study	Huawei, HiSilicon

- R4-2418034	Coexistence evaluation on D1T1 deployment scenario	Sony

- R4-2418035	Further considerations on the ambient IoT device implementation and RF aspect	Sony

- R4-2418121	TP for TR 38.769: CW cancellation feasibility company results	Qualcomm Incorporated

- R4-2418122	TP for TR 38.769: Testability aspects of A-IoT device	Qualcomm Incorporated

- R4-2418154	Topic summary for [113][132] FS_Ambient_IoT_solutions_part1	Moderator (CMCC)

- R4-2418155	Topic summary for [113][133] FS_Ambient_IoT_solutions_part2	Moderator (Huawei)

- R4-2418180	TP to TR 38.769  on the impact from AIoT CW to NR	vivo

- R4-2418181	TP  to TR 38.769  on D2T2 co-existence evaluation and analysis	vivo

- R4-2418182	Further evaluation on D2T2 co-existence	vivo

- R4-2418183	Discussion on the RF requirement for AIoT BS	vivo

- R4-2418184	Discussion on the RF requirement for AIoT device	vivo

- R4-2418185	TP to TR 38.769 on the intermediate UE Tx RF requirement	vivo,Huawei

- R4-2418186	Discussion on the RF requirement for intermediate UE	vivo

- R4-2418187	TP to TR 38.769 TP on the CW cancellation capability	vivo

- R4-2418293	TP to TR 38.769 on the RF Requirements for Ambient IoT device 1	Spreadtrum Communications

- R4-2418319	Discussion on RF requirements impact for ambient IoT device	Spreadtrum Communications

- R4-2418320	Discussion on RF requirements impact for intermediate node (UE)	Spreadtrum Communications

- R4-2418403	Discussion on co-existence evaluation results for Ambient IoT and NR	Spreadtrum Communications

- R4-2418723	Draft TP to TR38.769 on conclusion for A-IOT	CMCC

- R4-2418724	Evaluation methodology for ambient IOT co-existence	CMCC

- R4-2418725	Discussion on A-IoT co-existence evaluation	CMCC

- R4-2418726	Discussion on A-IoT BS RF requirements	CMCC

- R4-2418727	Discussion on A-IoT device RF requirements	CMCC

- R4-2418728	Discussion on A-IoT intermediate UE RF requirements	CMCC

- R4-2418801	A-IoT general aspects	Huawei, HiSilicon

- R4-2418802	A-IoT co-existence evaluation cases	Huawei, HiSilicon

- R4-2418803	On the RF requirements for A-IoT BS	Huawei, HiSilicon

- R4-2418804	On the RX RF requirements for A-IoT intermdiate UE	Huawei, HiSilicon

- R4-2418852	Discussion on Ambient IoT Coexistence Evaluations	Qualcomm Incorporated

- R4-2418978	Simultaneous operation of Ambient IoT and Uu in intermediate UE	LG Electronics UK

- R4-2418998	Draft TP to TR 38.769 for A-IoT intermediate node RX requirements	LG Electronics UK

- R4-2419199	Discussion on Co-existence evaluations	ZTE Corporation, Sanechips

- R4-2419200	TP to TR38.769: Ambient BS Rx requirements	ZTE Corporation, Sanechips

- R4-2419201	Discussion on RF requirement of Ambient IoT device	ZTE Corporation, Sanechips

- R4-2419202	Discussion on RF requirement of Intermediate node (UE)	ZTE Corporation, Sanechips

- R4-2419203	Discussion on the feasibility values of CW interference cancellation from BS perspective	ZTE Corporation, Sanechips

- R4-2419246	TP for CW interference cancelation	Huawei, HiSilicon

- R4-2419366	on co-existence evaluations	OPPO

- R4-2419367	on device RF requirements	OPPO

- R4-2419368	on intermediate UE RF requirements	OPPO

- R4-2419369	TP on device 2b RF requirement	OPPO

- R4-2419475	TP to TR 38.769:  A-IoT BS RF requirement	Ericsson

- R4-2419476	Coexisting study simulation assumptions and initial results	Ericsson

- R4-2419477	TP to TR 38.769:  A-IoT general overview	Ericsson

- R4-2419478	TP to TR 38.769:  RF overview for UE as reader	Ericsson

- R4-2419479	TP to TR 38.769:   A-IoT device OTA requirement and testing aspects	Ericsson

- R4-2419480	TP to TR 38.769:   On CW cancellation capability	Ericsson

- R4-2419481	Co-existence evaluation methodology and cases	Ericsson

- R4-2419553	TP to 38.769 on co-existence evaluation methodology	Qualcomm Incorporated

- R4-2419554	TP to 38.769 on co-existence simulation assumptions	Qualcomm Incorporated

- R4-2419638	On the RF requirements for Ambient IoT Device	Huawei, HiSilicon

- R4-2419649	On considerations of frequency shifter	Samsung

- R4-2420359	Draft TP to TR38.769 on conclusion for A-IOT	CMCC

- R4-2420360	TP to TR38.769: Co-existence simulation	Huawei, HiSilicon

- R4-2420362	TP to TR 38.769 for A-IoT BS TX requirements	CATT, Ericsson, Huawei, HiSilicon, LGE

- R4-2420363	TP to TR38.769: Ambient BS Rx requirements	ZTE Corporation, Sanechips, Ericsson, Huawei, HiSilicon

- R4-2420364	TP to TR 38.769:  A-IoT general overview	Ericsson

- R4-2420365	TP to TR 38.769 on the intermediate UE Tx RF requirement	vivo,Huawei

- R4-2420366	Draft TP to TR 38.769 for A-IoT intermediate node RX requirements	LG Electronics UK

- R4-2420367	TP to TR 38.769 on the RF Requirements for Ambient IoT device 1	Spreadtrum Communications

- R4-2420368	TP for TR 38.769 on RF requirements for device 2a	Xiaomi

- R4-2420369	TP on device 2b RF requirement	OPPO

- R4-2420370	TP for TR 38.769: Testability aspects of A-IoT device	Qualcomm Incorporated

- R4-2420371	TP for CW interference cancelation	Huawei, HiSilicon

- R4-2420372	TP for TR 38.769: RAN4 part for Ambient-IoT	CMCC

- R4-2420373	LS of RAN4 input on Ambient-IoT to TR 38.769	CMCC

- R4-2420470	WF on UE RF requirements for A-IoT	Huawei

- R4-2420504	TP to TR38.769: Co-existence simulation	Huawei, HiSilicon

10.01.2022		minor adaptations for RAN #95e,

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template