3GPP TSG RAN Meeting #106                                                                     RP-243170

Madrid, Spain, December 9-12, 2024

Source:	ZTE Corporation,Sanechips

Title:	New proposal for Rel-19 RRM enhancement

Agenda Item:	9.3.4.3

Document for:	Discussion/Decision

- 1	Introduction

In the latest SR for R19 RRM phase 5 WI , it indicated that the overall completion ratio is 60% and the objective of Fast SCell activation for UE supporting Rel-18 EMR was completed in RAN4#113 meeting. In other words, there would be some available TU left to accommodate other additional objective. Under this situation, we would like to propose to have an additional RRM objective based on the latest summary [1] in the RAN4 Rel-19 package discussion for RRM enhancement.

- 2 Discussion

## 2.1. Scell with uplink only transmission

Scell with uplink only transmission has been discussed in the previous release, lots of merits and applicable use cases (listed as follows) have been identified and well discussed. However due to the workload and limited TU restrictions, RRM requirement for Scell with uplink only transmission has not been specified yet until Rel-18. In order to enable this feature with full set of requirements in RAN4, it’s proposed to define the corresponding RRM requirement for it in Rel-19.

- In Japan, MIC has decided to allocate the 700MHz band to IMT service and the 1.2GHz and 2.3GHz bands to FPU service. Japan was planning to start assigning frequencies in the 2.3 GHz band by the end of 2023 and triple allocations across all compatible spectrum bands in 2025. It's essential to protect the incumbent service FPU from the foreseen IMT deployment, otherwise IMT BS DL CLI towards to FPU receiver would be significant. Scell with UL only transmission at 2.3GHz could mitigate the gNB to gNB CLI issue to enable the good coexistence performance with FPU.

- In certain regions, the deployment at 2.3GHz now is restricted to the indoor scenario due to the protection of the incumbent service. In order to better utilize the spectrum of 2.3GHz at the outdoor scenario, Scell with UL only transmission would  provide good coexistence performance with the incumbent service due to its limited transmission power and carrier emissions from UE transmitter.

- In USA, there are long discussion on coexistence issue between IMT deployment at C-band and aircraft radio altimeter at 4.2-4.4GHz. The FAA determined that radio altimeters cannot be relied upon to perform their intended function if they experience interference from wireless broadband operations in the 3.7-3.98 GHz frequency band (5G C-Band). Scell with UL only transmission at 5G C-band could provide good solution to enable the good coexistence performance with aircraft radio altimeter if there are no additional filtering implemented for the protection of interference from IMT network at the aircraft.

- With the deployment of NR network, operators may refarm some of the LTE bands to NR to improve UL coverage and throughput. Re-farm bands for UL transmission might be possible given NR coverage is UL limited. An operator may use other higher NR bands for capacity and the re-farming LTE band for coverage. Scell with UL only transmission can address such re-farming demands from operators.

- From network energy saving perspective, based on the study in Rel-18, UL only scell by shutting down the NW DL transmitter could achieve significant power saving. For BS Category 1, 65.5% to 71.7% of network power saving gain could be achieved and for BS Category 2, 80.7% to 83.3% of power saving gain could be achieved. Scell with UL only transmission could provide further network energy saving enhancement.

In addition, in Rel-18, there are following three options discussed for the band combination of CA_n5-n8, but options 2 & 3 are dropped and only option 1 was considered. Furthermore, the option 2 and option 3 are discussed in Rel-19. The conclusion in last RAN4 meeting was to adopt option 2 to enable UL inter-band CA for CA_n5-n8, and meanwhile RAN4 asked RAN2 via the LS to make necessary modifications in RAN2 spec and/or to add UE capability.

- Option 1: DL CA_n5-n8 with single UL in n5 only

- Option 2: UL/DL CA_n5-n8 with non-simultaneous Rx/Tx between n5 DL and n8 UL

- Option 3: UL/DL CA_n5-n8 with additional dedicated filter for the restricted frequency ranges for specific operator

Scell with UL only transmission could enable the operators to utilize the uplink spectrum in band n8 in addition to Rel-18 Option 1 solution.

The objectives of the work item are listed as follows:

  - Define the RRM requirement for Scell with UL only transmission [RAN4].

  - Potential RAN1/2 impact if any could be triggered by LS.

- 3	Conclusions

In this contribution, we propose to have an additional RRM objective for Rel-19 RRM enhancement as following:

Proposal: For Scell with uplink only transmission, propose to have the objective as follows:

  - Define the RRM requirement for Scell with UL only transmission [RAN4].

  - Potential RAN1/2 impact if any could be triggered by LS.

- 4	References

[1] RP-240019,Proposed Summary for RAN Rel-19 Package: RAN4 Part.