| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5D/TEMP/273(edited) |  |
|  | 12 February 2025 |
|  | English only |
| ITU-R Working Party 5D |  |
| REPLY LIAISON STATEMENT TO EXTERNAL ORGANIZATION (3GPP TSG-RAN WORKING GROUP 4 (RADIO)) |  |
| Parameters for 14 800 to 15 350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 |  |

3GPP TSG RAN Meeting #107						RP-250027

Incheon, Korea, March 12-14, 2025

Working Party (WP) 5D thanks TSG-RAN Working Group 4 (Radio) for the information provided in Document 5D/428 in response to the liaison statement (LS) from Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (Att. 7.4 to Doc. 5D/134).

Document 5D/428 provides information on recommended IMT technology related parameters for the frequency range 14 800 to 15 350 MHz. The referred document notes that AAS limits always apply Over-the-Air (OTA), and among the recommended parameters in the document, the spectral mask for AAS IMT base station has been provided in the table reproduced below.

TABLE 1A

AAS BS Spectral mask (Operating band unwanted emissions limits) for 
14 800-15 350 MHz operation (Category A)

| Frequency offset of measurement filter −3 dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2 dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | ‒14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | ‒13 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

The spectral mask provided in Document 5D/428 for AAS IMT base station in the frequency range 14 800 to 15 350 MHz is similar to the spectral mask of AAS IMT base station in the frequency range 7 125 to 8 400 MHz for 0 MHz £ Df < 50 MHz, and similar to the non-AAS IMT base station in the frequency range 7 125 to 8 400 MHz for Df ≥ 50 MHz.

Furthermore, the information on the spectrum mask and spurious emissions for Micro cell and Small cell indoor for 14.8-15.35 GHz were not provided in Document 5D/428.

Working Party 5D would welcome confirmation from TSG-RAN Working Group 4 (Radio) that the spectral mask provided in Table 1A is the recommended OTA limits for IMT base station spectral mask for all relevant frequency offset. Working Party 5D would also like to seek more information on the spectrum mask and spurious emissions for Micro cell and Small cell indoor for 14.8-15.35 GHz.

| Contact: Uwe Löwenstein | E-mail: Uwe.loewenstein@itu.int |
| --- | --- |

______________

____________________