3GPP TSG-RAN Meeting #107											RP-250118

Incheon, Korea, 12 - 14 March 2025

Agenda item:	15.2.1

Source:	NEC

Title: 	AIML for air interface Phase2

Document for:	Discussion and Decision

# Introduction

In the last RAN meeting [1], companies showed their views on having Phase2 for AI/ML for air interface and the following issues have been discussed. In this contribution, we provide our further views on AI/ML for air interface Phase2 for 5G-A in Rel-20.

AI/ML Air Interface - Whether/how to continue Artificial Intelligence (AI)/Machine Learning (ML) air interface in Rel-20 depends on the outcome of the Rel-19 study and possibly the leftovers of the normative work, if any. - Normative work on AI/ML air interface is expected, subject to successful completion of the Rel-19 study - E.g., two-sided model for CSI compression - Strong need to explore new use cases in Rel-20 for 5G-Advanced or not? - If so, these new use cases should be very limited. - Study of these new cases is expected to convert to normative work in Rel-20. - How to avoid overlapping with 6G study? - Any other aspects? - Important to keep the total amount of work related to AI/ML air interface within a reasonable load in Rel-20, and to avoid any overlap with the expected related 6G study

# Discussion

### CSI compression

In addition to Rel-18 study, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction. Meanwhile, Rel-18/19 studies identified and are trying to provide solutions to issues related to inter-vendor training collaboration. Furthermore, in RAN1, Rel-19 studies on model identification, data collection and model transfer for two-sided model are ongoing.

For CSI compression, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction.

For CSI compression, in RAN1, Rel-19 studies on model identification, data collection and model transfer for two-sided model are ongoing.

In Rel-20, it is important to specify CSI compression in 5G-Adv Rel-20 as the representative use case of two-sided model. As mentioned above, time domain aspects, cell/site specific models are essential to achieve the promised performance gain. The general procedure as in Figure 1 is needed to support CSI compression in Rel-20, which includes RAN1/RAN2 efforts to specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations (including model identification), data collection and model transfer (e.g., based on open format within 3GPP) for two-sided model. It is also required to alleviate/resolve issues on inter-vendor training collaboration and the consistency between model training and model inference.

Figure 1 General procedure to support CSI compression

Specify AI/ML based CSI compression with two-sided model [RAN1/RAN2]

Support both spatial/frequency CSI compression and spatial/temporal/frequency CSI compression, including measurement and report configurations, within CSI framework

Support LCM (including model identification), data collection and model transfer for two-sided model

Ensure inter-vendor training collaboration, as well as the consistency between model training and model inference

### Beam Management

Rel-19 supports AI/ML based beam management within CSI framework and inference configuration based LCM, and only works under a very basic assumption: single TRP, intra-cell, and Tx-beam prediction. It is not sufficient to enable AI/ML BM to be used in more practical FR2 scenarios with blockage, MTRP, inter-cell and Tx-Rx alignment requirements. Meanwhile, it is not studied on how to apply the prediction results to improve beam management procedures. To our understanding, the following needs to be supported to complete AI/ML based BM.

- Beam failure detection and new beam identification based on spatial/temporal beam prediction can solve the blockage issue more efficiently

- MTRP is a more typical deployment scenario and beam group prediction requires more designs on Set B/Set A pattern

- Inter-cell beam prediction can be done jointly with AI/ML mobility

- Tx-Rx beam alignment is a fundamental and realistic requirement and it needs more designs to enable Rx beam prediction

For AI/ML based beam management, Rel-19 specification only supports a very basic scenario, i.e., intra-cell single TRP Tx beam prediction.

Complete AI/ML based BM with considerations of BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction [RAN1/RAN2].

### Positioning

The Rel-19 WI objectives for AI/ML based positioning accuracy enhancement are as follows [4].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Since the workload in Rel-19, the specified 2nd priorities cases were not addressed in Q4 2024. However, given the numerous pending 1st priority cases that remain unresolved, and the fact that RAN2 has not and will not study Case 2a and Case 2b in Rel-19[5], it is unlikely to specify the remaining issues for 2nd priority cases for positioning in Rel-19, which undermines the potential of UE-assisted/LMF-based method in the AI/ML based positioning accuracy enhancement. However, Case 2a and Case 2b show the advantages in DL positioning, and there lack convincing technical reasons to deprioritize them, it is therefore proposed to complete the remaining work on these cases in Rel-20.

It is unlikely to specify the remaining issues for 2nd priority cases for positioning in Rel-19.

Complete 2nd priority AI/ML based positioning accuracy enhancement cases [RAN1/RAN2].

### Other use cases

To avoid overlaps between 5G-Adv and 6G, AI/ML for air interface for 5G-Adv should focus on some essential functions with limited scopes. To complete above objectives of CSI compression, BM, positioning, ~2TUs in RAN1 and ~1TU in RAN2 are needed. It is therefore proposed to have no new use case study in Rel-20 for 5G-Adv.

No new use case study in Rel-20 for 5G-Adv.

# Conclusion

In this contribution, we provided our views on Rel-20 AI/ML for air interface Phase2, and we have the following observations and proposals:

Observation 1:	For CSI compression, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction.

Observation 2:	For CSI compression, in RAN1, Rel-19 studies on model identification, data collection and model transfer for two-sided model are ongoing.

Observation 3:	For AI/ML based beam management, Rel-19 specification only supports a very basic scenario, i.e., intra-cell single TRP Tx beam prediction.

Observation 4:	It is unlikely to specify the remaining issues for 2nd priority cases for positioning in Rel-19.

Proposal 1:	Specify AI/ML based CSI compression with two-sided model [RAN1/RAN2]

−	Support both spatial/frequency CSI compression and spatial/temporal/frequency CSI compression, including measurement and report configurations, within CSI framework

−	Support LCM (including model identification), data collection and model transfer for two-sided model

−	Ensure inter-vendor training collaboration, as well as the consistency between model training and model inference

Proposal 2:	Complete AI/ML based BM with considerations of BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction [RAN1/RAN2].

Proposal 3:	Complete 2nd priority AI/ML based positioning accuracy enhancement cases [RAN1/RAN2].

Proposal 4:	No new use case study in Rel-20 for 5G-Adv.

In addition, model transfer, dataset transfer, and data collection for two-sided model may require collaboration with other TSGs (e.g., SA2, SA5), which can be triggered when necessary.

# References

- RP-242438, RAN Chair’s Summary of 5G-Advanced in Rel-20

- RP-242505, NEC, AIML for air interface Phase2, 3GPP TSG RAN #106, Madrid, Spain, December 9-12, 2024

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase

- ChairNotes, RAN2 #129, February 17-21, 2024