3GPP TSG-RAN Meeting #107												 RP-250119

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.2

Source: 	NEC

Title:	        Views on MIMO enhancement for NR

Document for:	 Discussion

# Introduction

In TSG RAN Meeting#106, majority companies support on further MIMO enhancement for 5G advanced. This contribution provides our views on the MIMO enhancement for NR Rel-20.

# Discussion

MIMO is one of the key technologies in NR systems and is successful in commercial deployment, and since Rel-15, MIMO has been enhanced on reference signals, CSI acquisition, beam management, and etc. While there are still some bottleneck to be enhanced for real commercial needs, and also considering the 6G study in Rel-20, it’s preferred to have a Rel-20 NR MIMO WI with ~1TU targeting real commercial needs as 6G study in parallel.

Support a Rel-20 NR WI on MIMO enhancement with ~1TU.

There are several topics discussed during RAN#106, and we show our views on the topics as follows:

- Frequency Selective Uplink Transmission:

Uplink performance and robust is still bottleneck due to frequency selective, and for uplink, subband or precoding resource block group (PRG) has not been discussed. To solve the issue, uplink frequency selective precoder (closed-loop) and uplink precoder cycling (open-loop) can be introduced to enhance the uplink transmission. And in NR phase, we can strive for a tradeoff between precoder indication overhead and performance, e.g. introducing uplink PRG with large size, and limiting the number of precoding matrix indication fields to be reasonable value, e.g. 4.

- UE initiated beam management (UEI BM) extension to multi-TRP:

Frequent periodic/semi-persistent or aperiodic beam reporting to timely acquire the best/preferred beam for data/control transmissions results in large UL reporting overhead and control signaling overhead. In Rel-19, UEI BM targeting single-TRP was introduced for more timely beam reports and reduced reporting overhead. Multi-TRP is a key feature for NR MIMO, which is feasible in implementation, especially for FR2. To fully utilize the benefit of UEI BM and also considering implementation feasibility, it’s natural to extend the UEI BM to multi-TRP.

- Early CSI or SRS triggering before RRC connection:

It was discussed that there are issues of late CSI or SRS for UEs transitioning from RRC idle/inactive to RRC connected during RAN1#106. We also think it can improve the system performance with good commercial need.

- Multi-layer DFT-s-OFDM:

It was proposed to extend multiple layers with DFT-s-OFDM in RAN#106, while whether >1 layer for DFT-s-OFDM (mainly targeting for coverage limited UE) a typical case still needs justification. If really necessary, current spec already supports dynamic waveform between DFT-s-OFDM and CP-OFDM, that means rank >1 scheduling can be achieved. Also considering this will cause big spec effort and implementation impact to discuss rank 2 (or more) codebook for DFT-s-OFDM.

In addition, there were also some topics, such as extending number of coordinated TRPs, number of DMRS ports, discussed in RAN#106, while we don’t think the topics needed at this late stage of NR, which can be discussed in 6G.

Support following topics for Rel-20 NR MIMO enhancement:

- Frequency selective uplink transmission

- Extension of UEIBM to multi-TRP

- Early SRS/CSI triggering

Not support following topics for Rel-20 NR MIMO enhancement, which can be discussed in 6G:

- Multi-layer DFT-s-OFDM

- Extension of number of TRPs

- Extension of number of DMRS ports

# Conclusion

In his contribution, we provide our views on the MIMO enhancement for NR Rel-20.

- Support a Rel-20 NR WI on MIMO enhancement with ~1TU.

Support following topics for Rel-20 NR MIMO enhancement:

- Frequency selective uplink transmission

- Extension of UEIBM to multi-TRP

- Early SRS/CSI triggering

Not support following topics for Rel-20 NR MIMO enhancement, which can be discussed in 6G:

- Multi-layer DFT-s-OFDM

- Extension of number of TRPs

- Extension of number of DMRS ports

# References

[1] RP-250002, Meeting Report for TSG RAN meeting #106