3GPP TSG-RAN Meeting #107	RP-250120

Incheon, Korea, 12 - 14 March 2025

Agenda Item:		15.2.3

Source:		NEC

Title:			Views on Rel-20 Ambient IoT

Document for:		Discussion and decision

# Introduction

In RAN#106, a discussion on Ambient IoT(AIoT) was given and a way forward RP-243280[1] was endorsed with brief discussion of Rel-20 AIoT scope as follows:

Workplan for Rel-19: - No impact to Rel-19 ASN.1 freeze, no impact to the timeline of Rel-20. Rel-20 Ambient IoT starts after the completion of the Rel-19 Ambient IoT WI. - Completion of the Rel-19 work (Core Part) is targeted at the stage-3 functional freeze for Rel-19 in Sept. 2025, with 3 quarters for RAN2/3/4 and 2 quarters for RAN1. Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase. For both Rel-19 and Rel-20, co-ordination with SA2/3 is expected

Due to limited time left in Rel-19, lots of contents were excluded from the scope of Rel-19 WID [2], and those contents are potential objective to be analysed in Rel-20 AIoT. Besides Rel-19 leftover, Rel-20 AIoT may further target for extended scenarios, such as outdoor application, DO-A service, etc. And those extended scenarios require additional efforts.

# Discussion

## 2.1	Rel-20 Justification

In Rel-19, AIoT has covered the service of indoor inventory/command with only one topology and only one device type and, i.e., topology 1 and device 1. However, according to TR 38.848 [3], various outdoor use cases could not be fulfilled by Rel-19 AIoT. The outdoor use cases include rUC5(outdoor inventory), rUC6(outdoor sensors), rUC7(outdoor positioning), rUC8(outdoor command). The details of those use case are shown by Table 1.

Table 1: Mapping between RAN representative use cases and SA1 use cases [3]

| rUC | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC5: Outdoor inventory | 5.2 Medical instruments inventory management and positioning 5.4 Non-public network for logistics 5.7 Airport terminal / shipping port 5.16 Automated supply chain distribution |
| rUC6: Outdoor sensor | 5.3 Smart grids 5.19 Forest Fire Monitoring 5.22 Dairy farming 5.24 Smart manhole cover safety monitoring 5.25 Smart bridge health monitoring |
| rUC7: Outdoor positioning | 5.8 Finding remote lost item 5.9 Location service 5.12 Personal belongings finding |
| rUC8: Outdoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.30 Controller in smart agriculture |

Though Rel-19 AIoT targets for indoor inventory/command, due to only 6 months for work item, its scope is minimized to cater for the limited time. However, device 1 has the lowest transmission coverage, which means the usage of Rel-19 AIoT is restricted. As mentioned in RP-243280[1], at least topology 2 and device 2 are listed as workplan for Rel-20. In other words, even for indoor service, there are still plenty of enhancement work expected to be done in Rel-20 AIoT.

Proposal 1: Rel-20 AIoT scope should cover both outdoor use case and enhancements on R19 AIoT indoor use case.

## 2.2	SI/WI objectives

Below we analyse details of our preferred Rel-20 scope.

- Outdoor use cases

The outdoor use cases include 4 general categories, i.e., rUC5(outdoor inventory), rUC6(outdoor sensors), rUC7(outdoor positioning), rUC8(outdoor command). Since Rel-19 AIoT targets for indoor inventory and indoor command, rUC5 and rUC8 would be enhancement for those inventory and command service. For example, when only indoor inventory is equipped, inventory has to be done per warehouse, and those goods in transportation may be regarded as “missing”. With both indoor and outdoor inventory, those goods can be inventoried regardless of their location. For rUC6, outdoor sensors have new application in forest fire monitoring, dairy farming, etc. As for rUC7 with outdoor positioning, since Rel-19 AIoT performs positioning at the granularity of each reader, whether to support sophisticated positioning is questionable and may require further discussion.

Proposal 2: Rel-20 AIoT should contain outdoor inventory and outdoor command as prioritized outdoor use cases.

- Topology 2 support

Topology 2 for Outdoor Usage:

The structure of topology 2 is shown by Fig.1. And details of topology 2 were discussed in Rel-19 AIoT study phase with outputs in TR 38.769.

Figure 1: Topology 2 structure

Due to limited time, topology 2 is excluded from Rel-19 AIoT WI, and is expected in workplan of Rel-20 AIoT as the endorsed workplan in RP-243280[1]. One benefit of topology 2 is extended coverage by the help of intermediate node. Since AIoT device is cost-sensitive and power-limited, the link between AIoT device to reader can sustain a maximum distance of 50m. However, for outdoor usage, the coverage requires in units of km. Therefore, an intermediate node is beneficial to extend the coverage distance from tens of meters to several kms.

Proposal 3: Topology 2 should be supported in Rel-20 AIoT.

RRC-Based Solution:

In TR 38.769, there are 3 solutions for Topology 2 architecture.

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information: -RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. -NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. -UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data. NOTE 1: The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any.

Among the three solutions, the RRC-based solution offers awareness to the RAN node, which is advantageous for more precise reader control. Given the need for down-selection from the aforementioned three architectures during Rel-20, the RRC-based solution should be supported.

Proposal 4: RRC-based solution should be supported for Topology 2.

To support topology 2, the impacts on the immediate UE must be taken into account. This includes inventory and command data transmission, the authorization of the UE reader, resource allocation, resource validation, RAN-CN interface design. These issues also encompass several key points, such as the RRC states in which the immediate UE can operate, as well as the mechanisms for activating or terminating the reader function or A-IoT resources. All these aspects require collaborative efforts from both RAN2 and RAN3.

Fundamental Signaling Requirements:

The following fundamental messages must be carefully designed:

- Inventory Request/Response: For initiating and acknowledging inventory procedures.

- Inventory Report: For transmitting collected inventory data.

- Command Request/Response: For executing control commands.

- Resource Allocation Procedures: For managing resources over both the CN-RAN interface and the Uu interface between the UE reader and AIoT-enabled gNB.

Additionally, the information contained within these messages must be defined to ensure interoperability and efficiency.

Proposal 5: Specify signaling over both the CN-RAN interface and the Uu interface between the UE reader and AIoT-enabled gNB.

Support for RRC_INACTIVE State:

RRC states for UE reader to perform A-IoT operations are expected to be determined during the Rel-20 WI phase.  Besides RRC_CONNECTED, which is the baseline, supporting the RRC_INACTIVE state is also crucial for power saving, as no Uu signaling exchange is expected during the inventory procedure for AIoT devices.

Proposal 6: The UE reader's operation in the RRC_CONNECTED and RRC_INACTIVE state should be supported for Topology 2.

- DO-A service

In TR 38.769 [4], DO-A service is assessed as follows.

## 6.10 DO-A assessment The study included an assessment of whether the harmonized air interface design can address the DO-A use case, only to identify which part(s) of the harmonized air interface design is/are not sufficient for the DO-A use case. From the RAN1 perspective, at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type: For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. From RAN2 perspective, the DO-A traffic type/use case cannot be supported with the current design in the study item. It is assessed that, from RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

Rel-19 AIoT design targets for DO-DTT and DT traffic types and is not suitable for DO-A traffic both from RAN1 and RAN2 perspective. To equip Rel-20 AIoT with DO-A service, efforts should be given by at least RAN1 and RAN2 working groups.

Moreover, DO-A service is crucial for a wide area of applications. One benefit of DO-A service is low traffic delay. For example, fire alarm application does not need frequent reporting from sensors, but when hazards happen, those sensors are expected to have quick feedback to reader, which could not be fulfilled by Rel-19 AIoT design.

Proposal 7: DO-A support is important for commercial usage and should be in scope of Rel-20 AIoT.

- Device 2 support

Similar to topology 2, device 2 is also excluded from Rel-19 AIoT WI due to time limit, and is expected in workplan of Rel-20 AIoT as the discussion in RP-243280[1]. Device 2 contains two sub-types: device 2a and device 2b. Device 2a can be regarded as an enhancement of device 1 with ability to reflect carrier wave. Device 2b, on the contrary, has the ability to generate independent signals without requirement of carrier wave. Device 2b is expected to have larger coverage than device 2a and device 1. Hence, for the benefits of outdoor use cases and DO-A service, device 2b should be first priority among all device types. Device 2a can be adopted if time allows.

Proposal 8: Device 2 should be in scope of Rel-20 AIoT, and device 2b should be prioritized.

- Carrier wave transmission

In Rel-19 study item, it is identified that carrier wave (CW) waveform characteristics, including the transmission timing, power, and frequency resource, need control of CW node. In Rel-19 WI, only topology 1 is supported, and case 1-4 of CW is adopted. Since the CW node is outside topology and it can be an implementation issue, there is no specification impact of CW control in Rel-19 WI. However, in Rel-20, topology 2 should be considered, and the intermediate node will take the role of reader, and it is straightforward that the reader UE can also transmit CW signal to device. Therefore, case 2-2 (i.e., CW is transmitted from inside the topology by reader UE in UL spectrum) should be supported. In addition, control information of CW transmission may be provided from gNB to the reader UE in order to control the CW characteristics identified in Rel-19 SI.

Proposal 9: Support carrier wave transmitted from inside the topology by reader UE in UL spectrum (i.e., case 2-2 in TR 38.769) in Rel-20 AIoT.

Proposal 10: Support network to provide control information of carrier wave transmission to reader UE in Rel-20 AIoT.

Based on the analysis above, the objectives of the new Rel-20 SI are summarized as below.

Proposal 11: RAN to approve a new Rel-20 SI/WI on AIoT as captured below.

The scope aims to cover both leftover of Rel-19 AIoT and investigate new outdoor use case. The detailed objectives are listed as follows: - Outdoor use cases - Prioritize outdoor inventory and outdoor command; - Topology 2 - Support RRC-based solution; - Signaling over both the CN-RAN interface and the Uu interface between the UE reader and AIoT-enabled gNB; - Support UE reader's operation in the RRC_CONNECTED and RRC_INACTIVE state; - DO-A service - Device 2 - Device 2b should be prioritized for outdoor service; - Carrier wave - Support carrier wave from reader UE in UL spectrum; - Support network to provide control information of carrier wave transmission to reader UE; Note: Topology 2 and device 2 have been studied in Rel-19, and may not require further discussion in study phase. Note: Co-ordination with SA2/3 is expected.

## 2.3	Expected time scale

The contents of Rel-20 AIoT scope has been discussed as Rel-19 AIoT enhancement and outdoor services not covered by Rel-19 AIoT. For the enhancement part of Rel-19, study-phase outcomes have been captured in TR 38.769[4], and not further study efforts are needed. But for outdoor services, some preliminary discussion is still needed. Therefore, a short study phase with 3 or 6 months and a following work phase are expected.

Proposal 12: RAN to allocate 3 or 6 months for Rel-20 AIoT SI on outdoor services and leave remaining time for a following-up WI on both outdoor services and Rel-19 AIoT enhancement.

# Proposals

In this contribution, we shared our further considerations on Rel-20 AIoT and the following proposals were drawn from the analysis.

Proposal 1: Rel-20 AIoT scope should cover both outdoor use case and enhancements on R19 AIoT indoor use case.

Proposal 2: Rel-20 AIoT should contain outdoor inventory and outdoor command as prioritized outdoor use cases.

Proposal 3: Topology 2 should be supported in Rel-20 AIoT.

Proposal 4: RRC based solution should be supported for Topology 2.

Proposal 5: Specify signaling over both the CN-RAN interface and the Uu interface between the UE reader and AIoT-enabled gNB.

Proposal 6: The UE reader's operation in the RRC_CONNECTED and RRC_INACTIVE state should be supported for Topology 2.

Proposal 7: DO-A support is important for commercial usage and should be in scope of Rel-20 AIoT.

Proposal 8: Device 2 should be in scope of Rel-20 AIoT, and device 2b should be prioritized.

Proposal 9: Support carrier wave transmitted from inside the topology by reader UE in UL spectrum (i.e., case 2-2 in TR 38.769) in Rel-20 AIoT.

Proposal 10: Support network to provide control information of carrier wave transmission to reader UE in Rel-20 AIoT.

Proposal 11: RAN to approve a new Rel-20 SI/WI on AIoT as captured below.

The scope aims to cover both leftover of Rel-19 AIoT and investigate new outdoor use case. The detailed objectives are listed as follows: - Outdoor use cases - Prioritize outdoor inventory and outdoor command; - Topology 2 - Support RRC-based solution; - Signaling over both the CN-RAN interface and the Uu interface between the UE reader and AIoT-enabled gNB; - Support UE reader's operation in the RRC_CONNECTED and RRC_INACTIVE state; - DO-A service - Device 2 - Device 2b should be prioritized for outdoor service; - Carrier wave - Support carrier wave from reader UE in UL spectrum; - Support network to provide control information of carrier wave transmission to reader UE; Note: Topology 2 and device 2 have been studied in Rel-19, and may not require further discussion in study phase. Note: Co-ordination with SA2/3 is expected.

Proposal 12: RAN to allocate 3 or 6 months for Rel-20 AIoT SI on outdoor services and leave remaining time for a following-up WI on both outdoor services and Rel-19 AIoT enhancement.

# Reference

[1] RP-243280 Way forward for Ambient IoT for REL-19 and REL-20

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] TR 38.848 Study on Ambient IoT (Internet of Things) in RAN v18.0.0

[4] TR 38.769 Study on solutions for ambient IoT (Internet of Things) v2.0.0