3GPP TSG-RAN Meeting #107	RP-250124

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.7.1

Source:	NEC

Title:	Views on Rel-20 Coverage Enhancement for 5G-Adv

Document for:	Discussion and Decision

1	Introduction

Coverage is very important for operators’ network deployment. It has strong operator support from a variety of regions. 3GPP have enhanced NR coverage in Rel-17 coverage enhancement and in Rel-18 further coverage enhancement. Many physical channels and signals are coverage enhanced. However, there are still some physical channels are identified to be needed to enhance but not specified due to limited time during Rel-18. Those could be done in Rel-20 as an additional RAN1-led topics based on the guidance of RAN Chair’s Summary of 5G-Advanced in Rel-20 [1].

In this paper, we discuss and give our views on Rel-20 Coverage Enhancement for 5G-Adv.

2	Discussion

For Rel-20 5G-Adv scope, coverage enhancement should be included due to strong market interest. Some areas have been discussed in Rel-19 WID work but no consensus was reached. Those areas could be the starting point of Rel-20 coverage enhancement, they are:

- Multiple PRACH TX over different beams including beam selection for Msg3

- PUSCH repetition scheduled by DCI 0_0 with C-RNTI

2.1	Multiple PRACH transmissions with different beams

Beam correspondence accuracy has large impact on the performance of PRACH transmission and the followed msg3 transmission. Sometimes, UE could not select the best beam for PRACH transmission. In such case, when multiple PRACH transmissions are done with different beams, thanks to the opportunity diversity, gains can be realized compared with PRACH transmissions with the same beam.

UE with beam correspondence can take advantage of the preferred UL Tx beam based on the DL Rx beam. This decreases the need for beam sweeping for preferred UL Tx beam that legacy operation currently may require.

During the study item and work item of PRACH coverage enhancement. Multiple PRACH transmission with different beams associated with the same SSB has been discussed. Due to the limited time, they are not specified. For Rel-20 coverage enhancement, it could be the starting point for the further discussion. It is shown as in fig.1.

Fig.1 Multiple PRACH transmissions through different Tx beams

2.2	Msg5 PUSCH repetition

Field tests have shown that Msg5 is the bottleneck for initial access, due to the large payload size of Msg5 when compared to other messages. Msg5 transmission performance may be additionally negatively impacted due to Msg3 repetition, especially when msg5 and Msg3 retransmission happening at a similar time which can lead to network congestion.

To enhance the coverage and efficient of msg5 transmission, some normative works are needed. Msg5 during initial access is scheduled by DCI format 0_0 with C-RNTI. Even though some PUSCH coverage enhancement has been specified, those enhancements are not applied for PUSCH scheduled by DCI format 0_0. That means PUSCH repetition for msg5 transmission is not supported. To support PUSCH repetition for msg5 transmission, we need to support PUSCH repetition procedures scheduled by DCI 0_0 with C-RNTI. Of course, we could discuss whether PUSCH repetition scheduled by DCI format 0_0 for UL transmission other than msg5 transmission is needed. At least for msg5 transmission, we should support PUSCH repetition scheduled by DCI format 0_0 with C-RNTI.

3	Conclusion

In this contribution, we give our views on Rel-20 Coverage Enhancement for 5G-Adv. We propose that:

Proposal 1: Support coverage enhancement for Rel-20 5G-Adv for RAN1-led work, including:

- Further PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions through different Tx beams for 4-step RACH procedure

  - UL beam indication based on different Tx beams from multiple PRACH transmissions for Msg3

- Support PUSCH repetition scheduled by DCI format 0_0 with C-RNTI (RAN1, RAN2)

References

[1]	RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, TSG RAN#106.