3GPP TSG RAN Meeting #107		RP-250125

Incheon, Korea, 12 - 14 March, 2025

Title: 	Support of On-demand SSB for Idle/Inactive UEs in Rel-20

Source: 	NEC, CEWiT

Agenda item:	15.2.7.2

Document for:	Discussion and decision

# Introduction

The rapid expansion of 5G networks and the growing demand for data-intensive applications make energy efficiency a critical concern.  Minimizing environmental impact and operational costs are key drivers for innovation.  Releases 18 and 19 of the 5G standard introduced significant enhancements to optimize energy usage, addressing both dynamic and static power consumption.  Rel-18 focused on features like SSB-less SCell operation, enhanced cell DTX/DRX, and spatial and power domain optimizations [1]. Rel-19 extended these improvements with on-demand SSB and SIB1 transmissions, and adaptation of common signal/channel transmissions in the time and spatial domains [2].  Building on these advancements, Rel-20 should explore additional opportunities to maximize network energy savings.

# On-Demand SSB for Idle/Inactive Operation

Although Rel-18 and Rel-19 introduced various energy-saving enhancements, including spatial domain adaptation, cell DTX/DRX, and on-demand SIB1 for idle/inactive mode UEs, further opportunities exist to improve PCell energy efficiency, particularly regarding SSB operation.  Notably, SSB-based enhancements like on-demand SSB and SSB adaptation were limited to SCells in these releases. This limitation significantly hinders overall network energy savings, as PCell power consumption remains high due to the need for recurrent SSB transmissions with short periodicity values (e.g., 20ms) for supporting idle/inactive operation.

The lack of SSB based enhancements for PCells in Rel-18/19 hinders network energy efficiency by requiring continuous SSB transmissions when UEs are idle.

To address this, Rel-20 should introduce on-demand SSB operation for PCell CD-SSBs, applicable for at least idle/inactive operation.

In single-carrier deployments, on-demand SSB can complement the periodic SSB transmissions of the NES PCell. The NES PCell can transmit periodic SSBs with a longer periodicity (e.g., 80ms or 160ms) for general operation and utilize on-demand SSBs with shorter periodicities when a higher SSB density is required for delay-critical operations.

Note that 3GPP specification currently allows SSB periodicity greater than 20ms for cell-reselection based on configuration of SMTC2-LP which may provide larger SSB periodicities for coverage cells. The same mechanism may be reused for configuring larger SSB periodicities for NES Rel-20 users as well. This also allows simultaneous existence of legacy UEs and NES Rel-20 UEs in the same cell.

In this scenario, we assume that at least network based trigger should be supported for on-demand SSB for the UEs in idle/inactive mode. Network based trigger would allow triggering on-demand SSB when cell load is high or when there is a need to high mobility requirements. Further study is required for support of UE-triggered or UE WUS based on-demand SSB.

The specification work should consider the following aspects for standardization:

- Transmission characteristics of on-demand SSB including operating frequency, relationship with always-on SSB

- Specifying signaling for network trigger for on-demand SSB

- UE procedure for receiving on-demand SSB

Support on-demand SSB operation for UEs in idle/inactive mode in Rel-20 with at least following aspects

  - Always-on SSB transmission along with on-demand SSB transmission

  - Network-triggered on-demand SSB

Following aspects need to be considered for specification of on-demand SSB for idle/inactive mode

  - Transmission characteristics of on-demand SSB including operating frequency, relationship with always-on SSB

  - Specifying signalling for network indication of on-demand SSB to UE

  - UE procedure for receiving on-demand SSB

# Conclusion

Based on the discussion above, we have the following observations and proposals.

Observation 1: 	The lack of SSB based enhancements for PCells in Rel-18/19 hinders network energy efficiency by requiring continuous SSB transmissions when UEs are idle.

- Proposal 1:	Support on-demand SSB operation for UEs in idle/inactive mode in Rel-20 with at least following aspects

  - Always-on SSB transmission along with on-demand SSB transmission

  - Network-triggered on-demand SSB

- Proposal 2:	Following aspects need to be considered for specification of on-demand SSB for idle/inactive mode

  - Transmission characteristics of on-demand SSB including operating frequency, relationship with always-on SSB

  - Specifying signalling for network indication of on-demand SSB to UE

  - UE procedure for receiving on-demand SSB

# References

1. RP-223540, New WID: Network energy savings for NR

2. RP-242354, Revised WID: Enhancements of network energy savings for NR