3GPP TSG-RAN Meeting #107 												 RP-250126

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.8.3

Source: 	NEC

Title:	        Views on UAV enhancement for NR

Document for:	 Discussion

# Introduction

In TSG RAN Meeting#106, UAV have received great attention and support even if it is listed as additional topics. This contribution provides our views on the UAV enhancement for NR Rel-20.

# Discussion

## 2.1	Deployment Scenarios and Motivation

Rel-18 NR UAV represents a significant advancement in enabling UAV connectivity and supporting vertical use cases. By reusing LTE UAV functionality and introducing A2X communication, NR UAV lays the foundation for more sophisticated applications. However, tighter target requirements for NR UAV, as outlined in the 3GPP technical specification TS 22.125, highlight the need for continuous innovation to meet the demands of vertical industries, such as industrial automation, precision agriculture, and public safety. Consequently, future enhancements in mobility performance, flight path management will be critical to unlocking the full potential of UAV technology in both commercial and industrial domains.

Additionally, it is important to note that we need to leverage the full potential of 5G NR to meet the demands of modern UAV applications within a relatively limited time frame. Therefore, we believe this new WI should focus exclusively on NR.

Support a new WI on UAV enhancement.

The target of the new WI is NR only.

## 2.2 Proposed Functions for Enhancing UAV Performance

To address the unique challenges of UAV connectivity and mobility, two key functions are proposed:

- RRC_IDLE/RRC_INACTIVE mobility enhancement

- UAV-specific CHO for mobility performance improvements

These functions aim to optimize network performance for UAVs by considering their altitude, flight path, and vertical mobility characteristics.

### RRC_IDLE/RRC_INACTIVE Mobility Enhancement

In Rel-18, height-dependent measurements have been introduced to address the challenges of vertically changing radio environments for UAVs in the RRC_CONNECTED state. However, similar needs exist for UAVs in the RRC_IDLE and RRC_INACTIVE states. UAVs in these states may need to perform cell reselection or measurements based on their altitude. For example, like in RRC_CONNECTED, altitude based SSB measurement configurations can be applied to Idle/Inactive measurements for cell reselection purpose.

In addition, flight path may need to be updated to maintain serving quality. Currently, when RRC_IDLE/RRC_INACTIVE state UAVs merely have flight path plan to report, they still have to enter RRC_CONNECTED state firstly and send the report according to specified procedures. In such a case, RRC connection setup and subsequently release to RRC_IDLE/INACTIVE state happens every time when updating flight path plan.  When small size flight path plan is frequently provided, current mechanism may cause unnecessary power consumption and signaling overhead. In general, we observe that enabling flight path plan update in RRC_IDLE/RRC_INACTIVE is beneficial from network performance/efficiency and UE battery performance perspectives.

The proposed enhancements for RRC_IDLE/RRC_INACTIVE mobility include:

- Extend height-dependent RRM parameters to RRC_IDLE/RRC_INACTIVE state (e.g., for cell reselection purpose, or Idle/Inactive measurements);

- Extend Flight path update mechanism to RRC_IDLE/RRC_INACTIVE state: (e.g., report flight path information updated during RRC_IDLE/RRC_INACTIVE).

### UAV-Specific CHO for Mobility Performance Improvements

According to 3GPP TR 36.777, UAVs can be served by geographically distant cells compared to ground-based UEs. This is due to the LoS propagation conditions at higher altitudes, which allow UAVs to connect to cells that are farther away. For CHO, this means that the candidate cells for handover may vary depending on the UAV's altitude and flight path. Additionally, cells along the flight path can assist in making more informed handover decisions. Therefore, CHO mechanisms can be enhanced by incorporating UAV-specific parameters, such as altitude and flight path information, into candidate cell preparation and handover triggering.

The proposed UAV-specific CHO would introduce triggering events that consider the UAV's flight path, including height-, distance-, or time-based conditions relative to predefined waypoints. For example, as in Figure 2.2-1, A drone is flying along a predefined flight path with multiple waypoints. As it approaches waypoint #2, which is near the edge of a neighboring cell, the network evaluates the distance between the drone and the waypoint. If the distance meets the predefined threshold, the network triggers a handover to the neighboring cell, ensuring connectivity as the drone enters the new cell's coverage.

Figure 2.2-1 Example: A drone is flying along a predefined flight path with multiple waypoints

The scope of the new WI includes the following aspects:

- Enhancements on RRC_IDLE/RRC_INACTIVE:

  - Extend height-dependent RRM parameters to RRC_IDLE/RRC_INACTIVE state

  - Extend Flight path update mechanism to RRC_IDLE/RRC_INACTIVE state

- Mobility enhancements on CHO:

  - Specify UAV specific triggering event(s) for CHO

# Conclusion

Proposal 1:	Support a new WI on UAV enhancement.

Proposal 2:	The target of the new WI is NR only.

Proposal 3:	The scope of the new WI includes the following aspects:

	Enhancements on RRC_IDLE/RRC_INACTIVE:

-	Extend height-dependent RRM parameters to RRC_IDLE/RRC_INACTIVE state

-	Extend Flight path update mechanism to RRC_IDLE/RRC_INACTIVE state

	Mobility enhancements on CHO:

-	Specify UAV specific triggering event(s) for CHO

# References

[1] RP-250002, Meeting Report for TSG RAN meeting #106