3GPP TSG RAN Meeting #107	RP-250134

Incheon, Korea, March 12 – 14, 2025

Agenda item:	15.2.1

Source:	SK Telecom

Title:	View on Rel-20 AI/ML for air interface for 5G-Advanced

Document for:	Discussion and Decision

1	Introduction

Rel-20 is the last specification release of 5G before 6G normative work. At the same time, there is no doubt that AI/ML is one of key components and enabling technologies of 6G air interface. Therefore, for each candidate items of AI/ML for air interface enhancements, it is very important to determine the right place between 5G-Advanced and 6G study to avoid unnecessary overlap and efficient standardization works in Rel-20. Since items classified to Rel-20 6G study can only be ready for implementation in the next release Rel-21 (i.e., first 6G normative work), we think that one of important criterion is that how urgent market needs and commercialization opportunities exists in near future for each candidate items. In this contribution, we discussed our views on the urgency of AI/ML based CSI prediction and remaining issues of Rel-19 AI/ML based CSI prediction which needs standard support for commercialization. Finally, we proposed candidate objectives for Rel-20 AI/ML for air interface for 5G-Advanced to address it.

2	Discussion

2.1	Why AI/ML based CSI prediction is urgently needed in 5G?

In mid-band TDD, at cell center, uplink SRS (Sounding Reference Signal) estimation results are normally used for downlink beamforming weight design (i.e., channel reciprocity based beamforming) due to its huge performance gain compared to other method if there are available SRS antenna switching resource for UEs. However, at cell edge, only CSI report from UE is available for downlink beamforming weight design (i.e., codebook based beamforming). Each SRS is uplink 1Tx signal and the purpose of receiving SRS at the base station is to obtain channel state information between each Tx antenna port of UE and each Rx antenna port of base station and thus it is impossible to do any Rx combining for enhancing signal quality at the gNB receiver (e.g., maximum ratio combining among 64/32 Rx ports in base station for uplink data channel) as discussed in [1].

[Figure 1. Key use cases of AI/ML based CSI prediction at network and UE]

From Rel-19 evaluation summary, AI/ML based CSI prediction shows 3.8%~20.7% gain over non-AI/ML based CSI prediction in terms of 5% UPT which represents cell edge performance gain. It is much higher than 0~7.8% mean UPT gain which represents overall performance gain including cell center.

- AI/ML based CSI prediction has great potential to improve codebook-based DL beamforming performance at cell edge where UL SRS is not available.

Still, there is no new 5G killer service which requires high throughput performance. Therefore, roughly speaking providing a seamless video streaming service (ex. at least 5Mbps throughput) anywhere is still most important target of operator for 5G customer satisfaction. Below figure 2 is one of example of NR DL UE T-put distribution in SK telecom’s network (10 gNB @ some city center in Korea, daytime 1 hour average).

[Figure 2. Example of NR DL UE T-put distribution in SK telecom’s network]

As you can see, a large percentage of users in a cell are still having less than 5 Mbps throughput performance. In other words, any 5G features which can improve cell edge performance are urgently needed for operators perspective. So, we think that AI/ML based CSI prediction has good opportunities for future commercialization even in matured 5G market.

- Now, cell edge performance is limiting real-world customer 5G experiences. Any 5G features for enhancing cell edge performance are urgently needed.

2.2	Remaining issues of Rel-19 AI/ML based CSI prediction

For performance evaluation of CSI enhancement feature in 3GPP, the periodicity of CSI-RS transmission and CSI feedback is assumed to 5ms as a baseline evaluation methodology from Rel-16. However, 5ms periodicity is not practical setting for CSI-RS transmission and CSI feedback due to its huge overhead. In real commercial network, 20ms or longer periodicity is used in common. For example, DL peak throughput of NR TDD 100MHz (DL:UL=4:1) system is degraded by -4% when we reduce CSI-RS periodicity from 20ms to 5ms. Also, 5ms CSI-RS periodicity become more unrealistic in the future if we consider Rel-19 64/128 ports CSI codebook which has more higher overhead.

Rel-19 evaluation results of AI/ML based prediction gain with 20ms CSI-RS periodicity can be summarized in below table  [2].

[Table 1 - AI/ML based prediction gain evaluation with 20ms CSI-RS periodicity in Rel-19]

| CSI-RS config. | # of CSI predictions | Mean T-put | 5% UE T-put | UE Speed |
| --- | --- | --- | --- | --- |
| 20ms pCSI-RS | 1 | -18 ~ -7% | -26 ~ -8% | 3km/h |
| 20ms pCSI-RS | 4 | +1 ~ +6% | +6 ~ +17% | 3km/h |
| 20ms pCSI-RS + three apCSI-RS | 4 | +2 ~ +17% | +13 ~ +19% | 30km/h |

Firstly, with 20ms CSI-RS periodicity, AI based CSI prediction has no gain or marginal gain even at very low UE speed (i.e., 3km/h). If it is very difficult to make a positive gain of AI/ML based CSI prediction using periodic CSI-RS with 20ms periodicity only, combining periodic CSI-RS and aperiodic CSI-RS is another implementation option for AI/ML-based CSI prediction. For example, by using one periodic CSI-RS + three aperiodic CSI-RS, CSI prediction could provide meaningful gain at much higher UE speed (i.e., 30km/h) in Rel-19 evaluation.

- Even at very low UE speed (e.g., 3km/h), Rel-19 AI/ML based CSI prediction gain is insufficient when we use periodic CSI-RS with 20ms periodicity only.

In Rel-18 MIMO WI, aperiodic CSI-RS consisting of multiple NZP CSI-RS resources is specified to support UE-side prediction [3]. However, CSI report configuration to support UE-side prediction that use both periodic CSI-RS and aperiodic CSI-RS are not considered. K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource are only possible options [4].

- TS 38.214 V18.2.0 (2024-03): 5.2.1.4.1 Resource Setting configuration - … - A UE configured with a CSI-ReportConfig with the higher layer parameter N4 and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The K aperiodic CSI-RS resources are transmitted following the order of the CSI-RS resource IDs configured in the CSI-RS resource set.

Also, Rel-19 normative work for AI/ML based CSI prediction focus only on LCM (life-cycle management) support and there is no enhancement on CSI-RS configuration for prediction as in Rel-19 AI/ML for air interface WID [5].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In other words, it is impossible to use both periodic CSI-RS and aperiodic CSI-RS for UE-side CSI prediction (regardless of AI based or Non-AI based) now and thus additional specification is needed in Rel-20.

- UE-side CSI prediction (regardless of AI based or Non-AI based) operation which uses both periodic CSI-RS and aperiodic CSI-RS resource is not supported in current Rel-19 specification.

2.3	Potential solution

Based on above observation, we proposed as follow:

Support UE-side CSI prediction operation using both periodic CSI-RS and aperiodic CSI-RS at least for 20ms periodicity (as part of Rel-20 AI/ML for air interface WI)

    - 20ms periodicity is the baseline EVM of periodic CSI-RS and CSI feedback for this work

For example, the transmission of additional aperiodic CSI-RSs on top of periodic CSI-RS with longer periodicity (e.g., 20ms) to mimic CSI-RS periodicity was temporarily reduced to shorter period (e.g., 5ms) for target UE (e.g., fast moving UE) is illustrated in Figure 3 below.

[Figure 3. AI/ML-based CSI prediction using both periodic CSI-RS and aperiodic CSI-RS]

3	Conclusions

In this contribution, we discussed our views on the urgency of AI/ML based CSI prediction and remaining issues of Rel-19 AI/ML based CSI prediction which needs standard support for commercialization.

- AI/ML based CSI prediction has great potential to improve codebook-based DL beamforming performance at cell edge where UL SRS is not available.

- Now, cell edge performance is limiting real-world customer 5G experiences. Any 5G features for enhancing cell edge performance are urgently needed.

- Even at very low UE speed (e.g., 3km/h), Rel-19 AI/ML based CSI prediction gain is insufficient when we use periodic CSI-RS with 20ms periodicity only.

- UE-side CSI prediction (regardless of AI based or Non-AI based) operation which uses both periodic CSI-RS and aperiodic CSI-RS resource is not supported in current Rel-19 specification.

Based on above observations, we proposed as follow:

- Support UE-side CSI prediction operation using both periodic CSI-RS and aperiodic CSI-RS at least for 20ms periodicity (as part of Rel-20 AI/ML for air interface WI)

    - 20ms periodicity is the baseline EVM of periodic CSI-RS and CSI feedback for this work

References

- RP-232011, On CSI-RS extension beyond 32 ports in R19 MIMO evolution, SK Telecom, Deutsche Telekom

- RP-242398, Status report for WI AI/ML for NR air interface, Qualcomm Innovation Center Inc

- RP-232914, WI summary for WI Core part: NR MIMO evolution for downlink and uplink, Samsung

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- 3GPP TS 38.214 V18.2.0 (2024-03), Physical layer procedures for data (Release 18)