3GPP TSG RAN Meeting #107                                                        RP-250153

Incheon, South Korea, March 12 – 14, 2025

Source:	Indian Institute of Technology (Madras), CEWiT, IIT Kanpur, Tejas Networks, Indian Institute of Technology (Hyderabad), WiSig Networks

Title:	Views on the study of IMT-2030 requirements

Agenda Item:	16 – Study on 6G Scenarios and Requirements

Document for:	Discussion and Decision

Introduction

In RAN #106:

- A reply Liaison Statement (LS) [1] was sent to ITU-R WP5D, which contained an initial list of candidate minimum Technical Performance Requirements (TPRs), with the understanding that additional candidate TPRs will be finalized in RAN #107.

- A Study Item entitled “Study on 6G Scenarios and Requirements” [2] was approved with the following scope and objectives:

  - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160 [3], and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

    - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent 6G study in RAN

In this contribution, we provide our views on:

- The list of TPRs that should be included in 3GPP studies

- Considerations for possible modifications to definitions of some of the TPRs

- Definitions of new TPRs for IMT-2030

- Mapping between IMT-2030 usage scenarios, test environments and TPRs

Current Status of work on IMT-2030 in ITU-R WP5D

The ITU-R recommendation M.2160 describes six usage scenarios for IMT-2030. These usage scenarios can be divided into two subcategories; Part 1 which includes the expansion of IMT-2020 usage scenarios from Enhanced Mobile Broadband to Immersive Communication (IC), Ultra Reliable Low Latency Communication to Hyper Reliable Low Latency Communication (HRLLC) and Massive Machine Type Communication to Massive Communication (MC). Part 2 consists of three new usage scenarios - Ubiquitous Connectivity (UC), Integrated Sensing & Communication (ISAC) and Artificial Intelligence & Communication (AI&C) - which are expected to support new services beyond IMT-2020. Figure 1(a) shows the six usage scenarios as depicted in M.2160 [3].

Furthermore, fifteen capabilities were proposed, as shown in Figure 1(b), including nine capabilities enhanced from IMT-2020 and six new capabilities for IMT-2030, to support new usage scenarios and services. These capabilities are being discussed in the form of minimum TPRs for IMT-2030 in ITU-R WP5D Sub-Working Group (SWG) Radio Aspects.

- Usage scenarios                                                                              (b) Capabilities

Figure 1: Usage scenarios and Capabilities of IMT-2030

Figure 2 shows the ITU-R WP5D timeline for IMT-2030. Meeting #48 concluded in February 2025, where the focus in SWG-Radio Aspects was on (1) agreeing on an initial list of TPRs to submit to SWG Evaluation, and (2) making progress on definitions of AI-related capabilities, energy efficiency and the joint requirement of data rate, latency, reliability and capacity. In SWG Evaluation there were discussions on (1) the mapping between usage scenarios and test environments, and (2) the mapping between test environments and TPRs. The discussions in both SWGs were driven by contributions from different countries, alliances, research institutes and companies. These contributions, as well as those from prior meetings are captured in Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] [4] and Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] [5].

Figure 2: ITU-R WP5D timeline

Furthermore, as per the work plan of SWG Radio Aspects [6], discussions on TPR definitions will continue in June 2025 (Meeting #49), along with initial discussions on target values. These discussions will progress till February 2026 (Meeting #51). The SWG Evaluation [7] will stabilize its document in February 2026 (Meeting #51) and finalize it in June 2026 (Meeting #52). This work plan indicates the time window to be followed by 3GPP for its deliberations on IMT-2030 studies.

Observation 1: The work in WP5D SWG Radio Aspects towards minimum TPR is expected to be completed in February 2026. In the February 2025 meeting, an initial list of TPRs was agreed with the plan to update this list in June 2025. Further discussions on definitions and values will also continue in June 2025.

Observation 2: The work in WP5D SWG Evaluation, towards guidelines for the procedure, methodology and criteria (technical, spectrum and service) to be used in evaluating the candidate IMT-2030 Radio Interface Technologies (RITs) or Set of RITs (SRITs) is to be stabilized in February 2026 and finalized in June 2026.

General Views on IMT-2030 Study

Based on the progress of the TPR definitions and evaluation criteria in ITU-R WP5D, it is evident that, since part 1 of the IMT-2030 usage scenarios (IC, HRLLC and MC) also have corresponding usage scenarios in IMT-2020 (eMBB, URLLC, mMTC), the same TPR-usage scenario mapping of IMT-2020 can be reused with some modifications as necessary. For example, new test environments such as Indoor Factory-HRLLC should be considered for applicable TPRs.

Part 2 of the usage scenarios, i.e. UC, ISAC and AI&C may require new TPRs and test environments which need to be discussed further. For example, for Ubiquitous Connectivity, a new test environment is necessary to evaluate new TPRs such as Coverage, and possibly some legacy TPRs such as 5th percentile spectral efficiency and average spectral efficiency. The same is true for ISAC where positioning and sensing related TPRs may need to be newly introduced. AI&C is essential for new services and capabilities, over and above existing 5G technologies. This is already being considered in stage 1 discussions in the 3GPP SA1 group. Considering the future commercial impact of AI&C, it is important to study the necessary RAT impact in terms of TPRs and necessary evaluation criteria, to be able to bring a clear distinction of IMT-2030 technology compared to IMT-2020 technology, from a commercial standpoint.

- Proposal 1: RAN study on IMT-2030 should:

- Consider all six usage scenarios - Immersive Communication, Hyper Reliable and Low Latency Communication, Massive Communication, Ubiquitous Connectivity, Integrated Sensing and Communication and AI & Communication.

- Align with the progress in SWG Radio Aspects and SWG Evaluation as indicated by the Working Document Towards a Preliminary Draft New Report ITU-R M. [IMT-2030.TECH PERF REQ] and the Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL].

- Consider KPI(s) for all six usage scenarios across RAN and SA.

Proposal 2: Part 1 of the usage scenarios including IC, HRLLC & MC, can use IMT-2020 TPRs and evaluation criteria as a baseline. The necessary modifications over the baseline can be studied further.

Proposal 3: Part 2 of the usage scenarios including UC, ISAC and AI&C, should be evaluated for new TPRs and for possible impact on legacy TPRs, with new test environments and evaluation criteria.

Views on the IMT-2030 Technical Performance Requirements

Proposal 4: The following TPRs are essential to be considered by 3GPP in its study and specification efforts for IMT-2030 (6G):

- Peak Data Rate

- User Experienced Data Rate

- Peak Spectral Efficiency

- Average Spectral Efficiency

- 5th Percentile Spectral Efficiency

- Area Traffic Capacity

- Connection Density

- Mobility

- Mobility Interruption Time

- Control Plane Latency

- User Plane Latency

- Reliability

- Bandwidth

- Coverage

- Positioning

- Sensing-related Capabilities

- AI-related Capabilities

- Energy Efficiency

- Security

- Other TPRs are TBD

## 4.1 Views on the definitions of selected TPRs

- Peak Data Rate and Peak Spectral Efficiency: Even though the peak data rate and peak spectral efficiency TPRs represent upper bound values in ideal conditions, it is important to define realistic and achievable values.  The way to do this is by duly considering realistic overheads such as guard bands/times, TDD structures as well as realistic carrier aggregation strategies.

Proposal 5: For the definition of Peak Data Rate and Peak Spectral Efficiency, further study should 	consider (1) guard bands/times (2) uplink/downlink ratio for TDD and (3) the bands over which the 	bandwidth is aggregated.

- Ubiquitous Connectivity (UC) and Coverage: As specified in ITU-R Recommendation M.2160, Ubiquitous Connectivity relates to providing basic services with extended coverage, including presently uncovered or scarcely uncovered areas, particularly rural, remote and sparsely populated areas. Typical use cases include, but are not limited to, IoT and mobile broadband communications. ITU-R WP5D is also specifically looking to extend UL coverage in terrestrial networks. This should be considered when defining study and work items for Rel 20/21.

Given the importance attached to the UC usage scenario in M.2160, UC must have a TPR associated with it. Based on the above paragraph, coverage (a measure of distance) is the most applicable TPR for UC, along with 5th percentile and average spectral efficiency to indicate the level of service achieved by a Radio Interface Technology (RIT), at a certain distance. To cater to the rural and sparsely populated areas, it is especially important for IMT-2030 RITs to be able to support large cells. Consequently, 3GPP studies should focus on developing and specifying technologies that improve terrestrial coverage in large cells.

Proposal 6: For the Ubiquitous Connectivity usage scenario, the study should consider coverage, 5th 	percentile and average spectral efficiency as TPRs. A new test environment should be defined for	Ubiquitous Connectivity which includes at least the Rural geographical area, with an emphasis on large 	cells.

Proposal 7: 3GPP should consider both UL and DL coverage as Day 1 requirements in 6G and this consideration should be well reflected in the definitions of new study and work items (Rel 20/21).

Proposal 8: 3GPP should develop appropriate evaluation methodologies for UL and DL coverage in 6G and convey to ITU-R WP5D by LS.

- Positioning: Positioning should be a fundamental capability of the RIT. Several studies and specification efforts in 3GPP have already focused on both conventional and AI/ML methods for localization of active devices. These efforts should continue for IMT-2030 RITs.

Proposal 9: Positioning should be a TPR, so that it motivates RIT based technologies that improve the ability to estimate the position of active devices in both horizontal and vertical directions. This TPR can be characterized by the 90th percentile on the CDF curve of positioning accuracy for both horizontal and vertical directions.

In 3GPP, positioning is considered as an external service and the related architecture evolved in 5G NR. In 6G, along with service-based use cases, positioning should also be considered as a network feature (like network measurement) and should be something that can be measured in the radio access network itself.

Proposal 10: Positioning should be considered as a network feature (like network measurement) and should be something that can be measured in the radio access network itself.

- Sensing-related Capabilities: These capabilities should include support for tasks such as passive object presence/absence detection, object localization and movement characterization. IMT-2030 studies should consider sensing-related capabilities in general, and also specific TPRs that characterize the above tasks. For object presence/absence detection, the applicable TPRs are Sensing Range, Resolution, Probability of Detection and False Alarm Probability. For object localization, the applicable TPRs are Sensing Range, Probability of Detection, False Alarm Probability, Horizontal and Vertical Localization Accuracy. For movement characterization, the applicable TPRs are Sensing Range, Probability of Detection, False Alarm Probability, Horizontal and Vertical Localization Accuracy and Horizontal and Vertical Velocity Accuracy.

Proposal 11: Sensing-related Capabilities should be included in IMT-2030 studies along with the following TPRs to measure the capabilities (as applicable to a specific sensing task): Sensing Range, Resolution, Horizontal/Vertical Sensing Accuracy, Horizontal/Vertical Velocity Accuracy, Probability of Detection and False Alarm Probability.

- AI-related Capabilities: ITU-R Recommendation M.2160 defines AI-related capabilities as the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications.

In our view, AI enabled applications include both (1) AI for Network and (2) Network for AI.

AI for Network refers to communication aspects of the RIT being enhanced by AI frameworks, via AI models on both the UE side and the base station side, either by replacing or working in conjunction with non-AI frameworks.

Network for AI refers to use cases employing AI frameworks supported by the RIT/SRIT.

Given that the integration of AI into the RIT is a relatively new effort, there should be sufficient emphasis in 3GPP on performance comparison with and without AI.  Furthermore, despite the interest in true native AI technologies, fallback from fully AI-based solutions to conventional approaches should be an essential feature of IMT-2030 RITs. Fallback is important from the perspective of performance comparison, security and interoperability.

Proposal 12: 3GPP should consider use cases related to both AI for Network and Network for AI. 3GPP work should also emphasize performance evaluation of use-cases with and without AI.

Proposal 13: When specifying AI-based use cases, 3GPP should consider fallback (switching AI capabilities on and off) mechanisms in order to enable performance comparison, security and interoperability.

Proposal 14: The comparison of 6G technology features with and without AI can be provided using the table below,

| Test environment | User Experienced Data Rate |  | Avg. Spectral Efficiency |  | Latency |  | …… |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | With AI | Without AI | With AI | Without AI | With AI | Without AI | …. |
| Dense Urban – IC |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |

## 4.2 Views on the target values for selected TPRs

The table below shows our initial views on possible target values for selected TPRs.

| TPR | Preliminary Values |
| --- | --- |
| Peak Data Rate | 1.5-5x compared to IMT-2020 |
| User Experienced Data Rate | 2-3x compared to IMT-2020 Note: For UC, lower values compared to IMT-2020 can be considered. |
| Peak Spectral Efficiency | 1.5-3x compared to IMT-2020 |
| Area Traffic Capacity | 2-3x compared to IMT-2020 |
| Connection Density | 1-10x compared to IMT-2020 |
| Mobility Interruption Time | 0 ms |
| User Plane Latency | 0.5-1 ms |
| Control Plane Latency | 5-20 ms |
| Reliability | 1-10−5 to 1-10−7 |
| Coverage | > 6 km |
| Bandwidth | 200 (minimum value for the maximum bandwidth) |
| Positioning | 0.5-10 m |

Views on the IMT-2030 Test Environments

IMT-2020 used an outdoor test environment (Urban Macro-URLLC) to evaluate the TPRs under the URLLC usage scenario. In IMT-2030, HRLLC is envisioned to encompass more indoor use cases (e.g. industrial automation), than outdoor.

In 3GPP 5G NR, the Indoor Factory environment is used extensively to evaluate multiple features. The Indoor Factory evaluation has matured enough, and can be used for 6G feature evaluation as well. Therefore, in IMT-2030 (6G), for HRLLC, we propose to replace the outdoor test environment (Urban Macro-URLLC equivalent) of IMT-2020 with an Indoor Factory-HRLLC test environment. 3GPP should consider developing the evaluation methodology, including configuration parameter tables and TPR evaluation steps. The findings can be shared with ITU-R WP5D by LS.

Proposal 15: For the HRLLC usage scenario, an Indoor Factory test environment should be considered in 3GPP. 3GPP should consider developing the evaluation methodology, including configuration parameter tables and TPR evaluation steps. The findings can be shared with ITU-R WP5D by LS.

Further, the detailed mapping between usage scenarios, test environments and minimum technical performance requirements is provided in the proposal below.

Proposal 16: The following mapping between usage scenarios, test environments and TPRs should be considered for IMT-2030 as shown in the table below.

| Usage Scenarios | Test Environments | Applicable TPRs |
| --- | --- | --- |
| Immersive Comm (IC) | Indoor Hotspot-IC | 5th percentile SE, Avg SE, Mobility, Positioning |
|  | Dense Urban-IC | 5th percentile SE, Avg SE, User Exp Data Rate, Mobility, Positioning |
|  | Rural-IC | 5th percentile SE, Avg SE, Mobility, Positioning |
| HRLLC | Indoor Factory-HRLLC | Reliability, Mobility Interruption Time, Latency, Positioning |
| Massive Comm (MC) | Urban Macro-MC | Connection Density |
| Ubiquitous Connectivity (UC) | Rural-UC | Coverage, 5th percentile SE, Avg SE, User Exp Data Rate, Mobility |
| ISAC | Dense Urban-ISAC | Sensing-related Capabilities |
|  | Rural-ISAC | Sensing-related Capabilities |
| AIAC (Reuse TEs) | Dense Urban-IC, TBD | AI-related Capabilities |

Conclusions

In this contribution, we presented our views on (1) the progress of IMT-2030 TPRs and Evaluation Criteria & Methodologies in ITU WP5D, (2) the list of TPRs that should be included in 3GPP studies, (3) considerations for possible modifications to definitions of some of the TPRs, (4) definitions of new TPRs for IMT-2030 (5) mapping between IMT-2030 usage scenarios, test environments and TPRs.

We made the following observations and proposals:

Observation 1: The work in WP5D SWG Radio Aspects towards minimum TPR is expected to be completed in February 2026. In the February 2025 meeting, an initial list of TPRs was agreed with the plan to update this list in June 2025. Further discussions on definitions and values will also continue in June 2025.

Observation 2: The work in WP5D SWG Evaluation, towards guidelines for the procedure, methodology and criteria (technical, spectrum and service) to be used in evaluating the candidate IMT-2030 Radio Interface Technologies (RITs) or Set of RITs (SRITs) is to be stabilized in February 2026 and finalized in June 2026.

- Proposal 1: RAN study on IMT-2030 should:

- Consider all six usage scenarios - Immersive Communication, Hyper Reliable and Low Latency Communication, Massive Communication, Ubiquitous Connectivity, Integrated Sensing and Communication and AI & Communication.

- Align with the progress in SWG Radio Aspects and SWG Evaluation as indicated by the Working Document Towards a Preliminary Draft New Report ITU-R M. [IMT-2030.TECH PERF REQ] and the Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL].

- Consider KPI(s) for all six usage scenarios across RAN and SA.

Proposal 2: Part 1 of the usage scenarios including IC, HRLLC & MC, can use IMT-2020 TPRs and evaluation criteria as a baseline. The necessary modifications over the baseline can be studied further.

Proposal 3: Part 2 of the usage scenarios including UC, ISAC and AI&C, should be evaluated for new TPRs and for possible impact on legacy TPRs, with new test environments and evaluation criteria.

Proposal 4: The following TPRs are essential to be considered by 3GPP in its study and specification efforts for IMT-2030 (6G):

- Peak Data Rate

- User Experienced Data Rate

- Peak Spectral Efficiency

- Average Spectral Efficiency

- 5th Percentile Spectral Efficiency

- Area Traffic Capacity

- Connection Density

- Mobility

- Mobility Interruption Time

- Control Plane Latency

- User Plane Latency

- Reliability

- Bandwidth

- Coverage

- Positioning

- Sensing-related Capabilities

- AI-related Capabilities

- Energy Efficiency

- Security

- Other TPRs are TBD

Proposal 5: For the definition of Peak Data Rate and Peak Spectral Efficiency, further study should consider (1) guard bands/times (2) uplink/downlink ratio for TDD and (3) the bands over which the bandwidth is aggregated.

Proposal 6: For the Ubiquitous Connectivity usage scenario, the study should consider coverage, 5th percentile and average spectral efficiency as TPRs. A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area, with an emphasis on large cells.

Proposal 7: 3GPP should consider both UL and DL coverage as Day 1 requirements in 6G and this consideration should be well reflected in the definitions of new study and work items (Rel 20/21).

Proposal 8: 3GPP should develop appropriate evaluation methodologies for UL and DL coverage in 6G and convey to ITU-R WP5D by LS.

Proposal 9: Positioning should be a TPR, so that it motivates RIT based technologies that improve the ability to estimate the position of active devices in both horizontal and vertical directions. This TPR can be characterized by the 90th percentile on the CDF curve of positioning accuracy for both horizontal and vertical directions.

Proposal 10: Positioning should be considered as a network feature (like network measurement) and should be something that can be measured in the radio access network itself.

Proposal 11: Sensing-related Capabilities should be included in IMT-2030 studies along with the following TPRs to measure the capabilities (as applicable to a specific sensing task): Sensing Range, Resolution, Horizontal/Vertical Sensing Accuracy, Horizontal/Vertical Velocity Accuracy, Probability of Detection and False Alarm Probability.

Proposal 12: 3GPP should consider use cases related to both AI for Network and Network for AI. 3GPP work should also emphasize performance evaluation of use-cases with and without AI.

Proposal 13: When specifying AI-based use cases, 3GPP should consider fallback (switching AI capabilities on and off) mechanisms in order to enable performance comparison, security and interoperability.

Proposal 14: The comparison of 6G technology features with and without AI can be provided using the table below,

| Test environment | User Experienced Data Rate |  | Avg. Spectral Efficiency |  | Latency |  | …… |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | With AI | Without AI | With AI | Without AI | With AI | Without AI | …. |
| Dense Urban – IC |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |

Proposal 15: For the HRLLC usage scenario, an Indoor Factory test environment should be considered in 3GPP. 3GPP should consider developing the evaluation methodology, including configuration parameter tables and TPR evaluation steps. The findings can be shared with ITU-R WP5D by LS.

Proposal 16: The following mapping between usage scenarios, test environments and TPRs should be considered for IMT-2030 as shown in the table below.

| Usage Scenarios | Test Environments | Applicable TPRs |
| --- | --- | --- |
| Immersive Comm (IC) | Indoor Hotspot-IC | 5th percentile SE, Avg SE, Mobility, Positioning |
|  | Dense Urban-IC | 5th percentile SE, Avg SE, User Exp Data Rate, Mobility, Positioning |
|  | Rural-IC | 5th percentile SE, Avg SE, Mobility, Positioning |
| HRLLC | Indoor Factory-HRLLC | Reliability, Mobility Interruption Time, Latency, Positioning |
| Massive Comm (MC) | Urban Macro-MC | Connection Density |
| Ubiquitous Connectivity (UC) | Rural-UC | Coverage, 5th percentile SE, Avg SE, User Exp Data Rate, Mobility |
| ISAC | Dense Urban-ISAC | Sensing-related Capabilities |
|  | Rural-ISAC | Sensing-related Capabilities |
| AIAC (Reuse TEs) | Dense Urban-IC, TBD | AI-related Capabilities |

References

[1] Reply LS from RAN #106 to ITU-R WP5D #48

[2] RP-243327 SID on IMT 2030 from RAN #106

[3] ITU-R M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

[4] R23-WP5D-C-0563!H5-N5.09!MSW-E: Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] - Minimum requirements related to technical performance for IMT-2030 radio interface(s).

[5] R23-WP5D-C-0563!H5-N5.02!MSW-E: Working Document Towards Preliminary Draft New Report ITU-R M.[IMT-2030.EVAL] - Guidelines for evaluation of radio interface technologies for IMT-2030

[6] R23-WP5D-C-0563!H2-N2.09.16!MSW-E: Draft detailed work plan for development of the draft new Report ITU-R M.[IMT-2030.TECH PERF REQ]

[7] R23-WP5D-C-0563!H2-N2.09.15!MSW-E: Detailed work plan for development of the draft new Report ITU-R M.[IMT-2030.EVAL]