3GPP TSG RAN Meeting #107							         RP-250174

Incheon, Korea, March 12-14, 2025

Agenda item:	15.2.1

Title:	Potential work scope for AIML air interface in Rel-20

Source:	OPPO

Document for:	Discussion

# Introduction

In Rel-18, the technologies of AI/ML have been introduced into 3GPP RAN1 working group and intensively studied. At the beginning of Rel-19, for the case of beam management and positioning, RAN1 step into normative work. Later on, the use case of CSI prediction has been studied from Q1 2025. As for CSI compression with two-side model, the study will proceed until the end of Rel-19 [1].

With two continuous study phases on CSI compression, we tend to believe RAN working group can complete its normative work in Rel-20 as 5G-Advance.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this contribution, we will briefly discuss how to handle the use case of CSI compression with two-side model in Rel-20 5G-A for AI/ML.

# CSI compression

During the 2nd study phase of CSI compression in Rel-19, two major issues have been identified. The first issue is to balance the performance and complexity/overhead. The second issue is to address the training collaboration among vendors.

## Evaluation results

During the Rel-18 and Rel-19 study phase, evaluations have been conducted by many companies. Some companies adopted the intermediate KPI, e.g. SGCS as performance metric to demonstrate its necessity, other companies dived into the final output, e.g. throughput, to justify the two-sided model for CSI compression.

From our evaluations, one may observe the performance gain of CSI compression as shown in Figure 1 when compared to legacy CSI reporting.

Figure 1: UPT gain of CSI compression over Rel-16 eType II

Observation 1: Performance gain of CSI compression on UPT can be achieved when compared with legacy CSI feedback.

## Inter-vendor collaboration

In the current Rel-19 study phase of two-side model, the issue of inter-vendor collaboration has been widely discussed and evaluated in RAN1. Various directions to address this issue have been identified. Those directions can be listed below.

- Direction A: Sharing reference model parameters/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a-1 and option 4-1)

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b)

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

From our understanding, anyone of the above directions can solve the problem of inter-vendor collaboration and what RAN1 needs to do next is to figure out which one is most technically reasonable.

Observation 2: For two-sided model, the inter-vendor collaboration issue is addressable via any of Direction A/B/C in RAN1.

## Normative work for CSI compression

With the above being said, we believe the issues of tradeoff and inter-vendor collaboration can be addressed in RAN1 and other related working groups. In Rel-20, it is time to deliver the study/evaluation results of two-side model into reality.

Proposal 1: Considering the fact that the above two issues are addressable, we have a good reason to specify CSI compression in Rel-20 5G-advance.

As far as we know, there could be parallel study/working items on AI/ML in Rel-20. Specifically, AI/ML could be a totally new study item for 6G, whereas there could be another AI/ML normative work (e.g. CSI compression with two-side model) for NR. Considering the large effort companies contributed to AI/ML during Rel-18 and Rel-19, it makes sense to avoid duplicated work as much as possible during Rel-20. For instance, if the RAN working group(s) specify the CSI compression with two-side model (if supported) for NR, then we don’t have to study it again for 6G CSI feedback in parallel. From this sense, the workload is manageable and reasonable.

Proposal 2: It makes sense to avoid duplicated work between Rel-20 NR WI and 6G SI, e.g. on CSI compression with two-side model.

# Potential scope

If RAN agrees to specify the use case of CSI compression, the following design should be completed, and potential solution can be listed for now briefly.

Inter-vendor collaboration

On Direction A/B/C, different options have been identified and analyzed in previous meetings. Compared with Direction B/C, direction A has the largest freedom of optimizing the actual model and can fully explore the potential of AI/ML to acquire the highest performance gain. Meanwhile, Direction C with specifying both of reference model structure and parameters also results in unaffordable spec efforts within limited time budget in the 5G-A normative phase in Rel-20.

Furthermore, regarding Direction A, two promising options with majority support are as follows.

- Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback).

- Option 3a-1: Model exchanged from the NW-side to UE-side is CSI generation part.

In our understanding, for Option 3a-1, the reference model structure needs to be specified, while Option 4-1 comes with the benefits of less specification effort, i.e. no need to specify reference model and/or corresponding parameters. Meanwhile, given the transferred dataset, what performance can be achieved highly depends on the model implementation and optimization. Its upper bound will not be limited by the reference model designed in Rel-20, if specified. In addition, due to no restrictions on model implementation, Option 4-1 can also provide total flexibility when compared with other options.

Proposal 3: To address the inter-vendor collaboration, support dataset exchange (i.e. Option 4-1) in Rel-20 5G-Advance for CSI compression with two-sided model.

Data collection for training

In terms of data collection for training, it is necessary for NW to collect the ground-truth label. For Option 4-1 to resolve the inter-vendor collaboration issue, NW-side CSI generation part and CSI reconstruction part models are trained firstly based on NW-side dataset. Basically, NW-side data are collected with ground-truth CSI reporting from multiple UEs. Therefore, mechanisms/signaling of the ground-truth reporting should be normalized to facilitate the NW-side data collection for model training.

Proposal 4: For model training at NW-side, specify mechanism/signaling for ground-truth reporting in Rel-20 5G-Advance.

As for model training at UE-side, it is necessary to normalize the mechanisms/signaling to facilitate the UE-side data collection, so that UE can train its own CSI generation part without NW-side data sharing. Since different UEs/UE-vendors may have different requirements for UE-side data collection, it is better up to UE to trigger the procedure for data collection.

Proposal 5: For model training at UE-side, support UE requested RS configuration/transmission.

Performance monitoring

After the functionality of CSI compression is activated with two-side model, there should be mechanisms to monitor its performance. Since the encoder and decoder are deployed at UE-side and NW-side respectively, it is straightforward to monitor its performance at both UE-side and NW-side.

Moreover, to monitor its performance, at least one performance metric is needed. Some would say system level metric, e.g. throughput is needed. However, such metric may not be directly related to the model of CSI compression, and it takes time to calculate it. Instead, the intermediate KPI e.g. SGCS could serve as a metric to determine whether the paired model works well or not.

Proposal 6: To monitor the performance of CSI compression, support both NW-side and UE-side performance monitoring with intermediate KPI as performance metric in Rel-20 5G-Advance.

# 3. Conclusion

To draw attention from readers, allow us to repeat observations and proposals in what follows.

Normative work for Rel-20 5G-A

Observation 1: Performance gain of CSI compression on UPT can be achieved when compared with legacy CSI feedback.

Observation 2: For two-sided model, the inter-vendor collaboration issue is addressable via any of Direction A/B/C in RAN1.

Proposal 1: Considering the fact that the above two issues are addressable, we have a good reason to specify CSI compression in Rel-20 5G-advance.

Proposal 2: It makes sense to avoid duplicated work between Rel-20 NR WI and 6G SI, e.g. on CSI compression with two-side model.

Potential scope for CSI compression with two-sided model

Proposal 3: To address the inter-vendor collaboration, support dataset exchange (i.e. Option 4-1) in Rel-20 5G-Advance for CSI compression with two-sided model.

Proposal 4: For model training at NW-side, specify mechanism/signaling for ground-truth reporting in Rel-20 5G-Advance.

Proposal 5: For model training at UE-side, support UE requested RS configuration/transmission.

Proposal 6: To monitor the performance of CSI compression, support both NW-side and UE-side performance monitoring with intermediate KPI as performance metric in Rel-20 5G-Advance.

# 4. References

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface