3GPP TSG RAN Meeting #107						   RP-250183

Incheon, Korea, March 12-14, 2025

Agenda Item:	10.3.1

Source:	Apple

Title:	On Rel-19 IoT NTN

Document for:	Discussion/Decision

# Introduction

The Release-19 IoT NTN enhancement work item was approved in RAN #102 meeting [1] and updated in RAN #105 meeting [2]. The updated objectives are shown below.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we present our views on the topics of PWS and PRACH capacity enhancement.

# Discussion

Regarding supporting PWS notification for UE in RRC_CONNNECTED, RAN2 sent an LS to RAN1 in [3] to check whether it is feasible to introduce support in Rel-19 for NB-IoT UEs reception of PWS notifications (in paging, direct indication, and system information) while in RRC_CONNECTED. From RAN1 perspective, the following aspects should be taken into account.

- UE capability

According to the Table 8.2-2b of TS 36.302, an RRC_CONNECTED IoT UE is not required to support parallel receptions in the same subframe. However, to support the reception of PWS notifications, the UE may need the capability to simultaneously monitor the CSS and receive the NPDSCH. This requirement for parallel reception capability appears to be inconsistent with the IoT UE design goals of low cost and low complexity.

- Collision handling

If no UE capability is introduced, a collision handling rule should be defined when HD-FDD IoT UE monitoring CSS for PWS notification collides with NPDSCH reception or uplink transmission. As a result, the performance of NPDSCH reception or uplink transmission will be degraded.

- Additional power consumption for frequency CSS monitoring

In short, to support PWS notification, additional RAN1 standard work is required. During the last RAN1 discussion, no consensus can be reached to support PWS from RAN1 perspective, and an LS [4] was sent RAN plenary to seek guidance on how to proceed.

From RAN1 perspective, it may be possible to find solutions with specification impact for supporting NB-IoT UEs in RRC connected mode monitor PWS notifications, while some companies expressed concerns due to increased implementation complexity, power consumption at UE side and timely delivery of PWS notifications in NTN context. However, the amount of RAN1 workload for specifying this may not fit in the Rel-19 TU allocation for NTN. RAN1 would like to ask RAN plenary to decide whether RAN1 should work on specifying support of NB-IoT UEs in RRC connected mode monitor PWS notifications in Rel-19.

Considering the RAN1 NTN TU allocation and its overloaded workload, PWS solutions without RAN1 involvement can be considered.

Proposal 1: RAN2 to consider the PWS solution without requiring RAN1 involvement.

Regarding the NPRACH capacity enhancement, after several meetings study, RAN1 made the following conclusion.

Conclusion RAN1 did not reach consensus on whether OCC for NPRACH is beneficial or not. RAN1 will not specify support for OCC for NPRACH in Rel-19 IoT NTN.

According to the above conclusion, NPRACH capacity enhancement shall be removed from the corresponding objective, and the objective shall be updated.

Proposal 2: Update the uplink capacity enhancement objective as the following

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4]

# Conclusion

Based on the discussions above, we have the following proposals.

Proposal 1: RAN2 to consider the PWS solution without requiring RAN1 involvement.

Proposal 2: Update the uplink capacity enhancement objective as the following

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4]

# Reference

- RP-234077, “New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Dec. 2023.

- RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Sep. 2024.

- R1-2500009, “LS on PWS support in RRC_CONNECTED for NB-IoT NTN”, RAN2, InterDigital, Feb. 2025

- R1-2501613, “Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN”, RAN1, Feb. 2025