3GPP TSG-RAN Meeting #107                                         RP-250270

Incheon, Korea, March 12-14, 2025

Agenda Item:	9.3.2.1

Source:	CMCC

Title:	Views on scope of Rel-19 mobility enhancements

Document for:	Discussion and Decision

- Introduction

In RAN#105, the scope of Rel-19 Mobility enhancements WI was revised as in [1], and there are one objective need to be checked in RAN#107, including whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases as following:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide our views on the scope of Rel-19 mobility enhancements.

- Discussion

Based on the guideline of Rel-19 Mobility enhancements WID, we need to review progress at RAN#107 to see if any modification of objectives is required to support CLTM in DC scenario.

In Rel-18 mobility, intra-CU LTM can be applied in CA or DC case, as specified in TS 38.300 [2]:

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported: - PCell change in non-CA scenario and non-DC scenario; - PCell and SCell(s) change in CA scenario; - Dual connectivity scenario, PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

Observation 1: In Rel-18 intra-CU LTM, LTM can be supported in DC scenario.

Based on Rel-18 discussion, the limitation that LTM for simultaneous PCell and PSCell change is not supported can be interpreted with the following two ways:

- Since SCG configuration can be part of LTM configuration, therefore LTM configured for PCell change may lead to the PSCell change and this is not supported;

- MCG LTM and SCG LTM is configured separately and they cannot be triggered by MN and SN simultaneously.

Observation 2: For legacy intra-CU LTM, there’s limitation that LTM for simultaneous PCell and PSCell change is not supported. And the limitation can be interpreted with the following two ways:

1.	Since SCG configuration can be part of LTM configuration, therefore LTM configured for PCell change may lead to the PSCell change and this is not supported;

2.	MCG LTM and SCG LTM is configured separately and they cannot be triggered by MN and SN simultaneously.

For the first interpretation, it is mainly considering that in LTM for PCell change, only PCell related measurement is taken into consideration, while measurement for PSCell is not considered. Therefore, it is not reasonable to change PSCell in a PCell LTM procedure. As to the second interpretation, it is because that PCell change may lead to the change of SCG configuration, and vice versa. That to say, for MCG LTM and SCG LTM configured separately by different node, one type LTM happens may cause the validity of the other type LTM. Therefore, LTM for PCell change and LTM for PSCell change cannot be triggered simultaneously.

In current discussion, CLTM is only supported in standalone scenario, in other words it is applied only for UE’s PCell. When it comes to DC scenario, if CLTM is supported in this scenario, there can be three cases as following:

Case 1: CLTM is only configured in MCG for PCell change.

Case 2: CLTM is only configured in SCG for PSCell change.

Case 3: CLTM is configured both in MCG for PCell change and in SCG for PSCell change.

For Case 1, we think it can be achieved by the current CLTM framework for standalone case, and configuration of SCG can be part of LTM configuration. But considering the first interpretation of legacy LTM limitation in Observation 2, the SCG configuration cannot be used for PSCell change.

For Case 2, the CLTM frame work can be used with some specification changes, like introduce new IEs for conditional SCG LTM, for example, SCG LTM execution condition related IE. And this may introduce extra workload in RAN2.

For Case 3, it is achieved based on the support of Case1 and Case 2. The execution condition for PCell change and PSCell change may be satisfied at the same time or different time. As discussed above for the second interpretation of Observation 2, for DC case, the PCell change may lead to the change of SCG configuration, and vice versa. Therefore, the separate change of PCell and PSCell at the same time is not expected. If intra-CU conditional LTM is supported in DC case, one issue should be discussed is how to avoid the simultaneous change of both PCell of PSCell. For legacy intra-CU LTM, it may be achieved by network implementation since it’s network triggered mobility, while for CLTM, it may introduce extra UE complexity since it is performed by UE itself. Besides, even the two types of LTM is executed by the UE sequentially, considering the invalidity issue mentioned previously, RRC reconfiguration for LTM configuration updating is needed. While this is contradict to the original intention of LTM in some sense.

Considering the limited time of Rel-19, additional work and large spec impact are not expected, especially that intra-CU CLTM in SA scenario, which is the baseline for DC scenario is still under discussion. Therefore, we propose to limit Rel-19 CLTM scope to only support SA scenario, while DC scenario is not supported.

Proposal 1: Conditional LTM is not supported for DC scenario.

The WID can be revised as following:

Proposal 2: Revise the WID as follows:

- Specify support of conditional Intra-CU LTM for Standalone [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

- Conclusions

In this contribution, we discuss the scope of Rel-19 mobility enhancement. Following observations and proposals are made in this contribution:

Observations

Observation 1: In Rel-18 intra-CU LTM, LTM can be supported in DC scenario.

Observation 2: For legacy intra-CU LTM, there’s limitation that LTM for simultaneous PCell and PSCell change is not supported. And the limitation can be interpreted with the following two ways:

1.	Since SCG configuration can be part of LTM configuration, therefore LTM configured for PCell change may lead to the PSCell change and this is not supported;

2.	MCG LTM and SCG LTM is configured separately and they cannot be triggered by MN and SN simultaneously.

Proposals

Proposal 1: Conditional LTM is not supported for DC scenario.

Proposal 2: Revise the WID as follows:

- Specify support of conditional Intra-CU LTM for Standalone [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

- References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024;

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18).