3GPP TSG-RAN WG Meeting #107	                                          RP-250275

Incheon, Korea, March 12-14, 2025

Source: 	Xiaomi

Title:  	Views on R20 AI/ML for Mobility

Agenda Item:	15.2.4

Document for:	Discussion and Decision

# Introduction

AI/ML for mobility is studied by RAN2 in R19. In this contribution, we discuss the scope of R20 AI/ML for mobility.

# Discussion

The R19 SI includes following use cases,

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

Regarding RRM prediction, RAN2 has done lots of simulation results on prediction accuracy. The simulation results showed the prediction can be close to the ground truth in some cases. The RSRP difference can be very small. RAN2 is going to further simulate the generalization and system level performance. Based on the simulation results, RAN2 has captured that AI can provide gain compared to Non-AI from different dimensions. Therefore, RRM prediction should be included in the R20 WI. Furthermore, the RRM prediction is categorized into following cases,

| Case number | Evaluation scenario combination | target study goal |
| --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | FR2 to FR2 intra-frequency spatial domain | 1st goal |

RAN2 simulation focused on temporal and frequency domain prediction i.e. case 2, 3, 4. RAN2 didn’t discuss much on spatial domain prediction, due to lack of simulation results. In RAN2#129, RAN2 has agreed that the temporal and frequency prediction will be considered as higher priority as following.

- For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

Therefore, the RRM prediction on temporal and frequency domain should be prioritized over spatial domain for WI.

Proposal 1: RRM prediction on temporal and frequency domain should be included in R20 WI.

Event prediction can be done in direct or indirect way. RAN2 has agreed to prioritize indirect prediction and the related simulation assumptions. Based on the simulation results, RAN2 concluded that the prediction result is accurate under certain scenarios. In addition, the event prediction can improve the mobility performance or save UE power by reducing the measurement.

Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. Agreements on Measurement event case B - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Therefore, event prediction should be included in R20 WI.

Proposal 2: Event prediction should be included in the R20 WI.

As agreed in last meeting, RLF predication has been deprioritized in Rel-19.

- Agreements on priority

- RLF prediction will not be studied in Rel-19

However, the event prediction discussed in SI phase is based on RSRP which may not reflect the situation of HOF and RLF very well since the HOF and RLF evaluation should be performed based on SINR. The RSRP based event prediction may not be able to avoid potential HOF and RLF. To our understanding, HOF and RLF is a key performance KPI in mobility, which would result in bad network performance, worse user experience. HOF and RLF prediction is useful to reduce the failure and interruption time. RAN2 should continue the study on SINR based failure prediction.

Proposal 3: Further study on SINR based failure prediction should be considered in R20.

The work is mainly on RAN2. Besides RAN2, RAN4 should also be involved to specify the necessary RRM/test requirement.

# Conclusions

According to the discussion above, we have the following proposals:

Proposal 1: RRM prediction on temporal and frequency domain should be included in R20 WI.

Proposal 2: Event prediction should be included in the R20 WI.

Proposal 3: Further study on SINR based failure prediction should be considered in R20.

# Reference

[1] RP-242393