3GPP TSG RAN Meeting #107	RP-250296

Incheon, South Korea, March 12-14, 2025

Agenda item:	15.2.6

Source:	Nokia

Title:	AI/ML for NG-RAN in Rel-20

Document for:	Discussion

# 1	Introduction

At RAN#106, a categorization of Rel-20 5G Advanced topics was endorsed in RP-243292 [1]. The categorization was based on contributions submitted to the 5GA workshop and placed particular emphasis on addressing “real and critical commercial deployment needs”.

AI/ML for NG-RAN was categorized as a strong candidate for inclusion in the release package with the following guidance:

From slide 13 of [1]: It is expected to continue evolving AI/ML for Ng-RAN in Rel-20 - Rel-19 Leftovers? Additional use cases? If so, very limited additional use cases. - Consider a study first, followed by WI conversion

In this paper, we provide our views on the scope and objectives for AI/ML for NG-RAN in Rel-20.

# 2	Discussion

NG-RAN AI/ML work has addressed a number of use cases in Rel-18 and Rel-19. However, in our view, several important aspects related to the existing use cases remain, which should be further considered before we proceed on new topics and additional use cases. Therefore, we think that in Rel-20 a RAN3-led work item is needed to address those issues.

## 2.1	Further discussion on Release 19 leftovers and additional use cases

In Rel-18, RAN3 worked on the three prioritized use cases (Load Balancing, Mobility Optimization, and Energy Saving), with AI/ML Model Training located in either the OAM or in the NG-RAN and with AI/ML Model Inference located in the NG-RAN. In addition, Rel-19 study item included two new use cases (AI/ML Slicing and AI/ML Coverage and Capacity Optimization (CCO)) and a number of leftover topics from Rel-18 including Energy Saving Enhancements, Continuous MDT collection targeting the same UE across RRC states, and Multi-hop UE Trajectory across gNBs. In Rel-19 work item phase the topics of Energy Saving and Multi-hop UE Trajectory dropped. This down-scoping was done in order to adapt the work item load to the available TUs, and not due to missing importance or relevance of those topics. In our analysis of Rel-20 work item scope, we therefore consider topics related to the prioritized use cases of Rel-18 or Rel-19 that were either:

a)	discussed during Rel-19 and explicitly postponed; or

b)	extensions of functionality agreed during Rel-19 that do not require study.

Proposal 1:	Rel-20 work item scope should include topics related to the Rel-19 use cases that do not require further study.

### 2.1.1	Mobility Optimization use case

In Rel-18 UE Trajectory prediction is limited to the cells of the target NG-RAN node and it is calculated on a cell-level granularity. Predicted UE Trajectory over the cells of the target NG-RAN node is limiting since it does not exploit the full potential of predicted trajectory. It would be very useful for a node to obtain trajectory information not only over its own cells but also over the cells of other neighbouring NG-RAN nodes. In this way, it can better determine how traffic is expected to depart from its coverage. In Rel-19 NG-RAN AI/ML study item, we looked into multi-hop trajectory prediction and corresponding multi-hop measured UE Trajectory, but the topic was dropped due to necessary down-scoping.

In addition, the current predicted trajectory utilizes information only from connected mode UEs. However, we can assume that mobility patterns for idle mode and connected mode UEs are the same and therefore, additional idle or inactive mode input could be used for UE Trajectory prediction.

Furthermore, in Rel-18 UE Trajectory prediction is limited to cell-based UE Trajectory. In our view, finer granularity of predicted UE trajectory in terms of beams will enable improved per-beam decisions in terms of mobility and resource allocation.

We believe that the discussions in RAN3 in Rel-18 work item phase, as well as in Rel-19 study item phase, on the Mobility Enhancements use case were extensive and therefore, this topic can be concluded in a Rel-20 work item.

Proposal 2:	Rel-20 enhancements for the mobility optimization use case should include UE Trajectory prediction enhancements, i.e. support for multi-gNB, RRC_IDLE/RRC_INACTIVE states, and beam-level (predicted) UE Trajectory granularity.

### 2.1.2	Energy Saving use case

The AI/ML Energy saving solution in Rel-18 assumes that a node wishing to initiate an action (cell activation/deactivation) is able to determine the energy cost value at the target node that this action will incur, in particular the related offloading of UEs. However, given that the energy cost metric is defined on a node level granularity, the energy cost reported by the NG-RAN node receiving offloaded traffic doesn’t necessarily reflect the actual cost pertaining to the offloading action, but will in many cases include energy cost resulting from other handovers to/from other neighbouring nodes.

In Rel-19 study item phase, RAN3 attempted to address finer granularity Energy Cost. Even though no conclusion could be drawn on this topic, a number of possible approaches to improve Energy Cost granularity were discussed such as Energy Cost per group of cells based on energy consumption measurements associated to the hardware serving the group of cells, Energy Cost per cell based on energy consumption estimations, Energy Cost per one or more handovers based on energy consumption estimations, Energy Cost per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU. Thus, we believe that in Rel-20, RAN3 should include an objective to address finer granularity Energy Cost.

We believe that this topic can be concluded in Rel-20 work item phase, since it was extensively discussed in Rel-19 in RAN3.

Proposal 3:	Rel-20 enhancements for the energy saving use case should include finer granularity of Energy Cost.

### 2.1.3	Further support of AI/ML Model Training in the NG-RAN

The current data collection framework provides support for AI/ML Model training located in the OAM. This is in particular enabled by MDT. MDT is a key component for data collection in the NG-RAN, and, from Rel-10, the MDT framework was designed to make MDT measurements available at the OAM (in the Trace Collection Entity – TCE).

However, the current data collection framework doesn’t provide full support of the use case of AI/ML Model Training located in the NG-RAN. A natural solution would be to enhance the MDT framework to also support the use case where data collection is terminated in the RAN. The MDT framework is jointly specified by SA5, RAN2 and RAN3. However, due to impact on the NG-RAN and à priori absence of UE impact it seems natural that RAN3 drives MDT enhancements for this important use case and provides requirements to SA5 as needed. We believe it is important that this aspect is covered in 5G Advanced time frame.

Proposal 4:	Enhance the MDT framework to support AI/ML Model Training in the NG-RAN.

### 2.1.4	Leftovers from the on-going Rel-19 work

Depending on the progress of Rel-19 some aspects may not be possible to fully complete on time. Also, it may not be possible to cover all aspects of the new Rel-19 use cases, e.g. energy efficiency aspects should be considered for all features, but time might not allow to handle such aspects. One such particular example is for the AI/ML Coverage and Capacity Optimization (CCO) use case.

We think that those topics should be considered part of Rel-20 work item, with decision taken at the end of Rel-19.

Proposal 5:	RAN to determine at the end of Rel-19 any leftovers from currently ongoing Rel-19 work that should be included in Rel-20 work item phase.

# 3	Potential work item objectives

1)	Enhanced support of Rel-18 and Rel-19 use cases:

-	Mobility optimization, including:

-	UE Trajectory prediction enhancements, namely multi-hop UE Trajectory prediction across gNBs, across RRC states, and beam level granularity of (predicted) UE Trajectory.

-	Energy saving, including:

-	Finer granularity Energy Cost

2)	Enhance the MDT framework to support AI/ML Model training in the NG-RAN.

3)	Support of leftovers from the Rel-19 AI/ML for NG-RAN.

Note: The Rel-19 leftovers to be included in Rel-20 (if any) will be identified at RAN#109.

# 4	Conclusions

In this contribution, we discussed the potential objectives for the Rel-20 AI/ML for NG-RAN WI, following the guidance endorsed at RAN#106 in [1] that the candidate topics for Rel-20 should address “real and critical commercial deployment needs”. We make the following proposals:

Proposal 1:	Rel-20 work item scope should include topics related to the Rel-19 use cases that do not require further study.

Proposal 2:	Rel-20 enhancements for the mobility optimization use case should include UE Trajectory prediction enhancements, i.e. support for multi-gNB, RRC_IDLE/RRC_INACTIVE states, and beam-level (predicted) UE Trajectory granularity.

Proposal 3:	Rel-20 enhancements for the energy saving use case should include finer granularity of Energy Cost.

Proposal 4:	Enhance the MDT framework to support AI/ML Model Training in the NG-RAN.

Proposal 5:	RAN to determine at the end of Rel-19 any leftovers from currently ongoing Rel-19 work that should be included in Rel-20 work item phase.

Based on the above proposals, we have provided a list of work item objectives in section 3, Thus, the following is further proposed:

Proposal 6:		Adopt the objectives in section 3 for the Rel-20 AI/ML for NG-RAN work item.

# References

- RP-243292 Summary of 5G-Advanced in Rel-20, RAN Chair