3GPP TSG RAN Meeting #107							RP-250303

Incheon, Korea, March 12-14, 2025

Agenda item:        15.2.2

Source: 	Tejas Networks, CEWiT, Reliance Jio, WiSIG Networks, IISC Bangalore, IIT Madras, IIT Hyderabad, IIT Kanpur, IIT Bhilai

Title: 	                   Views on Rel-20 5G-Advanced MIMO Enhancements

Document for:      Discussion and Decision

Introduction

In this contribution, we provide our views on a set of potential MIMO Phase 6 enhancements for Rel-20 work item. A key aspect we have identified involves DMRS enhancements, incorporating ports and port allocation combinations. Another important aspect is the reduction of CSI-RS overhead for more than 32 CSI-RS ports. We discuss each of the identified items in detail in the subsequent sections.

DMRS Enhancements

This section proposes objectives and justifications for enhancing Demodulation Reference Signal (DMRS) in 5G New Radio (NR), with a primary focus on addressing the increasing demands of large-scale multiple-input multiple-output (MIMO) systems. Fixed Wireless Access (FWA) has emerged as a critical use case within the 5G NR landscape, providing high-speed internet connectivity in areas where traditional broadband infrastructure is limited and enabling an additional monetization option for operators. However, these deployments introduce new challenges, including the need for higher-density cell configurations, improved channel estimation in scenarios involving high Modulation and Coding Scheme (MCS) indices, and significant delay spread. The proposal emphasizes DMRS port allocation strategies: including asymmetric and symmetric combinations across Code-Division Multiplexing (CDM) groups—to improve throughput and performance, particularly for FWA with outdoor Customer Premises Equipment (CPE).

## Justification

Fixed Wireless Access (FWA) has evolved as an extremely useful 5G NR use case, offering substantial advantages for residential and SME connectivity solutions. By leveraging 5G capabilities, FWA delivers high-speed internet without extensive wired infrastructure, which is particularly challenging in rural and underserved regions. Release 17 introduced 1024-Quadrature Amplitude Modulation (QAM) in the downlink to boost data rates and spectral efficiency, primarily targeting FWA devices. However, optimizing FWA deployment requires careful consideration of coverage, capacity, and economic feasibility, especially given the strain on backhaul networks from increased capital expenditure and operational expenditure.

Non-line-of-sight (NLoS) FWA deployments using massive MIMO (mMIMO) base stations (gNBs) must support advanced MCS levels (256QAM and beyond) in high-delay-spread environments. This necessitates improvements in throughput for high-MCS, high-delay-spread scenarios. A key area for optimization lies in re-evaluating DMRS port allocation principles across CDM groups. Current specifications for uplink Physical Uplink Shared Channel (PUSCH) and downlink Physical Downlink Shared Channel (PDSCH) in TS 38.212 limit port allocations to symmetric distributions within CDM groups, which restricts performance gains achievable through alternative allocation strategies.

As discussed in Appendix Section 5, our studies demonstrate that introducing asymmetric and symmetric port allocation combinations spanning multiple CDM groups enhances Mean Squared Error (MSE) performance in channel estimation. This improvement occurs when user-assigned layers partially occupy CDM groups, allowing receivers to bypass de-spreading steps by treating Frequency Domain (FD) references as distinct reference signals rather than Orthogonal Cover Code (OCC)-multiplexed reference signals. Transmitter-side CDM spreading remains unchanged.

#### Link-Level Simulation Results:

The following Figure 1 and Figure 2 show improvement in throughput and channel estimation MSE based on enhanced DMRS port allocation.

Figure 1 Comparison between proposed DMRS port allocation and legacy port allocation for 3 port case

Figure 2 Comparison between proposed DMRS port allocation and legacy port allocation for 4 port case

#### Observation

Rank-3 configurations using enhanced port allocation, [0,2,4] (Type 2, Length 2), show 40% higher throughput compared to legacy port allocations, [0,1,2] (Type 1, Length 1).

Rank-4 configurations using enhanced port combination, [0,2,4,6] (Type 2, Length 2), demonstrate 20% throughput gains over legacy port allocations, [0,2,4,6] (Type 1, Length 2).

Combined with the growing emphasis on FWA performance, these findings justify exploring additional DMRS port allocation principles and defining new port combinations for enhancement.

Across Releases 17,18 and 19, significant enhancements toward joint transmission and multi-TRP objectives were achieved. These include a unified TCI Framework, improved CSI Reporting for NJCT, tight synchronization across TRPs for CJT (coherent joint transmission), and advanced codebooks. These developments reflect a concerted effort to optimize joint transmission techniques and multi-TRP operations.

To further improve communication quality across the cell edge area, increasing the number of TRPs that can be coordinated is required. Increased TRPs bring motivation for the increased DMRS ports. The port allocation principle incorporates DMRS port combinations, which provide an asymmetric advantage across ports and benefit TRPs with uneven performance across layers. In Release-19, enhancements were made to the specifications to support hybrid beamforming and fully digital gNB implementation options, enabling large antenna arrays with many CSI-RS ports for CSI reporting (up to 128 CSI-RS ports). However, to proportionally increase the multiplexing capacity of the downlink DMRS for various use cases, there is a further need to increase the number of orthogonal ports for DMRS beyond 24 ports.

Summary of Justifications for Increased number of DMRS ports

- Enhanced Multiplexing requirements due to Enhanced CSI ports to 128 or greater.

- To support a higher number of transmit-receive points (TRP) in mTRP (multiple-TRP) cases.

Summary of Justifications for Enhanced DMRS port combinations

- Performance enhancements in high MCS, high delay spread to cater to FWA, CPE use cases

Improvement in the performance for deployments using higher SCS (larger impact of delay spread).

Enhance mTRP deployment to manage uneven layer performance from issues like time misalignment.

Address DMRS port allocation for increased DMRS ports.

## Proposed Objectives

Introduce DMRS enhancement to support beyond 24 orthogonal DMRS ports

- No change in the number of DMRS Symbols

- Enhance number of DM-RS ports by increasing the OCC length.

Enhance DMRS/Antenna port combinations and indication

- Add port allocation rows into existing or new tables in 38.212.

- Update co-scheduling procedures in 38.214.

CSI-RS Overhead Reduction

This section presents the objectives and justifications for proposing CSI-RS resource overhead reduction as a key issue to be addressed in Rel-20 5GA as part of MIMO Phase 6 enhancements.

## Justification

For DL channel sounding, NR Rel-19 agreed to enhance the CSI framework to support large antenna systems by configuring CSI-RS resources up to 128 ports. To realize this enhancement, a maximum of four 32-port CSI-RS resources can be aggregated in a single resource set and sounded as a 128-port reference signal. In general, K M-port resources can be configured in a single resource set to sound  PCSI-RS=N CSI-RS ports, where N= K*M, such that K=N/M is the number of resources in a resource set.

Increasing CSI-RS resources by K to support larger ports requires extending the CSI-RS pattern in time or frequency or both. Consequently, a gNB must allocate more resource elements (REs) for transmitting reference signals, which increases the downlink CSI-RS resource overhead. This increase occurs when the number of CSI-RS ports exceeds 32. As a result, the network experiences a significant reduction in DL throughput, influenced by factors such as the number of aggregated resources K, the frequency domain density of resources (0.5 or 1), and the periodicity of CSI-RS (the rate of CSI occasions).

As compared to rural deployments, where the CSI-RS resource overhead problem may be trivial, we believe that the problem is critical in networks like densely populated urban environments, smart city infrastructure, and industrial IoT, among others, where the signal quality, interference levels, propagation characteristics, and network conditions are dynamic and where frequent CSI occasions are necessary to provide accurate and reliable network quality.

To avoid the resulting degradation of the network load and capacity in 128 port systems, this proposal introduces a method of occasional sounding of full port CSI-RS and frequent sounding of reduced CSI-RS ports that aid full port sounding and full port CSI acquisition. This method of deployment-aware channel sounding is very beneficial in 128-port massive MIMO narrow-beam systems, as it is otherwise challenging to reduce the rate of full-port CSI occasions in such systems due to their increased channel variability.

Current specifications prevent exploiting the benefits that can be obtained through the flexible sounding of full and reduced CSI-RS ports, wherein the term "reduced ports" in this context is defined as a subset of the full CSI-RS ports. Certain modifications in the CSI-RS resource and report configurations and settings are essential to implement the proposed method of solving the CSI-RS resource overhead problem.

## Proposed Objectives

The objective of this proposal is to modify the NR CSI framework by enhancing CSI-RS resource and report configurations and settings to enable flexible sounding of full and reduced CSI-RS ports. Specifically, this involves occasional sounding of full-port CSI-RS and frequent sounding of a subset of CSI-RS ports that aid full-port sounding and full-port CSI acquisition for CSI-RS overhead reduction.

The proposed method of flexible sounding of full and reduced CSI-RS ports to achieve CSI-RS overhead reduction is briefly explained as follows;

- Channel sounding of DL antenna ports will involve sounding of full CSI-RS ports occasionally and a subset of the full CSI-RS ports frequently.

CSI-RS transmissions in PDSCH will be such that the CSI occasions linked to the configured full CSI-RS ports will repeat themselves after a certain number of instances of CSI occasions linked to a subset of the configured full CSI-RS ports.

- All the CSI-RS resources configured for aggregating full ports, i.e., K M-port CSI-RS resources, will have a coarse period/lower rate of CSI occasions, whereas a subset of CSI-RS resources that are part of the aggregated resources, i.e., L M-port CSI-RS resources, will have a relatively fine period/higher rate of CSI occasions.

The coarse and fine periods may be identified by the gNB alone, UE alone, or through an association between a UE and gNB.

Figure 3: An illustration of the finer and coarser period for the resources indicated in the disclosure.

Figure 4: The proposed CSI framework sequence flow illustrated for CSI-RS resource configuration set as aperiodic, periodic, or semi-persistent, illustrating resource overhead reduction.

As a sample illustration, we demonstrate the proposed strategy in scenarios requiring enhanced beamforming resolution in the horizontal domain (azimuth plane), by considering a dual-polarized large antenna panel with 128 ports (N1=8, N2=4).

Figure 5: (a) An example mapping method (b) Illustration of reference beam indication in the first CSI occasion and the next CSI occasion (c) using reduced resources (d) Illustration of updated beam generated with new i1,1 indicator and pre-stored i1,2

Based on our observations, the beam variations in the domain that require higher beamforming resolution are well captured using the proposed method. Furthermore, due to the reduced CSI-RS resource elements (REs) in the fine occasions where only a subset of resources is sounded, more data can be packed into the REs, thereby increasing the data throughput.

As noted, the modified CSI framework with the proposed enhancements will support occasional full port CSI-RS sounding and frequent sounding of a subset of CSI-RS ports. This approach aids full port sounding and CSI acquisition while reducing overall CSI-RS overhead.

For MIMO Phase 6, introduce flexible sounding of full and reduced CSI-RS ports to achieve CSI-RS overhead reduction.

Conclusions

For Rel-20 5GA MIMO Phase 6 enhancements, we propose the following enhancements,

- Introduce DMRS enhancement to support beyond 24 orthogonal DMRS ports.

- Enhance DMRS/Antenna port combinations and indication.

- Introduce flexible sounding of full and reduced CSI-RS ports to achieve CSI-RS overhead reduction.

Appendix

## DMRS Port Allocation Proposal

### DMRS Port Allocation Principle

Adopt a new principle for the DMRS port allocation, where the ports for layer assignment are chosen across CDM groups such that at least one layer is allocated to each group before additional layers are assigned within the same group. This approach provides a Mean Square Error (MSE) advantage to layers in Channel Estimates for partially allocated CDM groups.

Figure 6 Proposed strategy for port allocation

### Proposed DMRS Port Combinations

Based on the port allocation principle described in section 5.1.1, we propose additional rows for the Antenna (DMRS) port allocation tables in TS 38.212. Following Table 1 shows the proposed DMRS port combinations.

To leverage the performance benefit, the receiver must bypass the de-spreading step in Channel Estimation (CE) for partially allocated CDM groups by treating references on corresponding frequency and time domain resources as distinct rather than OCC.

Modification is required only on the receiver side (Software upgrade), and no changes are required on the transmitter.

Table 1 Proposed port allocation combinations

| Number of DMRS CDM group(s) without data | Proposed DMRS/Antenna port combination | Number of Front Loaded DMRS Symbols | Tables Impacted (TS 38.212) | DL/UL |
| --- | --- | --- | --- | --- |
| 2 | [0,1,2,4,5] | 2 | Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 2 | [0,2,4] | 2 | Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,2,4] | 1 | Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1 | DL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1 | DL |
| 3 | [0,1,2,4,6] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,2,4] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,1,2,4] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2 | DL |
| 2 | [0,1,2,8] | 1 | Table 7.3.1.2.2-7: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 2 | [0,1,2,4,5] | 2 | Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 2 | [0,1,2,4] | 2 | Table 7.3.1.2.2-8: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,1,2,4,12] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,1,2,4,12,13] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,2,4] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.2.2-9: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1 | DL |
| 3 | [0,1,2,4,6] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,1,2,4,6,7] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,2,4] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.2.2-10: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2 | DL |
| 2 | [0,2,4] | 2 | Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=1, dmrs-TypeEnh is not configured, maxLength=2, rank = 3 | UL |
| 3 | [0,2,4] | 1 | Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1, rank =3 | UL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=1, rank =4 | UL |
| 3 | [0,2,4] | 2 | Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, multipanelSchemeSDM is not configured, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=3 | UL |
| 3 | [0,1,2,4] | 2 | Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=4 | UL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank=4 | UL |
| 3 | [0,1,2,4,6] | 2 | Table 7.3.1.1.2-23A: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank = 5 | UL |
| 3 | [0,1,2,4,6,7] | 2 | Table 7.3.1.1.2-23B Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is not configured, maxLength=2, rank = 6 | UL |
| 2 | [0,1,2,8] | 1 | Table 7.3.1.1.2-41: Antenna port(s), transform precoder is disabled, dmrs-Type=1, dmrs-TypeEnh is configured, maxLength=1, rank = 4 | UL |
| 3 | [0,1,2,4] | 1 | Table 7.3.1.1.2-57: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1, rank = 4 | UL |
| 3 | [0,1,2,4,12] | 1 | Table 7.3.1.1.2-58: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1, rank = 5 | UL |
| 3 | [0,1,2,4,12,13] | 1 | Table 7.3.1.1.2-59: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=1, rank = 6 | UL |
| 3 | [0,2,4,6] | 2 | Table 7.3.1.1.2-65: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2, rank = 4 | UL |
| 3 | [0,1,2,4,6] | 2 | Table 7.3.1.1.2-66: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2, rank = 5 | UL |
| 3 | [0,1,2,4,6,7] | 2 | Table 7.3.1.1.2-67: Antenna port(s), transform precoder is disabled, dmrs-Type=2, dmrs-TypeEnh is configured, maxLength=2, rank = 6 | UL |

### Views on Specification Impact

- New port allocation combinations are proposed for inclusion in the following tables of TS 38.212

  - Uplink PUSCH Channel: Tables 7.3.1.1.2-6 to 7.3.1.1.2-23D and Tables 7.3.1.1.2-38 to 7.3.1.1.2-69

  - Downlink PDSCH Channel: Tables 7.3.1.2.2-1 to 7.3.1.2.2-10A

- A new UE capability parameter is proposed for addition in TS 38.306 to indicate whether a device can utilize partially allocated CDM groups by bypassing the de-spreading step.

- Minor refinements to co-scheduling procedures in TS 38.214 are proposed, based on the new capability parameter.

### Additional Link Level Simulation Results

The following Figures show improvement in throughput and channel estimation MSE based on enhanced DMRS port allocation.

Figure 7 Comparison between proposed port combination [0,2,4] for DMRS Length 2 (Type1 and Type2) and legacy port combination [0,1,2] for DMRS Length 1 (Type 1) with 3 DMRS ports de-spreading benefit

Figure 8 Comparison between proposed port combination [0,1,2,4] for DMRS Length 1 and 2 (Type2) and legacy port combination [0,1,2,3] for DMRS Length 1 (Type 1) with 2 DMRS ports de-spreading benefit

Figure 9 Comparison between proposed port combination [0,2,4,6] for DMRS Length 2 (Type2) and legacy port combination [0,2,4,6] for DMRS Length 2 (Type 1) with 2 DMRS ports de-spreading benefit

Figure 10 Comparison between proposed port combination [0,2,4] for DMRS Length 1 and 2 (Type2) and legacy port combination [0,1,2] for DMRS Length 1 (Type 1) with 3 DMRS ports de-spreading benefit

Figure 11 Comparison between proposed port combination [0,1,2,4] for DMRS Length 1 and 2 (Type2) and legacy port combination [0,1,2,3] for DMRS Length 1 (Type 1) with 2 DMRS ports de-spreading benefit