3GPP TSG RAN meeting #107										RP-250307

Incheon, S. Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.2.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_AIML_air_Ph2 |  |  |  |  |
| Unique ID | 1060078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243245 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: Sept ‘25 | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 50% | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### 2.1.1.1	RAN1#120

# CSI Compression

Conclusion

Under 10% UCI loss,  Case 2 shows small to large performance drop (by Comparing Scenario B and Scenario A). Signaling support to mitigate the UCI loss can improve the performance and helps preserving the gain of Case 2 over Case 0 (by Comparing Scenario C and Scenario A and benchmark schemes).

Agreement

Investigate the following approaches for signaling support for mitigating the impact of UCI loss

- NW-signaling to reset of historical CSI information at UE

- NW-triggered CSI retransmission

Conclusion

For direction A, performance impact, if any, due to NW / UE data distribution mismatch with respect to UE side additional condition can be addressed.

Agreement

In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then

- The same ID can be used for UE to collect UE-side target CSI for UE-side training

- The same ID can be used for applicability inquiry and reporting

- The same ID can be used for inference configuration

- FFS: whether ID/even same ID is needed for monitoring configuration

- FFS: where the ID is assigned

- Note: whether the purpose for pair will be specified will be discussed separately.

Agreement

For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, performance target is confirmed as additional information along with the exchanged dataset or the model parameters.

- FFS: type of performance metric

- FFS: input data for evaluating the performance

Conclusion

- Direction C with standardized model ensures a minimum performance requirement.

  - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

- Direction A with standardized model parameter / dataset exchange from NW-side to UE-side may achieves better performance.

Conclusion

For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following

- Data distribution mismatch that is identified by either NW side or UE side

- Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

Agreement

For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose,

Encoder description:

| Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer) | Number of tokens Feature dimension of each token Output dimension dmodel per token |
| --- | --- |
| Positional encoding | Number of tokens positions Dimension of positional encoding for each token position: dmodel |
| Transformer blocks | Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice |
| Output linear layer | Reshape the matrix to a vector Output dimension zdim |
| Quantization | Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment |

Decoder description:

The decoder has a mirroring design as the encoder. Details are to be discussed.

Agreement

For model structure scalability study for temporal domain Case 0,

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension

    - The number of tokens varies with the number of subbands.

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension

    - The number of tokens varies with the number of Tx ports.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens.

    - The number of tokens varies with the number of Tx ports and the number of subbands.

- For scalability over the feature dimension,

  - Alt1: specific embedding layer for each feature size

  - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values)

- For scalability over the token dimension,

  - Alt1: positional embedding specific to each token index

    - Ntoken,active tokens out of  Ntoken token positions are used as input.

  - Alt2: Padding at the input

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration

  - Alt2: truncation/masking of the output linear layer output

  - Alt3: by varying quantization parameters

- Notes

  - Other Alternatives are not precluded.

  - Different Alternatives may be used in combination.

  - Same/similar approach is applied at the decoder side.

- Evaluations to consider:

  - Case 1 (scalable structure): Scalable model structure described above

    - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure.

    - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports)

  - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations.

    - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths.

  - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}.

Agreement

For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values.

- Number of Tx ports

  - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports)

  - [N1, N2, P] = [4,4,2] (32 CSI-RS ports)

  - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports)

- CSI feedback payload size

  - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z.

- Bandwidth & subband size

  - All subbands

  - A subset of all the subbands.

Agreement

Capture the following observations into TR:

For the evaluation of performance impact due to UE-side / NW-side data distribution mismatch with respect to UE-side additional condition (issue 4 and 6),

When dataset A include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 1 source [Panasonic] observes minor performance loss (-0.22% ~ -1.09%) relative to case 1 with NW side target CSI sharing.

When dataset A include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing,

- 1 2 source [ZTE,vivo] observes minor performance loss (-0.67 -0.006% ~ -0.97%) related to case 1 without NW side target CSI sharing.

- 1 source [OPPO] observes moderate performance loss (-3.3% ~ -4%) relative to case 1 without NW side target CSI sharing

When dataset A include dataset B, for case 3 (Direction B),

- 1 2 source [ZTE,vivo] observes minor performance loss  (-0.6% ~ -0.9%  -0.9% ~ +0.01%) relative to case 1.

When dataset A does not include dataset B, for case 2, option 3a-1, with NW-side target CSI sharing

- 3 sources [QC, Apple, Samsung] observe similar performance (-0.4% ~ +0.24%) as case 1 for Alt1 UE training.

- 3 sources [vivo, Xiaomi, ETRI] observe minor performance loss up to -2.5% relative to case 1 for Alt1 training.

- 1 2 source [Ericsson, ZTE] observe moderate performance loss of (-3.8 ~ -8.3%) relative to case 1 for Alt1 training.

- 1 source [OPPO] observe significant performance loss (-51% ~ -62.5%) relative to case 1 for Alt1 UE training.

- 2 sources [CATT, Futurewei] observe minor performance loss (-1.62% -2.78%  ~ -3.2%) relative to case 1 for Alt2 UE training

- 1 source [Apple] observe moderate loss of -6.7% relative to case 1 for Alt2 UE training.

When dataset A does not include dataset B, for case 2, option 3a-1, without NW-side target CSI sharing

- 4  5 sources [CATT, ZTE, Xiaomi, ETRI, vivo] observe minor to moderate performance loss (-0.4% ~ -3.9%) relative case 1.

- 2  3 source [QC, Ericsson, Apple] observe moderate performance loss (-6.7% ~ -8.6%) relative to case 1.

- 1 source [OPPO] observes significant performance loss of -62.1% relative to case 1.

When dataset A does not include dataset B, for case 2, option 4-1,

- 4 sources [QC, Apple, Xiaomi, ETRI] observe zero to minor performance loss (-2.4% ~ 0%) relative to case 1 for Alt1 UE training.

- 1 source [Ericsson] observe minor to moderate performance loss (-2.9% ~ -7.9%) relative to case 1 for Alt1 UE training depending on whether dataset A applies augmentation using various phase normalization methods.

- 4 sources [CATT, Xiaomi, Futurewei, ETRI] observe minor performance loss (-1.41% ~ -3.52%) relative to case 1 for Alt2 UE training.

- 1 source [Apple] observes moderate loss of -7.9% relative to case 1 for Alt2 UE training.

- 1 source [vivo] observes moderate loss of -7.7% relative to case 1 for 3a-1 for Alt1 UE training when backbone are different.

When dataset A does not include dataset B, for case 3, Direction B,

- 4 sources [CATT, vivo, ZTE, ETRI] observe minor loss to positive gain (-3.7% ~ 1%) relative to case 1.

- 1 source [QC] observes significant loss (-20%) relative to case 1.

Agreement

Capture the following observations into TR:

For the evaluation of performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution (Issue 9),

For case 2 (model trained on dataset S but tested on dataset B),

- 2 sources [vivo, QC] observe significant performance loss (-7.2% ~ -17.4%) relative to case 1, where dataset B consists of actual field data.

- 1 source [ETRI] observes significant performance loss of -37.3% relative to case 1, where dataset S and B are different by TxRU mapping.

- 3 resources [ZTE, Panasonic, Ericsson] observe moderate performance loss (-2.12% ~ -4.75%) relative to case 1, where dataset S and B are generated from different scenarios, antenna layout or UE location. E.g., Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

For case 2A (model further finetuned on dataset B),

- If finetune at NW and fix encoder at UE

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [vivo] observe moderate loss (-4.63% ~ -7.66%) relative to case 1. The improvement compared to case 2 is minor to moderate (2.58% ~ 9.17%).

    - When dataset A / B mismatch is not considered, 2 1 sources [vivo, QC] observes minor to moderate loss of (-3.5% ~ -7.66%) loss relative to case 1. The improvement compared to case 2 is moderate to significant (2.6% ~ 13.9%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is considered but dataset B does not contain A, 1 source [ZTE] observes moderate loss to case 1 (-4.32% ~ -4.75%). The improvement compared to case 2 is negative (-0.68% ~ -1.02%).

    - When dataset A includes or is the same as dataset B, 3 sources [ZTE, Panasonic, Ericsson] observe minor to moderate loss relative to case 1 (-0.97% ~ -4.67%). The improvement compared to case 2 is negative to minor (-0.61.01% ~ 1.6149%). Dataset S and B are Uma and InH respectively, or dataset S and B are different by antenna layout and indoor/outdoor ratio.

- If finetune at UE and fix decoder at NW

  - For using actual field data as dataset B,

    - 2 sources [vivo, QC] observe moderate to significant loss (-6.8% ~ -12%) relative to case 1. The improvement compared to case 2 is minor moderate (0.4% ~ 7.1%) depending on the specific scenario where the field data is collected.

  - For using synthetic data as dataset B

    - 2 source [Panasonic, Ericsson] observe minor to moderate loss (-1.56% ~ -3.81%) relative to case 1. The improvement compared to case 2 is marginal (0.03% ~ 0.75%). Dataset S and B are different by antenna layout and indoor/outdoor ratio.

    - 1 source [ETRI] observes significant loss (-23.8%) relative to case 1. The improvement compared to case 2 is significant (13.5%). Dataset S and B are different by TxRU mapping

- If finetune at both UE and both NW sides

  - For using actual field data as dataset B

    - When dataset A / B mismatch is considered and dataset A does not contain B, 1 source [vivo] observes significant loss (-8.5% ~ -44.85%) relative to case 1 depending on specific scenarios for data collection. The improvement compared to case 2 is negative (-28% ~ -1.3%). The loss relative to finetune at one side is negative (-1.72% ~ -37.19%).

    - When dataset A / B mismatch is not considered and A is equal to B, 1 2 sources [vivo, QC] observes moderate to significant loss of (-7.3% ~ -56.1%) relative to case 1. The improvement compared to case 2 is negative to significant (-39.3% ~ 10.1%). The loss relative to finetune at one side is minor to significant (-1.7 ~ 48.4%) negative to moderate (-3.82~ 4.67%).

  - For using synthetic data as dataset B

    - When dataset A / B mismatch is modelled by different scenarios (i.e., by NW side condition),

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-29.0% ~ -30.5%) relative to case 1 depending on whether data Set S is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.3% ~-1.3%)

    - When dataset A includes B or is equal to B or has same distribution as B,

      - If the input is in spatial-frequency domain, and dataset A does not contain B, 2 sources [Panasonic, Ericsson] observe minor to moderate loss (-1.1% ~ -3.69%) relative case 1 depending on the modelling of synthetic data. The improvement compared to case 2 is minor (0.57% ~ 1.3%). Compared to finetune at one side, the performance improvement is similar to minor (-0.35% ~ 1%).

      - If the input is in spatial-frequency domain, 1 source [Samsung] observes minor to significant loss (-3.67% ~ -11.18%) relative to case 1 depending on whether data Set S is used in the finetune.

      - If the input is in angle-delay domain, 1 source [Samsung] observes similar performance to case 1 (-0.89% ~1.51%).

    - When dataset A / B mismatch is modelled by Rx antenna spacing and dataset A does not contain B and dataset A has same distribution as S (i.e., synthetic/field data distribution mismatch modeled only at the UE side (via UE-side additional condition) but not modeled at the NW-side), 1 source [Samsung] observes similar performance (-0.89% ~ 0.21%) as case 1.

Agreement

Study performance-complexity trade-off by comparing different AI/ML models, e.g. by optimizing existing designs, and/or by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler), by considering the following aspects

- Performance comparison between different AI/ML designs and benchmark schemes

- Complexity numbers (FLOP, calculated/projected latency or power consumption if available, measured latency or power consumption if available) of different AI/ML designs and benchmark schemes

## Other aspects of AI/ML model and data

Agreement

From RAN1 perspective, for the study of delivery/transfer Case z4, if the known structured model is specified in 3GPP, at least consider the following for the open format

- Define an open format within 3GPP

Conclusion

- From RAN1 perspective, the study progress of model identification and model transfer/delivery in Agenda item 9.1.4.2 are sufficient.

  - New work/discussion in AI 9.1.4.2 can be triggered by LS from other working group(s) or other TSG(s), if any

  - The work on pCR (if any) to capture the output of AI 9.1.4.2 to be done in future meeting(s)

  - Other on-demand work (if any)

- From RAN1 perspective, there is no consensus in Rel-19 on the need of standardized solution for model delivery/transfer of one-sided model

- In RAN1, if needed, whether model identification is needed for CSI compression or not is to be decided in Agenda item 9.1.4.1

- In RAN1, if needed, whether standardized solution(s) for model transfer/delivery are needed for CSI compression or not is to be decided in Agenda item 9.1.4.1

#### 2.1.2	Remaining Open issues

- Finalize study on CSI compression

- Finalize study on other aspects of AI/ML model and data with RAN1 impact

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.1.1	RAN2#129

RAN2#129 discussed the topic of model transfer/delivery, focusing on the evaluation of the feasibility of solutions related to the transfer of dataset/model parameters of two-sided models that RAN1 identified in the LS to RAN2 (R1-2410922). However, no agreement was reached on this topic.

The topic of CN/OAM/OTT collection of UE-sided model training data was not discussed.

Further, related to the topic of model transfer/delivery, the following post-meeting email discussions was agreed:

[POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)

Intended outcome: Identify the options for OTA and non-OTA, based on TR, contributions and considering data collection discussion.

Deadline:  Mar.  21st 10:00 UTC

#### 2.2.2	Remaining Open issues

Related to the topic of model transfer/delivery, RAN2 can continue the discussion on the feasibility evaluation of the RAN1 solutions in the RAN1 LS (R1-2410922).
Related to the topic of CN/OAM/OTT collection of UE-sided model training data, RAN2 can continue the discussion based on the SID.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#114

Topic 1: Post-deployment testing

Way forward:

The following table presents the description of options 1 and 2 and sub-options as already agreed.

- A new name for “option 1” and “option 2” is proposed to reduce ambiguity when discussing in the future.

- The descriptions of the options are the same as the already agreed description.

- The “further potential clarifications” box captures issues that have been raised that could further clarify the options. It does not represent any agreement on the issues, or that all are relevant, but is intended to stimulate input and discussion to future meetings.

- The “significant issues” line is intended to capture issues that have been raised that should be answered in order to make a decision.

|  | Option 1 | Option 2 |
| --- | --- | --- |
| New name | Pre-activation functionality/model update testing | Post deployment management based on LCM |
| Description | Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs • Validation takes into account the UE hardware in which the model is to be deployed/activated. | Using performance monitoring and LCM procedures • Performance monitoring will be designed in other groups • RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. |
| Sub-options | Possible collection of input data to a model during conformance testing for use later on. |  |
| Further potential clarifications (not resolved) | The following proposals have been made for option 1 but not discussed. o Whether the testing of a new model is on a device or in a lab. o Whether models that have been updated post conformancetesting should be kept in a device. o Whether parallel operating of one model and (in-device) testing of another can be assumed. o Whether signaling from a UE of changes/updates are needed. - Whether there is any relation to GCF procedures. - Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes. - Definition of Fine-tuning and Model update - Model update can have a performance impact. - Fine-tuning might have little performance impact. - Hybrid Validation approach - A hybrid approach integrating both pre-activation functionality/model update testing (Option 1) and post-deployment management based on LCM (Option 2) aims to provide a balanced validation strategy. | The following proposals have been made for option 1 but not discussed. - Whether the metric used for performance testing and the metric used for monitoring can be aligned. - Whether monitoring used for post-deployment testing should be NW sided only. |
| Significant issues | Whether the option is reasonable or would cause too much complexity and test time burden when introducing updates. | Whether LCM monitoring can actually unambiguously identify individual model performance with reasonable complexity (considering other variations due to e.g. channel, interference, scheduling, other AI models etc.) Whether LCM monitoring can have RAN4 requirements to ensure consistent monitoring between different UEs. |

Topic 2: Testing for one sided models

Issue 4-1: Baseline test setup

Agreement:

- Discuss both single AoA based and multiple AoA based set up

  - Continue to study the two multiple AoA candidate setups

    - One option to continue the discussion proposed as following:

      - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability

        - AI/ML BM performance centric, not CDL channel emulation centric.

        - Define test system candidates which are already feasible

      - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance.

  - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

Issue 4-2: Channel model

Agreement:

	Use CDL-based channel model as starting point

Topic 3: CSI compression

Sub-topic 1: Refinement of option 3 track 1

Agreements on quantization:

Companies shall provide decoders with 2 bit quantization, and with no quantization

- No quantization aware training when model is unquantized

- Quantization aware training when model is quantized

- For the model with quantization:

  - Agree on a scaling and quantization codebook

  - Include Sigmoid in model file in the encoder, and inverse Sigmoid in the decoder

  - Codebook (1/8, 3/8, 5/8, 7/8) in model file

  - Do not including mapping to 2 bits. Quantizer function of converting to 2 bits should not be in the model file

- For the model without quantization

  - For no quantization models, do not include sigmoid at the model output and no inverse Sigmoid in the decoder

Agreements regarding the input / output dimensionality:

- define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size

- define Encoder Output and Decoder Input as data dimension (n, 32) with data type float

- No additional inputs

How to continue with track 1 until RAN4#114bis

- Companies should provide quantized and unquantized models according to the advice above, in particular Samsung, CATT and Huawei.

- Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results

Sub-topic 2: Refinement of option 3 track 2

Dataset agreement:

Take mixed dataset comprising Mediatek, Ericsson, Vivo, Oppo, Nokia

How to continue with track 2 until RAN4#114bis:

The mixed dataset will be revised such that it comprises data from the 5 above companies and uploaded to the folder for RAN4#114.

Companies requested to provide new models based on this mixed dataset together with SGCS within 2 weeks

The model with median SGCS will be selected for the frozen decoder. If an even number, choose the model below the median

Once the frozen decoder is selected, companies are invited to train their own encoders using the mixed dataset or/and private dataset and test using the mixed test dataset

Sub-topic 3: Refinement of option 4

Agreement

The following update to the procedure description to option 4 is agreed

Option 4a(Dataset based) for 2-sided model

    - Step 1-3: Reuse results of Option 3

    - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3

      - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1

      - Or mixed dataset from track 2

      - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately

    - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3.

    - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s)

    - Performance alignment to be checked/discussedStep 7: Conclude on overall feasibility of Option 4a

      - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

      - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

Option 4b (encoder based) for 2-sided model

- Step 1-3: Reuse results of Option 3

- Step 4: Select one or more encoder(s) for further analysis based on option 3

  - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1

  - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2

- Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s)

  - Performance alignment to be checked/discussed

  - FFS what is the dataset assumed for the training

- Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5

  - FFS what is the dataset assumed for the training

- Step 7: Conclude on overall feasibility of Option 4b

  - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ).

  - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset

Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

Sub-topic 4: What would be standardized for option 3

By definition, a test decoder is standardized for option 3. The need to define a reference encoder was discussed and the following agreement reached

Agreement:

- To verify the performance of CSI compression test decoder for calibration of test equipment, agree to define the reference encoder including encoder structure and parameters in RAN4

  - FFS on whether to specify it in TS or TR, and capture the conclusion in the TR conclusion part

  - FFS on criterion for selection of reference encoder

  - FFS on decision whether RAN4 or RAN5 will specify it

Way forward:

There was also a discussion on whether to capture a training dataset, but no agreements. Some open questions from the discussion (not agreements):

- Whether it is useful to capture a training dataset

- Where and how the training dataset would be captured (TR, TS, some other public place…)

#### 2.4.2	Remaining Open issues

- General aspects

  - Post deployment handling

  - Granularity for RAN4 requirements LCM handling

  - LCM related requirements

  - Relation to legacy

- Testing for one sided models

  - Baseline test setup

  - Channel model

  - Set B and Set A beams

  - Other requirements for the test setup

  - Positioning test feasibility for case 1

- Testability and interoperability issues for CSI compression

  - Simulation results

  - ”Track 1” further refinement

  - ”Track 2” further refinement

  - Option 3 other considerations

    - Need for reference encoder

    - Need for training dataset

    - Representation of models

    - Model structure

    - Principles for defining test decoders

  - Option 4 procedure

  - Other option 4 considerations

    - What should be specified for option 4a

    - What should be specified for option 4b

    - Variation of test decoder and encoder structures

  - Other considerations that are independent of option 3 / option 4

    - Generalization

    - Monitoring

    - Storage of data/models

    - Capture model parameters in the TR

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

#### 4.1.1	RAN1#120

# CSI compression

R1-2501472	Final summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2501471	Final summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2501470	Final summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2501469	Final summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2501468	Final summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2500051	Discussion of CSI compression on AI/ML for NR air interface	FUTUREWEI

R1-250008	AI/ML for CSI compression	Ericsson

R1-2500069	Discussion on study for AI/ML CSI compression	ZTE Corporation, Sanechips

R1-2500151	Discussion on AI/ML for CSI compression	Huawei, HiSilicon

R1-2500161	Discussion on AIML for CSI compression	Spreadtrum, UNISOC

R1-2500204	Further study on AI/ML-based CSI compression	CATT

R1-2500277	Discussion on AI/ML for CSI compression	CMCC

R1-2500340	Discussion on CSI compression	vivo

R1-2500407	Discussion on AI/ML for CSI Compression	Tejas Networks Limited

R1-2500468	Additional study on AI/ML-based CSI compression	OPPO

R1-2500534	On AI/ML-based CSI compression	InterDigital, Inc.

R1-2500548	AI/ML based CSI Compression	Google

R1-2500558	Discussion on AIML CSI compression	TCL

R1-2500567	Study on CSI compression	LG Electronics

R1-2500601	Discussion CSI compression	NEC

R1-2500638	On AI/ML for CSI compression	Lenovo

R1-2500689	Addtional study on AI-enabled CSI compression	NVIDIA

R1-2500713	Views on two-side AI/ML model based CSI compression	Xiaomi

R1-2500769	Discussion on AI based CSI compression	Apple

R1-2500817	Discussion on AI/ML for CSI compression	Panasonic

R1-2500837	Views on additional study for AI/ML based CSI compression	Samsung

R1-2500903	Discussion on AI/ML for CSI compression	ETRI

R1-2500928	Discussion on CSI compression with AI/ML	Fujitsu

R1-2500973	AI/ML for CSI Compression	Nokia

R1-2501015	Additional study on AI/ML for NR air interface - CSI compression	MediaTek Inc.

R1-2501119	AI/ML for CSI feedback enhancement	Mavenir

R1-2501147	Additional study on CSI compression	Qualcomm Incorporated

R1-2501193	Discussion on AI/ML for CSI compression	NTT DOCOMO, INC.

R1-2501274	Discussion on AI/ML for CSI compression	CEWiT

R1-2501350	Discussion on two-sided AI/ML model based CSI compression	IIT Kanpur

# Other aspects of AI/ML model and data

R1-2501404	Summary#2 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2501403	Summary#1 for other aspects of AI/ML model and data 	Moderator (OPPO)

R1-2500052	Discussion on other aspects of AI/ML model and data on AI/ML for NR air-interface				FUTUREWEI

R1-2500070	Discussion on other aspects of AI/ML model and data	ZTE Corporation, Sanechips

R1-2500152	Discussion on other aspects of the additional study for AI/ML	Huawei, HiSilicon

R1-2500162	Discussion on other aspects of AI/ML model and data	Spreadtrum, UNISOC

R1-2500205	Further study on AI/ML for other aspects	CATT, CICTCI

R1-2500255	Discussion on other aspects of AI ML model and data	China Telecom

R1-2500278	Discussion on other aspects of AI/ML model and data	CMCC

R1-2500341	Other aspects of AI/ML model and data	vivo

R1-2500392	Discussion on other aspects of AI/ML	Ericsson

R1-2500408	Other aspects of AI/ML Model and Data 	Tejas Networks Limited

Withdrawn

R1-2500469	Additional study on other aspects of AI/ML model and data	OPPO

R1-2500549	AI/ML Model and Data	Google

R1-2500559	Discussions on other aspects of AlML In NR air interface	TCL

R1-2500568	Discussion on other aspects of AI/ML model and data	LG Electronics

R1-2500591	Discussion on other aspects of AI/ML model and data	NEC

R1-2500639	Discussion on other aspects of AI/ML model and data	Lenovo

R1-2500690	Additional study on other aspects of AI model and data	NVIDIA

R1-2500714	Further study on AI/ML model and data	Xiaomi

R1-2500770	Discussion on other aspects of AI/ML models and data	Apple

R1-2500815	Discussion on the terminology alignment TR in SA	Panasonic

R1-2500838	Views on additional study for other aspects of AI/ML model and data	Samsung

R1-2500904	Discussion on other aspects of AI/ML model and data	ETRI

R1-2500929	Discussion on other aspects of AI/ML model and data	Fujitsu

R1-2500974	Other aspects of AI/ML for two-sided model	Nokia

R1-2500976	Discussion on other aspects of AI/ML model and data	Continental Automotive

R1-2501079	Other Aspects of AI/ML framework	AT&T

R1-2501148	Other aspects of AI/ML model and data	Qualcomm Incorporated

R1-2501194	Discussion on other aspects of AI/ML model and data	NTT DOCOMO, INC.

## 4.2	RAN2

### 4.2.1	RAN2#129

36 contributions (for details see agenda items 8.1.4 and 8.1.5 in Tdoc list)

## 4.3	RAN4

#### 4.3.1	RAN4#114

// General aspects

R4-2500230	Discussion on General Aspects on Work Plan

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2501096	Discussion on general aspects for study on AI/ML

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501530	Discussion on general aspects on AIML for NR air interface phase 2

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501958	General aspects of AI/ML for NR air interface Phase 2

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

// Testing framework and procedure for one-sided models

R4-2500167	FR2 test setup for AI/ML beam management evaluation

Type: other		For: Approval
					Source: Anritsu Corporation

Abstract:

In this contribution, we share our findings regarding the associated discussion on AI/ML beam management evaluation among RAN1 and RAN4, and we provide our views on them.

Decision:		Noted.

R4-2500226	Testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500273	Testing framework and procedure for one-sided models

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2500403	Testability for AI/ML beam management

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2500617	Discussion on test framework and procedure for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2500825	Discussion on testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2500889	Discussion on testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2501038	Discussion on testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501097	Discussion on testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501531	Discussion on testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501959	Testing framework and procedure for one-sided models

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2501965	Discussion on testing framework and procedure for AI-BM

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2502174	On Test Systems and Testability Aspects for FR2 AI/ML Beam

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

// Testing issues for CSI compression two-sided models

R4-2500227	Study on CSI Compression: Testability and Interoperability issues

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500271	Testing issues for CSI compression two-sided models

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2500339	Discussion on Testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2500826	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2501039	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501098	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501497	On 2-sided compression study and simulations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Presents simulation results and some further considerations for 2-sided CSI

Decision:		Noted.

R4-2501509	Discussion on testability and interoperability issues for CSI compression

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2501532	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501960	Testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2501966	Study on testing issues for CSI compression two-sided models

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2502169	CSI Compression: Testability and Interoperability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Moderator summary and conclusions

R4-2500684	Topic summary for [114][132] NR_AIML_air_part2

Type: other		For: Information
					Source: Moderator(Ericsson)

Abstract:

Summary for AI 7.19.1, 7.20, 7.20.1, 7.20.3

Decision:		Noted.

Newly allocated tdocs in the first round

R4-2503011	Ad hoc minutes on AI/ML topic thread [132]

Type: other		For: Approval
					Source: Ericsson

Decision:		Noted.

R4-2502953	WF on AI/ML air interface study

Type: other		For: Approval
					Source: Ericsson

Decision:		Approved.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of the first round on Wednesday:

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_114/Inbox/Drafts/%5B114%5D%5B100%5D%20Main%20Session/3.Wednesday/2.%5B132%5D_R4-2500684%20Moderator%20summary%20for%20%5B114%5D%5B132%5D%20NR_AIML_air_part2_v03.docx

Please refer to the hyperlink below for the detailed minutes of the first round on Friday:

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template