3GPP TSG RAN Meeting #107	RP-250335

Incheon, Korea, March 12 – 14, 2025

Agenda item:	15.2.1

Source:	SEU

Title:	View on Rel-20 AI/ML for air interface for 5G-Advanced

Document for:	Discussion and Decision

1	Introduction

Rel-20 marks the final specification release of 5G prior to the commencement of normative work on 6G. It is undeniable that AI/ML stands as a pivotal component and enabling technology within the 6G air interface. Given that performance monitoring remains an unresolved aspect of LCM, we introduce a confidence-based performance monitoring approach tailored for UE-side predictions. This method leverages uncertainty estimation to assign a confidence value to the predicted CSI, enabling the UE to determine the viability of using this predicted CSI.

2	Discussion

The following potential specification aspects for performance monitoring are identified in Rel-18 SI:

In CSI prediction using UE-sided model use case: Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

During the RAN1#116bis and RAN1#117 meetings, the following agreements have been achieved :

Agreement (RAN1#116bis) For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects. - To clarify the boundary between type 1 and type 3 - To clarify definition of monitoring output and performance metric

Agreement (RAN1#117) For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured.

Observation (RAN1#117) For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

For Type 1 UE-side monitoring, the development of an effective monitoring mechanism, performance metric, and threshold criterion remains an ongoing challenge. To address this, we propose a confidence-based performance monitoring method. The framework of our proposed method is depicted in Figure 1 and comprises two key components: a confidence estimation network and a determination network. During the monitoring process, the UE first inputs historical CSI samples into the uncertainty estimation network, which computes a confidence value based on these samples. Subsequently, the determination network utilizes this confidence value to determine the next course of action for the UE, by comparing it against a predefined confidence threshold τ.

Specifically, we consider uncertainty estimation to provide the confidence value. Uncertainty estimation has proven to be a valuable technique for evaluating the reliability of prediction outcomes, providing a metric that indicates the AI model's confidence in its predictions. Our approach enhances the architecture of the CSI prediction network to quantify this uncertainty, incorporating technologies such as Monte Carlo (MC) dropout. MC dropout is a widely adopted method for uncertainty estimation that utilizes dropout as a regularization technique to evaluate predictive uncertainty. In this approach, dropout is applied not only during the training phase but also during inference. As a result, for the same input, the prediction model can produce multiple outputs due to the randomness introduced by dropout. The variability among these outputs serves as an indicator of uncertainty.

Figure 1 Confidence-based performance monitoring

Figure 2 Framework of MC-dropout based performance monitoring

The proposed uncertainty estimation network, illustrated in Figure 2, is an enhanced version of the original prediction model. It is designed to perform both uncertainty estimation and prediction, thereby eliminating the need for a separate uncertainty estimation network. This network produces a prediction outcome along with an uncertainty value.

We have incorporated two dropout layers, each with a dropout rate of 0.3, into the network. During inference, the model is run multiple times (n times) for each input, generating n distinct outcomes. These outcomes are then averaged to produce the final output, which improves both prediction accuracy and reliability. Subsequently, we calculate the SGCS values between each individual outcome and the averaged final result. The average of these SGCS values, which represents the dispersion among the outcomes, serves as an uncertainty metric that indicates the model's confidence in its prediction.

The simulation results are presented in Table 1. To generate our datasets, we use the QuaDRiGa simulator. In the simulation setup, the UE moves along a straight-line trajectory at a constant speed of 36 km/h, remaining within a 100-meter radius of the BS. For training the network, we utilize the UMi_NLOS channel model, while testing is conducted using the UMa_NLOS model to highlight the challenges posed by out-of-distribution scenarios. The transmit and receive antenna arrays are configured with 32 and 4 antennas, respectively, and the carrier frequency is set at 2.1 GHz. We assume that the UE performs CSI feedback every 5 milliseconds. In our prediction network, we use three historical CSI samples to predict a future CSI sample. Based on our evaluation, the inference time n is set to 4, as the improvement in monitoring accuracy becomes insignificant when the number of outcomes exceeds 4. To evaluate the accuracy of performance monitoring, we utilize the true positive rate (TPR) and true negative rate (TNR) metrics, which provide insights into the ability of distinguishing between qualified and unqualified predictions.

Table 1 Evaluation on MC-dropout based performance monitoring

| SGCS Threshold | 0.6 | 0.65 | 0.7 | 0.75 | 0.8 |
| --- | --- | --- | --- | --- | --- |
| Confidence Thresholdτ | 0.915 | 0.933 | 0.946 | 0.957 | 0.968 |
| TPR | 82.8% | 82.0% | 82.3% | 82.3% | 82.4% |
| TNR | 80.4% | 80.2% | 80.6% | 80.8% | 80.8% |

The balance between TPR and TNR can be adjusted by modifying the confidence threshold τ. In our results, we set τ to achieve a balanced TPR and TNR. Our proposed method consistently attains TPR and TNR values within the range of 80% to 85%, demonstrating the effectiveness of the proposed approach.

- Observation 1 Confidence-based performance monitoring leveraging uncertainty estimation can achieve TPR and TNR values within the range of 80% to 85%.

- Confidence-based performance monitoring could be a viable option for implementing Type 1 UE-side monitoring.

3	Conclusions

In this contribution, we discussed our views on a confidence-based performance monitoring method for Type 1 UE-side monitoring.

- Confidence-based performance monitoring leveraging uncertainty estimation can achieve TPR and TNR values within the range of 80% to 85%.

Based on above observations, we proposed as follow:

- Confidence-based performance monitoring could be a viable option for implementing Type 1 UE-side monitoring.

References

- RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.