3GPP TSG-RAN Meeting #107			RP-250365

Incheon, Korea, March 12-14, 2025

Agenda item:	16

Title:	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

At the last RAN plenary meeting, the following guidance for RAN 107 meeting was provided by RAN chair [1]

- Aim to finalize the candidate set of TPRs - Initial discussion on the associated values of the identified TPRs - A corresponding summary of the values as an input for future progress - Possible LS to ITU-R as necessary, depending on the progress of the study

# 2	Discussions

## 2.1	Candidate items for minimum TPRs

Following is the guidance for candidate items for minimum TPRs (MTPRs) study provided by RAN chair [1].

- The ITU-focused study should be based on the Recommendation ITU-R M.2160 (not necessarily aligned) and cover the following aspects - Identified candidate set of minimum TPRs - Including the associated high-level assumptions, if any - Including legacy and new TPRs and the evaluation method (simulations, analysis or inspection) - Including whether or not to skip some TPRs for this study - Associated values of the identified TPRs - Including whether the values are quantitative or qualitative or a mix therein - Including whether or not some values need to be jointly considered

According to latest ITU-R WP 5D discussion, the following list of candidate items for MTPRs have been agreed.

- AI-related capabilities (Method: TBD)

- Area traffic capacity (Method: Quantitative)

- Average Spectral efficiency (Method: Quantitative)

- Peak spectral efficiency (Method: Quantitative)

- 5th percentile user Spectral efficiency (Method: Quantitative)

- Bandwidth (Method: Quantitative)

- Connection density (Method: Quantitative)

- Control plane latency  (Method: Quantitative)

- User plane latency  (Method: Quantitative)

- Energy Efficiency  (Method: Quantitative)

- Mobility (Method: Quantitative)

- Mobility interruption time (Method: Quantitative)

- Peak data rate (Method: Quantitative)

- User Experienced Data Rate (Method: Quantitative)

- Sensing-related capabilities (Method: Quantitative)

- Positioning (Method: Quantitative)

- Reliability (Method: Quantitative)

Proposal 1: RAN plenary should discuss the above candidate items that have been agreed by ITU-R WP 5D for MTPRs and associated values.

According to latest ITU-R WP 5D discussion, the discussion of following list of candidate items are still ongoing. Therefore, it is proposed to discuss whether to include the following list of candidate items for MTPRs.

Proposal 2: RAN plenary should discuss whether to include the following list of candidate items for MTPRs.

- Joint requirement on data rate, latency, reliability and capacity

- Coverage

- Resilience

- Interoperability

- Security

- Sustainability

Regarding associated values, based on [4] as well as operator commercial deployment experiences, we think it is difficult to significantly improve bandwidth, MIMO layer and modulation order compared with 5G IMT-2020 target values such as for peak spectral efficiency, peak data rate, etc. Therefore, it is proposed to take 5G IMT-2020 MTPR target values as baseline for the following items to study whether/how much improvement should be targeted for IMT-2030 MTPRs.

Proposal 3: Take 5G IMT-2020 MTPR target values as baseline for the following items to study whether/how much improvement should be targeted for IMT-2030 MTPRs.

- Area traffic capacity (Method: Quantitative)

- Average Spectral efficiency (Method: Quantitative)

- Peak spectral efficiency (Method: Quantitative)

- 5th percentile user Spectral efficiency (Method: Quantitative)

- Bandwidth (Method: Quantitative)

- Connection density (Method: Quantitative)

- Control plane latency (Method: Quantitative)

- User plane latency (Method: Quantitative)

- Mobility (Method: Quantitative)

- Mobility interruption time (Method: Quantitative)

- Peak data rate (Method: Quantitative)

- User Experienced Data Rate (Method: Quantitative)

- Reliability (Method: Quantitative)

For positioning, we think the evaluation can be conducted based on [5] as follows. In [6], horizontal accuracy, vertical accuracy and latency have been considered for positioning evaluation. Therefore, it is proposed that at least positioning scenario, horizontal accuracy, vertical accuracy and latency should be taken into account for MTPR on positioning.

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

Proposal 4: For MTPR on positioning, at least scenario, horizontal accuracy, vertical accuracy and latency should be taken into account.

Additionally, for scenarios, the following scenarios could also be taken into account for positioning evaluation [6].

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Proposal 5: For MTPR on positioning, the following scenarios should be taken into account.

      - Scenario 1. Indoor Office for FR1 and FR2 (Open office)

      - Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

      - Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Regarding accuracy and latency, the following requirements should be taken into account for MTPR as baseline[6].

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

-	End to end latency < 1s

Proposal 6: For positioning accuracy and latency related MTPRs, the following requirements should be taken into account.

      - Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

      - Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

      - Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

      - Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

      - End to end latency < 1s

For energy efficiency, according to the latest ITU-R WP 5D discussion, energy efficiency can be categorised into network energy efficiency and Device energy efficiency. Also, the test environments, selected load case, the reference case and relative power saving gain should be taken into account for the evaluation of energy efficiency related MTPRs.

Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

Table

The minimum requirements for Network Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| TBD | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [X1%, X2%, ...] |

*empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100

** needed only in case of simulation

Table

The minimum requirements for Device Energy efficiency

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| TBD | [empty load, low load, light load and/or medium load] * | Fully loaded IMT-2030 case * | [A1%, A2%, …] |

Proposal 7: For energy efficiency, it can be categorized into network energy efficiency and Device energy efficiency. Also, the test environments, selected load case, the reference case and relative power saving gain should be taken into account for the evaluation of energy efficiency related MTPRs.

In [4] subclause 5.8, sleep ratio is used for energy efficiency evaluation. Similarly, it is proposed to use sleep ratio for 6G energy efficiency TPR.

The sleep ratio is the fraction of unoccupied time resources (for the network) or sleeping time (for the device) in a period of time corresponding to the cycle of the control signaling (for the network) or the cycle of discontinuous reception (for the device) when no user data transfer takes place. The sleep duration is the continuous period of time with no transmission (for network and device) and reception (for the device).

Table 5.8.1.1.1-1 NR network sleep ratio in slot level

| SSB configuration |  | SSB set periodicity PSSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | Number of SS/PBCH block per SSB set, L | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| 15kHz | 1 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |
|  | 2 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |
| 30kHz | 1 | 95.00% | 97.50% | 98.75% | 99.38% | 99.69% | 99.84% |
|  | 4 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |
| 120kHz | 8 | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% | 99.69% |
|  | 16 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |
| 240kHz | 16 | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% | 99.69% |
|  | 32 | 80.00% | 90.00% | 95.00% | 97.50% | 98.75% | 99.38% |

Table 5.8.1.1.1-2 NR network sleep ratio in symbol level

| SSB configuration |  | SSB set periodicity PSSB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [kHz] | Number of SS/PBCH block per SSB set, L | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| 15kHz | 1 | 93.57% | 96.43% | 97.86% | 98.93% | 99.46% | 99.73% |
|  | 2 | 87.14% | 92.86% | 95.71% | 97.86% | 98.93% | 99.46% |
| 30kHz | 1 | 96.79% | 98.21% | 98.93% | 99.46% | 99.73% | 99.87% |
|  | 4 | 87.14% | 92.86% | 95.71% | 97.86% | 98.93% | 99.46% |
| 120kHz | 8 | 94.29% | 97.14% | 98.57% | 99.29% | 99.64% | 99.82% |
|  | 16 | 88.57% | 94.29% | 97.14% | 98.57% | 99.29% | 99.64% |
| 240kHz | 16 | 94.29% | 97.14% | 98.57% | 99.29% | 99.64% | 99.82% |
|  | 32 | 88.57% | 94.29% | 97.14% | 98.57% | 99.29% | 99.64% |

Proposal 8: Sleep ratio at network side and device side should be taken into account for energy efficiency related MTPRs.

For sensing related capabilities and AI related capabilities, since there are ongoing discussions in ITU-R, it is possible to wait for further progress (e.g. definition and evaluation method) at ITU-R side.

# 3	Conclusion

In this contribution, the following proposals are provided.

Proposal 1: RAN plenary should discuss the following candidate items that have been agreed by ITU-R WP 5D for MTPRs and associated values.

- AI-related capabilities (Method: TBD)

- Area traffic capacity (Method: Quantitative)

- Average Spectral efficiency (Method: Quantitative)

- Peak spectral efficiency (Method: Quantitative)

- 5th percentile user Spectral efficiency (Method: Quantitative)

- Bandwidth (Method: Quantitative)

- Connection density (Method: Quantitative)

- Control plane latency (Method: Quantitative)

- User plane latency (Method: Quantitative)

- Energy Efficiency (Method: Quantitative)

- Mobility (Method: Quantitative)

- Mobility interruption time (Method: Quantitative)

- Peak data rate (Method: Quantitative)

- User Experienced Data Rate (Method: Quantitative)

- Sensing-related capabilities (Method: Quantitative)

- Positioning (Method: Quantitative)

- Reliability (Method: Quantitative)

Proposal 2: RAN plenary should discuss whether to include the following list of candidate items for MTPRs.

- Joint requirement on data rate, latency, reliability and capacity

- Coverage

- Resilience

- Interoperability

- Security

- Sustainability

Proposal 3: Take 5G IMT-2020 MTPR target value as baseline for the following items to study whether/how much improvement should be targeted for IMT-2030 MTPRs.

- Area traffic capacity (Method: Quantitative)

- Average Spectral efficiency (Method: Quantitative)

- Peak spectral efficiency (Method: Quantitative)

- 5th percentile user Spectral efficiency (Method: Quantitative)

- Bandwidth (Method: Quantitative)

- Connection density (Method: Quantitative)

- Control plane latency (Method: Quantitative)

- User plane latency (Method: Quantitative)

- Mobility (Method: Quantitative)

- Mobility interruption time (Method: Quantitative)

- Peak data rate (Method: Quantitative)

- User Experienced Data Rate (Method: Quantitative)

- Reliability (Method: Quantitative)

Proposal 4: For MTPR on positioning, at least scenario, horizontal accuracy, vertical accuracy and latency should be taken into account.

Proposal 5: For MTPR on positioning, the following scenarios should be taken into account.

      - Scenario 1. Indoor Office for FR1 and FR2 (Open office)

      - Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

      - Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

Proposal 6: For positioning accuracy and latency related MTPRs, the following requirements should be taken into account.

      - Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

      - Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

      - Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

      - Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

      - End to end latency < 1s

Proposal 7: For energy efficiency, it can be categorized into network energy efficiency and Device energy efficiency. Also, the test environments, selected load case, the reference case and relative power saving gain should be taken into account for the evaluation of energy efficiency related MTPRs.

Proposal 8: Sleep ratio at network side and device side should be taken into account for energy efficiency related MTPRs.

# 4	References

[1] RP-243276, RAN Chair’s Guidance for 6G study ITU focused, RAN chair

[2] RP-243202, Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R Working Party 5D

[3] Recommendation ITU-R M.2160, Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU-R

[4] TR 37.910

[5] TS 22.104

[6] TR 38.855

# 5	Annex (Summary of candidate items for minimum TPRs listed by Recommendation ITU-R M.2160.)

### 5.2.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the user peak data rate Rp is given by:

Rp = W × SEp	(1)

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi 	(2)

where Wi and SEpi (i = 1,…Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.2	User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(3)

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target values for the user experienced data rate are as follows in the TBD test environment(s):

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.3	Spectral efficiency

5.2.3.1	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

5.2.3.2	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to equation (4).

(4)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency are as follows:

–	Downlink average spectral efficiency is TBD bit/s/Hz.

–	Uplink average spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

5.2.3.3	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to equation (5).

(5)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency are as follows:

–	Downlink 5th percentile user spectral efficiency is TBD bit/s/Hz.

–	Uplink 5th percentile user spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.4	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6).

Carea = ρ × W × SEavg	(6)

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target value for Area traffic capacity in downlink is TBD Mbit/s/m2 in the TBD test environment.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.5	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a limited bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is TBD devices per km2.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.6	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

[TBD]

Table 1 defines the mobility classes that shall be supported in the respective test environments.

TABLE 1

Mobility classes

|  | Test environments for TBD |  |  |
| --- | --- | --- | --- |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 2. This assumes the user is moving at the maximum speed in that mobility class in each of the test environments.

This requirement is defined for the purpose of evaluation in the Immersive communication and Massive Communication usage scenarios.

According to latest ITU discussion, the target value should be quantitative.

TABLE 2

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |

5.2.6.1	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signaling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the TBD usage scenarios.

The minimum requirement for mobility interruption time is TBD ms.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.7	Latency

5.2.7.1	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	TBD ms for TBD

assuming unloaded conditions (i.e. a single user) for TBD IP packets), for both downlink and uplink.

According to latest ITU discussion, the target value should be quantitative.

5.2.7.2	Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is TBD ms. Proponents are encouraged to consider lower control plane latency, e.g. TBD ms.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.8	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a TBD data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is TBD success probability of transmitting a layer 2 PDU (protocol data unit) of TBD bytes within TBD ms in channel quality of coverage edge for the Hyper Reliable and Low Latency Communication test environment, assuming TBD application data .

According to latest ITU discussion, the target value should be quantitative.

### 5.2.9	Joint requirement on data rate, latency, reliability and capacity

The joint performance requirement relates to the capability of transmitting a given amount of traffic within a predetermined latency with high success probability. In addition, the capacity is defined as the number of users or connections per cell that can be satisfied with the fulfilled joint requirement of data rate, latency and reliability.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The joint minimum requirement for various test environments is summarized in Table 3.

TABLE 3

Joint requirement on data rate, latency, reliability and capacity

| Test environment | Data rate |  | Latency |  | Reliability |  | Number of satisfied users |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |

### 5.2.10	Coverage

Coverage refers to the ability to provide access to communication services for users in a desired service area. Coverage is defined as the cell edge distance of a single cell.

The RIT/SRIT shall have a function to provide a larger cell coverage in specific area than that supported by a ground IMT base station using the IMT-2020 technologies. This function may include, but not limited to, inherent functionalities within the RIT/SRIT and utilization of higher altitude applications (e.g. UAS and HIBS).

### 5.2.11	Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.12	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is TBD.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.13	Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, mapping, and composite sensing tasks such as tracking, SLAM, etc.

A sensing task is a processing function ISAC-capable system, which is performed to achieve a certain sensing goal with respect to one or more sensing target(s), based on radio frequency measurements. Performance of a sensing task is characterized by at least one minimum requirement.

According to latest ITU discussion, the target value should be quantitative.

5.2.13.1	Detectability (Detection / False alarm probability)

Detection probability, PD, is the probability of correctly detecting the object status (e.g., presence, category, and state) within sensing range, and False alarm probability, PFA, is the associated probability that an object status is detected given that the object is not present within sensing range.

5.2.13.2	Localization accuracy

Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value within sensing range and is the difference between the calculated location and the actual location of an object.

5.2.13.3	Velocity accuracy

Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target within sensing range.

5.2.13.4	Sensing resolution

Sensing resolution is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects.

### 5.2.14	AI-related capabilities

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc.

### 5.2.15	Sustainability / Energy efficiency

Sustainability, or more specifically environmental sustainability, refers to the ability of both the network and devices to minimize greenhouse gas emissions and other environmental impacts throughout their life cycle.

Energy efficiency is a quantifiable metric of sustainability. It refers to the quantity of information bits transmitted or received, per unit of energy consumption (in bit/Joule). Energy efficiency is expected to be improved appropriately with the capacity increase in order to minimize overall power consumption.

Network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics.

Energy efficiency of the network and the device can relate to the support for the following two aspects:

- Efficient data transmission in a loaded case;

- Low energy consumption when there is no data, namely, in an un-loaded case.

Efficient data transmission in a loaded case is demonstrated by the average spectral efficiency.

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

A quantitative requirement on Energy efficiency can be defined in the following

- A quantitative evaluation to demonstrate the level of sleep ratio and long sleep duration supported by the RIT (Analysis; With TPR limits on sleep ratio and sleep duration)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Analysis without user data traffic; With TPR limits on relative energy savings)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Simulation with user data traffic; With TPR limits on relative energy savings)

According to latest ITU discussion, the target value of energy efficiency should be quantitative.

### 5.2.16	Security and resilience

Security refers to preservation of confidentiality, integrity, and availability of information, such as user data and signaling, and protection of networks, devices and systems against cyberattacks such as hacking, distributed denial of service, man in the middle attacks, etc.

Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc.  The RIT/SRIT shall have functionalities to continue operations in emergency situations, either within itself or with the assistance of specific reliable applications with minimal ground network infrastructure (e.g. UAS, HIBS and shipboard base stations), etc.

### 5.2.17	Interoperability

Interoperability refers to the radio interface being based on member-inclusivity and transparency, so as to enable functionality(ies) between different entities of the system.