3GPP TSG RAN Meeting #107			RP-250381

Incheon, Korea, March 12-14, 2025

Agenda item:		15.2.3

Source:	China Telecom

Title:	Views on Rel-20 Ambient IoT

Document for:		Discussion

Introduction

The ambient IoT (Internet of Things) work item in Rel-19 will be completed by 3GPP RAN plenary #109 meeting, and a Rel-20 ambient IoT topic will be followed immediately. Based on the way forward [1], Device 2 and D2T2 will be included in the Rel-20 A-IoT WI scope, and other considerations are also necessary to be discussed as follows:

Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase.

In this contribution, we will provide our views on the detailed Rel-20 scope for ambient IoT based on the RAN plenary meeting guidance.

Views on the ambient IoT work plan

Since the indoor scenario deployment is urgent for the market application such as factory inventory, the Rel-19 work item focuses on the indoor scenario and DT/DO-DTT traffic type, studies their transmission performance and potential channel design, and completes the normative work for only D1T1 with the device 1 scenario due to the limitation of the workload and TU. Based on the guidance in the last RAN plenary meeting [2], a WG-level work item is expected in Rel-20, with details depending on the progress of Rel-19, thus the remaining normative work based on the Rel-19 study item shall be continued in Rel-20 ambient IoT, such as D2T2 and device 2 related design. Moreover, new scenarios also draw wide attention to the current market customer. The potential application scenario will be based on sensing and position in outdoor deployment which cannot be satisfied by the current technology, such as the outdoor logistic tracking, environment monitoring etc. To meet the market demand, enhancement or a new design is needed based on current ambient IoT design such as the potential whole-network level synchronization procedure, and a higher capability device (i.e., device 3) with higher power consumption may also be needed to improve the outdoor coverage performance. Meanwhile, evaluation is necessary based on the coverage performance and coexistence condition to identify whether the potential outdoor deployment of ambient IoT is reasonable and applicable or not. Based on the above considerations, a study phase is needed in Rel-20 at least for outdoor scenario. In addition, since we are not sure of the progress and outcome of the study phase, a checkpoint can also be set to confirm whether a normative work phase is feasible to be converted.

Proposal 1: Support to start a work item for Rel-20 ambient IoT.

- Rel-19 leftovers if any could be specified in Rel-20 WI.

- A new study phase on outdoor scenario and the support of DO-A will be included in Rel-20 WI.

- A checkpoint can be set for the new study phase to confirm whether the normative work can be converted if needed.

Potential scope of Rel-20

General aspects of scenario and deployment

- Deployment

During the Rel-19 stage, RAN evaluates coverage performance of D1T1 and D2T2 deployment scenario and selects D1T1 deployment scenario for the work item scope due to its higher coverage performance and lower system design complexity in the signalling and network architecture aspects. In Rel-20, the left work of the Rel-19 study item shall be continued, such as D2T2-related design and further down-selection of architecture options for topology 2 as mentioned in the work plan. Moreover, to study the DO-A services, outdoor deployment D4 shall also be considered in the scope. Since only T1 and T2 are applied in the Rel-19 study item scope, similar topologies can also be selected for Rel-20 outdoor deployment, which means D4T1 and D4T2 will be considered for the Rel-20 outdoor deployment.

Figure 1 The proposed outdoor deployment scenario for Rel-20

- Carrier frequency

During the Rel-19 stage, FR1 licensed spectrum in FDD is proposed, and 900MHz is selected for the work item scope. When the deployment scenario changes to outdoor, co-site is expected for the ambient IoT network deployment to reduce deployment overhead, where 2GHz is a typical carrier frequency for the outdoor FDD spectrum utilized for LTE FDD deployment such as Band 1 and Band 3. Since a higher capability device will be used in the outdoor scenario and some potential coverage enhancement solutions studied in the Rel-19 stage, 2GHz coverage performance may be improved compared with current evaluation results and can be regarded as a candidate carrier frequency in the Rel-20 outdoor study. Therefore, both 900MHz and 2GHz can be studied for the outdoor scenario.

- CW characteristic

The one or multiple unmodulated single-tone waveform is utilized for CW waveform due to a simpler interference condition and better recoverability based on the CW characteristic study in the Rel-19, and the single unmodulated single-tone waveform is selected as the CW generation waveform in the work item stage for a simpler implementation. In the Rel-20, the remaining multiple unmodulated single-tone waveform can be utilized for other deployment scenarios for a better CW transmission distance. Moreover, considering higher capability devices such as device 2b or device 3 will be used in the outdoor scenario, the D2R transmission can be performed without the backscatter method, thus CW scenario C shall be included in the Rel-20 scope.

- Device type

Based on the TR 38.769 [3], both device 1 and device 2 are studied in the Rel-19 study item, and device 1 is selected in the work item due to a simpler implementation than device 2a. In the Rel-20, both device 1 and device 2a should be included in the work item scope to complete the remaining normative work on the D1T1 (with device 2a) and D2T2 scenario. Moreover, considering DO-A traffic type will also be studied in the Rel-20 stage, device 2b or a potential higher-capability device 3 may be introduced for a stronger coverage performance in the outdoor scenario. The potential device 3 may have a larger power consumption value that is up to several mw level, with the capability to support a coverage level of several hundred meters under a lower SFO or CFO level.

Proposal 2: Considering the following scope of scenario and deployment for Rel-20 ambient IoT WI:

Table 1 Potential Rel-20 scope for different characteristics

| Characteristics | TR 38.848 [4] in Rel-18 | TR 38.769 in Rel-19 | WI scope in Rel-19 | Proposed Rel-20 scope |
| --- | --- | --- | --- | --- |
| Represent use case | Inventory, sensors, positioning, command | Indoor inventory, Indoor command | Indoor inventory, Indoor command | Indoor inventory, Indoor command, Focus on Outdoor sensors and Outdoor positioning |
| Deployment and topology | D1, D2, D3, D4, D5; T1, T2, T3, T4 | D1T1, D2T2 | D1T1 | D1T1 (for device 2) D2T2, D4T1, D4T2 |
| Spectrum and carrier frequency | FDD, TDD, unlicensed | Licensed FDD, | Licensed FDD, 900MHz | Licensed FDD, 900MHz, 2GHz |
| CW generation method | - | Single unmodulated single-tone or multiple unmodulated single-tones | Single unmodulated single-tone | Single unmodulated single-tone or multiple unmodulated single-tones |
| CW transmission case | - | CW scenario A1, CW scenario A2, CW scenario B, CW scenario C | CW scenario B | CW scenario B, CW scenario C |
| Device characteristic | Device A, Device B, Device C | Device 1 and Device 2 | Device 1 | Device 1(for D2T2), Device 2, [Device 3] |
| Traffic model | DO-A, DO-DTT, DT | DT, DO-DTT | DT, DO-DTT | DO-A, DO-DTT, DT |

Views on the scope for Rel-20 Ambient IoT

In this section, we will provide our views on the potential scope related to different RAN working groups.

Evaluation assumptions and methodology for outdoor scenario

Evaluation assumptions and methodologies are made in the Rel-19 study item stage for indoor scenario, and some conclusions can be reused for Rel-20 outdoor study, such as the link-level budget template and latency evaluation model. However, the necessary modifications need to be discussed due to the difference between indoor and outdoor deployment scenario, such as the new applicable maximum distance target values for outdoor scenario, or the 2D distribution of devices based on outdoor scenario. Moreover, since the interference conditions are quite different between indoor and outdoor deployment scenarios, coexistence evaluation and analysis based on the current work in Rel-19 shall be discussed in RAN1 and RAN4 for the outdoor scenario.

RAN1-lead objectives

- Bandwidth and numerologies

System design parameters such as bandwidth size, sub-carrier space, and CP-related content shall be identified first in the Rel-20 work. Since D1T1 and D2T2 are designed only for indoor deployment scenario, system parameters in D1T1 don’t have obvious differences and can be reused for D2T2 scenario, such as bandwidth or the CP handling method. However, considering a high-capability device will be introduced for the outdoor scenario and DO-A traffic type, a larger bandwidth size and a potential new CP adding/handling method may need to be considered.

- Physical channel signal design

The rough PRDCH or PDRCH generation process including coding, modulation or waveform generation has been studied during the Rel-19 SI stage and can be reused for Rel-20 indoor transmission design. For the outdoor scenario, some processes can be reused with a harmonized design principle, especially for the PRDCH which needs to guarantee compatibility and minimize the modifications. In the PDRCH channel, considering active transmission is supported in the outdoor scenario, some necessary changes may be needed, such as the modulation method or generated waveform.

- Synchronization signal design

The whole-network synchronization is impossible for device 1 and device 2a due to a high SFO level and backscatter communication method, thus asynchronous system is designed in the Rel-19, whose synchronization process is implemented based on the synchronization signal, i.e., preamble, midmable, and/or postamble. In Rel-20, similar design can be reused in the indoor scenario, but whole-network synchronization can be possible with considering high-capability and non-backscatter devices introduced in the outdoor scenario whose SFO level may decrease by several orders of magnitude. Therefore, the new synchronization process and synchronization signals for outdoor scenario shall be studied in the Rel-20 scope.

- Scheduling and timing relationship aspects

The scheduling information carried out in PRDCH and PDRCH is discussed in the Rel-19 SI stage, which contains higher layer and L1 control information and includes TDRA, MCS-like, and other indications. Since the device availability and unavailability scheduling scheme is not considered in the Rel-19 WI stage, only device 1 is used and the potential scheduling scheme is not considered in the WI scope to simplify the system design. In the Rel-20 indoor scenario, the corresponding availability and unavailability scheme can be further discussed, thus the potential scheduling information for device availability and unavailability may need to be added, such as wake-up or charging indication. Moreover, considering that whole-network synchronization may be used in the outdoor scenario, the synchronization process may differ from the current design, and some new synchronization-related scheduling information may also be needed.

In addition, the timing relationship parameter may also differ from the current design due to the difference between indoor and outdoor scenario. For example, a time interval between two consecutive PDRCH may be needed to define separately since PDRCH can be transmitted actively to the reader side, and the time interval range may also differ from the current value due to a potentially shorter processing time of device 2b or device 3 compared with device 1 or device 2a.

RAN2-led objectives

For R20 A-IoT WI, the R19 SI and WI on ambient IoT shall be considered as baseline. For outdoor scenario, the general procedure of random access and paging, the design of MAC layer shall be reused as much as possible. Further enhancement could be discussed in working group meetings if necessary. For the Topology 2 specific aspects, RAN2 focuses on the architecture options and management of radio resources in the ambient IoT radio interface in R19 SI. Forwarding of ambient IoT upper layer data and ambient IoT radio resource management should be specified, and any additional impacts from other WGs will also be discussed in the R20 WI. In addition, the support of DO-A could be considered in R20. Potential enhancement on support of DO-A, including paging, random access, data transmission could be discussed and specified in R20 if needed.

RAN3-led objectives

Due to the limited remaining time in Rel-19, the work item only focused on RAN architecture and network signalling design for Topology 1. From the perspective of RAN3, Rel-20 ambient IoT normative works is based on the assumption of Rel-19 A-IoT system design framework, and it is recommended to further study and specify enhancements to support Topology 2 deployment scenario.

- Figure 2 Logical system architecture for topology 2

Support Topology 2 requirement to specify architecture and procedures to conduct ambient IoT operations with intermediate UE, we can follow the conclusions already made in TR 38.769, and further down-selection on solutions (i.e., RRC-based, UP-based) for conveying ambient IoT upper layer information may be needed.

For ambient IoT Device Locating, only "Reader ID granularity" positioning accuracy is studied in Rel-19 foreseen. Finer granular techniques for locating the A-IoT device will be expected to support indoor and outdoor deployment scenarios according to TR 38.848.

In addition, remaining issues for Rel-19 can also be considered for Rel-20, depending on Rel-19 ambient IoT WI progress. There are mainly two parts of work in Rel-20 ambient IoT, including Rel-19 remaining normative work and Rel-20 new study work. Hence, we have the following proposal.

Proposal 3: Considering the following scope for Rel-20 ambient IoT work item:

- Rel-19 remaining normative work.

Reuse Rel-19 design as much as possible and specify the necessary enhancements based on TR 38.848 and TR 38.769

- Specify the procedure on support of device availability and unavailability via direction 2, including monitoring, wake up indication and other necessary procedures.

- Specify the procedure on proximity determination via solution 2 if needed.

- Specify the necessary architectural aspects, protocol, signaling and procedures to support A-IoT operations with intermediate UE for Topology 2, including

  - Forwarding of A-IoT upper layer data and A-IoT radio resource management related signalling procedure between the UE reader and the gNB

  - Signalling support for UE Reader authorization

- Note: Further down-selection on solutions (i.e., RRC-based, UP-based) for conveying A-IoT upper layer information may be needed.

- Rel-20 new study work.

Study and specify the necessary and feasible solutions for DO-A and outdoor scenario.

Evaluation assumptions:

- Reuse the evaluation assumptions and methodology for coverage and coexistence in TR 38.769 with minimized and necessary modification

- Define the ambient IoT device architecture for the higher-capability device within the low-power and low-complexity requirements if needed.

  - a few mW peak power consumption.

RAN1-led:

- Study and specify the parameter and value for the bandwidth, numerologies, and other aspects of outdoor scenario (if needed).

- Study and specify the design of PRDCH and PDRCH channels for outdoor scenario

  - Frame structure, synchronization and timing, random access

  - Waveforms and modulation

  - Coding

  - Multiple access, scheduling and timing relationship

RAN2/3-led:

- Study and specify potential enhancement on support of DO-A and outdoor scenario, including the procedure of paging, random access, data transmission and the MAC layer design, CN-RAN interface and RAN architecture if needed.

- Study and specify RAN-CN signaling to support other A-IoT use cases (e.g., outdoor sensors, outdoor positioning)

- Study and specify finer granular techniques for locating the A-IoT device if needed.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: Support to start a work item for Rel-20 ambient IoT.

- Rel-19 leftovers if any could be specified in Rel-20 WI.

- A new study phase on outdoor scenario and the support of DO-A will be included in Rel-20 WI.

- A checkpoint can be set for the new study phase to confirm whether the normative work can be converted if needed.

Proposal 2: Considering the following scope of scenario and deployment for Rel-20 ambient IoT WI:

Table 1 Potential Rel-20 scope for different characteristics

| Characteristics | TR 38.848 [4] in Rel-18 | TR 38.769 in Rel-19 | WI scope in Rel-19 | Proposed Rel-20 scope |
| --- | --- | --- | --- | --- |
| Represent use case | Inventory, sensors, positioning, command | Indoor inventory, Indoor command | Indoor inventory, Indoor command | Indoor inventory, Indoor command, Focus on Outdoor sensors and Outdoor positioning |
| Deployment and topology | D1, D2, D3, D4, D5; T1, T2, T3, T4 | D1T1, D2T2 | D1T1 | D1T1 (for device 2) D2T2, D4T1, D4T2 |
| Spectrum and carrier frequency | FDD, TDD, unlicensed | Licensed FDD, | Licensed FDD, 900MHz | Licensed FDD, 900MHz, 2GHz |
| CW generation method | - | Single unmodulated single-tone or multiple unmodulated single-tones | Single unmodulated single-tone | Single unmodulated single-tone or multiple unmodulated single-tones |
| CW transmission case | - | CW scenario A1, CW scenario A2, CW scenario B, CW scenario C | CW scenario B | CW scenario B, CW scenario C |
| Device characteristic | Device A, Device B, Device C | Device 1 and Device 2 | Device 1 | Device 1(for D2T2), Device 2, [Device 3] |
| Traffic model | DO-A, DO-DTT, DT | DT, DO-DTT | DT, DO-DTT | DO-A, DO-DTT, DT |

Proposal 3: Considering the following scope for Rel-20 ambient IoT work item:

- Rel-19 remaining normative work.

Reuse Rel-19 design as much as possible and specify the necessary enhancements based on TR 38.848 and TR 38.769

- Specify the procedure on support of device availability and unavailability via direction 2, including monitoring, wake up indication and other necessary procedures.

- Specify the procedure on proximity determination via solution 2 if needed.

- Specify the necessary architectural aspects, protocol, signaling and procedures to support A-IoT operations with intermediate UE for Topology 2, including

  - Forwarding of A-IoT upper layer data and A-IoT radio resource management related signalling procedure between the UE reader and the gNB

  - Signalling support for UE Reader authorization

- Note: Further down-selection on solutions (i.e., RRC-based, UP-based) for conveying A-IoT upper layer information may be needed.

- Rel-20 new study work.

Study and specify the necessary and feasible solutions for DO-A and outdoor scenario.

Evaluation assumptions:

- Reuse the evaluation assumptions and methodology for coverage and coexistence in TR 38.769 with minimized and necessary modification

- Define the ambient IoT device architecture for the higher-capability device within the low-power and low-complexity requirements if needed.

  - a few mW peak power consumption.

RAN1-led:

- Study and specify the parameter and value for the bandwidth, numerologies, and other aspects of outdoor scenario (if needed).

- Study and specify the design of PRDCH and PDRCH channels for outdoor scenario

  - Frame structure, synchronization and timing, random access

  - Waveforms and modulation

  - Coding

  - Multiple access, scheduling and timing relationship

RAN2/3-led:

- Study and specify potential enhancement on support of DO-A and outdoor scenario, including the procedure of paging, random access, data transmission and the MAC layer design, CN-RAN interface and RAN architecture if needed.

- Study and specify RAN-CN signaling to support other A-IoT use cases (e.g., outdoor sensors, outdoor positioning)

Study and specify finer granular techniques for locating the A-IoT device if needed.

- References

- RP-243280, Way forward for ambient IoT for Rel-19 and Rel-20, 3GPP TSG RAN Plenary meeting #106, Madrid, Spain, December 9-12, 2024.

- RP-243292, RAN chair’s summary of 5G-Advanced in Rel-20, 3GPP TSG RAN Plenary meeting #106, Madrid, Spain, December 9-12, 2024.

- TR 38.769, Study on solutions for ambient IoT (Internet of Things) (Release 19) v2.0.0.

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18) v18.0.0.