3GPP TSG RAN Meeting #107			RP-250388

Incheon, Korea, March 12-14, 2025

Agenda item:		16

Source:	China Telecom

Title:	Views on IMT-2030 Technical Performance Requirements (TPRs)

Document for:		Discussion

Introduction

In the last 3GPP TSG RAN meeting #106, a new SID on 6G scenarios and requirements has been approved [1]. It is planned to revise SID to add a part 2 that goes beyond the part 1 which is already there and investigates IMT-2030 requirements of Recommendation ITU-R M.2160. The objective of SID is described as follows.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - [To fill in additional objectives, on general 6G RAN study, in March’2025]

In the last meeting, 3GPP RAN Plenary received an LS from ITU-R WP5D providing the inputs on the minimum technical performance requirements for IMT-2030 radio interface technologies [2]. The motivation for the development of IMT-2030 is to continue to build an inclusive information society to achieve innovation, sustainability, interworking and other important goals for 6G era. The usage scenarios of IMT-2030 are envisaged to expand on those of IMT-2020 into broader use requiring evolved and new capabilities. In addition, IMT-2030 is envisaged to enable new usage scenarios, such as artificial intelligence and sensing. There are fifteen capabilities defined in IMT-2030 based on the traditional capabilities in IMT-2020 and new requirements for emerging fields.

Based on the discussion in the last 3GPP TSG RAN meeting #106, a draft reply to ITU-R WP5D on IMT-2030 TPR has been made [3]. The candidate IMT-2030 TPRs under discussion in ITU-R WP5D has been listed as follows [4]. Additional candidates and definitions will be further discussed and included, with considering commercial needs for 6G expanded new scenarios and capabilities. In addition, the responding evaluation methodology and assumptions will of course need discussion in 3GPP RAN Plenary.

1.	Peak data rate

2.	Peak spectral efficiency

3.	User experienced data rate

4.	5th percentile user spectral efficiency

5.	Average spectral efficiency

6.	Sustainability/Energy efficiency

7.	Area traffic capacity

8.	User plane latency

9.	Control plane latency

10.	Connection density

11.	Reliability

12.	Mobility

13.	Mobility interruption time

14.	Bandwidth

Furthermore, 3GPP adopted the study plan based on the 6G study timeline in 3GPP and co-collaboration with ITU-R WP5D [3]. It started in RAN#106 meeting (December 2024) for the initial discussion on the possible TPRs. In this RAN#107 meeting (March 2025), it aims to finalize the candidate set of TPRs and initial discussion on the associated values of the identified TPRs. In RAN#108 meeting (June 2025), it will finalize the associated values of the identified TPRs. The remaining issues can be further continued to discuss after RAN#108 meeting, if needed. Based on the above plan, we will focus on providing our views on IMT-2030 Technical Performance Requirements (TPRs) in this contribution.

Views on the candidate set of IMT-2030 TPRs and associated target values

In addition to the existing candidate set of TPRs included in ITU-R WP5D, we would like to propose other essential items for IMT-2030 TPRs with considering new requirements for emerging fields. The proposed candidate items and preliminary target values for IMT-2030 TPRs are listed as follows. The targets for IMT-2030 should be determined with reasonable values to satisfy the 6G real commercial and deployment requirements.

Table 1: The proposed candidate items and preliminary target values for IMT-2030 TPRs

| Index | Items | Proposed target values for IMT-2030 | Note |
| --- | --- | --- | --- |
| 1 | Peak data rate | DL: 100Gbps, UL: 50bps | 5 x IMT-2020 |
| 2 | Peak spectrum efficiency | DL: 45~90bps/Hz, UL: 22.5~45bps/Hz | 1.5~3 x IMT-2020 |
| 3 | User experienced data rate | DL: 300Mbps, UL: 150Mbps | 3 x IMT-2020 |
| 4 | 5% percentile user spectrum efficiency | Indoor Hotspot: 0.45~0.9 (DL) /0.315~0.63 (UL) Dense Urban: 0.3375~0.675 (DL) /0.225~0.45 (UL) Rural: 0.18~0.36 (DL) /0.0675~0.135 (UL) | 1.5~3 x IMT-2020 |
| 5 | Average spectrum efficiency | Indoor Hotspot: 13.5~27 (DL) /10.125~20.25 (UL) Dense Urban: 11.7~23.4 (DL)/8.1~16.2 (UL) Rural: 4.95~9.9 (DL)/2.4~4.8 (UL) | 1.5~3 x IMT-2020 |
| 6 | Sustainability/Energy efficiency | TBD | 3~5x IMT-2020 |
| 7 | Area traffic capacity | 100 Mbit/s/m2 | 10 x IMT-2020 |
| 8 | User plane latency | HRLLC: 0.1~0.5ms |  |
| 9 | Control plane latency | 10ms | 1/2 x IMT-2020 |
| 10 | Connection density | 107~108 devices/km2 | 10~100 x IMT-2020 |
| 11 | Reliability | 10-6~10-7 | 10~100 x IMT-2020 |
| 12 | Mobility | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicle: 10 km/h to 120 km/h High speed vehicle: 120 km/h to 700 km/h Ultra speed aerial vehicle: 700 km/h to 1000 km/h. SINR: TBD |  |
| 13 | Mobility interruption time | 0ms |  |
| 14 | Bandwidth | 200MHz/400MHz TBD |  |
| 15 | Coverage | Coverage distance derived from MPL, the detailed values corresponding specific scenarios are TBD |  |
| 16 | Positioning | 1-10cm |  |
| 17 | Sensing-related: 17.1 False alarm probability/Missed detection probability 17.2 Localization accuracy 17.3 Velocity accuracy 17.4 Sensing resolution 17.5 Maximum sensing service latency | TBD |  |
| 18 | AI-related: 18.1 AI service requirements 18.2 AI service accuracy 18.3 AI service latency | TBD |  |

The candidate items for IMT-2030 TPRs are listed in the above table. The detailed definitions and target values can be further discussed. Generally, we think the reasonable TPRs are essential for 6G study and subsequent 6G commercial operation. For legacy TPRs in IMT-2020, e.g., data rate, we think the target values should be pragmatic to meet only 6G day-1 requirements first. Too high values need to be avoided to reduce system design complexity and multiple technologies for 6G day-1. Then additional higher targets can be set to satisfy 6G day-2 features. Hence, we have the following proposal.

Proposal 1: Adopt the candidate items and preliminary target values in Table 1 for IMT-2030 TPRs. The remaining values and definitions can be further study.

Consideration on the potential evaluation for sensing-related capabilities

For new TPRs, AI-related capabilities and sensing-related capabilities are very close to use cases and implementation. It may be hard to have quantitative targets or unified evaluation criteria. We prefer to identify essential scenarios first and use necessary inspection for evaluation if needed.

As the low-altitude economy is rapidly developing and is becoming an emerging industry, many cities in China have planned several low altitude logistics routes for trial. Sensing-related capabilities refer to the ability to provide functionalities in the radio interface including range/velocity/angle estimation, object detection, localization, imaging, mapping, etc., which are essential for 6G scenarios and technologies. We propose five kinds of candidate items for sensing, including False alarm probability/Missed detection probability, Localization accuracy, Velocity accuracy, Sensing resolution and Maximum sensing service latency. The sensing resolution refers to the minimum difference between any two distinguishable target objects in the measured aspects of range, velocity and angle. However, the target values for each candidate item are closely related to test environments. Hence, it would be reasonable to first determine the typical test environments for sensing. The detailed assumptions for each test environment can be further discussed. Based on the speed and type of sensing target objects, there are three kinds of test environments proposed, i.e., ISAC-UAV, ISAC-Automotive and ISAC-Human. The descriptions are as follows.

- Test environments

ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicle, e.g., smart vehicles, UGVs.

ISAC_Human: A test environment targeting at sensing the human motions or target objects which are stationary or low speed.

Hence, we have the following proposal.

Proposal 2: Adopt the following test environments for sensing scenario. The detailed assumptions can be further discussed.

- ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

- ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicles, e.g., smart vehicles, UGVs.

- ISAC_Human: A test environment targeting at sensing human motions or target objects which are stationary or low speed.

- Evaluation methodology

Regarding the preliminary consideration on evaluation methodology for sensing scenarios, we think it would be reasonable to reuse 5G-A ISAC as much as possible, especially the achieved progress on ISAC channel model and assumptions for identified typical scenarios. No need to do the duplicated work for 6G sensing evaluation. In our understanding, sensing should be evaluated under ISAC test environments using system-level simulation or system-level simulation followed by link-level simulation. Relevant sensing performance metrics and identified TPRs can be acquired from sensing signal processing and evaluation. Hence, we have the following proposal.

Proposal 3: Reuse 5G-A ISAC channel model and assumptions as much as possible, with adopting system-level simulation or system-level simulation followed by link-level simulation under ISAC test environments.

Conclusions

In this contribution, we have the following proposals:

Proposal 1: Adopt the candidate items and preliminary target values in Table 1 for IMT-2030 TPRs. The remaining values and definitions can be further study.

Proposal 2: Adopt the following test environments for sensing scenario. The detailed assumptions can be further discussed.

- ISAC_UAV: A test environment targeting at sensing the medium or high speed vehicle, e.g., UAVs.

- ISAC_Automotive: A test environment targeting at sensing the low and medium speed vehicles, e.g., smart vehicles, UGVs.

- ISAC_Human: A test environment targeting at sensing human motions or target objects which are stationary or low speed.

Proposal 3: Reuse 5G-A ISAC channel model and assumptions as much as possible, with adopting system-level simulation or system-level simulation followed by link-level simulation under ISAC test environments.

- References

- RP-243327, New SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- RP-243202, Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG RAN Meeting#106, Madrid, Spain, December 9-12, 2024.

- RP-243325, DRAFT LTI Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to 5D/413 Report on the 47th meeting of Working Party 5D.