3GPP TSG RAN Meeting #107								RP-250436

Incheon, Korea, March 12-14, 2025

Status Report to TSG

Agenda item:			9.3.4.2

| WI / SI Name | # NR base station (BS) RF requirement evolution for FR1/FR2 and testing |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_BS_RF_req_evo |  |  |  |  |
| Unique ID | 1030078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243270 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2025 | Performance part: 03/2026 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 50% | Performance Part: 30% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Xue, fei |
|  | Company | ZTE Corporation |
|  | Email | Xue.fei25@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #114:

Core and conformance testing for CLTA

Sub-topic 1-1: BS-to-BS isolation analysis

Way forward:

BS-to-BS isolation data collection to prioritize the following scenarios:

- Horizontal domain prioritized (vertical BS stacking analyses not precluded);

- Same-band analysis (inter-band analysis not precluded),

- Array-to-array case (array to sub-array not precluded);

- Same BS type for aggressor and victim (mixed cases of various BS types not precluded).

Guidance for collection of future results: Companies to clarify on the statistical distribution of the results provided (i.e., worst case, CDF, etc.)

For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements.

BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats:

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (M, R4-2501875): -54 | ZTE (R4-2501875): -52 | ZTE (M, R4-2501875): -56 |  |
| Isolation in horizontal domain (dB) | ZTE (M, R4-2501875): -31 | ZTE (M, R4-2501875): -34 Huawei (M, R4-2502135): -46 | ZTE (M, R4-2501875): -46 |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz --> 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | S: Ericsson (R4-2418398): A-to-SA: -55±20 dB |  | S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20 |

For the deprioritized scenarios, reuse table templates as above.

- Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116).

- Data collection for other carrier frequencies is not precluded.

- Any future conclusion on isolation numbers is subject to availability of sufficient amount of data from multiple companies and can be decided case-by-case.

Sub-topic 1-2: Specifications set baseline

Way forward: Baseline set of specifications for implementation of the BS-BS isolation conclusions:

- TS 37.105

- TS 37.145-1/-2

- TS 38.104

- TS 38.141-1/-2

- TS 38.174

- TS 38.176-1/-2

- TS 38.106

- TS 38.115-1/-2

Sub-topic 1-3: Requirements set baseline

Way forward: Baseline set of requirements for implementation of the BS-BS isolation conclusions:

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- Tx IMD requirement

- Co-located OOBB requirement

Sub-topic 1-4: TX IMD co-location

Way forward:

Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements.

Sub-topic 1-5: Capturing CLTA-related agreements

Way forward:

As there is a dedicated CLTA section in TR 37.941 on RF conformance testing background for OTA BS requirements, consider TR 37.941 as the placeholder for the CLTA-related agreements collection. Rel-19 findings to be ca in a brand new sub-clause.

Sub-topic 1-6: Horn antenna aspects

Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available).

Sub-topic 1-7: Assumptions for the beam steering scenario analyses

Way forward: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration:

Data collection to prioritize the following scenarios:

- Beam steering range +/- [60] degrees

- Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.

For U6GHz BS EEIRP mask (the work has been basically completed):

Conformance testing:

Issue 2-2: CAR and mechanical tilt related issue

- Agreement:

  - Agree to use the test beam as proposed in D.3a for the conformance testing

Issue 2-4: Test beams for EEIRP mask

- Agreement:

  - Agree the following testing beam direction pattern 5x5 without 4 corner beams for the EEIRP conformance testing

Issue 2-5: Conformance testing for different mechanical tilts

- Agreement:

  - The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

  - If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

        - Minimum mechanical down tilt and the reduced electrical steering range

        - Maximum electrical steering range and increased mechanical down tilt

  - To capture in the note of the declaration table

Issue 2-6: Measurement uncertainty

- Agreement:

  - Agree to capture the Plane Wave Synthesizer (PWS) for EEIRP conformance testing

  - For dynamic range error, propose to use Standard uncertainty 0.3dB  as staring point.

  - For EEIRP sampling error, agree to use the following summarized error information to do the statistical analysis for the distribution and std based on the agreed test beam;

| Company A |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant configuration 1, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
| Company B |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant configuration 1, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |

| Company A |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant configuration 1, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
| Company B |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant configuration 1, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |

  - For C1-1 Uncertainty of the RF power measurement equipment for spectrum analyzer at band n104, further discuss it from Rel-17 band n104.

Issue 2-7: CI

- Recommended WF:

  - To capture the CI calculation in the TR.

  - For CI of agreed testing beam pattern, agree to use the  following summarized error information to do the statistical analysis for the distribution and Confidence interval based on the agreed test beam;

    - Where error in the following table is defined as  EEIRP per bin of agreed test beam pattern minus EEIRP per bin of reference testing beam (e.g. rectangular reference or ellipse reference)

    - Step 1: to do the statistical analysis of  the distribution of collected errors for the inputs based on different antenna configuration and steering range according to the agreed test beam pattern;

    - Step 2: to calculate the standard deviation of collected errors;

    - Step 3: to calculate the confidence interval based on the standard deviation in the step 2 and distribution of the errors in step 1.

| Company A |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant configuration 1, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
| Company B |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant configuration 1, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for Rect Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for Rect Ref |  |  |  |  |  |  |  |

| Company A |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ant configuration 1, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
| Company B |  | Bin index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Ant configuration 1, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 1, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 1 | Error for ellipse Ref |  |  |  |  |  |  |  |
|  | Ant configuration 2, beam steering rang 2 | Error for ellipse Ref |  |  |  |  |  |  |  |

Conformance testing for OTA test scope reduction:

Issue 1-1-1: test scope reduction for Tx requirement

- Agreement:

  - Focus on test scope reduction for Tx IMD requirement

  - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements;

    - 1ST interference freq offset for ACLR;

    - 3rd interference freq offset for spurious emission;

    - TBD interference freq offset for OBUE requirement

Issue 1-1-2: Reduction of  test configuration for Tx requirement

- Agreement:

  - Proposal 1: for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

  - FFS on the maximum output power for the conformance testing.

Issue 1-2-1: Out of band blocking

- Agreement:

  - To increase the step size for OOBB conformance testing for FR1 BS

  - FFS on the step size for OOBB conformance testing

Conformance testing for BS TRP Annex simplification:

Issue 2-1: TRP test methods

- Agreement:

  - To keep the current TRP testing method, TRP sampling grid and testing chamber in the specification

  - Further discuss how to improve the test procedure and the applicability of testing method for different requirements in Annex N of TS 38.141-2.

2.4.2	Remaining Open issues

Core part:  Open issues for co-location reference antenna

Sub-topic 1-1: BS-to-BS isolation analysis

- Continue the BS-to-BS isolation data collection

Sub-topic 1-4: TX IMD co-location

Way forward:

- Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements.

Sub-topic 1-6: Horn antenna aspects

- Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available).

Per part:  open issues for U6GHz EIRP mask

Issue 2-6: Measurement uncertainty

- To finalize the MU value

Issue 2-7: CI

- To finalize the CI based on collected inputs from companies.

Perf part: Open issues for the reduction of testing scope

test scope reduction for Tx requirement

  - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements;

    - TBD interference freq offset for OBUE requirement

Reduction of  test configuration for Tx requirement

- Agreement:

  - for AAS TS 37.145-2, only ANTCR7 is needed for C and NC capable BS with identical parameters, so ATCR7 can be removed from the tests.

  - FFS on the maximum output power for the conformance testing.

Out of band blocking

  - FFS on the step size for OOBB conformance testing

Open issues for the reduction of simplification BS TRP test methods for FR1 and FR2

  - Further discuss how to improve the test procedure and the applicability of testing method for different requirements in Annex N of TS 38.141-2.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#114

- R4-2500074	Simplification of BS TRP test methods	Nokia

- R4-2500199	Draft CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR	Nokia

- R4-2500202	On BS OTA test scope reduction	Nokia

- R4-2500203	Conformance testing of Expected EIRP mask requirement	Nokia

- R4-2500510	Inclusion of Plane Wave Synthesizer (PWS) method for Expected EIRP test procedures	ROHDE & SCHWARZ

- R4-2500548	Plane Wave Synthesizer (PWS) method inclusion for Expected EIRP test procedures	ROHDE & SCHWARZ

- R4-2500563	Topic summary for [114][302] NR_BS_RF_Part1_E_EIRP	Moderator (ZTE)

- R4-2500564	Topic summary for [114][303] NR_BS_RF_Part2_CLTA	Moderator (Huawei)

- R4-2500565	Topic summary for [114][304] NR_BS_RF_Part3_OTA_TRP	Moderator (ZTE)

- R4-2500881	General aspects related to BS RF evolution WI	Ericsson

- R4-2501033	Discussion on BS RF OTA test scope	CATT

- R4-2501165	TP to TR 38.908 clause 6.2 The calculation for EEIRP mask	Nokia

- R4-2501166	TP to TR 38.908 clause 6.3 Test beams	Nokia

- R4-2501190	Draft CR to TS 38.141-2 with updates to Annex N for Expected EIRP calculation	Nokia

- R4-2501191	Draft CR to TS 38.141-2 with updates to OTA spatial emission requirement test procedure	Nokia

- R4-2501323	On OTA test enhancements	Nokia

- R4-2501647	TS 38.141-2 Annex I: Discussion on improvement of TRP measurement methods	Ericsson

- R4-2501648	TP for TR 38.908: Conformance test aspects for EEIRP	Ericsson

- R4-2501649	TP for TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104	Ericsson

- R4-2501850	Further discussion on Expected EIRP test	Huawei, HiSilicon

- R4-2501851	Expected EIRP conformance declarations	Huawei, HiSilicon

- R4-2501852	TP to TR 38.908 – section 6.5	Huawei, HiSilicon

- R4-2501853	TP to TR 38.908 – section 6.6	Huawei, HiSilicon

- R4-2501854	OTA test case reduction	Huawei, HiSilicon

- R4-2501874	TR 38.908 v1.3.0 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz	ZTE Corporation, Sanechips

- R4-2501875	Further discussion on OTA test enhancement	ZTE Corporation, Sanechips

- R4-2501876	Discussion on conformance testing for U6GHz BS EEIRP mask	ZTE Corporation, Sanechips

- R4-2501877	Draft CR for introduction of U6GHz EIRP conformance testing	ZTE Corporation, Sanechips

- R4-2501878	Discussion on the OTA testing scope reduction	ZTE Corporation, Sanechips

- R4-2501879	Discussion on the applicability improvement of TRP measurement	ZTE Corporation, Sanechips

- R4-2501927	Big draft CR for introduction of U6GHz EIRP conformance testing	ZTE Corporation, Sanechips

- R4-2502135	Analysis of BS-to-BS coupling values	Huawei, HiSilicon

- R4-2502159	OTA test scope reduction	Ericsson

- R4-2502160	Simulation of BS-to-BS isolation for edge-to-edge co-location isolation at 7 GHz	Ericsson

- R4-2502279	Way Forward for [114][303] NR_BS_RF_Part2_CLTA	Huawei

- R4-2502304	Way Forward for [114][304] NR_BS_RF_Part3_OTA_TRP	ZTE

- R4-2502308	Ad-hoc meeting minutes for [114][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2502309	Way Forward for [114][302] NR_BS_RF_Part1_E_EIRP	ZTE

- R4-2502316	TP to TR 38.908 clause 6.2 The calculation for EEIRP mask	Nokia

- R4-2502317	Draft CR to TS 38.141-2 with updates to Annex N for Expected EIRP calculation	Nokia

- R4-2502318	TP for TR 38.908: Conformance test aspects for EEIRP	Ericsson

- R4-2502319	TP for TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104	Ericsson

- R4-2502320	TP to TR 38.908 – section 6.6	Huawei, HiSilicon

- R4-2502321	Draft CR for introduction of U6GHz EIRP conformance testing	ZTE Corporation, Sanechips

- R4-2502375	Way Forward for [114][304] NR_BS_RF_Part3_OTA_TRP	ZTE

- R4-2502379	TP for TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104	Ericsson