3GPP TSG-RAN Meeting 107	RP-250453

Incheon, South-Korea, March 12-14, 2025

Agenda Item:	15.2.4

Source:	Huawei, HiSilicon

Title:	Discussion for AI/ML based mobility

Document for:	Discussion

1	Introduction

In the last TSG-RAN meeting 106, the majority of the companies showed their interests in AI/ML based mobility work in Rel-20 and various motivations and scope proposals were mentioned. Meanwhile, RAN2 is still discussing the AI mobility in Rel-19 via SI project and some initial conclusions can be drawn from this study. In this paper, we provide our opinion on AI/ML based mobility scope for Rel-20.

2	Discussion

In the past meetings of Rel-19, RAN2 agreed on simulations methodology based on which companies performed simulations work to verify RRM measurement and mobility event prediction accuracies, as well as impact of using predicted measurements/events on mobility performance. RAN2 has already drawn conclusions that in many configurations and scenarios the prediction using AIML models is accurate and that thanks to AIML based mobility the gains such as reduction of measurement effort and overhead or HO performance improvement can be achieved. What is still to be discussed by RAN2 is a potential impact on the LCM procedures which will be based on the LCM framework developed in AIML for air-interface WI.

Observation 1: RAN2 simulations show that AIML allows to achieve good RRM measurement and mobility event prediction accuracy which translates into gains in terms of reduction of measurement effort and overhead and HO performance improvement.

Based on this, we think it is justified to assume that AIML based mobility work should continue in a WI phase in Rel-20.

Proposal 1: AI/ML based mobility work should continue as a WI in Rel-20.

In the following sub-clauses, we provide our consideration about the potential WI scopes.

2.1 Analysis for scenarios selection

In the Rel-19 SI, the following three use cases are included [1]:

- Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

  - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

- HO failure/RLF prediction (UE sided model) [RAN2]

- Measurement events prediction (UE sided model) [RAN2]

According to the latest RAN2 process, the cell-level prediction and measurement event prediction are discussed, based on the analysis of several rounds of simulation results.

For the RRM prediction, there are 6 scenarios proposed at first as the following table shows.

| scenario number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Among these scenarios, scenario 2/3/4 are agreed as high priority. Most of the simulations provided by the companies are also about these three scenarios. More specifically, scenarios 2/3 and scenario 4 respectively present the AI advantage in measurement cost reduction and HO performance improvement. Meanwhile, RAN2 #129 also agreed that temporal and frequency domain prediction will be considered with higher priority for detailed study, which means scenario 6 is arranged with lower priority. Therefore, we think Rel-20 WI should continue to focus on these three scenarios.

Another important issue is about the choice of RRM prediction sub-use cases. In Rel-19, there are the following three use cases, describing the method to obtain predicted measurement results.

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In RAN2 meetings, companies have provided results for all the three use cases. Most of the provided results used sub-use case 1 or sub-use case 2, and their accuracy is comparable. Considering the input dimension, sub-use case 2 has the minimum calculation complexities. Besides, it is close to the legacy L3 HO procedures, where the cell-level results are used to trigger mobility events and used by the gNB to make HO decisions. On the other hand, the beam-level prediction is helpful for the access beam selection in the target cell, which can further improve the resource allocation efficiency during HO procedures. Thus, we think in Rel-20 WI stage, both sub-case 1 and sub-case 2 should be considered.

Proposal 2: For RRM measurement prediction, Rel-20 WI should focus on:

- Scenarios 2/3/4, i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency (frequency domain), and FR2 to FR2 intra-frequency temporal domain case A.

- sub-use case 1 (i.e. L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated) and sub-use case 2 (i.e. Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)).

2.2 Analysis for LCM framework

Until now, there was no discussion on LCM for AI/ML based mobility, and RAN2 plans to discuss LCM in the final three Rel-19 RAN2 meetings. So far, RAN2 has agreed that the LCM framework for R19 AI for PHY should be re-used as much as possible. Considering the HO procedures, we also think that the existing measurement configuration/reporting procedures should be used as the baseline for LCM discussions. The exact scope of the work which needs to be done for LCM depends on further RAN2 discussions, but there will certainly be some impact to measurement/mobility framework as well as to LCM procedures developed for AIML based air interface.

Therefore, Rel-20 should focus normative work on a general LCM framework and corresponding procedures for both UE-sided model and NW-sided model.

Proposal 3: Rel-20 to focus normative work on a general LCM framework and corresponding procedures for both UE-sided model and NW-sided model, with Rel-19 AI for PHY and existing measurement procedures as baselines.

2.3 RAN4 work scope

In the SID objectives of AI/ML mobility in Rel-19, RAN4 was asked to analyse the following items

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

  - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed

  - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

  - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In Rel-20, we think RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI mobility. The followings could be reference, and the details may be further adjusted based on Rel-19 progress.

- Relation to legacy requirements

- Performance monitoring and LCM aspects considering use-case specifics

- Generalization aspects

- UE processing capability and limitations

- Post-deployment validation due to model change/drift

Proposal 4: In Rel-20, RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI mobility.

2.4 Scope of the Work Item

Beside RRM measurement prediction and Measurement events prediction, which have been studied in Rel-19 with simulations and evaluations, there could be other aspects that AIML may also help, but these aspects may need further studies and evaluations, including but not limited to:

- HO failure/RLF prediction

- Conditional HO;

- LTM

- Conditional LTM,

- etc.

We think the Work item should focus on normative work for those use cases that have been studied in Rel-19.

Proposal 5: Rel-20 Work Item should focus on normal work for use cases that have been studied in Rel-19.

3. Conclusions

In this paper, we provide our further considerations for AI/ML based mobility study in Rel-20 and give the following observation and proposals:

Observation 1: RAN2 simulations show that AIML allows to achieve good RRM measurement and mobility event prediction accuracy which translates into gains in terms of reduction of measurement effort and overhead and HO performance improvement.

Observation 2: Beam level prediction results from Rel-18 AIML for PHY and Rel-19 from AIML for mobility work performed by RAN1 and RAN2 confirm the feasibility of using AIML models to predict L1 RSRP on beam level.

Proposal 1: AI/ML based mobility work should continue as a WI in Rel-20.

Proposal 2: For RRM measurement prediction, Rel-20 WI should focus on:

- Scenarios 2/3/4, i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency (frequency domain), and FR2 to FR2 intra-frequency temporal domain case A.

- sub-use case 1 (i.e. L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated) and sub-use case 2 (i.e. Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)).

Proposal 3: Rel-20 to focus normative work on a general LCM framework and corresponding procedures for both UE-sided model and NW-sided model, with Rel-19 AI for PHY and existing measurement procedures as baselines.

Proposal 4: In Rel-20, RAN4 should finalize the testing framework and procedures for the typical one-sided models in AI mobility.

Proposal 5: Rel-20 Work Item should focus on normal work for use cases that have been studied in Rel-19.

References

RP-242939 Revised SID on AIML for mobility in NR-cl.