3GPP TSG-RAN Meeting #107		RP-250509

Incheon, Korea, March 12-14, 2025

Source:	CATT

Title:	Consideration on NR NTN in 5G-A Rel-20

Agenda Item:	15.2.8.4

Document for:	Discussion and Decision

# Introduction

NR NTN and IoT-NTN have been specified in 3GPP RAN Rel-17, since then been optimized in Rel-18 and Rel-19. Commercial deployment is ongoing at this moment. Based on real deployment or deployment plans, further evolution of NR NTN and IoT-NTN are needed.

In this contribution, we provide our views on NR NTN evolution in 5G-A Rel-20 with some potential enhancements.

# Discussion

## Leftovers from previous releases

### 2.1.1 Msg-5 repetition for UL coverage enhancement

In Rel-18 NTN, UL enhancement has been specified for PUSCH and PUCCH at the initial access. However, there is one missing channel, Msg-5 PUSCH, which may have coverage problem. The size of Msg-5 content may be larger than that of Msg-3, while the Msg-3 has been enhanced. For handheld UE, UL transmission power is quite limited, hence, all UL transmission channels should be improved to keep good coverage.

As another alternative, Msg-5 channel enhancement may be one common requirement for TN and NTN, in that sense, this issue can be discussed jointly with UL coverage enhancement for TN.

Proposal 1: Consider UL Msg-5 transmission enhancement for NTN in Rel-20.

### 2.1.2 Wide beam and narrow beam association

In Rel-19 NTN, there was one discussion point for the application of wide beam and narrow beam. Technically, wide beam has smaller beamforming gain compared to narrow beam, but due to larger coverage area, wide beam is suitable for system information broadcasting and mobility control, while narrow beam is suitable for dedicated data channel transmission. gNB can apply different beam types to support signal transmission of common channel and dedicated channel separately. Another use case is that DL beam is wide but the UL beam is narrow, and the underlying reason is that UL transmission power or UL RX beam gain of wide beam is limited. It may cause UL coverage enhancement issue.

In both cases, the narrow beam index associated of UE should be indicated to gNB. After then, gNB can apply suitable narrow beam to perform DL transmission and UL reception.

Figure 1： Wide beam and narrow association in NTN

Proposal 2: Consider wide beam and narrow beam association to improve beam coverage for NTN scenario in Rel-20.

## New enhancements for NR NTN

### 2.2.1 UE centric positioning to support GNSS relaxation

In 5G NR NTN, UE is assumed with good GNSS position information and is able to conduct timing and frequency compensation to access the network. However, evidence has been shown that GNSS jamming (leading to denial of service) and spoofing (leading to incorrect location reporting and potentially denial of service) may occur on a temporary basis. Also, a terminal or group of terminals attempting to connect over access channels or once connected to the network could also cause denial or degradation of service for all other users in the cell if using an altered GNSS position.

Therefore, there is a need to increase the robustness of uplink time and frequency synchronization in NR NTN based access against of GNSS temporary unavailability. One simple method is to consider network assisted UE positioning to help UE to get its position. For example, network can indicate emperies information to help UE to get satellite location, and UE can measure the reference signal or other DL signals from satellite to determine its location. With network assistance, using legacy DL-TDOA technique can help UE to estimate its location.

For UE centric positioning, UE can derive its location information based on network assisted information and no need to send measurement report to gNB, consequently, the standardization efforts are quite limited.

Proposal 3: Consider UE centric positioning to support GNSS relaxation based on network assisted information to enable initial access and UE connection.

### 2.2.2 Signaling enhancements to support E-UTRA TN to NR NTN handover

In Rel-19, the E-UTRA TN to NR NTN mobility WI (RP-240924) included inter-RAT IDLE mode mobility enhancement only. On including the support of inter-RAT CONNECTED mode mobility in this WI, discussions have been carried out in RAN#103 ~ RAN#105, summarized as follows:

- In RAN#103: a number of Sat. and TN operators first raised interests and proposals to include also support of inter-RAT connected mode mobility, when the WI scope was discussed.

- Nonetheless, what necessary Spec impact is needed was unclear to companies (See discussion on RP-240061)

- In RAN#104: there was operator’s proposal (RP-241324), but not treated due to no expectation on WI expansion.

- In RAN#105: the proposal to support multiple SMTCs for connected mode mobility was proposed by a number of companies (including Sat. and TN operators) but was concluded as not pursued (See RP-241954 and RP-241756)

Observation 1: There have been a large number of operator interests to support inter-RAT E-UTRA TN to NR NTN mobility in CONNECTED mode in Release 19.

Observation 2: It is not clear till what necessary RAN2 Spec impacts are needed to support this inter-RAT connected mode mobility feature (other than multiple SMTCs)

From RAN2 perspective, the minimum set of necessary Spec impact to enable this inter-RAT connected mode mobility in NR Uu is justified as follows. In current TS 36.331, measurement configurations for inter-RAT mobility to NR are specified separately from E-UTRAN (i.e. measObjectNR/ReportConfigInterRAT). Based on our analyses, current E-UTRA specifications may not be able to support the “E-UTRA TN to NR NTN” handover, due to the lack of (at least) following necessary Satellite assistance information in inter-RAT measurement configuration:

- DL polarization information

This information is needed to avoid 3dB loss for measurements due to antenna polarization mismatch. It corresponds to NR NTN parameter ntn-PolarizationDL in TS 38.331, and agreed to be introduced for IDLE mode mobility

- Satellite Ephemeris information

This information is needed for the UE to do frequency compensation when performing DL measurement (which is the case for both IoT NTN and NR NTN). Note that this is Irrelevant to location-based CHO specified for NR NTN in TS 38.331

Also Note that we have no intention to support multiple SMTC for this inter-RAT E-UTRA TN to NR NTN handover.

Proposal 4a: Support E-UTRA TN to NR NTN handover in Rel-20 5G-A.

Proposal 4b: As potential RAN2 objective, E-UTRAN is enabled to provide satellite assistance information (e.g. polarization information, Ephemeris data, etc.) into NR-specific measurement configurations.

### 2.2.3 RRM requirements for various "TN-NTN" and "NTN-TN" mobility

From RAN4 perspective, the RRM requirements for mobility only defined for the following scenarios in current TS 38.133:

- NTN to NTN:

NR NTN to NR NTN mobility for IDLE / INACTIVE/CONNECTED mode

- NTN to TN:

NR NTN to NR TN mobility only for IDLE / INACTIVE mode

NR NTN to E-UTRAN TN mobility only for IDLE / INACTIVE mode

- TN to NTN：

NR TN to NR NTN mobility only for IDLE / INACTIVE mode

According to the above background, it is pointed that only the scenario of "E-UTRAN TN to NR NTN" was missed, including IDLE / INACTIVE/CONNECTED mode requirements. And the requirements for CONNECTED mode in all existing "TN to NTN" and "NTN to TN" scenarios supported by RAN4 were missed.

In addition, from the perspective of operator demand and deployment, considering the same operator will also have a demand for Space-Terrestrial Integrated Networks and it will deploy both TN and NTN networks at the same time, it is necessary to define the missed mobility requirements. Therefore, in terms of the completeness of mobility scenarios, the mobility requirements for missing scenarios need to be added.

Observation 3: Only the scenario of "E-UTRAN TN to NR NTN" was missed, including IDLE / INACTIVE/CONNECTED mode requirements.

Observation 4: The requirements for CONNECTED mode in all other existing "TN to NTN" and "NTN to TN" scenarios supported by RAN4 were missed.

Based on the above discussion, we propose to support E-UTRA TN to NR NTN handover in Rel-20 5G-A with following proposals.

Proposal 5a: As potential RAN4 objective, RAN4 need to define RRM requirements for mobility for the scenario of "E-UTRAN TN to NR NTN", including IDLE / INACTIVE/CONNECTED mode.

Proposal 5b: As potential RAN4 objective, RAN4 need to define RRM requirements for CONNECTED mode in all other existing "TN to NTN" and "NTN to TN" scenarios supported by RAN4.

# Conclusion

According to the discussions above, it’s proposed to consider the following potential enhancements for NR NTN in 5G-A Rel-20:

- Support UL Msg-5 transmission enhancement (can be jointly considered with UL CE in TN)

- Specify Wide beam and narrow beam association to improve the coverage

- Enable UE centric positioning to support GNSS relaxation

- Support E-UTRA TN to NR NTN handover

For the reference, the observations and proposals are listed as follows:

Proposal 1: Consider UL Msg-5 transmission enhancement for NTN in Rel-20.

Proposal 2: Consider wide beam and narrow beam association to improve beam coverage for NTN scenario in Rel-20.

Proposal 3: Consider UE centric positioning to support GNSS relaxation based on network assisted information to enable initial access and UE connection.

Proposal 4a: Support E-UTRA TN to NR NTN handover in Rel-20 5G-A.

Proposal 4b: As potential RAN2 objective, E-UTRAN is enabled to provide satellite assistance information (e.g. polarization information, Ephemeris data, etc.) into NR-specific measurement configurations.

Proposal 5a: As potential RAN4 objective, RAN4 need to define RRM requirements for mobility for the scenario of "E-UTRAN TN to NR NTN", including IDLE / INACTIVE/CONNECTED mode.

Proposal 5b: As potential RAN4 objective, RAN4 need to define RRM requirements for CONNECTED mode in all other existing "TN to NTN" and "NTN to TN" scenarios supported by RAN4.

Observation 1: There have been a large number of operator interests to support inter-RAT E-UTRA TN to NR NTN mobility in CONNECTED mode in Release 19.

Observation 2: It is not clear till what necessary RAN2 Spec impacts are needed to support this inter-RAT connected mode mobility feature (other than multiple SMTCs)

Observation 3: Only the scenario of "E-UTRAN TN to NR NTN" was missed, including IDLE / INACTIVE/CONNECTED mode requirements.

Observation 4: The requirements for CONNECTED mode in all other existing "TN to NTN" and "NTN to TN" scenarios supported by RAN4 were missed.

# Appendix TU Estimation

In the following we provide TU estimation based on the proposed scope.

| WG | Estimated TU |
| --- | --- |
| RAN1 | 1 |
| RAN2 | 0.75 |
| RAN3 | 0.5 |
| RAN4 | TBD |