3GPP TSG-RAN Meeting #107                                              	RP-250520

Incheon, Korea, March 12-14, 2025

Source:	CATT

Title:	Discussion on NR mobility enhancements Phase 4

Agenda Item:	9.3.2.1

Document for:	Discussion and Decision

Introduction

As per the WID [1], there is one checkpoint that should be reviewed at RAN#107 regarding the objectives of conditional LTM. This contribution discusses this checkpoint.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion

According to the WID [1], the work on conditional LTM has been limited in non-DC before RAN#107.

Considering that neither MCG LTM with SN change nor SCG LTM with MN change is supported, the intra-CU DC scenarios for conditional LTM mainly involve the following 2 cases:

- Scenario 1: conditional intra-CU MCG LTM without SN change

- Scenario 2: conditional intra-CU SCG LTM without MN involvement

Below, the two scenarios are analyzed respectively.

- Scenario 1: conditional intra-CU MCG LTM without SN change

According to current RAN2 progress, the conditional LTM and network based LTM can coexist, without candidate cell configuration specific to conditional LTM additionally introduced. That is, the LTM candidate configuration list configured for one UE can include either candidate configuration for network based LTM or candidate configuration for conditional LTM. It is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

Hence the following case could exist: Candidate cells 1/2/3 are configured for UE, and the cell 1 provides execution condition for cell 2, but cell 3 does not provide execution condition for cell 2. That is, the candidate cell 2 can function as a C-LTM candidate cell when the serving cell is cell 1, but cannot be a C-LTM candidate cell when the serving cell is cell 3. For network based MCG LTM, it supports intra-CU MCG LTM with SCG. Hence if the conditional intra-CU MCG LTM with SCG were not supported, it would mean the restriction that the candidate configuration for cell 2 only applies to non-DC case, which will then decrease the performance when the UE performs cell switch from cell 3 to cell 2 based on network command only.

Observation 1:  If conditional intra-CU MCG LTM without SN change were not supported, extra restrictions would be needed on the configuration of the candidate cell for C-LTM. This will further degrade the performance when UE performs cell switch to this candidate cell which can be only triggered based on network command.

The main differences between conditional intra-CU LTM and network based intra-CU LTM are mainly located in the early TA acquisition, execution condition configuration and trigger of the cell switch; these aspects have been discussed and resolved by RAN2 discussion so far, irrespective of whether SCG is configured or not. Since the procedure of cell switch execution for C-LTM is the same as that for network based LTM, the current design/conclusions made in RAN2 for intra-CU conditional LTM in non-DC case can be directly reused for conditional intra-CU MCG LTM with SCG, without need of additional work in RAN2. In contrast, if the conditional intra-CU MCG LTM with SCG is not supported, the restriction should be reflected in the spec.

Considering that both the MN and SN are unchanged before and after the cell switch procedure, the only difference compared with network based MCG LTM is on the moment when the MN informs SN the execution of cell switch, but this would not have impact on the internode message between MN and SN. Someone may have concerns on early data forwarding, but with the node unchanged, we do not think it is an essential issue and can be well handled by NW implementation. Hence we can assume extra workload due to support of Scenario 1 is not needed for RAN3 either.

Observation 2: To support conditional intra-CU MCG LTM without SN change, current design/conclusions in RAN2 and RAN3 made for conditional intra-CU LTM in non-DC case can be directly reused without extra workload/enhancements expected.

Also, we think conditional intra-CU MCG LTM without SN change, if supported, can share the same applicable scenarios with the network based MCG LTM in DC case (i.e. including both SCG changed and SCG unchanged cases).

Observation 3: Conditional intra-CU MCG LTM without SN change can share the same applicable scenarios with the network based MCG LTM in DC case (i.e. including both SCG changed and unchanged cases).

- Scenario 2: conditional intra-CU SCG LTM without MN involvement

To support conditional intra-CU SCG LTM without MN involvement, at least the following issues should be additionally discussed technically:

- whether current agreements made for intra-CU MCG LTM in non-DC case could be reused;

- the coexistence issue of intra-CU SCG LTM without MN involvement with other scenarios (e.g. intra-CU MCG LTM, inter-CU MCG LTM, inter-CU SCG LTM, etc.).

From Spec impact perspective, if intra-CU SCG LTM without MN involvement were to be supported, impacts on TS37.340 would be additionally needed on top of the Specs already with Cat.B running CRs.

Observation 4: To support conditional intra-CU SCG LTM without MN involvement, additional technical discussions are needed, and extra Spec will be impacted (e.g. TS 37.340).

Based on above discussions, we propose to support only conditional intra-CU MCG LTM without SN change in DC, and propose no extra workload/enhancements needed. The other DC scenarios are not supported in this Release.

Proposal 1: In Rel-19, only support the conditional intra-CU MCG LTM with SN unchanged in DC case. No extra case is supported.

Text Proposal to WID

Based on above proposals and observations, we draft a TP to WID [1] in the Annex. It is proposed that RAN plenary adopts the TP, if above proposals and observations are agreeable.

Proposal 2: Adopt the revised WI objectives in Annex A.

Conclusion

According to the discussions in Section 2 and 3, we have the following observations and proposals:

Observation 1:  If conditional intra-CU MCG LTM without SN change were not supported, extra restrictions would be needed on the configuration of the candidate cell for C-LTM. This will further degrade the performance when UE performs cell switch to this candidate cell which can be only triggered based on network command.

Observation 2: To support conditional intra-CU MCG LTM without SN change, current design/conclusions in RAN2 and RAN3 made for conditional intra-CU LTM in non-DC case can be directly reused without extra workload/enhancements expected.

Observation 3: Conditional intra-CU MCG LTM without SN change can share the same applicable scenarios with the network based MCG LTM in DC case (i.e. including both SCG changed and unchanged cases).

Observation 4: To support conditional intra-CU SCG LTM without MN involvement, additional technical discussions are needed, and extra Spec will be impacted (e.g. TS 37.340).

Proposal 1: In Rel-19, only support the conditional intra-CU MCG LTM with SN unchanged in DC case. No extra case is supported.

Proposal 2: Adopt the revised WI objectives in Annex A.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- Annex A

----------------------------------------------------------------------------------------------------------------------------------------

### 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1]

  - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

NOTE: 	RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario:

    - non-DC

    - MCG LTM without SN change in DC.

NOTE x: Conditional intra-CU MCG LTM without SN change shall reuse the design of conditional intra-CU LTM in non-DC case.

NOTE y: MCG LTM without SN change for conditional LTM share the same applicable scenarios as network based MCG LTM supported in Rel-18/19 (including both SCG changed and SCG unchanged).

- Specify RRM requirements related to the above objectives as necessary [RAN4]