3GPP TSG RAN Meeting #107	RP-250531

Incheon, South Korea, March 12-14, 2025

Agenda Item:	9.3.2.3

Source:	Futurewei

Title:	Check point and WID update proposal for R19 XR WI

Document for:	Discussion and Decision

# Introduction

The following check points were set for RAN#107 in the latest R19 XR WID [1]:

- Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#107 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#107 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q2/25 onwards.

In this contribution, we discuss some adjustments to the R19 XR WID that are needed in accordance with the progress made in the WGs.

# Discussion

In SA2#167, CR to TS 23.501 has been approved by SA2 to support multi-modality services with the following text [2]:

If the AF provides a Multi-modal Service ID for data flows to 5GS, then the PCF may include it, based on operator configuration, in PCC rules, and the SMF forwards it to NG-RAN in N2 SM information when establishing and/or updating the corresponding QoS Flows.

NOTE X:	Based on an SLA between the network operator and an application service provider, the Multi-modal Service ID can uniquely identify a specific multi-modal communication service instance. The related 5GS behavior is not further specified in this specification and is assumed to be determined by the operator configuration related to the SLA.

In addition to the features that are provided for the case that the data flows are associated with a single UE, the following features are provided for the case where the data flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service should be selected by each of the involved UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF should use the same Multi-modal Service ID in the interactions with the PCF(s) for all the involved UEs that relate to a multi-modal service. The PCF may take this information into account (e.g. to apply a specific QoS policy) when processing each AF request independently. The data flows contribute to the service experience, but are still valid stand-alone, as they are transmitted over separate PDU Sessions to/from the involved UEs.

As highlighted above, the AF-provided MMSID appears advantageous, in two aspects, over the UE-provided MMSID (via UAI), the latter of which has been discussed in RAN2 but not fully specified yet. First, the AF-provided MMSID is uniquely assigned based on SLA between the network operator and the application service provider. Therefore, it may provide more information about the multi-modal service to the network provider, for optimizing network behaviors, than an MMSID that may be generated the UE randomly or without a unified rule defined and implemented by the UE vendors. Secondly, the AF-provided MMSID supports not only the use cases of single-UE multi-modal service but also the use cases of multi-modal service involving multiple UEs. Meanwhile, the UE-provided MMSID information only works for the single-UE multi-modal service, unless there is inter-UE service-level coordination among the UEs involved in the multi-UE multi-modal use cases. Therefore, the AF-provided MMSID is sufficient and the preferred solution for assisting RAN in jointly admission control and DRB mapping for multi-modal services.

In addition, SA4 has “recommended that the 5GS not jointly discard packets across multi-modal flows” [3]. Therefore, there is no need for RAN2 to further explore the UE-provided multi-modal traffic information or using the inter-dependency information for discarding UL XR packets.

Proposal 1. RAN2 no longer needs to work on the UE-provided multi-modal information nor using the inter-dependency information for discarding UL XR packets.

Meanwhile, RAN3 still needs to study, and if needed, specify the signaling support for the delivering the CN-provided MMSID information to the gNB.

Proposal 2. RAN3 to study, and if needed, specify the signaling support for the delivering the CN-provided MMSID information to the gNB.

Therefore, we propose that Rel-19 XR WID be revised as below:

-	Specify support for multimodality in RAN for UL and DL [RAN3]:

- Study, and if needed, specify signaling support for delivering the CN-provided MMSID information to the gNB.

Proposal 3. Revise Rel-19 XR WID as below:

-	Specify support for multimodality in RAN for UL and DL [RAN3]:

- Study, and if needed, specify signaling support for delivering the CN-provided MMSID information to the gNB.

# Conclusion

We propose the following:

Proposal 1. RAN2 no longer needs to work on the UE-provided multi-modal information nor using the inter-dependency information for discarding UL XR packets.

Proposal 2. RAN3 to study, and if needed, specify the signaling support for the delivering the CN-provided MMSID information to the gNB.

Proposal 3. Revise Rel-19 XR WID as below:

-	Specify support for multimodality in RAN for UL and DL [RAN3]:

- Study, and if needed, specify signaling support for delivering the CN-provided MMSID information to the gNB.

# References

RP-243318, WID XR Revised, Nokia (Rapporteur).

S2-2502463, 23.501 CR6148, Qualcomm, et al.

R2-2500057_S4-242223, LS Reply on multi-modality awareness, SA4.