3GPP TSG RAN Meeting #107	RP-250567

Incheon, Korea, March 12-14, 2025

Source:	ZTE Corporation, Sanechips

Title:	Views on RAN study for IMT-2030

Agenda Item:	16

Document for:	Discussion and Decision

- Introduction

In RAN#103 meeting, the following RAN plenary work plan was agreed for IMT-2030 aspects [1] where the RAN study includes two phases corresponding to ITU IMT-2030 TPR and 6G general aspects respectively.

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In RAN#106 meeting, the new SID was endorsed for Study on 6G scenarios/use cases and requirements where the first phase on ITU TPR part is included but the second phase on general 6G aspect will be filled in this March RAN plenary meeting.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - [To fill in additional objectives, on general 6G RAN study, in March’2025]

In this contribution, we elaborate our views on both study phases.

- TPR candidates and values

During ITU-R WP5D#47, an LS was formulated and output to external organizations including 3GPP featuring invited input as below [3]

Working Party 5D kindly invites External Organizations to provide their inputs on the minimum technical performance requirements for IMT-2030 radio interface technologies, including but not limited to, – Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025). – Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

3GPP TSG RAN#106 in December 2024 adopted the study plan as below:

- RAN#106 (December 2024) - Initial discussion on the possible TPRs - RAN#107 (March 2025) - Aim to finalize the candidate set of TPRs - Initial discussion on the associated values of the identified TPRs - RAN#108 (June 2025) - Aim to finalize the associated values of the identified TPRs. - After RAN#108 - Any remaining issues on 6G focusing on ITU-R may continue, if needed

Based on the  LS from ITU-R WP5D and RAN study plan, our proposals are provided to facilitate discussion regarding the following two aspects:

- Candidate IMT-2030 TPR list

- Target values of proposed TPRs

  - Candidate IMT-2030 TPRs

During RAN#106 meeting in December 2024, an initial view of candidate IMT-2030 TPRs was agreed and a list was provided to ITU-R WP5D:

1. Peak data rate 2. Peak spectral efficiency 3. User experienced data rate 4. 5th percentile user spectral efficiency 5. Average spectral efficiency 6. Sustainability/Energy efficiency 7. Area traffic capacity 8. User plane latency 9. Control plane latency 10. Connection density 11. Reliability 12. Mobility 13. Mobility interruption time 14. Bandwidth

In ITU-R WP5D#48 in February 2025, input documents including candidate TPR list provided by 3GPP RAN were discussed in SWG radio aspects. A preliminary list of Minimum Technical Performance Requirements (MTPRs) [4] was agreed and sent to SWG Evaluation from SWG radio aspects as shown below:

Preliminary list of Minimum Technical Performance Requirements

(Note: Square brackets indicate further discussion required)

| Minimum Technical Performance Requirement | Method |
| --- | --- |
| AI-related capabilities | (to be determined) |
| Area traffic capacity | (Quantitative) (Note 1) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) (Note 2) |
| [Joint requirement on data rate, latency, reliability and capacity] | To be determined |
| [Coverage] | To be determined |
| [Security] | To be determined |
| [Resilience] | To be determined |
| [Sustainability] | To be determined |
| [Interoperability] | To be determined |

Note 1: The evaluation technique for quantitative requirements is determined by SWG Eval (e.g., analysis, simulation, etc.).

Note 2: The reliability requirement may be replaced by the “Joint Requirement” (if agreed).

On top of the above agreed preliminary list from RAN plenary and ITU-R WP5D, we support the following proposal (detailed analysis can be found in subsections under section 2.1):

Proposal 1: Support the following candidate TPRs for IMT-2030

1. Peak data rate 2. Peak spectral efficiency 3. User experienced data rate 4. 5th percentile user spectral efficiency 5. Average spectral efficiency 6. Sustainability/Energy efficiency 7. Area traffic capacity 8. User plane latency 9. Control plane latency 10. Connection density 11. [Reliability] 12. Mobility 13. Mobility interruption time 14. Bandwidth 15. Composite requirement, i.e. joint requirement 16. AI-related capabilities 17. Sensing-related capabilities 18. Positioning

### 2.1.1 Composite requirement

Immersive communication extends the eMBB of IMT-2020 and as elaborated in M.2160, certain immersive communication use cases require support of high reliability and low latency for responsive and accurate interaction with real and virtual objects, as well as higher system capacity for simultaneously connecting numerous devices.

For 6G immersive communications, a notable feature is multi-modal or mult-traffic transmission. The minimum technical requirements for 6G immersive communications should cater to the performance requirement of the most representative modality/traffic, along with their corresponding traffic characteristics. For example, the use cases of immersive live shows / AR enable immersive experience / Mobile metaverse for immersive gaming, they focus on providing a visual, auditory, tactile feast in a hot spot area. In this kind of scenarios, multiple streams are transmitted, consisting of video streams, audio streams, haptic streams and so on. In multi-modality transmission, video traffic is the main data stream and major technical performance requirement lies on the sum throughput or overall spectral efficiency within the geographical area. For immersive live shows / AR enabled immersive experience, composite requirement on data rate, latency reliability and capacity should be considered for both UL and DL traffic under  typical test environments.

For eMBB in IMT-2020, area traffic capacity defined as the total traffic throughput served per geographic area (in Mbit/s/m2) is adopted for the purpose of evaluation in the related eMBB test environments. However, only full buffer traffic was considered for area traffic capacity, and target values for limited Test environment (only one Indoor Hotspot-eMBB) was defined. Area traffic capacity can also be used for immersive communication as immersive communication is an extension of eMBB in IMT-2020. However, for immersive communication in IMT-2030, non-full buffer traffic and the additional requirement of data rate, low latency and high reliability should be considered when determining the capacity in order to align with the real deployment. Composite requirement on data rate, latency, reliability and capacity can be adopted for immersive communication, and the capacity of composite requirement is the total traffic throughput of all users that fulfil requirement of data rate, latency and reliability.

For Hyper Reliable and Low-Latency Communication (HRLLC), similarly, as elaborated in M.2160, this usage scenario would require support of enhanced reliability and low latency, and depending on the use case, precise positioning, and connection density. Moreover, according to TR 38.824 [5] on URLLC in IMT-2020 in RAN1, the following performance metrics are used:

The performance metric for the system level evaluations in this study item, including evaluation of the baseline performance achievable with Rel-15 NR URLLC and evaluation of the performance achievable with potential enhancement(s) for Rel-16 URLLC, is either option 1 or option 2 as below: - Option 1: Percentage of users satisfying reliability and latency requirements - Intend for the case with fixed number of UEs and fixed traffic model per UE - Option 2: URLLC capacity and URLLC/eMBB multiplexing capacity - Definition: URLLC system capacity is calculated as follows: - C(L, R) is the maximum offered cell load under which Y% of URLLC UEs in a cell operate with target link reliability R under L latency bound - X= (100 – Y) % is the percentage of UEs in outage - A UE in outage is defined as the UE cannot meet both latency L and link reliability R bound - Companies report their assumption on X (either ~5% or 0%) - Companies report their assumption on the number of eMBB UEs deployed together with the URLLC UEs - Intend for the case that the number of UEs and/or the data arrival rate is adjustable - Adjusting the number of UEs should be applied to periodic deterministic traffic model

Similarly, considering the industrial need of HRLLC and the feasibility of evaluating capability under HRLLC (already used in 3GPP RAN1 URLLC study), composite requirement on data rate, latency, reliability and capacity can be adopted for both immersive communication and HRLLC.

It should be noted that the data rate and traffic throughput requirements for 6G may be much higher than those of NR in which it is assumed 32 bytes data only within 1 ms for reliability evaluation according to ITU-R report M.2410. For example, some new use cases, such as 3D factory model-based AR-guided tasks, cooperating mobile robots, etc. [7] [8] [9] [10] [11] [12] [13], were agreed upon in the SA1#109 meeting as 6G use cases on industry verticals, which are typical HRLLC traffic scenarios. The required data rate could range from 10 Mbits/s to 22 Gbits/s, and the area traffic throughput could even reach as high as 100 Gbits/s/km². In other words, high data rate and traffic capacity should be jointly considered along with latency and reliability for HRLLC as well as immersive communication. The requirement on 3D factory model-based AR-guided tasks is copied below.

Table 11.x.6-1 Performance requirements for simultaneous high data rate, lower latency and higher reliability in factory Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL); Area traffic capacity (UL); Network transmission Latency; Reliability; Overall UE density; Activity factor; UE speed; Coverage. AR guided task; 240-500M bit/s; 50M bit/s; [50 - 100]Gbit/s/km2; [10]G bit/s/km2; 1 to 5ms (note 4); Up to 99.999%; [200 00]/km2 (note 2); (note 1); Pedestrians; Factory.

Proposal 2: Composite requirement on data rate, latency, reliability and capacity can be adopted for both immersive communication and HRLLC.

- This requirement is defined as the total traffic throughput of all users within a geographic area that fulfil the required data date, latency and reliability.

Specifically, this composite requirement is defined as the total traffic throughput of all users within a geographic area that fulfil the required success probability (reliability) of transmitting a layer 2/3 packet within a required latency, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper-Reliable Low Latency Communication usage scenario. The minimum performance requirement for various test environments is summarized in Table X in the latest ITU WP 5D report [4]. Here is our suggestion where the required values can be further studied.

TABLE X

Composite requirement

| Test environment | Data rate (Mbits/s) | Latency (ms) | Reliability | Total traffic throughput (Mbits/s/km2) |
| --- | --- | --- | --- | --- |
| Dense Urban/Indoor Hotspot - Immersive Communication |  |  |  |  |
| Dense Urban/Indoor – HRLLC |  |  |  |  |

### 2.1.2 Energy efficiency

Energy efficiency instead of sustainability can be listed as a TPR. We understand that sustainability is a foundational aspiration of future IMT systems and energy efficiency is only one part of sustainability. But the concept of sustainability is too broad and hard to define corresponding TPR (e.g., the capability of network or UE to minimize greenhouse gas emission to address climate change). We prefer to only keep energy efficiency as a TPR and apply similar wording as Working document towards a preliminary draft new Report ITU-R M.[IMT-2030.TECH PERF REQ] to address concerns on sustainability: “Energy efficiency is an important metric of sustainability.”.

In IMT-2020, energy efficiency of the network and the device can relate to the support for two aspects: a) Efficient data transmission in a loaded case (demonstrated by the average spectral efficiency); b) Low energy consumption where there is no data (estimated by sleep ratio). However, the sleep ratio and/or sleep duration can not fully represent the energy consumption of the network/UE, and the energy consumption of a network/UE varies in different states (e.g., active, deep sleep, light sleep, micro sleep), also average spectral efficiency cannot indicate the energy consumption.  Similar as energy saving study using power model in TR 38.864 and TR 38.840, a quantitative energy efficiency TPR using power model can be introduced for IMT-2030 network and device.

Details can refer to ITU-R Document 5D/459-E, the E(energy consumption) could be defined as average energy consumption during the observation time.

E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal=Pon ×Ton + Poff ×Toff+Ptransition ×TtransitionTtotal (in energy unit/time unit)

Ei (in energy unit) is the energy computation category at network or device for transmission and/or reception in loaded and unloaded cases; As an illustration, it is assumed at least three energy consumption categories:

Eon 	is the energy consumption for active transmission and/or reception at the network or device during the observation/evaluation time. Pon denotes the power consumed at network or device during Ton wherein Ton is the time for the transmission and/or reception.

Eoff 	is the energy consumption at network or device when there is no transmission and/or reception. Poff denotes the power consumed at network or device during Toff wherein Toff is the time without transmission or reception.

Etransition 	is the energy consumption at network or device during transition between on and off.

Ttotal (in time unit)	is the total observation/evaluation time.

Ptransition denotes the power consumed at network or device during transition and Ttransition is the total transition time.

- For unloaded case (when there is no data):

EEno_data=E

- For loaded case:

EEloaded=v/E

where v refers to the traffic per second served by a base station or traffic per second of a device, i.e. data rate.

Based on the above analysis, the following proposal is suggested to define energy efficiency:

Proposal 3: Quantitative energy efficiency TPR can be adopted for IMT-2030 communication for network and device:

- For unloaded case: EEno_data=E

- For loaded case: EEloaded=v/E

where E is defined as average energy consumption during the observation time, v refers to the traffic per second served by a base station or traffic per second of a device

### 2.1.3 Sensing

For ISAC, numerous use cases have been specified in SA1 TS 22.137 [6]. Furthermore, the channel modeling is under discussion in RAN1 and will be completed in Rel-19 for positioning-type use cases including UAV, AGV, pedestrian, vehicles and hazardous objects invasion and tracking. For this type of sensing targets, the requirements have been defined in ST 22.137. From TPR perspective, the detectability, localization accuracy, velocity accuracy and sensing resolution can be considered.

- Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the presence of the object, and False alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy - Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

- Horizontal/Vertical Localization Accuracy (HSA/VSA): It is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of an object

- Velocity Accuracy - Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

- Sensing Resolution – Is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects. (evaluated by an analytical approach).

In addition to the above mentioned sensing-related TPR, new use cases, e.g. micro-deformation can also be considered. Micro-deformation is the translation, tilt, bending and other deformation of the architectural structure of buildings (such as bridges and skyscraper). This type of deformation is not easily observable by the naked eye (on the order of centimeters), but has a significant impact on the health and life of the building structure. Displacement accuracy can be defined as the closeness of the measured deformation displacement to the true deformation displacement.

Proposal 4: Support the following quantitative sensing-related TPRs:

- Detectability (Detection / False Alarm Probability)

- Localization accuracy

- Velocity accuracy

- Sensing resolution

- Displacement accuracy

### 2.1.4 AI

The following description from ITU WP5D report [4] can be used as a starting point:

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support [mechanisms and/or signalling/ related to] AI functionalities over the radio interface (between UE and Network). This should include support for one or more of the following: - Distributed data processing - Distributed learning - AI computing - AI model execution - AI model inference - or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Editor’s note: The following is a placeholder for a possible quantitative requirement on “Network for AI”, that would require further input and discussions. AI Service Latency AI service latency is the time from beginning to the end of a given AI inference/learning task with a target service accuracy. Editor’s note: The following is a placeholder for a possible quantitative requirement on “AI for Network”, that would require further input and discussions. AI assisted Network: This refers to the communication aspects of the RIT/SRIT being enhanced by AI frameworks, via AI models on both the mobile station side and the TRxP side, either by replacing or working in conjunction with non-AI frameworks. The performance improvements with and without AI can be reported wherever applicable.

For AI assisted network, we think it can be reflected in other performance requirement items such as data rate, spectrum efficiency, energy efficiency etc., hence it may not be needed to separately define a dedicated TPR item.

AI latency can reflect the 6G system processing capability with AI function. It can ensure that AI applications are achievable in valid time by setting measurable and enforceable performance targets for AI tasks within the 6G ecosystem. Hence, we support the following proposal:

Proposal 5: For AI-related TPRs:

- At least support a qualitative TPR to reflect AI-related capability

- AI service latency can be introduced for network assisted AI

### 2.1.5 Coverage, Security, Resilience, Interoperability

For coverage: Coverage refers to the ability to provide access to communication services for users in a desired service area. Similar as IMT-2020, there is no need to define TPR for coverage and instead link budget can be provided in submission template.

For security, resilience and interoperability, submission template can also be considered since it is not practical to define corresponding TPR.

Proposal 6: Do not define TPR(s) for coverage, security, resilience and interoperability.

  - Target values of proposed TPRs

In this section, we provide our views for target values on the proposed TRP items. The justification is briefly described in the third column for some TPR items.

Proposal 7: Consider Table 2.2-1 regarding target values of TPRs for IMT-2030.

Table 2.2-1 Preliminary target values of TPRs for IMT-2030

| ITU candidate TPR | Target value of IMT 2030 | Notes | Target value of IMT2020 |
| --- | --- | --- | --- |
| Peak data rate | DL: 100, UL: 50 Gbit/s. | 5xIMT-2020 Related to the increase of bandwidth, number of antenna elements, modulation order, etc | – Downlink peak data rate is 20 Gbit/s. – Uplink peak data rate is 10 Gbit/s. |
| Peak spectral efficiency | DL: 75, UL: 37.5 bit/s/Hz | 2.5xIMT-2020 Related to the increase of BS/UE antenna elements number, modulation order, etc | – Downlink peak spectral efficiency is 30 bit/s/Hz. – Uplink peak spectral efficiency is 15 bit/s/Hz. |
| User experienced data rate | DL: 500, UL: 250 Mbit/s. | 5x IMT-2020 | – Downlink user experienced data rate is 100 Mbit/s. – Uplink user experienced data rate is 50 Mbit/s. |
| 5th percentile user spectral efficiency | Indoor Hotspot 0.75(DL), 0.63 (UL) Dense Urban 0.563(DL), 0.45(UL) Rural 0.36(DL), 0.135(UL) bit/s/Hz | 2.5~3x IMT2020 Preliminary simulation result can be found in Appendix | (bit/s/Hz) Indoor Hotspot – eMBB 0.3(DL) 0.21(UL) Dense Urban – eMBB (NOTE 1) 0.225 0.15 Rural – eMBB 0.12 0.045 |
| Average spectral efficiency | Indoor Hotspot 22.5(DL) 16.875(UL) Dense Urban 19.5(DL) 13.5(UL) Rural 8.25(DL) 4(UL) bit/s/Hz/TRxP | 2.5xIMT-2020 Preliminary simulation result can be found in Appendix | (bit/s/Hz/TRxP) Indoor Hotspot – eMBB 9 (DL) 6.75(UL) Dense Urban – eMBB (Note 1) 7.8 5.4 Rural – eMBB 3.3 1.6 |
| Area traffic capacity | Downlink 50 Mbit/s/m2 in the Indoor Hotspot-IC | 5x IMT-2020 | Downlink 10 Mbit/s/m2 in the Indoor Hotspot – eMBB |
| User plane latency | 1ms for IC 0.5ms HRLLC | At least 1/2 x IMT-2020 Refer to M.2160: target latency could be 0.1-1ms | 4 ms for eMBB 1 ms for URLLC |
| Control plane latency | 10ms for 4 step RACH 5ms for 2-step or more direct random access | 1/2 or 1/4 x IMT-2020 | 20 ms consider 10ms |
| Connection density | 10^8 devices per km2 for Massive | 100x IMT-2020 | 1 000 000 devices per km2 |
| Mobility | TBD |  | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h. |
| Mobility interruption time | 0ms | Same as IMT 2020 | 0ms |
| Bandwidth | 400 MHz | 4x IMT-2020 | at least 100MHz |
| Energy efficiency | TBD |  | / |
| Sensing related capability - Detectability (Detection / False Alarm Probability) - Localization accuracy - Velocity accuracy - Sensing resolution - Displacement accuracy | - Detection: 95% - False alarm: 5% - Horizontal accuracy: 2m - Vertical localization accuracy: 5m - Velocity accuracy :1m/s. - Displacement accuracy: 1 cm | Refer to TS 22.137 sensing service category 2 Preliminary simulation result can be found in Appendix | / |
| Composite requirement on data rate, latency, reliability and capacity | TBD | Preliminary simulation result can be found in Appendix | / |
| Positioning | Indoor factory: 0.05m-1m outdoor: 1m-3m | Refer to TR 38.859 | / |
| AI service latency | TBD |  | / |

- General 6G RAN study

To gain some useful insight and facilitate potential scope discussion for the Rel-20 SI focusing ITU IMT-2030, it's worthwhile to review the 5G radio requirement study work, i.e. Study on Scenarios and Requirements for Next Generation Access Technologies. The study consisted of the following major milestones:

- Approval of the SID RP-152257 [2] in RAN#70, Dec. 2015

- Approval of TR 38.913 v0.4.0 after RAN#72 (endorsed via Email discussion after RAN#72) with sufficient inputs on section 6 Scenarios and section 7 Key performance indicators

- Further discussion till RAN#75 and finalization of TR 38.913 considering input from ITU-R WP5D work

The objectives of the 5G RAN-level SID [2] are shown as follows:

The study item aims to develop deployment scenarios and requirements of next generation access technologies, and to provide guidance to the technical work to be performed in RAN WGs. In order to achieve this, the study item should fulfil the following objectives: - Identify the typical deployment scenarios associated with attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, etc. - Develop specific requirements for next generation access technologies for the identified deployment scenarios. To complete the above work, the inputs from other organizations could be referred to.

For 6G, the first study will focus on ITU IMT-2030 which refers to IMT-2030 Technical Performance Requirements (TPRs) being discussed in ITU-R WP5D as described in section 1. The second study will focus on 6G general part which can be similar to the 5G SID [2].  Different from the first SI, the scenarios and requirements discussed in the second SI are not limited to TPR discussed in ITU-R WP5D. Our suggestion on objectives is as follows

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - develop deployment scenarios and requirements of next generation access technologies, and to provide guidance to the technical work to be performed in RAN WGs. In order to achieve this, the study item should fulfil the following objectives: - Identify the typical deployment scenarios associated with attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, etc. - Develop specific requirements (in addition to the identified minimum TPRs in phase 1) for next generation access technologies for the identified deployment scenarios.

The output of this SI can be a single TR similar as TR 38.913.  While RAN can provide guidance to the technical work to be performed in RAN WGs, it should be avoided to move technical study from WG level to RAN level.

Proposal 8: The general 6G RAN study should aim to develop deployment scenarios and requirements of 6G access technologies, and to provide guidance to the technical work to be performed in RAN WGs

- The scenarios and requirements are not limited to TPR discussed in ITU-R WP5D

- The output of the whole study can be a single TR similar as TR 38.913.

- Conclusion

In this contribution, we elaborate our views on RAN study for IMT-2030. Our proposals are listed as follows.

Proposal 1: Support the following candidate TPRs for IMT-2030

1. Peak data rate 2. Peak spectral efficiency 3. User experienced data rate 4. 5th percentile user spectral efficiency 5. Average spectral efficiency 6. Sustainability/Energy efficiency 7. Area traffic capacity 8. User plane latency 9. Control plane latency 10. Connection density 11. [Reliability] 12. Mobility 13. Mobility interruption time 14. Bandwidth 15. Composite requirement, i.e. joint requirement 16. AI-related capabilities 17. Sensing-related capabilities 18. Positioning

Proposal 2: Composite requirement on data rate, latency, reliability and capacity can be adopted for both immersive communication and HRLLC.

- This requirement is defined as the total traffic throughput of all users within a geographic area that fulfil the required data date, latency and reliability.

Proposal 3: Quantitative energy efficiency TPR can be adopted for IMT-2030 communication for network and device:

- For unloaded case: EEno_data=E

- For loaded case: EEloaded=v/E

where E is defined as average energy consumption during the observation time, v refers to the traffic per second served by a base station or traffic per second of a device

Proposal 4: Support the following quantitative sensing-related TPRs:

- Detectability (Detection / False Alarm Probability)

- Localization accuracy

- Velocity accuracy

- Sensing resolution

- Displacement accuracy

Proposal 5: For AI-related TPRs:

- At least support a qualitative TPR to reflect AI-related capability

- AI service latency can be introduced for network assisted AI

Proposal 6: Do not define TPR(s) for coverage, security, resilience and interoperability.

Proposal 7: Consider Table 2.2-1 regarding target values of TPRs for IMT-2030.

Proposal 8: The general 6G RAN study should aim to develop deployment scenarios and requirements of 6G access technologies, and to provide guidance to the technical work to be performed in RAN WGs

- The scenarios and requirements are not limited to TPR discussed in ITU-R WP5D

- The output of the whole study can be a single TR similar as TR 38.913.

- References

- RP-240823 Additional Considerations for 6G Timeline Source: TSG Chairs

- RP-152257 “New Study Item Proposal: Study on Scenarios and Requirements for Next Generation Access Technologies”, CMCC

- RP-243202	Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

- ITU-R WP5D 563 Report on the 48th meeting of Working Party 5D (Geneva, 4-13 February 2025) Chair, WP 5D

- 3GPP TR 38.824 Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)

- 3GPP TS 22.137 Service requirements for Integrated Sensing and Communication

- S1-250977 3D factory model based AR guided task  ZTE Corporation, China Unicom, Huawei

- S1-250970 New use case on Cooperating Mobile Robots Siemens, TNO, KPN, Fraunhofer IIS, Airbus, Robert Bosch GmbH

- S1-251015 Collaborative Awareness in Dynamic Environments - Enhancing Mutual Decision-Making through Real-Time Data Sharing LG Electronics

- S1-250973 Use cases on high-rate aircraft communication services in 6G Airbus, Deutsche Telekom, Ericsson, Fraunhofer IIS

- S1-250972 Immersive Media Services for AAM Enabled by 6G NTN LG Electronics

- S1-250971 New use case on Realtime Digital Twins Nokia, TNO, KPN, Telefonica, Orange, Siemens, nvidia, Vodafone

- S1-250815 Pseudo-CR on on-board communication for UAM aircrafts Nokia, Huawei

- Appendix: preliminary simulation

## Spectral efficiency

Table: Spectral efficiency of different MIMO configurations for dense urban for DL@U6G

| Tx/Rx antenna configuration |  | Average spectral efficiency (bit/s/Hz; multiple of IMT-2020 requirement) | 5th percentile user spectral efficiency (bit/s/Hz;multiple of IMT-2020 requirement) | ITU IMT-2020 spectral efficiency requirements (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| 8R | BS: [32,16,2,1,1], 256 port UE: [2,2,2,1,1] | 33.806 (4.33*) | 9.721 (43.20*) | 0.225 for 5th percentile user 7.8 for average |
|  | BS: [32,16,2,1,1], 128 port UE: [2,2,2,1,1] | 28.420 (3.64*) | 7.438 (33.06*) |  |
|  | BS: [12,4,2,1,1], 32 port UE: [2,2,2,1,1] | 10.409 (1.33*) | 0.738 (3.28*) |  |
| 4R | BS: [64,16,2,1,1], 512 port UE: [1,2,2,1,1] | 24.385 (3.12*) | 2.017 (8.96*) |  |
|  | BS: [32,16,2,1,1], 256 port UE: [1,2,2,1,1] | 15.937 (2.04*) | 1.335 (5.93*) |  |
|  | BS: [32,16,2,1,1], 128 port UE: [1,2,2,1,1] | 12.054 (1.54*) | 1.009 (4.48*) |  |
|  | BS: [12,4,2,1,1], 32 port UE: [1,2,2,1,1] | 3.527 (0.45*) | 0.169 (0.75*) |  |

## Composite requirement

For the value of composite requirement for immersive communication, we provide some candidate values shown as follows:

- For test environment of Dense Urban-IC, the DL capacity can be 70Gbits/s/km2.

- For test environment of Urban Macro-IC, the DL capacity is 10G bits/s/km2

  - Traffic model: Video: 45Mbps@60FPS, PER=99%, PDB = 10ms; Haptic: 10 sensors, fixed 100 Byte per sensor, PER = 99.9%, PDB=5ms, pareto distribution arrival

For DL composite requirement in test environment UMa-IC, we provide the following statistics of transmitted XR packet sizes and service time for each XR packet, respectively.

| Figure a) transmitted XR packet sizes | Figure b) service time for each packet |
| --- | --- |

The candidate value of capacity in composite requirement can be calculated by:

(Average transmitted XR packet size / average service time for each packet / A) * UE density,

where average transmitted XR packet size can be obtained in Figure a), average service time for each packet can be obtained in Figure b). UE density is average 5 UE per cell

## Sensing

| Sensing mode | TRP monostatic |
| --- | --- |
| Carrier frequency | 6GHz |
| bandwidth | 100 MHz |
| SCS | 30kHz |
| BS Tx power | 49 dBm |
| BS antenna configuration | (M, N, P, Mg, Ng) = (16, 16, 2, 1, 1), dH = 0.5 λ，dV = 0.5 λ |
| BS height | 35m |
| BS noise figure | 5 dBi |
| Thermal noise | -174dBm/Hz |
| UAV height (sensing target) | 200m |
| UAV velocity | 0-60km/h uniform in horizontal |
| UAV distribution | 5 targets in center sector |
| Number of Drops | 200 |

(a) Velocity

(b) Pos_H_Cluster                          (c) Pos_V_Cluster

Figure: TRP monostatic (coordinated sensing) sensing CDF with (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1)

Table: TRP monostatic (coordinated sensing) sensing results

| Horizontal localization accuracy | Vertical localization accuracy | Velocity accuracy | Detection probability |
| --- | --- | --- | --- |
| 0.850 | 0.565 | 0.332m/s | 98.0% |