3GPP TSG RAN Meeting #107	RP-250571

Incheon, Korea, 12 – 14 March, 2025

Agenda item: 	    16

Source: 	China Unicom

Title: 	Considerations on candidate IMT2030 TPRs for sustainability

Document for:	Discussion

1	Introduction

ITU-R WP5D is developing a Report on IMT-2030 Technical Performance Requirements (TPRs), aiming to finalize it by February 2026. Meanwhile, RAN Plenary started the Rel-20 Study Item [1] focusing on ITU-R IMT-2030 TPRs on RAN#106 meeting.

At the RAN#106 meeting, IMT-2030 TPRs including candidate items for IMT-2030 TPRs, definition of candidate IMT-2030 TPRs and target value of candidate IMT-2030 TPRs have been discussed. It was concluded in RAN#106 that the sustainability/energy efficiency would be considered as candidate items for IMT-2030 TPRs, where the definition of the item will be further discussed.

This contribution provides our views on candidate IMT2030 TPRs for sustainability.

2	Discussion

According to ITU-R M.2410 (Minimum requirements related to technical performance for IMT-2020 radio interface(s)), network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics. This requirement is defined for the purpose of evaluation in the eMBB usage scenario. In IMT-2020, there is no quantitative TPR requirement for energy efficiency. Energy efficiency is evaluated by inspection of support for high sleep ratio and duration.

Sustainability is one of the fundamental TPRs of IMT-2030. One of the principles of IMT2030 system design is to improve usage resources efficiency by minimizing power consumption. It is necessary to define quantitative technical performance requirements to evaluate the energy efficiency of candidate RIT.

Proposal 1: It is preferred to define quantitative TPR  for sustainability.

Considering the feasibility and effectiveness of evaluation, it is recommended to consider the following two candidate methods for quantitative definition of sustainability.

Option A:

The candidate TPR for sustainability is defined as the relative energy saving, that is, the percentage of the energy consumption of the candidate RIT at a certain load level relative to the energy consumption in the full-load state. The minimum requirement for the relative energy saving is  a percentage.

This method involves use of  power model, which includes the relative power consumption of a base station or device in different power states. For example, according to the differences in power consumption, the power states of the base station could be defined as deep sleep, light sleep, and micro sleep. And the power consumption of the base station in the deep sleep state can be taken as the reference value of one power unit, and the relative power consumptions in other power states could be calculated.

Option B:

The candidate TPR for sustainability can be defined as energy efficiency in relation to traffic capacity, that is the traffic capacity per unit of power consumption (in Mbps/Power Unit) while meeting certain quality of service (QoS) target, such as data rate, latency, reliability, etc.

Let Cavg denote the average traffic capacity (in Mbps)  during the evaluation time. Further, let Pavg denote the average power consumption (in Power Unit) of network during the evaluation time. Energy efficiency in relation to traffic capacity is denoted as:

EEavg= CavgPavg

where EEavg is the estimated average energy efficiency.

This method also involves use of  power model, besides Option B requires detailed reference configuration and traffic model to ensure that the same simulation assumptions are used in the evaluation from different sourcing companies.

Proposal 2: Relative energy saving or energy efficiency could be considered as representative TPR for sustainability.

Both Option A and Option B require using power model to evaluate the relative energy consumption. Currently, there is no unified power model which can be applied to different RATs. Power models for IMT-2030 should be further discussed and decided. Power model defined in 3GPP TR 38.864 can be used as the baseline or starting point for future discussion.

Compared with Option B, Option A can only be used to evaluate the effect of energy saving between different load status within the same RAT, and it’s difficult to reflect the performance enhancement of IMT-2030 over IMT2020. Operators hope that 6G will improve system performance and enhance energy efficiency beyond 5G. However, Option B requires detailed reference configuration and traffic model, which may need more time to disucss and agree the final traffic models.

Proposal 3: Power model defined in 3GPP TR 38.864 can be used as the baseline or starting point for future evaluations, so as to compare the differences in energy efficiency between 5G and 6G.

3	Conclusion

Proposal 1: It is preferred to define quantitative TPR for sustainability.

Proposal 2: Relative energy saving or energy efficiency could be considered as representative TPR for sustainability.

Proposal 3: Power model defined in 3GPP TR 38.864 can be used as the baseline or starting point for future evaluations, so as to compare the differences in energy efficiency between 5G and 6G.

4   References

- RP-242873: Scope of RAN-level SID focusing on ITU IMT-2030 and initial considerations on IMT-2030 TPRs; Huawei, HiSilicon, CAICT, 3GPP TSG#106, December 9-12, 2024

- RP-243279: Moderator's summary for discussion on IMT-2030 TPRs;  Moderator (CMCC), 3GPP TSG#106 , December 9-12, 2024

- Report ITU-R M.2410-0 (11/2017): “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”