3GPP TSG-RAN4 Meeting #114	R4-2500164

# Athens, Greece, 17th Feb. - 21st Feb.

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0710 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_perf_enh-Perf) CR to CQI-PMI-RI Delay in 6.2A.3.1.1 Test1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, there is no clear description of counting CQI delay in a case of CA and mixed numerology situation between PCell and SCell in the RAN1 specification. Thus, there are two possibilities of delays with CQI/PMI/RI Delay for TDD Scell when FDD Pcell (Test 1), 9.5 ms or 10.5 ms. Refer to the following sheet in attached file FR1_Perf-CQI-CA_CQI-PMI-RI_Delay_two-counting-methods.xlsx for more detail. --(Test1: TDD30khz PCell + FDD15kHz SCell) See “1-1. CQI DLCA TDD30k+FDD15kHz” sheet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note to describe that the specified CQI/PMI/RI Delay for TDD Scell when FDD Pcell (Test 1) is minimum in Table 6.2A.3.1.1-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clarification of the possibility that there is longer delay may be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.1 Isolated impact analysis: No change to UE requirements, changes test parameters only. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

6.2A.3.1	CQI reporting definition under AWGN conditions

6.2A.3.1.1	Minimum requirement for periodic CQI reporting

For each CA CQI reporting test defined in Table 6.2A.3.1.1-6, the test requirements and the test parameters are defined as below.

For each CC, the test parameters are specified in Table 6.2A.3.1.1-1. The additional parameters specified in Table 6.2A.3.1.1-2 are applicable for tests on FDD CC. The additional parameters specified in Table 6.2A.3.1.1-3 are applicable for tests on TDD CC.

For CA with 2 DL CC, for the SNR configuration specified in Table 6.2A.3.1.1-4, and using the downlink physical channels specified in Annex C.3.1 on each CC, the difference between the wideband CQI indices of PCell and SCell reported shall be such that

wideband CQIPCell – wideband CQISCell ≥ 2

for more than 90% of the time.

For CA with 3 or more DL CC, for the SNR configuration specified in Table 6.2A.3.1.1-5, and using the downlink physical channels specified in Annex C.3.1 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, 3… reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2, 3… ≥ 2

for more than 90% of the time.

Table 6.2A.3.1.1-1: CA CQI reporting test parameters for FDD and TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Propagation channel |  |  | AWGN |
| Antenna configuration |  |  | 1×2 with static channel specified in Annex B.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 1 |
|  | CDM Type |  | No CDM |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 2,(13) |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |
| ReportConfigType |  |  | Periodic |
| CQI-table |  |  | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI (Note 1) |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Csi-ReportingBand |  |  | 1111111 |
| aperiodicTriggeringOffset |  |  | Not configured |
| Codebook configuration |  |  | Not configured |
| Physical channel for CSI report |  |  | PUCCH |
| Maximum number of HARQ transmission |  |  | 1 |
| Measurement channel |  |  | Derived as per section 5.1.3.2 of TS 38.214 [12] |
| Note 1: The bitwidth of PMI for UCI on PUCCH in a case 1-port CSI-RS is configured as channel measurement resource is given in [10], section 6.3.1.1.2. |  |  |  |

Table 6.2A.3.1.1-2: Additional test parameters for FDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex Mode |  |  | FDD |
| Subcarrier spacing |  | kHz | 15 |
| ZP CSI-RS configuration | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |
|  |  |  | 10/1 if configured as SCell with TDD PCell (Test1) |
| CSI-IM configuration | CSI-IM timeConfig periodicity and offset | slot | 5/1 |
| CSI-Report periodicity and offset |  | slot | 5/0 if configured as PCell |
|  |  |  | 5/1 if configured as SCell with FDD PCell (Test2) |
|  |  |  | 20/18 if configured as SCell with TDD PCell (Test1) |
| CQI/RI/PMI delay |  | ms | 8 if configured as PCell |
|  |  |  | 12 if configured as SCell |
| Sub-band Size |  | RB | 8 for 5MHz and 10MHz, 16 for 15MHz, 20MHz and 25MHz, 32 for 30MHz, 35MHz, 40MHz, 45MHz and 50MHz |
| Note 1: NZP CSI-RS periodicity/offset slots are based on the carrier SCS and CSI reporting periodicity/offset slots are based on the PCell SCS. |  |  |  |

Table 6.2A.3.1.1-3: Additional test parameters for TDD CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex Mode |  |  | TDD |
| Subcarrier spacing |  | kHz | 30 |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| ZP CSI-RS configuration | CSI-RS periodicity and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 if configured as SCell with FDD PCell (Test1) |
|  |  |  | 20/1 |
| CSI-IM configuration | CSI-IM timeConfig periodicity and offset | slot | 10/1 |
| CSI-Report periodicity and offset |  | slot | 20/19 if configured as PCell |
|  |  |  | 20/18 if configured as SCell with TDD PCell (Test3) |
|  |  |  | 5/1 if configured as SCell with FDD PCell (Test1) |
| CQI/RI/PMI delay |  | ms | 14.5 if configured as PCell |
|  |  |  | 12.5 if configured as SCell with TDD PCell (Test3) |
|  |  |  | 9.5 if configured as SCell with FDD PCell (Test1) (Note 2) |
| Sub-band Size |  | RB | 8 for 10MHz, 15MHz, 20MHz and 25MHz, 16 for 30MHz, 40MHz and 50MHz, 32 for 60MHz, 80MHz, 90MHz and 100MHz |
| Note 1: NZP CSI-RS periodicity/offset slots are based on the carrier SCS and CSI reporting periodicity/offset slots are based on the PCell SCS. Note 2: The minimum delay between the NZP CSI-RS for CSI Acquisition transmission and application of the reported CQI/PMI/RI on PDSCH. |  |  |  |

<<End of change >>