3GPP TSG-RAN4 Meeting #114	R4-2501317

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0729 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ext_to_71GHz-Perf) CR for 38.101-4 on Correcting duplicate FRC names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | FRC name “R.PDSCH.5-12.1 TDD” in Table A.3.2.2.5-13 already exists in Table A.3.2.2.5-12 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated FRCs in tables Table A.3.2.2.5-13, Table A.3.2.2.5-14, Table A.3.2.2.5-15 to match the table index number and remove duplication. Updated testcases in 9.2A.1.1 to use the new FRC names. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two tables have the same FRC name defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2A.1.1, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

#### 9.2A.1.1	NR CA between FR1 and FR2-2

<Unchanged parts omitted>

Table 9.2A.1-5: Minimum performance for Rank 1 (FRC) in Scell CC

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-13.1 TDD | 400 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | 0.9 |
| 1-2 | R.PDSCH.5-14.1 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | 9.0 |
| 1-3 | R.PDSCH.5-14.1 | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 30 | 2.7 |
| 1-4 | R.PDSCH.5-15.1 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | 11.6 |
| 1-5 | R.PDSCH.8-3.1 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | 1.0 |
| 1-6 | R.PDSCH.8-4.1 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | 8.3 |

# <End of change 1>

# <Start of change 2>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<Unchanged parts omitted>

Table A.3.2.2.5-12: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and HST-DPS scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD | R.PDSCH.5-12.2 TDD |  |  |  |
| Channel bandwidth | MHz | 200 | 200 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 132 | 132 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 |  |  |  |
| For Slot i=1 |  | N/A (Note 1) | N/A(Note 4) |  |  |  |
| For Slot i=2 |  | 13 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 9 | N/A (Note 4) |  |  |  |
| Allocated slots per 2 frames |  | 126 | 124 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 17 | 17 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.43 | 0.43 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 |  |  |  |
| For Slot i = 1 |  | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 |  | 18 | N/A (Note 4) |  |  |  |
| For Slot i=3 |  | 18 | N/A (Note 4) |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | 57376 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | 90176 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 | Bits | 90176 | N/A (Note 4) |  |  |  |
| For Slot i=3 | Bits | 57376 | N/A (Note 4) |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i = 2 | Bits | 24 | N/A (Note 4) |  |  |  |
| For Slot i = 3 | Bits | 24 | N/A (Note 4) |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 7 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | 11 |  |  |  |
| For Slot i = 1 | CBs | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i=2 | CBs | 11 | N/A (Note 4) |  |  |  |
| For Slot i=3 | CBs | 7 | N/A (Note 4) |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 195696 | 181968 |  |  |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 137808 | 137808 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {8,…,84, 87,..., 159} | Bits | 146520 | 146520 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…, 159} | Bits | 210672 | 210672 |  |  |  |
| For Slot i = 1 | Bits | N/A (Note 1) | N/A (Note 4) |  |  |  |
| For Slot i= 2 | Bits | 210672 | N/A (Note 4) |  |  |  |
| For Slot i= 3 | Bits | 137808 | N/A (Note 4) |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 515.62 | 508.25 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 and slot #1 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #0, slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |  |

Table A.3.2.2.5-13: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-13.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 3624 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 16 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 1 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 12210 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 18282 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 7.229 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

Table A.3.2.2.5-14: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-14.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 11272 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 17424 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24420 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 36564 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 22.804 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

Table A.3.2.2.5-15: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-15.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 15112 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 23568 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 36630 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 54846 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 30.791 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

# <End of change 2>