3GPP TSG-RAN WG4 Meeting #114	R4-2501751

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5387 | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PL1_sharing for L1-RSRP intra-frequency and inter-frequency neighbour cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When number of neighboring cells to be measured is more than 1, the current conditions for PL1_sharing are incorrect. They are not aligned between intra-frequency L1-RSRP measurements and inter-frequency L1-RSRP measurements without gap. For both intra-frequency neighbor cells or inter-frequency neighbor cells without gap, if none of TCI state(s) of cells are in the active TCI state list, PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the conditions in the sections of SSB based intra-frequency and inter-frequency L1-RSRP reporting: When number of neighboring cells configured with SSB based L1-RSRP measurementto be measured is more than 1 When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are in the active TCI state list - PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap - Some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements of intra-frequency neighbour cell L1-RSRP measurement will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.14.5.1, 9.15.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

9.14.5.1	SSB based L1-RSRP Reporting

The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured.

-	The SSB from the neighbor cell completely contained in the active BWP of the UE

If a neighbor cell is known according 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3.

The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR1 in table 9.14.5.1-1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 and in table 9.14.5.1-2 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18. The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR2 in table 9.14.5.1-3 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18 and in table 9.14.5.1-4 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18, where

-	M=1.

-	N= 8.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1

-	When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list is the number of neigbour cells (including intra-frequency neighbor cell(s) and inter-frequency neighbor cell(s) without gap) whose TCI state(s) are in the active TCI state list. No requirements are defined for any other cell(s) whose TCI state(s) is not in the active TCI state list.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: K = 1.5. NOTE 3: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX*NLayer |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. NOTE 2: K = 1.5. NOTE 3: UE shall at least measure neighbour cells which are activated for TCI state within UE capability and additional cells to measure is up to UE implementation. |  |

Table 9.14.5.1-3: Intra-frequency L1-RSRP measurement period TIntra_ L1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. NOTE: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of these neighbor cells is in the active TCI state list, the requirements apply only to the cell(s) that have TCI state(s) is in the active TCI state list. |  |

Table 9.14.5.1-4: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR2 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of these neighbor cells is in the active TCI state list, the requirements apply only to the cell(s) that have TCI state(s) is in the active TCI state list. |  |

------------ END OF CHANGE 1--------------

------------ START OF CHANGE 2--------------

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements without gaps and without interruptions based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

If a neighbor cell is known according to clause 9.15.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in table 9.3.9.1-3 for FR1 and in table 9.3.9.1-4 for FR2.

The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap is defined for FR1 in table 9.15.6.1.1-1.  The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap  is defined for FR2 in table 9.15.6.1.1-2, where

-	M=1

-	N= 8.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells configured with SSB based L1-RSRP measurement is more than 1

-	When none of TCI state(s) of the intra-frequency neighbor cells or inter-frequency neighbor cells without gap are in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list is the number of neigbour cells (including intra-frequency neighbour cell(s) and inter-frequency neighbor cell(s) without gap) whose TCI state(s) are in the active TCI state list. No requirements are defined for any other neighbor cell(s) whose TCI state(s) is not in the active TCI state list.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.15.6.1.1-1: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)*NLayer) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX*NLayer |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NLayer = the number of inter-frequency layers which are configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18 and measured without measurement gaps + the number of intra-frequency layers configured for L1-RSRP measurement with LTM-CSI-ResourceConfig-r18. NOTE 2: K = 1.5. NOTE 3: Void. |  |

Table 9.15.6.1.1-2: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2 for UE capable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: void. NOTE 3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of the neighbor cells is in the LTM candidate cell active TCI state list, the requirements apply only to the intra-frequency cells and inter-frequency neighbor cells without gap whose TCI state(s) are in the LTM candidate cell active TCI state list. |  |

Table 9.15.6.1.1-3: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: K = 1.5 NOTE 3: The requirements apply for the cells configured by LTM-CSI-ResourceConfig-r18 on which UE is required to perform L1 measurements, when the max RTD between the serving cell and the inter-frequency cell whose SSB are completely contained in the DL active BWP is not larger than CP length |  |

Table 9.15.6.1.1-4: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2 for UE incapable of multiCellL1-measRTD-greaterThan-CP-r18

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320 ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| NOTE 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements apply for the cells configured by LTM-CSI-ResourceConfig-r18 on which UE is required to perform L1 measurements, when the max RTD between the serving cell and the inter-frequency cell whose SSB are completely contained in the DL active BWP is not larger than CP length NOTE 3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of the neighbor cells is in the LTM candidate cell active TCI state list, the requirements only apply to intra-frequency cells and inter-frequency neighbor cells without gap whose TCI state(s) are in the LTM candidate cell active TCI state list. |  |

------------ END OF CHANGE 2--------------