3GPP TSG-RAN WG4 Meeting #114	R4-2503025

Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2646 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on support of UL MIMO for PC2 n28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CBN, China Broadnet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n28_PC2_40MHz_CBW-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of PC2 and 40MHz was introduced for n28 in previous meeting. The support of UL MIMO for PC2 n28 was missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce UL MIMO support for PC2 n28. Add Note on the tolerance requirements for n28 PC2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of UL MIMO for PC2 n28 is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.2D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF 1st CHANGE>

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  | 26 | +2/-33 | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±23, 4 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via dual Tx. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

<END OF 1st CHANGE>

<START OF 2nd CHANGE>

## 6.2D	Transmitter power for UL MIMO

### 6.2D.1	UE maximum output power for UL MIMO

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO codebook-based transmission with precoding matrix of W=. or 4-layer UL MIMO transmission with codebook of . DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n5 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n8 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n13 |  |  |  |  | 23 | +2/-3 |  |  |
| n24 |  |  |  |  | 23 | +2/-41 |  |  |
| n25 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n26 |  |  |  |  | 23 | +2/-31 |  |  |
| n28 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n40 |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n41 | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n70 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n71 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n77 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n80 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n81 |  |  |  |  | 23 | +2/-31 |  |  |
| n83 |  |  |  |  | 23 | +2/-31 |  |  |
| n84 |  |  | 26 | +2/-31 | 23 | +2/-3 |  |  |
| n85 |  |  |  |  | 23 | +2/-31 |  |  |
| n86 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n95 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n98 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n99 |  |  |  |  | 23 | +2/-41 |  |  |
| n104 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n105 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: The transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

<END OF 2nd CHANGE>