3GPP TSG-RAN5 Meeting #106	R5-251325

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0008 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5G support for USIM parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current definition of USIM parameters does not support 5G |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of support for 5G in the definition of USIM parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G will not be supported due to lack of USIM parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-250841 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

### 5.5.10	Common MCS test USIM parameters

#### 5.5.10.1	General

The format and coding of elementary files of the USIM are defined in 3GPP TS 31.102 [73]. Those of the ISIM are defined in 3GPP TS 31.101 [79] and 3GPP TS 31.103 [80].

The present clause defines default MCS relevant parameters for programming the elementary files of the test USIM when running conformance test cases defined in TS 37.579-2 [2], TS 37.579-6 [84], or TS 37.579-7 [85].

For requirements to the test USIM/ISIM needed for the E-UTRA/EPC and MCS off-network ProSe operation see 3GPP TS 36.508 [6], clause 4.9. For NR5/GC see 3GPP TS 38.508-1 [132], clause 4.8.3.

#### 5.5.10.2	Default settings for the Elementary Files (EFs)

EFUST (USIM Service Table)

| Services | Discription | Activated | Version |
| --- | --- | --- | --- |
| Service n°109 | Mission Critical Services | Yes |  |
| NOTE: Only the relevant MCS related services indicated. |  |  |  |

EFMST (MCS Service Table)

This file shall be present. This EF indicates the coding of the MCS management objects and which MCS services are available.

Coding of the MCPTT management objects = '00' (XML format).

| Services | Discription | Activated | Version |
| --- | --- | --- | --- |
| Service n°1: | MCPTT UE configuration data | Yes |  |
| Service n°2: | MCPTT User profile data | Yes |  |
| Service n°3: | MCS Group configuration data | Yes |  |
| Service n°4: | MCPTT Service configuration data | Yes |  |
| Service n°5: | MCS UE initial configuration data | Yes |  |
| Service n°6: | MCData UE configuration data | Yes |  |
| Service n°7: | MCData user profile data | Yes |  |
| Service n°8: | MCData service configuration data | Yes |  |
| Service n°9: | MCVideo UE configuration data | Yes |  |
| Service n°10: | MCVideo user profile data | Yes |  |
| Service n°11: | MCVideo service configuration data | Yes |  |

EFMCS_CONFIG (MCS configuration data)

This file shall be present.

Encoded in XML format (as specified in the MCS Service Table).

| MCPTT configuration data objects | Tag Values | Condition |
| --- | --- | --- |
| MCPTT UE configuration data | '80' | Shall be present. The content of the MCPTT UE configuration data object shall be as specified in Table 5.5.8.2-1. |
| MCPTT user profile data | '81' | Shall be present. The content of the MCPTT User configuration data object shall be as specified in Table 5.5.8.3-1. |
| MCS Group configuration data | '82' | Shall be present. The content of the MCS Group configuration data object shall be as specified in Table 5.5.7.1 for MCPTT, Table 5.5.7.2-1 for MCVideo, and Table 5.5.7.3-1 for MCData. |
| MCPTT Service configuration data | '83' | Shall be present. The content of the MCPTT Server configuration data object shall be as specified in Table 5.5.8.4-1. |
| MCS UE initial configuration data | '84' | Shall be present. The content of the MCS UE initial configuration data object shall be as specified in Table 5.5.8.1-1 for MCPTT, Table 5.5.8.5-1 for MCVideo, and Table 5.5.8.9-1 for MCData, |
| MCData UE configuration data | '85' | Shall be present. The content of the MCData UE configuration data object shall be as specified in Table 5.5.8.10-1. |
| MCData user profile data | '86' | Shall be present. The content of the MCData user profile data object shall be as specified in Table 5.5.8.11-1. |
| MCData service configuration data | '87' | Shall be present. The content of the MCData service configuration data object shall be as specified in Table 5.5.8.12-1. |
| MCVideo UE configuration data | '88' | Shall be present. The content of the MCVideo UE configuration data object shall be as specified in Table 5.5.8.6-1. |
| MCVideo user profile data | '89' | Shall be present. The content of the MCVideo user profile data object shall be as specified in Table 5.5.8.7-1. |
| MCVideo service configuration data | '8A' | Shall be present. The content of the MCVideo service configuration data object shall be as specified in Table 5.5.8.8-1. |